| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G042_r1 WG11 Number: m22193 |
| --- | --- |

| Title: | CE12: Summary report of core experiment on deblocking filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Xun Guo Byeungwoo Jeon Matthias Narroschke | Email: | andrey.norkin@ericsson.com xun.guo@mediatek.com bjeon@skku.edu matthias.narroschke@eu.panasonic.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

This contribution is a summary report of Core Experiment 12 on deblocking filtering. The goal of the proposals is to enhance deblocking filtering techniques in the HEVC Test Model. This is not limited to improving coding efficiency and subjective quality but also reducing complexity and harmonization of the various schemes that are technically feasible. The CE comprises five subtests. All the proposals are based on HM4.0 and experiments are performed according to the common test conditions provided in JCTVC-F900. Additionally, proposals from subtest 5 provide results for higher QPs. All results are verified by cross-checkers.

# Document list

## Summary document (this document)

- JCTVC-G042 [CE coordinators] CE12: Summary report of core experiment on deblocking filtering

## Proposals and cross-verification documents

Subtest 1: Subjective and objective quality improvements

- [JCTVC-G409, Microsoft] CE12, Subset 1: Report of Deblocking for Large Size Blocks

  - [JCTVC-G673, MediaTek] Cross-check: CE12 Subtest 1: Crosscheck for Microsoft's Deblocking for Larger Blocks in JCTVC-G409

- [JCTVC-G286, MediaTek, Ericsson] CE12 Subtest 1: Chroma Deblocking Filter

  - [G410, Microsoft] CE12, Subset 1: Cross-verification report for MediaTek and Ericsson's proposal (JCTVC-G286)

- [JCTVC-G287, MediaTek, Ericsson] CE12 Subtest 1: Simplified Chroma Deblocking Filter in JCTVC-G286

  - [JCTVC-G411, Microsoft] CE12, Subset 1: Cross-verification report for MediaTek and Ericsson's proposal

- [JCTVC-G383, Qualcomm] CE12 Subtest 1: Deblocking of New Non-Square Blocks: Edge Shift for AMP

  - [JCTVC-G485, MediaTek] CE12.1.5: Crosscheck for Qualcomm's deblocking of new non-square blocks: edge shift for AMP in JCTVC-G383

- [JCTVC-G590, Panasonic] CE12 Subtest 1: Results for modified decisions for deblocking

  - [JCTVC-G238, Sony] CE12 Subtest1: Cross-verification of Panasonic's proposal JCTVC-G590

  - [JCTVC-G621, Ericsson] CE12: Cross-verification of deblocking filtering decisions from Panasonic (JCTVC-G590) by Ericsson

- [JCTVC-G087, TI] CE12 subset 4.10: Test results on unification of luma and chroma filtering

  - [JCTVC-G249, Sony] CE12 Subtest1: Cross-verification of TI's proposal JCTVC-G087

Subtest 2: Parallel deblocking decision solutions

- [JCTVC-G255, Sony] CE12 Subtest2: Parallel deblocking improvement based on JCTVC-F214 tool1

  - [JCTVC-G585, Panasonic] CE12 Subtest 2: Cross-check results of the parallel deblocking tool 1 of Sony (JCTVC-G255)

  - [JCTVC-G103, TI] CE12 subset 2: cross-verification of SONY’s parallel de-blocking filtering proposals JCTVC-G255 and JCTVC-G256

- [JCTVC-G256, Sony] CE12 Subtest2: Parallel decision improvement and reduction based on JCTVC-F214 tool2

  - [JCTVC-G103, TI] CE12 subset 2: cross-verification of SONY’s parallel de-blocking filtering proposals JCTVC-G255 and JCTVC-G256

  - [JCTVC-G622, Ericsson] CE12: Cross-verification of parallel deblocking from Sony (JCTVC-G256) by Ericsson

  - [JCTVC-G587, Panasonic] CE12 Subtest 2: Cross-check results of the parallel deblocking tool 2 of Sony (JCTVC-G256)

- [JCTVC-G088, TI] CE12 subset 5.6: Test results and architectural study on de-blocking filter without parallel on/off filter decision

  - [JCTVC-G588, Panasonic] CE12 Subtest 2: Cross-check results of the deblocking proposal of Texas Instruments (JCTVC-G088)

  - [JCTVC-G746, Samsung] CE12 subset 5.6: Cross-check report on parallel deblocking decision (JCTVC-G088)

Subtest 3: Line memory reduction

- [JCTVC-G257, Sony] CE12 Subtest3: Deblocking vertical tap reduction for line buffer based on JCTVC-F215

  - [JCTVC-G486, MediaTek] CE12.3.1: Crosscheck for Sony's deblocking vertical tap reduction for line buffer in JCTVC-G257

- [JCTVC-G228, MediaTek] CE12.3.2: Reducing pixel line buffers by modifying DF for horizontal LCU edges

  - [JCTVC-G180, LG] CE12.3.2: Cross-check report of Reducing pixel line buffers by modifying DF for horizontal LCU edges (JCTVC-G228)

  - [JCTVC-G253, Sony] CE12 Subtest3: Cross-verification of MediaTek's proposal JCTVC-G228

- [JCTVC-G229, MediaTek] CE12.3.3: Reducing motion data line buffers

  - [JCTVC-G292, Qualcomm] CE12 Subtest3: Cross Check of Mediatek’s Motion Data Line Buffer Reduction Proposal JCTVC-G229

Subtest 4: Reduction of computational complexity of deblocking filter

- [JCTVC-G086, TI] CE12 subset 7.4: Test results on decreasing worst case complexity of deblocking filter

  - [JCTVC-G251, Sony] CE12 Subtest4: Cross-verification of TI's proposal JCTVC-G086

  - [JCTVC-G623, Ericsson] CE12: Cross-verification of deblocking simplification JCTVC-G086 from TI by Ericsson

Subtest 5: Controlling strength of deblocking filter at slice level

- [JCTVC-G174, Toshiba, Qualcomm] CE12: Deblocking filter parameter adjustment in slice level

  - [JCTVC-G645, Mitsubishi] CE12: crosscheck of deblocking filter parameter adjustment in slice level

- [JCTVC-G466, Mitsubishi Electric] CE12: results on signaling of boundary filtering strength

  - [JCTVC-G388, Toshiba] CE12: Cross-check of Mitsubishi's deblocking filter JCTVC-F175/G466 (JCTVC-G466, Mitsubishi Electric)

- [JCTVC-G574, Panasonic] CE12 Subtest 5: Deblocking filter using adaptive weighting factors

  - [JCTVC-G237, Sony] CE12 Subtest5: Cross-verification of Panasonic’s proposal relating to JCTVC-F405

# Experimental results

Based on common test conditions [1], results for all six mandatory test cases shall be provided for CE12 proposals. The following evaluation criteria are used:

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement)

- Subjective video quality

In the tables below, the following abbreviations are used:

AI – all intra

RA – random access

LD B – low delay (B)

LD P – low delay (P)

## Subtest 1: Subjective and objective quality improvements

Table 1 Cross-verification assignment for Subtest 1

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| MediaTek/Ericsson (G286) | Microsoft |
| MediaTek/Ericsson (G287) | Microsoft |
| Microsoft (G409) | MediaTek |
| Panasonic (G590) | Sony, Ericsson |
| Qualcomm (G383) | MediaTek |
| TI (G087) | Sony |

### Subtest 1: Deblocking filter modifications

Table 2 Parts that have been modified compared to HM4.0 are marked with an “x”

| Proponent | MediaTek/ Ericsson (G286) | MediaTek/ Ericsson (G287) | Microsoft (G409) | Panasonic (G590) | Qualcomm (G383) | TI (G087) |
| --- | --- | --- | --- | --- | --- | --- |
| Number of lines for block boundary decisions | 2 lines for each 4-pixel chroma boundary |  |  |  |  |  |
| Line-wise filtering decisions (on/off) | Added filtering decision for chroma | Added filtering decision for chroma |  | 2 lines for each 4-line-segment |  |  |
| Decision weak/strong filter |  |  | A stronger filter is added. | 2 lines for each 4-line-segment | no decision for 16x4/4x16 AMP edge |  |
| Luma filter operation (weak) |  |  |  |  | p0,q0 weak filter for 16x4/4x16 AMP edge | Changed back to HM3.0 |
| Luma filter operation (strong) |  |  | A stronger filter is added. |  |  |  |
| Chroma filter operation |  |  |  |  |  | Shares the same delta calculation with luma weak filter |
| Bs derivation | Added derivation of Bs = 2 for chroma |  |  |  |  |  |
| Tc values |  |  |  |  |  |  |
| Beta values |  |  |  |  |  |  |
| Optimization of original source code parts |  |  |  |  |  |  |
| Remarks |  |  |  |  | edge shift in case of 16x4/4x16 AMP |  |

### Subtest 1: Results

Table 3 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| MediaTek/Ericsson (G286) | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| MediaTek/Ericsson (G287) | 0.0% | 0.0% | -0.1% |  | 0.0% | 0.0% | -0.1% |  |
| Microsoft (G409) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Panasonic (G590) | 0.0% | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Qualcomm (G383) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| TI (G087) | -0.2% | -0.1% | -0.1% |  | -0.2% | -0.1% | -0.1% |  |

Table 4 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| MediaTek/Ericsson (G286) | -0.4% | -0.7% | -1.3% |  | -0.1% | -0.8% | -0.5% |  |
| MediaTek/Ericsson (G287) | -0.6% | -1.5% | -1.5% |  | -0.3% | -1.7% | -0.9% |  |
| Microsoft (G409) | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | -0.1% |
| Panasonic (G590) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |
| Qualcomm (G383) | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| TI (G087) | -0.1% | -0.2% | 0.0% |  | -0.3% | -0.3% | 0.0% |  |

Table 5 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| MediaTek/Ericsson (G286) | -0.4% | -0.6% | -1.1% |  | -0.1% | -0.7% | -0.6% |  |
| MediaTek/Ericsson (G287) | -0.5% | -1.4% | -1.5% |  | -0.2% | -1.4% | -0.9% |  |
| Microsoft (G409) | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% |
| Panasonic (G590) | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Qualcomm (G383) | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | -0.1% |
| TI (G087) | -0.1% | -0.2% | 0.1% |  | -0.3% | -0.3% | -0.2% |  |

## Complexity – relative decoding time against the anchor

Table 6 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| MediaTek/Ericsson (G286) | 100% | 100% | 100% |  | 99% | 99% | 99% |  |
| MediaTek/Ericsson (G287) | 100% | 100% | 100% |  | 99% | 99% | 99% |  |
| Microsoft (G409) | 101% | 102% | 102% | 102% | 102% | 104% | 104% | 104% |
| Panasonic (G590) | 99% | 99% | 99% | 100% | 99% | 99% | 100% | 100% |
| Qualcomm (G383) | 100% | 101% | 102% | 102% | 101% | 101% | 99% | 101% |
| TI (G087) | 100% | 98% | 101% |  | 101% | 101% | 102% |  |

### Subtest 1: comments from cross-checkers

- [JCTVC-G409, Microsoft] CE12, Subset 1: Report of Deblocking for Large Size Blocks

  - [MediaTek] The BD-rates exactly match those provided by the proponent. The source code also matches the algorithm described in the contribution. Subjective quality is reviewed. In some smooth areas, less block artifacts are observed compared with the anchor. Overall, the proposed method has similar subjective quality to the anchor.

- [JCTVC-G286, MediaTek, Ericsson] CE12 Subtest 1: Chroma Deblocking Filter

  - JCTVC-G410 [Microsoft]: The BD-rates are matched. The encoding and decoding times are similar.

- [JCTVC-G287, MediaTek, Ericsson] CE12 Subtest 1: Simplified Chroma Deblocking Filter in JCTVC-G286

  - JCTVC-G411 [Microsoft] The BD-rates are matched. The encoding and decoding time are similar.

- [JCTVC-G383, Qualcomm] CE12 Subtest 1: Deblocking of New Non-Square Blocks: Edge Shift for AMP

  - [MediaTek] The BD-rate results are perfectly matched. The source code and algorithm are matched. It’s a good suggestion to think about what modification of deblocking is required for AMP. The subjective qualities are viewed. The subjective quality is very similar to anchor. No undesirable artifact is observed.

- [JCTVC-G590, Panasonic] CE12 Subtest 1: Results for modified decisions for deblocking

  - JCTVC-G621 [Ericsson]: BD-rates match perfectly and the decoding times are similar to what was provided by the proponent. The source code matches the description in general expect a minor point missed in the description. The subjective quality is similar to that of the anchor.

  - JCTVC-G238 [Sony]: The BD-rates completely identical with those provided by Panasonic. The encoding and decoding time are similar with those provided by Panasonic. The source code seems to be as described in G590. The subjective quality is similar with anchor. G089 from TI is similar with this proposal.

- [JCTVC-G087, TI] CE12 subset 4.10: Test results on unification of luma and chroma filtering

  - JCTVC-G249 [Sony]: The BD-rate are completely identical with those provided by TI. The encoding and decoding time are a little bit different from those provided by TI. The source code seems to be as described in G087. The subjective quality loss is observed in vidyo3 (LB_HE, QP=37, frame=227) as shown in G087.

## Subtest 2: Parallel deblocking decision solutions

Table 7 Cross-verification assignment Subtest 2

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| Sony tool 1 (G255) | Panasonic, TI |
| Sony tool 2 (G256) | Panasonic, TI, Ericsson |
| TI (G088) | Panasonic, Samsung |

### Subtest 2: Deblocking filter modifications

Table 8 Modifications compared to to HM4.0

| Proponent | Sony tool 1 (G255) | Sony tool 2 (G256) | TI (G088) |
| --- | --- | --- | --- |
| Order of filtering |  |  |  |
| Modifications of deblocking decision |  | 2 lines of 8 line segment are used for on/off, strong and weak decision | Horizontal edge filter decision uses intermediate samples |
| Modifications of filtering operations | Both H and V filtering is applied to reconstructed pixels, and then two results are added | Clip of tc to strong filter is added | No |
| Additional buffers required |  | Buffers for parallel decision are not required | Reduced for a same throughput |
| Degree of achieved parallelism (relative HM4.0) | Increase (dependency between H and V filtering is removed) | Same | Slightly better by allowing filter decision and filtering to use same input |
| Remarks | The same issue of decision buffers as HM4.0 can be improved with G590 from Panasonic. | Sony has improved proposal with minor changes | Increased throughput for same implementation cost, or reduced memory reads and buffer size for a same throughput. AVC fashion of storing intermediate samples in deblocking line buffer I is restored |

### Subtest 2: Results

Table 9 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony tool 1 (G255) | 0.1% | 0.1% | 0.1% |  | 0.1% | 0.1% | 0.1% |  |
| Sony tool 2 (G256) | 0.1% | 0.1% | 0.3% |  | 0.0% | 0.1% | 0.2% |  |
| TI (G088) | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |

Table 10 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony tool 1 (G255) | 0.1% | 0.1% | 0.1% |  | 0.2% | 0.1% | 0.1% |  |
| Sony tool 2 (G256) | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.1% | 0.1% |  |
| TI (G088) | 0.0% | -0.1% | 0.1% |  | 0.0% | 0.0% | 0.1% |  |

Table 11 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony tool 1 (G255) | 0.1% | 0.1% | 0.0% |  | 0.2% | 0.1% | -0.1% |  |
| Sony tool 2 (G256) | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.0% | -0.1% |  |
| TI (G088) | 0.0% | 0.0% | -0.1% |  | 0.0% | 0.0% | -0.1% |  |

## Complexity – relative decoding time against the anchor -

Table 12 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony tool 1 (G255) | 102% | 101% | 101% |  | 102% | 101% | 100% |  |
| Sony tool 2 (G256) | 99% | 99% | 99% |  | 99% | 98% | 98% |  |
| TI (G088) | 101% | 97% | 98% |  | 96% | 99% | 98% |  |

### Subtest 2: comments from cross-checkers

- [JCTVC-G255, Sony] CE12 Subtest2: Parallel deblocking improvement based on JCTVC-F214 tool1

  - [JCTVC-G585, Panasonic] Confirmed software implementation, BD-bit rate results and run-times.

  - [JCTVC-G103, TI] Confirmed software implementation, BD-bit rate results. The proposed changes are beneficial for speeding up vertical edge filtering if the parallel filter decision is in use.

- [JCTVC-G256, Sony] CE12 Subtest2: Parallel decision improvement and reduction based on JCTVC-F214 tool2

  - [JCTVC-G103, TI] Confirmed software implementation, BD-bit rate results, recommended architectural study.

  - [JCTVC-G622, Ericsson] The BD-rate matched those of the proponent and the decoding time is similar. The description corresponds to the software.

  - [JCTVC-G587, Panasonic] Confirmed software implementation, BD-bit rate results and run-times.

- [JCTVC-G088, part 2, TI] CE12 subset 2: Test results and architectural study on de-blocking filter without parallel on/off filter decision

  - [JCTVC-G588, Panasonic] Confirmed software implementation, BD-bit rate results and run-times.

  - [JCTVC-G746, Samsung]

## Subtests 3 and 4: Line memory and computational complexity reduction.

Table 13 Cross-verification assignment Subtest 3

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| Sony (G257) | MediaTek |
| MediaTek (G228) | Sony |
| MediaTek (G229) | Qualcomm |

Table 14 Cross-verification assignment Subtest 4

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| TI (G086) | Sony, Ericsson |

### Subtest 3: Deblocking filter modifications

Table 15 Parts that have been modified compared to HM4.0

| Proponent | Sony (G257) | MediaTek (G228) | MediaTek (G229) |
| --- | --- | --- | --- |
| Number of lines for modification | Three lines for luma | One line for luma One line for chroma | 0, only change the BS. |
| Number of lines for reading | Three lines for luma | One line for luma One line for chroma | 4 (the same as HM-4.0) |
| Applied to LCU boundaries or all horizontal boundaries? | All horizontal boundaries | Applied to LCU boundaries | Applied to LCU boundaries |
| Modifications of deblocking decision | Padding is used in strong filter decision | Yes | Yes, change the reference MV position of upper LCU row. No change inside LCU. |
| Modifications of filtering operations | Yes | Yes | No |
| Remarks | G230 is joint proposal with MediaTek, LG and Sony related to subtest-3 | R1W1 at LCU boundaries Padding was used in filtering operations | Compress the motion data line buffer. Only affect the BS result when the block size of upper LCU row is 4x8. BS value may be switched from 0 to 1 or 1 to 0. |

### Subtest 4: Deblocking filter modifications

TI (G086):

Bs derivation modified

No deblocking when ref_idx are the same and no transform coefficients even when motion vectors are different

### Subtests 3 and 4: Results

Table 16 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony (G257) | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.0% | 0.0% |  |
| MediaTek (G228) all | 0.0% | 0.1% | 0.2% |  | 0.0% | 0.1% | 0.2% |  |
| MediaTek (G228) Y | 0.0% | 0.1% | 0.2% |  | 0.0% | 0.1% | 0.2% |  |
| MediaTek (G228) Chr | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| MediaTek (G229) |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |
| TI (G086) | 0.0% | 0.2% | 0.2% | 0.4% | 0.0% | 0.2% | 0.2% | 0.4% |

Table 17 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony (G257) | 0.0% | -0.1% | 0.1% |  | 0.0% | 0.1% | 0.2% |  |
| MediaTek (G228) all | 0.0% | 0.0% | 0.2% |  | 0.0% | 0.0% | 0.1% |  |
| MediaTek (G228) Y | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.0% | -0.1% |  |
| MediaTek (G228) Chr | 0.0% | -0.1% | 0.2% |  | 0.0% | 0.0% | 0.0% |  |
| MediaTek (G229) |  | 0.0% | 0.1% |  |  | 0.0% | 0.2% |  |
| TI (G086) | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |

Table 18 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony (G257) | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.1% | -0.1% |  |
| MediaTek (G228) all | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.1% | 0.0% |  |
| MediaTek (G228) Y | 0.0% | 0.1% | 0.1% |  | 0.0% | 0.1% | -0.1% |  |
| MediaTek (G228) Chr | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.0% | -0.2% |  |
| MediaTek (G229) |  | 0.0% | 0.1% |  |  | 0.0% | -0.2% |  |
| TI (G086) | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |

## Complexity – relative decoding time against the anchor

Table 19 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony (G257) | 99% | 98% | 100% |  | 100% | 102% | 101% |  |
| MediaTek (G228) all | 100% | 100% | 97% |  | 101% | 100% | 101% |  |
| MediaTek (G228) Y | 100% | 100% | 99% |  | 100% | 101% | 92% |  |
| MediaTek (G228) Chr | 100% | 100% | 100% |  | 101% | 100% | 101% |  |
| MediaTek (G229) |  | 100% | 101% |  |  | 101% | 100% |  |
| TI (G086) | 100% | 98% | 100% |  | 100% | 99% | 101% |  |

### Subtests 3 and 4: comments from cross-checkers

- [JCTVC-G257, Sony] CE12 Subtest3: Deblocking vertical tap reduction for line buffer based on JCTVC-F215

  - [JCTVC-G486,MediaTek] Source code modifications match the CE descriptions. The BD-rates match those provided by Sony. The encoding time and decoding time are roughly the same as that provided by Sony. Informal visual test are performed on Kimono, BQMall, Vidyo4, quality are roughly the same as anchor, except some horizontal artifacts on Kimono.

- [JCTVC-G228, MediaTek] CE12.3.2: Reducing pixel line buffers by modifying DF for horizontal LCU edges

  - [JCTVC-G180, LG] Source code is well implemented following JCTVC-G228. The BD rates and processing time are matched with those provided by MediaTek. By the informal visual test, no visual artifacts are founded in both LCU boundaries and inside of LCU.

  - [JCTVC-G253, Sony] The BD-rates in both luma only and chroma only are completely identical with those provided by MediaTek. The encoding and decoding time in both cases are similar with those provided by MediaTek. Source code seems to be as described in G228. Some artifacts are observed in such as Kimono (RA_HE, QP=37).

- [JCTVC-G229, MediaTek] CE12.3.3: Reducing motion data line buffers

  - [JCTVC-G292, Qualcomm] The BD-rate results are cross checked and match those reported in JCTVC-G229. No artifacts that are significantly worse than the anchor are observed. The conclusion is that the qualities are very similar to the anchor.

- [JCTVC-G086, TI] CE12 subset 7.4: Test results on decreasing worst case complexity of deblocking filter

  - [JCTVC-G251, Sony] The BD-rates are completely identical with those provided by TI. The encoding and decoding time are a little bit different from those from TI. Source code seems to be as described in G086. Visual quality is improved in BQMall (RA_HE QP=37, frame=115) as shown in G086, but overall is similar with HM-4.0.

  - [JCTVC-G623, Ericsson] The BD-rates match perfectly to those of the proponent and the decoding time is similar. The additional results for LD-P (HE) and LD-P(LC) have been obtained during cross-verification, which showed 0.4% BD-rate increase for luma component. Source code matches the description in G086. Subjectively, the proposal seems to have slightly more artifacts than the anchor on average (even though some parts of particular sequences may look slightly better).

## Subtest 5: Controlling strength of deblocking filter at slice level

Table 20 Cross-verification assignment Subtest 5

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| Mitsubishi (G466) | Toshiba |
| Toshiba, Qualcomm (G174) | Mitsubishi |
| Panasonic (G574) | Sony |

### Subtest 5: Deblocking filter modifications

Table 21 Parts that have been modified compared to HM4.0

| Proponent | Mitsubishi (G466) | Toshiba, Qualcomm (G174) | Panasonic (G574) |
| --- | --- | --- | --- |
| Number of parameters in slice header | 2 | 2 | 6 |
| What parameters are signaled | Bs intra offset luma Bs intra offset chroma | tc offset beta offset | 1 On/off-flag 5 Weights to delta values of luma filters (3 for strong, 2 for weak) |
| Parameters change typically (in the provided software): every frame/every sequence/same for all sequences in the test set | same for all sequences and all frames (set in the configuration file) | same for all sequences and all frames (set in the configuration file) | Encoder control minimizes MSE for each frame and decides whether to send parameters (frame-wise). |
| Modifications of Bs | Yes | No | No |
| Modification of deblocking decisions |  |  |  |
| Modification of filtering operations | No | No | yes |
| Remarks |  |  | An additional encoder control minimizing MSE for subjectively important samples have been provided before the meeting (see section 3.5) |

### Subtests 5: Results

Test 1. Common test conditions QPs: QP = 22, 27, 32, 37

Table 22 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Mitsubishi (G466) | -0.2% | 0.4% | 0.7% | 0.6% | 0.2% | 0.6% | 0.6% | 1.0% |
| Toshiba, Qualcomm (G174) | -0.3% | 0.1% | 0.1% | 0.1% | 0.0% | 0.4% | 0.3% | 0.7% |
| Panasonic (G574) | -0.4% | -0.3% | -0.5% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% |

Table 23 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Mitsubishi (G466) | -0.2% | 0.2% | 0.7% | 0.6% | 0.1% | 0.0% | 0.2% | 0.7% |
| Toshiba, Qualcomm (G174) | -0.4% | -0.3% | 0.2% | 0.1% | -0.3% | -0.5% | 0.3% | 0.3% |
| Panasonic (G574) | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.1% |

Table 24 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Mitsubishi (G466) | -0.1% | 0.2% | 0.5% | 1.2% | 0.3% | 0.1% | 0.4% | 0.4% |
| Toshiba, Qualcomm (G174) | -0.4% | -0.2% | 0.3% | 0.2% | -0.2% | -0.4% | 0.0% | 0.2% |
| Panasonic (G574) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% |

## Complexity – relative decoding time against the anchor

Table 25 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Mitsubishi (G466) | 100% | 100% | 101% | 100% | 101% | 102% | 102% | 101% |
| Toshiba, Qualcomm (G174) | 100% | 100% | 100% | 100% | 100% | 101% | 101% | 101% |
| Panasonic (G574) | 101% | 99% | 102% | 101% | 101% | 100% | 100% | 101% |

Test 2. Higher QPs: QP = 32, 37, 42, 47

Table 26 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Mitsubishi (G466) | -0.1% | 0.1% | 0.2% |  | 0.0% | 0.1% | 0.1% |  |
| Toshiba, Qualcomm (G174) | -0.2% | 0.4% | 0.7% | 0.6% | 0.2% | 0.6% | 0.6% | 1.0% |
| Panasonic (G574) | -0.3% | -0.3% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% |

Table 27 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Mitsubishi (G466) | 0.0% | 0.1% | 0.7% |  | 0.1% | 0.1% | -0.2% |  |
| Toshiba, Qualcomm (G174) | -0.2% | 0.2% | 0.7% | 0.6% | 0.1% | 0.0% | 0.2% | 0.7% |
| Panasonic (G574) | 0.0% | 0.2% | 0.3% | 0.3% | 0.0% | 0.0% | -0.4% | 0.0% |

Table 28 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Mitsubishi (G466) | 0.0% | 0.2% | 0.2% |  | 0.1% | 0.2% | 0.1% |  |
| Toshiba, Qualcomm (G174) | -0.1% | 0.2% | 0.5% | 1.2% | 0.3% | 0.1% | 0.4% | 0.4% |
| Panasonic (G574) | 0.1% | 0.1% | 0.1% | 0.5% | 0.0% | 0.0% | 0.0% | 0.2% |

## Complexity – relative decoding time against the anchor

Table 29 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Mitsubishi (G466) |  |  |  |  |  |  |  |  |
| Toshiba, Qualcomm (G174) | 100% | 100% | 101% | 100% | 101% | 102% | 102% | 101% |
| Panasonic (G574) | 100% | 100% | 100% | 102% | 100% | 101% | 100% | 99% |

### Subtest 5: comments from cross-checkers

- [JCTVC-G174, Toshiba, Qualcomm] CE12: Deblocking filter parameter adjustment in slice level

  - [JCTVC-G645, Mitsubishi]

- [JCTVC-G466, Mitsubishi Electric] CE12: results on signaling of boundary filtering strength

  - [JCTVC-G388, Toshiba]

- [JCTVC-G574, Panasonic] CE12 Subtest 5: Deblocking filter using adaptive weighting factors

  - [JCTVC-G237, Sony] The BD-rates in both original and improved version are completely identical with those provided by Panasonic. The encoding and decoding time in both cases are similar with those provided by Panasonic. Source code seems to be as described in G574. Visual quality is similar with HM-4.0 under QP in common test condition, and is improved for high efficiency and slightly improved or similar with HM-4.0 for low complexity under high QP. It is necessary to compare Panasonic’s proposal with the others (G174 and G466) and to discuss these together.

## Later modifications to CE12 proposals

This section comprises modifications to CE12 proposals developed after the CE12 deadline and announced at the time close to JCT-VC meeting document deadline (in November 2011). Therefore the proposals presented in this section should be considered as non-CE12 proposals reportedly improving the CE12 proposals by the same authors (or less formally as further development of CE12 proposals).

Table 30 Cross-verification

| Proposals(s) | Cross/checker(s) |
| --- | --- |
| Sony tool 1 (G255) | Panasonic (G585) |
| Sony tool 2 (G256) | Panasonic (G587) |
| Panasonic (G574) | Sony (G237) |
| MediaTek, Sony, LG (G230) | Samsung (G844) |

### Modifications points relative to the original CE12 proposals

Table 31 Modification points relative to original CE12 proposal

| Proposal | Related CE12 subtest/proposal(s) | Modification points relative to the proposals |
| --- | --- | --- |
| Sony tool 1 (G255) | subtest 2: G255 | The combination with G590 from Panasonic, which proposes the decision for segment of 4 lines, is additionally presented to improve the buffer issue because G255 requires additional buffers in LCU boundary owing to on/off decision for segment of 8 lines. |
| Sony tool 2 (G256) | subtest 2: G256 | The improved one has two minor modifications compared with original one. One is line-2 and 5 are used instead of line-3 and 4 for horizontal decision (on/off, strong and weak). The other one is the clip of tc in strong filter is removed. |
| Panasonic (G574) | subtest 5: G547 | Additional encoder control provided including a subjective criterion in the estimation: MSE is minimized for subjectively important samples. |
| MediaTek, Sony, LG (G230) | Subset 3: G228, G257 | Combined version of the two proposals. Modified filter decision from G257 and applying to LCU boundaries from G228. The filter operations are modified from R3W3 to R3W2. |

### Further modifications t CE 12 proposals: Results

Table 32 BD-Rate (Y)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony tool 1 (G255) | 0.0 | 0.0 | -0.1 |  | 0.1 | 0.0 | -0.1 |  |
| Sony tool 2 (G256) | 0.1 | 0.1 | 0.2 |  | 0.0 | 0.1 | 0.1 |  |
| Panasonic (G574) new enc. control | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Panasonic(G574) new enc. control hiQP | -0.1% | 0.3% | 0.7% | 0.6% | 0.0% | 0.3% | 0.6% | 0.6% |
| MediaTek,Sony,LG (G230) | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |

Table 33 BD-Rate (U)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony tool 1 (G255) | 0.1% | 0.1% | 0.2% |  | 0.2% | 0.1% | 0.1% |  |
| Sony tool 2 (G256) | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.0% | 0.1% |  |
| Panasonic (G574) new enc. control | 0.0% | 0.1% | 0.2% | 0.1% | 0.0% | 0.2% | 0.3% | 0.2% |
| Panasonic(G574) new enc. control hiQP | 0.0% | 0.5% | 1.5% | 0.9% | 0.0% | 0.3% | 0.5% | 0.9% |
| MediaTek, Sony, LG (G230) | 0.0% | -0.1% | 0.0% |  | 0.0% | 0.0% | 0.2% |  |

Table 34 BD-Rate (V)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony tool 1 (G255) | 0.1% | 0.1% | 0.2% |  | 0.2% | 0.1% | 0.1% |  |
| Sony tool 2 (G256) | 0.0% | 0.0% | 0.2% |  | 0.0% | 0.0% | -0.1% |  |
| Panasonic (G574) new enc. control | 0.0% | 0.1% | 0.3% | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% |
| Panasonic(G574) new enc. control hiQP | 0.0% | 0.3% | 1.0% | 1.0% | 0.0% | 0.3% | 0.7% | 0.8% |
| MediaTek, Sony, LG (G230) | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | -0.2% |  |

## Complexity – relative decoding time against the anchor

Table 35 Summary of relative decoding time (%) (against HM4.0 anchor)

| Proponent | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Sony tool 1 (G255) | 102% | 100% | 100% |  | 102% | 100% | 102% |  |
| Sony tool 2 (G256) | 98% | 98% | 98% |  | 99% | 99% | 100% |  |
| Panasonic (G574) new enc. control | 101% | 99% | 102% | 101% | 101% | 100% | 100% | 101% |
| Panasonic(G574) new enc. control hiQP | 101% | 99% | 100% | 100% | 101% | 100% | 100% | 100% |
| MediaTek, Sony, LG (G230) | 99% | 100% | 100% |  | 99% | 101% | 102% |  |

# Subjective quality

A subjective quality test (informal viewing by experts) will be held during the Geneva meeting.

# Conclusions

In CE12, the proposals from five subtests are evaluated based on the common test conditions. Additionally, the proposals in subtest 5 provide the results for configurations with higher QPs in order to evaluate subjective quality impact at higher QPs. The results are summarized based on BD-rates, decoding time and modifications to the current HM4.0 deblocking. The informal subjective viewing by experts will be held in Geneva in order to assess how the proposals affect the subjective quality.

# References

- F. Bossen, Common test conditions and software reference configurations, JCTVC-F900, Torino, IT, July 2011.

- A. Norkin, X. Guo, B. Jeon, M. Narroschke, Description of Core Experiment 12: Deblocking filtering, JCTVC-F912, Torino, IT, July 2011.