| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC-G043 WG11 Number: m22194 |
| --- | --- |

| Title: | CE13: Summary report of Core Experiment 13 on motion data parsing robustness and throughput |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
|  | B. Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
|  | J. Chen SAMSUNG ELECTRONICS CO., LTD. | Tel: Email: | +82 10 279 8826 jianle.chen@samsung.com |
|  | P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 Cesson-Sevigne, FRANCE | Tel: Email: | +33 2 99 87 68 92 patrice.onno@crf.canon.fr |
|  | M. Zhou 12500 TI Blvd., MS8649 Dallas, TX-75093 | Tel: Email: | +1 214 480 3816 zhou@ti.com |
| Source: | CE coordinators |  |  |

# Introduction

This Core Experiment relates to motion data parsing robustness and throughput. During the 6th JCT-VC meeting, a simplified version of JCTVC-F470 has been adopted in HM4.0. The goal of this CE is to evaluate various configurations or simplifications of different methods proposed for HM3.0, and evaluate them in the context of HM4.0.

# Document List

| Test |  |  |  | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- | --- |
| CE13 Description (JCTVC-F913) |  |  | CE13 Report (this doc.) |  |  |
| JCTVC-F052 | 3.1 | 1 | M1 | MediaTek - JCTVC-G231 and JCTVC-G232 | Canon - JCTVC-G236 |
|  |  | 2d | M2 |  | Samsung - JCTVC-G845 |
|  |  | 2e | M3 |  | Canon - JCTVC-G236 |
|  |  | 3d | M4 |  | Samsung - JCTVC-G845 |
|  |  | 3e | M5 |  | Canon - JCTVC-G236 |
| JCTVC-F086 | 3.2.1 | 1a | T1 | TI - JCTVC-G091 | Orange Labs - JCTVC-G275 |
|  |  | 2a | T2 |  | Panasonic - JCTVC-G539 |
|  |  | 3a | T3 |  | Panasonic - JCTVC-G539 Sony – JCTVC-G514 |
|  |  | 4a | T4 |  |  |
|  |  | 1b | T5 |  | Panasonic - JCTVC-G540 |
|  |  | 2b | T6 |  |  |
|  |  | 3b | T7 |  |  |
|  |  | 4b | T8 |  |  |
|  | 3.2.2 | 1 | T9 |  |  |
|  |  | 2 | T10 |  |  |
|  | 3.2.3 | 1b | T11 |  |  |
|  |  | 2b | T12 |  |  |
|  |  | 3b | T13 |  |  |
|  |  | 4b | T14 |  |  |
| JCTVC-F402 | 3.4 | 1 | S1 | Samsung - JCTVC-G776 | Orange Labs - JCTVC-G275 |
|  |  | 3 | S2 |  | Fujitsu - JCTVC-G191 |
|  |  | remark | S3 |  | MediaTek - JCTVC-G487 |
| JCTVC-F474 | 3.5 | 1 | C1 | Canon - JCTVC-G240 | ETRI - JCTVC-G424 |
|  |  | 2 | C2 |  | Orange Labs - JCTVC-G275 |
|  |  | 3 | C3 |  | Samsung - JCTVC-G842 |
|  |  | 4 | C4 |  | ETRI - JCTVC-G424 |
|  |  | 5 | C5 |  | MediaTek - JCTVC-G489 |
|  |  | 6 | C6 |  |  |

## Remark regarding JCTVC-F341 experiments:

8 experiments were planned to be evaluated in this CE during the 6th JCTVC meeting, however:

- Description of the proposed experiments was late but since it was just a further investigation of an already adopted technique (additional merge candidates) it was agreed by the coordinators to add it to the CE description.

- The software has been provided in time, but macro settings have not been made available.

- The results and settings were reported late.

Consequently, the coordinators have agreed on removing the experiments from the CE and suggested the proponent to submit the results outside the CE (JCTVC-G288).

## Remark regarding JCTVC-F402 experiments:

- Description of the proposed experiments was late (Sept. 19, 8.36am GMT+1) but since it was just a further investigation of an already adopted technique (additional merge candidates) it was agreed by the coordinators to add it to the CE description.

# Experiment descriptions and results

## JCTVC-G231 and JCTVC-G232 - Replacing redundant MVPs and adaptive MVP list size

This experiment evaluates the parsing robustness solution based on redundancy removing and replacement. This technique includes two tools:

- Tool 1 is replacing the redundant MVPs by other substitutes.

- Tool 2 is an adaptive determination of the list size.

### Tool 1: redundancy replacement

For Inter mode and Merge/Merge-Skip mode, if the number of available MVPs is smaller than the list size, the redundant or empty MVP is replaced by the MVP plus a constant value or rounding the MVP to half or integer precision. Two candidates are added to the 12 candidates for non-redundant MVP. The first one is truncating the MV of MVP[0] (the first MVP in the list) to integer precision and the second one is adding an offset (1 pixel) to the MV in the horizontal dimension of MVP[0]. The exact replacement process is the following:

If the first available MVP in the MVP list has a list0 MV

- The first additional MVP is derived by truncating the list0 MV to integer precision. This additional MVP is added to the MVP list if it is not redundant.

- The second additional MVP is derived by adding one integer offset to the list 0 MV in the horizontal dimension. This additional MVP is added to the MVP list if it is not redundant.

Else (if the first available MVP in the MVP list has a list1 MV)

- The first additional MVP is derived by truncating the list1 MV to integer precision. This additional MVP is added to the MVP list if it is not redundant.

- The second additional MVP is derived by adding one integer offset to the list1 MV in the horizontal dimension. This additional MVP is added to the MVP list if it is not redundant.

The experiments where the list size is adapted by MotionID, or by prediction modes, described in the CE description JCTVC-F913 have been withdrawn.

(Results reported with ENC_MRG_FIX (on both proposed method and HM4.0 reference)) ENC_MRG_FIX corresponds to the bugfix described in experiment S0.

|  | Tool 1 | Tool 2 |  |  | Average BD Rate | Avg encoder time | Avg decoder time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Redundancy Replacement | Fixed List Size | Adaptive List Size by Merge Index | Adaptive List Size by PU Size |  |  |  |
| HM4.0 Reference | Off | (5,2) | Off | Off | - |  |  |
| M1 | On |  |  |  | -0.1 | 100 | 100 |

### Tool 2: adaptive list size

Test M2: additional MVPs combined with adaptive list size by neighboring Merge indices

The MVP list size of a current PU for Merge mode is determined according to the final coded MVP indices of its neighboring PUs. The MVP list size, S, is decided by the maximum value of the coded MVP indices of neighboring PUs plus a constant value, N, which is coded in the slice header. The MVP list size can be calculated using the following equation:

,

S = max( Idxleft , Idxabove ) + N

where Idxleft and Idxabove represent the Merge index of the left PU and that of the above PU, respectively. If the number of available MVPs is smaller than the list size determined by the Merge indices, two additional MVPs proposed in test 1 are added to the MVP list until the MVP list size has reached its adaptive list size or both additional MVPs have been tested.

Test M3: additional MVP combined with adaptive list size by the current CU size

The MVP list size of a current PU for Merge is determined according to the current CU size. When the current CU size is smaller than or equal to 8x8, the list size is reduced from 5 to 4; otherwise the list size is 5. If the number of available MVPs is smaller than the list size determined by the CU size, two additional MVPs proposed in test1 are added to reach its MVP list size.

|  | Tool 1 | Tool 2 |  |  | Average BD Rate | Avg encoder time | Avg decoder time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Redundancy Replacement | Fixed List Size | Adaptive List Size by Merge Index | Adaptive List Size by CU Size |  |  |  |
| HM4.0 Reference | Off | (5,2) | Off | Off | - |  |  |
| M2 |  | Off | On | Off | 0.1 | 98 | 100 |
| M3 |  | Off | Off | On | 0 | 99 | 99 |

### Combination of Tool1 and Tool2

In M4, M1is combined with M2, adaptive MVP list size by neighboring Merge indices.

In M5, M1 is combined with M3, adaptive MVP list size by current CU size.

|  | Tool 1 | Tool 2 |  |  | Average BD Rate | Avg encoder time | Avg decoder time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Redundancy Replacement | Fixed List Size | Adaptive List Size by Merge Index | Adaptive List Size by PU Size |  |  |  |
| HM4.0 Reference | Off | (5,2) | Off | Off |  |  |  |
| M4 | On | Off | On | Off | 0.1 | 99 | 100 |
| M5 | On | Off | Off | On | 0 | 100 | 100 |

### Detailed results

|  | RAHE | RALC | LDHE | LDLC |
| --- | --- | --- | --- | --- |
| M1 | -0.1 101/100 | -0.1 101/99 | -0.1 100/100 | -0.2 100/100 |
| M2 | 0.1 98/100 | 0.1 98/100 | 0.2 99/100 | 0.1 99/100 |
| M3 | 0 99/98 | 0 99/99 | 0.1 99/100 | 0.1 99/99 |
| M4 | 0.1 98/100 | 0 98/100 | 0.1 99/100 | -0.1 100/102 |
| M5 | 0 100/100 | -0.1 101/100 | 0 100/100 | -0.1 100/101 |

### Additional results

|  | LDHE_P | LDLC_P |
| --- | --- | --- |
| M1 | -0.2 103/99 | -0.5 103/98 |
| M2 | 0 100/100 | 0 100/100 |
| M3 | 0 100/99 | 0 99/100 |
| M4 | -0.1 102/99 | -0.4 103/100 |
| M5 | -0.2 102/100 | -0.5 102/101 |

## JCTVC-G091 - A study on HM3.0 parsing throughput issue

### Three experiments are proposed:

- signaling the maximum number of merge/skip MVP candidates (maxNumMergeCand) in slice header

- simplify the second pruning process of the merge/skip MVP list derivation process

- combination of both approaches

The signaling method enables the truncated unary table of accurate length to be used for merge_idx coding, while the simplification method is intended to reduce the complexity of the second pruning process, thanks to the reduction of the total number of combined, non-scaled and zero MVP candidates from 8 to 5.

### Coding efficiency study with different maximum truncated unary table sizes signaled in slice header

It is proposed to cut the number of merge/skip MVP candidates during the motion estimation to lower the encoder cost. The current HM design already supports such encoder design flexibility, as one can choose to do motion search only for high priority candidates and skip the rest of low priority candidates in the merge/skip MVP list. The drawback of the current design is that the truncated unary table for merge/skip MVP index coding is designed for a maximum number of 5 candidates. If fewer number of merge/skip MVP candidates is supported on the encoder side, the coding efficiency is reduced since low-priority entries of the coding table remain unused.

To improve the coding efficiency, it is proposed to add a parameter in slice header to signal the maximum merge MVP list size (maxNumMergeCand), so that the truncated unary table for merge/skip MVP index coding can exactly match the maximum number of merge/skip candidates supported on the encoder side. The following table highlights the difference of truncated unary table for merge_idx coding under different maximum number of merge/skip MVP candidates supported by encoder. The proposed method cuts the last entry of each table by 1 bit.

| Merge/skip MVP index | maxNumMergeCand = 4 |  | maxNumMergeCand = 3 |  | maxNumMergeCand = 2 |  | maxNumMergeCand = 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM4.0 | Proposed | HM4.0 | Proposed | HM4.0 | Proposed | HM4.0 | Proposed |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | N/A |
| 1 | 10 | 10 | 10 | 10 | 10 | 1 |  |  |
| 2 | 110 | 110 | 110 | 11 |  |  |  |  |
| 3 | 1110 | 111 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |

Truncated unary table for merge_idx coding in HM4.0 and proposed method when encoder restricts number of merge/skip candidates in motion estimation

Experiments T1 to T4: encoder chooses to support a smaller number of merge/skip MVP candidates for merge estimation, and signal maxNumMergeCand in slice header

Experiments T5 to T8: both encoder and decoder assume maxNumMergeCand = 5, and when the truncated unary code table of fixed length 5 (i.e.  0 / 10 / 110 / 1110 / 1111) is used for merge_idx coding. maxNumMergeCand is not signaled in slice header. And reduced number of merge/skip MVPs for merge estimation.

(Results reported without ENC_MRG_FIX (on both proposed method and HM4.0 reference))

|  | maxNumMerge Cand | Truncated unary table for merge_idx coding | Encoder restriction | Average BD Rate | Avg encoder time | Avg decoder time |
| --- | --- | --- | --- | --- | --- | --- |
| HM4.0 Reference | 5 | 0 / 10 / 110 / 1110 / 1111 | No |  |  |  |
| T1 | 4 | 0 / 10 / 110 /111 | No | 0.1 | 99 | 103 |
| T2 | 3 | 0 / 10 / 11 | No | 0.5 | 97 | 101 |
| T3 | 2 | 0 / 1 | No | 1.2 | 93 | 99 |
| T4 | 1 | N/A | No | 3.6 | 94 | 97 |
| T5 | 5 | 0 / 10 / 110 / 1110 / 1111 | Merge_idx restricted to [0:3] in ME | 0.3 | 99 | 103 |
| T6 | 5 | 0 / 10 / 110 / 1110 / 1111 | Merge_idx restricted to [0:2] in ME | 0.8 | 96 | 98 |
| T7 | 5 | 0 / 10 / 110 / 1110 / 1111 | Merge_idx restricted to [0:1] in ME | 1.9 | 94 | 99 |
| T8 | 5 | 0 / 10 / 110 / 1110 / 1111 | Merge_idx restricted to 0 in ME | 4.9 | 95 | 99 |

T1 shows relatively small loss. The HM4.0 design offers flexibility for encoder designs to choose proper cost and coding efficiency trade-offs.

|  | maxNumMergeCand | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- | --- |
| Test 1 vs. 5 | 4 | 0.0 | -0.2 | -0.1 | -0.4 |
| Test 2 vs. 6 | 3 | -0.1 | -0.3 | -0.2 | -0.7 |
| Test 3 vs. 7 | 2 | -0.1 | -0.8 | -0.1 | -1.4 |
| Test 4 vs. 8 | 1 | -0.2 | -1.6 | -0.3 | -2.6 |

### Reduction of the number of candidates for the 2nd pruning process

In HM4.0, there are two MVP pruning processes involved. The first pruning processing removes the redundant MVP candidates from a maximum of one temporal MVP and four spatial MVP incoming candidates. After the first pruning process, the combined, non-scaled and zero MVP candidates can be appended to the list if the number of valid (native) MVPs is less than 5. This leads to the second MVP pruning process, in which a maximum of four native (i.e. resulting MVP candidates after the first pruning process), five combined, one non-scaled and two zero MVP candidates need to be pruned in the worst case. Compared to the first pruning process, the second one is quite complex because in the worst case there are a total of 12 input candidates.

In the proposed algorithm, the total number of combined, non-scaled and zero MVP candidates in the worst for the second pruning process is limited to 5, while the maximum number of non-scaled and zero MVP candidates remains unchanged (i.e. 1 for non-scaled, and 2 for zero MVP candidates). In this way, the worst case number of MVP candidates for the second pruning process can be reduced from 12 to 9 or 8.

(Results reported without ENC_MRG_FIX (on both proposed method and HM4.0 reference))

|  | Maximum number of MVP (merge) candidates after the 1st pruning process | Maximum number of combined bi-predictive MVP (merge) candidates | Maximum number of non-scaled bi-predictive MVP (merge) candidates | Maximum number of zero MVP (merge) candidates | Average BD Rate | Avg encoder time | Avg decoder time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HM4.0 reference | 4 | 5 | 1 | 2 |  |  |  |
| T9 | 4 | 4 – n - z | 1 (n) | 2 (z) | 0.1 | 101 | 101 |
| T10 | 4 | 5 – n - z | 1 (n) | 2 (z) | 0 | 100 | 100 |

There is no coding loss when the total number of combined, non-scaled and zero MVP candidates is reduced from 8 to 5 (T10), while loss starts showing up (0.1%) when the total number of additional candidates is further reduced to 4 (T9).

### Combined tests for 1) and 2)

In the combined design, the maximum merge MVP list size (maxNumMergeCand) is signaled in slice header, the proposed truncated unary tables are used for merge_idx coding based on the value of maxNumMergeCand, and the total number of combined, non-scaled and zero MVP candidates in the worst for the second pruning process is limited to 5.

(Results reported without ENC_MRG_FIX (on both proposed method and HM4.0 reference))

|  | maxNumMergeCand | Truncated unary table for merge_idx coding | Maximum number of combined bi-predictive MVP candidates | Average BD Rate | Avg encoder time | Avg decoder time |
| --- | --- | --- | --- | --- | --- | --- |
| HM4.0 Reference | 5 | 0 / 10 / 110 / 1110 / 1111 | 5 |  |  |  |
| T11 | 4 | 0 / 10 / 110 /111 | 5 – n – z | 0.1 | 97 | 100 |
| T12 | 3 | 0 / 10 / 11 |  | 0.5 | 94 | 100 |
| T13 | 2 | 0 / 1 |  | 1.2 | 92 | 100 |
| T14 | 1 | N/A |  | 3.6 | 93 | 96 |

Adding the simplification of the second pruning process on the top of the signaling method does not introduce any coding loss (comparison of T11 -> T14 with T5 -> T8).

### Detailed results:

|  | RAHE | RALC | LDHE | LDLC |
| --- | --- | --- | --- | --- |
| T1 | 0.1 97/99 | 0.3 102/109 | 0.2 99/103 | 0.6 99/102 |
| T2 | 0.4 93/96 | 0.7 95/99 | 0.6 98/100 | 1.5 96/97 |
| T3 | 0.9 92/98 | 1.7 94/105 | 1.4 94/96 | 3.5 95/96 |
| T4 | 3.3 91/99 | 4.9 94/102 | 4.0 97/100 | 7.3 99/96 |
| T5 | 0.1 98/102 | 0.1 102/111 | 0.2 98/98 | 0.2 99/101 |
| T6 | 0.3 93/98 | 0.4 97/105 | 0.4 98/99 | 0.8 98/104 |
| T7 | 0.8 91/99 | 0.9 92/100 | 1.3 95/100 | 2.0 96/99 |
| T8 | 3.1 90/98 | 3.2 92/99 | 3.7 96/95 | 4.4 98/98 |
| T9 | 0.1 100/99 | 0.1 102/103 | 0.1 101/100 | 0.1 101/103 |
| T10 | 0 98/98 | 0 101/103 | 0 100/98 | 0 101/102 |
| T11 | 0.1 96/101 | 0.1 96/99 | 0.1 98/101 | 0.2 96/101 |
| T12 | 0.3 90/95 | 0.4 95/105 | 0.4 96/100 | 0.8 94/98 |
| T13 | 0.8 88/97 | 0.9 91/101 | 1.3 96/102 | 2.0 92/98 |
| T14 | 3.1 86/94 | 3.2 92/103 | 3.7 96/96 | 4.4 96/96 |

## JCTVC-F341 - Decoder Performance Restrictions due to Merge/MVP Index Parsing

See remark in section 2.

## JCTVC-G776 - Merge candidates list construction

### Experiment S0: encoder side bug fix

In HM4.0, the number of bits for merge index coding is not considered during merge estimation stage for normal merge modes. In this proposal, the number of bits for merge index coding is considered as estimated bits without entropy coding in merge estimation stage.

This experiment is also designated by ENC_MRG_FIX in the document.

### Experiment S1

In HM4.0, the merge list size is fixed to 5. In the proposed method, the maximum merge list size of each slice can be different in a sequence and a high-level parameter maxNumMergeCand is signaled in slice header. The value of maxNumMergeCand can vary from 1 to 5 and its value can be set according to user application or heuristically derived with encoder algorithm. In this test, the maxNumMergeCand of each slice is derived according to merge list candiates statistics of previous coded slices.  For each temporal layer, the average of the number of the first step candidates which are derived before combined candidates is calculated. This average value keeps being updated with the previous average value as an average of those two. This average value is used as the maximum merge list size after being added by the offset value which is determined on temporal layer or slice type.

### Experiment S2

In HM4 .0, the fixed list size of Merge/AMVP candidates is adopted with additional Merge/AMVP candidates insertion. However, it’s still possible that the Merge list is not fully filled with additional merge candidates. It is tested to insert additional merge candidates by adding ±1 offset to each component of existing MV candidate. If the motion candidate in current merge list is less than 5, the additional MV is generated by modifying the first merge candidate.  The horizontal component is modified by adding +- 1 in order and if additional candidates are still insufficient, the vertical component is modified by adding +- 1 in order.

### Experiment S3

Joint experiment of S1 and S2.

(Results reported with ENC_MRG_FIX (on both proposed method and HM4.0 reference) for S3 and S4)

(Results reported without ENC_MRG_FIX (on both proposed method and HM4.0 reference) for S1)

|  | Adaptive table size | Additional merge candidates with offset | Average BD Rate | Avg encoder time | Avg decoder time |
| --- | --- | --- | --- | --- | --- |
| HM4.0 Reference | Off | Off |  |  |  |
| S0 = ENC_MRG_FIX | Off | Off | -0.1 | 100 | 100 |
| S1 | On | Off | 0.1 | 98 | 100 |
| S2 | Off | On | -0.1 | 102 | 100 |
| S3 | On | On | 0.1 | 98 | 100 |

### Detailed results

|  | RAHE | RALC | LDHE | LDLC |  | LDHE_P | LDLC_P |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S0 = ENC_MRG_FIX | -0.1 100/101 | -0.1 100/100 | -0.2 100/99 | -0.1 100/101 |  | -0.1 100/100 | 0 100/100 |
| S1 | 0.1 97/100 | 0.1 97/100 | 0.2 98/100 | 0.2 98/101 |  | -0.1 96/100 | -0.1 95/100 |
| S2 | -0.1 102/100 | -0.2 102/99 | -0.1 102/101 | -0.1 102/100 |  | -0.2 104/100 | -0.4 104/101 |
| S3 | 0.1 98/100 | 0.1 98/99 | 0.2 99/101 | 0.2 99/101 |  | -0.1 100/101 | -0.3 100/100 |

## JCTVC-G240 – Non redundant predictors

Six experiments are proposed for both AMVP and Merge modes.

Experiments C1 to C4 replace some of the additional candidates of the current HM4.0 motion vector derivation by non-redundant candidates as previously proposed in JCTVC-F474. These four experiments correspond to different compromises between the coding efficiency and the complexity reductions in terms of number of operations for the Merge mode MVP derivation process compared to the HM4.0.

Experiment C5 to C6 deal with the modification of the motion vector predictor index parsing for AMVP.

JCTVC-G240 also includes additional experiments for fast encoding mode that are not part of CE13.

### Non-redundant offset predictors (tool2)

HM4.0 uses three types of new predictors for the Merge modes as described in JCTVC-F470: the combined predictors, the scaled predictors and the Zero predictors. All these new predictors need to be generated and compared to one another. This process increases the complexity of the Merge derivation process compared to HM3.0. Especially, both combined and scaled predictors need to be generated after a first pruning process.

It is proposed to replace duplicate candidates by new candidates which are always non-redundant. The offset predictors consist in adding an offset value to one component of the first candidate in the list. The offset value is fixed to 8. For instance, If the first predictor (mvx, mvy) is a unidirectional candidate, the offset candidates are derived as the following: 1st:  (mvx -8, mvy), 2d:  (mvx+8, mvy), 3d:  (mvx, mvy -8), 4th: (mvx, mvy+8).

The offset predictors have two characteristics:

- the list of predictors definitely reaches the maximum number of candidates and all predictors are non-redundant. This offers the possibility to efficiently use the signaling bits dedicated to the predictor index.

- less comparisons are needed.

The table below gives the related number of possible predictors for each type of additional predictors Each experiment C2, C3 and C4 derives fewer predictors compared to the HM4.0 Anchor.

|  | Merge Mode |  |  | AMVP/Merge |  |
| --- | --- | --- | --- | --- | --- |
|  | Maximun number of combined predictors | Scaled predictors | Zero predictors | Non-redundant predictors / Offset | Total |
| HM4.0 | 12 | 8 | 2 | 0 | 22 |
| C1 | 12 | 8 | 0 | 4 | 24 |
| C2 | 12 | 0 | 0 | 4 | 16 |
| C3 | 3 | 0 | 0 | 4 | 7 |
| C4 | 2 | 0 | 0 | 4 | 6 |

(Results reported with ENC_MRG_FIX (on both proposed method and HM4.0 reference))

|  | Merge mode |  |  | AMVP/Merge | Average BD Rate | Avg encoder time | Avg decoder time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Maximun number of combined predictors | Scaled predictors | Zero predictors | Non-redundant predictors / Offset |  |  |  |
| HM4.0 | 12 | On | On | Off | - |  |  |
| C1 | 12 | On | Off | On | -0.2 | 103 | 101 |
| C2 | 12 | Off | Off | On | -0.2 | 103 | 101 |
| C3 | 3 | Off | Off | On | -0.1 | 103 | 100 |
| C4 | 2 | Off | Off | On | 0 | 103 | 100 |

### Maximum number of candidates for AMVP (tool 1)

The current AMVP process uses 3 candidates: 2 spatial candidates and one temporal candidate. But the maximum number in the list of candidates is 2. The spatial candidates are the left and the above predictors. For the left predictor, 2 spatial positions are checked and for the above candidates, 3 spatial positions. If both spatial and temporal candidates are not available, the zero motion vector is added to the list.

This proposition consists in checking the availability of the spatial candidates and considers that the temporal one is always available. A spatial candidate is considered as available if at least one of its possible positions exists and it is not Intra coded.

|  | Merge mode | AMVP | Average BD Rate |
| --- | --- | --- | --- |
| C5 | HM4.0 | Tool 1 and Tool 2 | -0.1 |
| C6 | C3 | Tool 1 and Tool 2 | -0.1 |

### Detailed results

|  | RAHE | RALC | LDHE | LDLC |
| --- | --- | --- | --- | --- |
| C1 | -0.1 | -0.2 | -0.2 | -0.3 |
| C2 | -0.1 | -0.1 | -0.2 | -0.3 |
| C3 | -0.1 | -0.1 | -0.2 | -0.2 |
| C4 | 0 | -0.1 | 0 | 0 |
| C5 | -0.1 | -0.1 | 0 | -0.1 |
| C6 | -0.1 | -0.1 | -0.2 | -0.2 |

|  | Enc Time[%] |  |  |  |  | Dec Time[%] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RAHE | RALC | LDHE | LDLC |  | RAHE | RALC | LDHE | LDLC |
| Test 1 | 104% | 103% | 103% | 103% |  | 99% | 102% | 102% | 103% |
| Test 2 | 104% | 104% | 103% | 103% |  | 99% | 103% | 103% | 101% |
| Test 3 | 104% | 104% | 103% | 103% |  | 99% | 102% | 101% | 100% |
| Test 4 | 104% | 104% | 103% | 103% |  | 98% | 102% | 101% | 100% |

|  | Enc Time[%] |  |  |  |  | Dec Time[%] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RAHE | RALC | LDHE | LDLC |  | RAHE | RALC | LDHE | LDLC |
| Test 5 | 101% | 100% | 101% | 101% |  | 98% | 101% | 100% | 100% |
| Test 6 | 104% | 104% | 103% | 103% |  | 97% | 102% | 102% | 104% |

# Conclusion

General comments:

- Experiment S0 reports an encoder-side fix with average -0.1% BD rate saving without encoder/decoder time change.

- Experiment C1 to C4, S3, M1 show that performance can be improved by adding offsets to the existing candidate, and fewer comparisons are needed for the 2nd pruning.

- There is no coding loss when the total number of combined, non-scaled and zero MVP candidates is reduced from 8 to 5 (T10).

- Signaling the maximum number of merge/Skip candidates in the list offers a trade-off for the encoder between encoding complexity and coding efficiency.

Tentative recommendations:

- bugfix (experiment S0): recommend to adopt.

- no recommendation on the signalization of maxNumMergeCand.

- no recommendation regarding the reduction of the number of combined bi-predictive and non scaled, and the use non redundant / offset predictors.

Tentative recommendation for process during the meeting:

1- Both CE9 and CE13 establish a set of tools agreed to be adopted. CE9 and CE13 recommend adopting this set of tools.

2- A single BoG is created for CE9 and CE13.

  - A first round consists of evaluating this set of tools together (interaction CE9 / CE13) and produces a first software version.

  - A second round (if time available) consists of evaluating non CE proposals