| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G052 |
| --- | --- |

| Title: | CE9: Results of MRG_MVD series |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Motoharu Ueda Kazumi Arakage Satoru Sakazume JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@ jvckenwood.com ueda.motoharu@jvckenwood.com someya.kazumi@ jvckenwood.com sakazume.satoru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

The CE9 was established to evaluate the performance of motion vector coding and skip/merge operations at 6th JCT-VC meeting. Merge base MVD transmission was proposed at the meeting. This contribution reports the results of merge base MVD transmission under HM4.0 common test conditions.

The simulation results report that merge base MVD transmission provides 0.5% BD-rate gain for random access settings and 0.8% gain for low delay B settings under HM4.0 common test conditions.

# Introduction

The CE9 [1] was established to evaluate the performance of motion vector coding and skip/merge operations at 6th JCT-VC meeting. Merge base MVD transmission [2] was proposed at the meeting. This contribution reports the results of merge base MVD transmission under HM4.0 common test conditions [3].

# Algorithm Description

## Proposed algorithm

Table 1 presents transmitted motion information by each mode in the proposed algorithm.

Table 1 Transmitted motion information

|  | Inter Pred | Merge Index | Ref Index | MVP Index | MVD |
| --- | --- | --- | --- | --- | --- |
| Merge mode | - | ○ (1) | - | - | - |
| MergeMVD mode | ○ | ○ (1) | - | - | ○ (1) |
| AMVP mode | ○ | - | ○ (1 or 2) | ○ (1 or 2) | ○ (1 or 2) |

The AMVP mode means MVD transmission mode using AMVP as Pred_LC mode or Pred_BI mode. The merge mode and the AMVP mode are not changed from HM4.0. The merge MVD mode is newly introduced. In the merge MVD mode, one MVD is transmitted in addition to the merge index.

Table 2 presents inferred motion information in the merge MVD mode for each merge candidate type.

Table 2 Inferred motion information in the merge MVD mode

| Merge candidate type | Ref Pic | MVD Ref Pic |
| --- | --- | --- |
| Uni-pred | Same as Merge mode / Ref0 | Not merged Pic |
| Bi-pred | Same as Merge mode | Distance based |

Figure 1 illustrates the selection of the reference picture to add MVD for each merge candidate type.

Figure 1 Selection of the reference picture to add MVD

If merge candidate type which is indicated by merge index is uni-prediction, the reference index of the other list which is not selected by the merge index is set to 0 for bi-prediction. The reference picture on which transmitted MVD is added to MVP is the picture indicated by the reference index in the other list.

Otherwise (merge candidate type is bi-prediction), same reference picture as for the merge mode is used for the merge MVD mode. The reference picture on which transmitted MVD is added to MVP is selected based on the distance to the current picture.

The decoding process for the merge MVD mode is shared with the merge mode. The additional process for the merge MVD mode is as follows.

- Select the reference picture to add MVD

- (In case that merge candidate type is uni-prediction,) calculate MVP of the other list for bi-prediction

- Calculate motion vector by adding the MVD to MVP for the selected reference picture

# Experiments

## Test conditions

The common test conditions are used.

Tests and testers are presented in Table 3.

Table 3 Tests and testers

| Test | Description | Related Doc. | Tester1 | Tester2 |
| --- | --- | --- | --- | --- |
| MRG_MVD01 | five candidates No additional ME | JCTVC-F373 | JVC Kenwood | Fujitsu |
| MRG_MVD02 | five candidates Additional ME | JCTVC-F373 | JVC Kenwood | Panasonic |
| MRG_MVD03 | one candidate (merge_idx = 0) Additional ME | JCTVC-F373 | JVC Kenwood | Qualcomm |

## Experimental environment

The experimental environment used is presented in Table 4.

Table 4 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 5 and Table 6 present the simulation results of MRG_MVD01 for random access, and low delay B, respectively.

Table 7 and Table 8 present the simulation results of MRG_MVD02 for random access, and low delay B, respectively.

Table 9 and Table 10 present the simulation results of MRG_MVD03 for random access, and low delay B, respectively.

Table 5 Simulation results of MRG_MVD01 for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.5% | -0.4% | -0.2% | 0.1% | -0.1% |
| Class B | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class C | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6 Simulation results of MRG_MVD01 for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | 0.2% | -0.3% | -0.1% | -0.2% |
| Class C | -0.3% | -0.3% | -0.4% | -0.3% | -0.1% | -0.4% |
| Class D | -0.3% | 0.0% | 0.4% | -0.3% | 0.2% | -0.3% |
| Class E | -0.2% | -0.1% | 0.2% | -0.4% | 0.3% | -0.2% |
| Overall | -0.2% | -0.2% | 0.1% | -0.3% | 0.0% | -0.3% |
|  | -0.2% | -0.2% | 0.0% | -0.3% | 0.0% | -0.3% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 7 Simulation results of MRG_MVD02 for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.9% | -0.9% | -0.7% | -0.2% | -0.2% |
| Class B | -0.6% | -0.6% | -0.5% | -0.6% | -0.5% | -0.5% |
| Class C | -0.5% | -0.6% | -0.5% | -0.4% | -0.5% | -0.4% |
| Class D | -0.4% | -0.4% | -0.5% | -0.4% | -0.5% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | -0.6% | -0.6% | -0.5% | -0.4% | -0.4% |
|  | -0.5% | -0.6% | -0.6% | -0.5% | -0.4% | -0.4% |
| Enc Time[%] | 114% |  |  | 115% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 8 Simulation results of MRG_MVD02 for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -0.8% | -0.9% | -0.8% | -0.4% | -0.4% |
| Class C | -0.9% | -0.9% | -1.0% | -0.8% | -0.3% | -0.6% |
| Class D | -0.8% | -0.4% | -0.5% | -0.7% | -0.3% | -0.8% |
| Class E | -0.8% | -0.6% | -0.3% | -0.7% | -0.5% | -0.3% |
| Overall | -0.8% | -0.7% | -0.7% | -0.8% | -0.4% | -0.5% |
|  | -0.8% | -0.7% | -0.7% | -0.8% | -0.4% | -0.5% |
| Enc Time[%] | 111% |  |  | 111% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 9 Simulation results of MRG_MVD03 for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.8% | -0.7% | -0.6% | -0.1% | -0.3% |
| Class B | -0.4% | -0.4% | -0.4% | -0.5% | -0.4% | -0.5% |
| Class C | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.2% |
| Class D | -0.4% | -0.4% | -0.3% | -0.2% | -0.5% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -0.5% | -0.5% | -0.4% | -0.3% | -0.4% |
|  | -0.4% | -0.5% | -0.5% | -0.4% | -0.3% | -0.4% |
| Enc Time[%] | 109% |  |  | 110% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

Table 10 Simulation results of MRG_MVD03 for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | -0.6% | -0.8% | -0.7% | -0.4% | -0.4% |
| Class C | -0.7% | -0.7% | -0.7% | -0.7% | -0.2% | -0.6% |
| Class D | -0.8% | -0.2% | -0.9% | -0.6% | -0.2% | -0.5% |
| Class E | -0.7% | -0.7% | 0.0% | -0.7% | -0.1% | -0.3% |
| Overall | -0.7% | -0.6% | -0.6% | -0.7% | -0.3% | -0.4% |
|  | -0.7% | -0.5% | -0.6% | -0.7% | -0.3% | -0.4% |
| Enc Time[%] | 107% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The simulation results report that the proposed technique (MRG_MVD02) provides 0.5% BD-rate [4] gain for random access settings, and 0.8% gain for low delay B settings. (See the attached excel sheet)

# Conclusions

This contribution reports the proposed technique can bring substantial coding gain.

Therefore, we suggest that the proposed technique (MRG_MVD02) be adopted to the next HM.

# References

[1] “Core Experiment 9: MV Coding and Skip/Merge Operations”,

JCTVC-F909, Torino, July 2011

[2] “Merge based mvd transmission”,

JCTVC-F373, Torino, July 2011

[3] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[4] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

### 7.3.7   Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( (inter_pred_flag[ x0 ][ y0 ] = = Pred_BI) && |  |
| (entropy_coding_mode_flag ¦¦ combined_inter_pred_ref_idx == (MaxPredRef – NumPredRefL0*NumPredRefL1)) ) |  |
| merge_mvd_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if (merge_mvd_flag[ x0 ][ y0 ]){ /* Merge with one mvd mode */ |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_ls[ x0 ][ y0 ][ 0 ], mvd_ls[ x0 ][ y0 ][ 1 ]) |  |
| }else{ |  |
| mvd_ls[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_ls[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| else { /* MVP mode */ |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else{ /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### Prediction unit semantics

merge_mvd_flag[ x0 ][ y0 ] specifies whether the inter prediction parameters for the current prediction unit are derived by the merging candidate from neighboring inter-predicted partition and one difference of vector components. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When merge_mvd_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mvd_ls[ x0 ][ y0 ][ compIdx ] has the same semantics as mvd_l0, with l0 and list 0 replaced by a list s which is derived by merge mode when merge_mvd_flag[ x0 ][ y0 ] is equal to 1, respectively.

#### 8.4.2.1   Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as PredFlagL0 and PredFlagL1, the following applies.

- If PredMode is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause 8.4.2.1.3 is invoked with the luma location ( xP, yP ), variables nPSW, nPSH and the partition index PartIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1.

- Otherwise, if PredMode is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1,, the derivation process for luma motion vectors for merge mode as specified in subclause 8.4.2.1.3 is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and the partition index PartIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1.

- Otherwise, if PredMode is equal to MODE_INTER and merge_mvd_flag[ xP ][ yP ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause 8.4.2.1.1 is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and the partition index PartIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1 and in the syntax elements mvd_ls, the following applies.

The variable mvdLS is derived as follows.

mvdLS[ 0 ] = mvd_ls[ xP ][ yP ][ 0 ]		(8-xx)
			mvdLS[ 1 ] = mvd_ls[ xP ][ yP ][ 1 ]		(8-xx)

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX and in Pred_LX and in the syntax elements ref_idx_lX and mvd_lX, the following applies.

- The variables LcToLx, refIdxLX and predFlagLX are derived as follows.

- If inter_pred_flag[ xP ][ yP ] is equal to Pred_LC and PredLCToPredLx[ ref_idx_lc[ xP ][ yP ] ] is equal to Pred_LX,

refIdxLX = RefIdxLCToRefIdxLx[ ref_idx_lc[ xP ][ yP ] ]	(8-1)
predFlagLX = 1		(8-2)
mvd_lX[ xP ][ yP ][ 0 ] = mvd_lc[ xP ][ yP ][ 0 ]	(8-3)
mvd_lX[ xP ][ yP ][ 1 ] = mvd_lc[ xP ][ yP ][ 1 ]	(8-4)
mvp_idx_lX[ xP ][ yP ][ 0 ] = mvp_idx_lc[ xP ][ yP ]	(8-5)
LcToLx = LX		(8-6)

- Otherwise, if inter_pred_flag[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

refIdxLX = ref_idx_lX[ xP ][ yP ]		(8-7)
					predFlagLX = 1		(8-8)

- Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = -1		(8-9)
					predFlagLX = 0		(8-10)

- The variable mvdLX is derived as follows.

mvdLX[ 0 ] = mvd_lX[ xP ][ yP ][ 0 ]		(8-11)
				mvdLX[ 1 ] = mvd_lX[ xP ][ yP ][ 1 ]		(8-12)

- When predFlagLX is equal to 1, the variable mvpLX is derived as follows.

- The derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and refIdxLX as the inputs and the output being mvpLX.

- When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

mvLX[ 0 ] = mvpLX[ 0 ] + mvdLX[ 0 ]	(8-13)
				mvLX[ 1 ] = mvpLX[ 1 ] + mvdLX[ 1 ]	(8-14)

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause Error: Reference source not found is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

##### 8.4.2.1.1    Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1 or MODE_ INTER and merge_mvd_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-15)

predFlagLXCol = availableFlagLXCol	(8-16)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to 5.

- When slice_type is equal to B, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- When merge_flag [ xP ][ yP ] is equal to 1, the following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-17)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-18)

refIdxLX = refIdxLXN	(8-19)

predFlagLX = predFlagLXN	(8-20)

- When merge_mvd_flag [ xP ][ yP ] is equal to 1, the following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

  - When predFlagLXN is equal to 0, refIdxL0, refIdxL1, and mvpLS are derived as follows.

S = X

T = 1-S

refIdxLS = 0

refIdxLT = refIdxLTN

tx = ( 16384 + Abs( td / 2 ) ) /td

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )

mvpLS = ClipMv( Sign( DistScaleFactor * mvLTN ) *

( (Abs( DistScaleFactor * mvLTN ) + 127 ) >> 8 ) )

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – 　　　　　　　　RefPicOrderCnt( currPic , refIdxLT, LT ) )

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – 　　　　　　　　RefPicOrderCnt( currPic, refIdxLS, LS ) )

  - Otherwise (when predFlagL0N is equal to 1 and predFlagL1N is equal to 1), refIdxL0, refIdxL1, and mvpLS are derived as follows.

When abs(PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxL0, L0 )) is greater than abs(PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxL1, L1 ))

S = 1, else S = 0

T = 1-S

refIdxL0 = refIdxL0N

refIdxL1 = refIdxL1N

mvpLS = mvLSN

  - predFlagL0, predFlagL1, mvL0[0], mvL0[1], mvL1[0], and mvL1[1] are derived as follows.

predFlagL0 = predFlagL1 = 1

mvLS[ 0 ] = mvpLS[ 0 ] + mvdLS[ 0 ]

mvLS[ 1 ] = mvpLS[ 1 ] + mvdLS[ 1 ]

mvLT[ 0 ] = mvLTN[ 0 ]

mvLT[ 1 ] = mvLTN[ 1 ]

Table 9-3 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag, merge_mvd_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 | Error: Reference source not found |  | 0..1 | 2..3 |

| Table 9-4 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| merge_flag, merge_mvd_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |

| Table 9-5 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| merge_flag, merge_mvd_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |