| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G053 |
| --- | --- |

| Title: | CE9: Results of AMVP_SEL01 and AMVP _SEL02 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima Masayoshi Nishitani Hideki Takehara JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com fukushima.shigeru@ jvckenwood.com nishitani.masayoshi@ jvckenwood.com takehara.hideki@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE9 AMVP_SEL01 and AMVP_SEL02. The simulation results report that AMVP_SEL01 and AMVP_SEL02 provide no coding loss for both random access settings and low delay B settings.

# Introduction

At the 6th JCT-VC meeting, it was proposed to modify the spatial candidates’ selection process on AMVP using one group for the MVP candidates (JCTVC-F419 [1]).

In CE9 [2] which was established at the 6th JCT-VC meeting in Torino, two methods were tested as AMVP_SEL01 and AMVP_SEL02.

# Algorithm Description

## Current algorithm (HM4.0)

In HM4.0, two spatial candidates A and B are derived by scanning spatial neighboring positions A0, A1, B0, B1 and B2 in two groups, one for A and one for B. The first available candidate in the scanning order gets chosen. The scanning orders for candidates A and B are as follows.

A:

A0 non-scaled, A1 non-scaled, A0 scaled, A1 scaled

B:

B0 non-scaled, B1 non-scaled, B2 non-scaled, B0 scaled (if A is not scaled), B1 scaled (if A is not scaled), B2 scaled (if A is not scaled)

Figure 1 Spatial motion vector neighbors for HM4.0

## Proposed algorithm

It is proposed to derive two spatial candidates S0 and S1 by scanning spatial neighboring positions A0, A1, A2, A3 and A4 in one group. The first available candidate in this scanning order is S0 and the second available one is S1. For application of the restriction that the number of scaled spatial MVP candidate is less or equal to one, AMVP_SEL01 and AMVP_SEL02 will be tested.

Figure 2 Spatial motion vector neighbors for this proposal

AMVP_SEL01 one-pass scanning

To derive S0 and S1, non-scaled and scaled candidates are checked by one-pass scanning with the scanning order as follows.

A0 non-scaled,  A0 scaled (if S0 is not scaled), A1 non-scaled, A1 scaled (if S0 is not scaled), A2 non-scaled, A2 scaled (if S0 is not scaled), A3 non-scaled, A3 scaled (if S0 is not scaled), A4 non-scaled, A4 scaled (if S0 is not scaled)

When S0 and S1 are derived, scanning process is terminated.

AMVP_SEL02 two-pass scanning

To derive S0 and S1, non-scaled and scaled candidates are checked separately by two-pass scanning. In the first pass scanning, non-scaled candidates are checked - in the second pass scanning, scaled candidates are checked.

first pass: 	A0 non-scaled, A1 non-scaled, A2 non-scaled, A3 non-scaled, A4 non-scaled

second pass: 	A0 scaled, A1 scaled, A2 scaled, A3 scaled, A4 scaled

When S0 and S1 are derived, scanning process is terminated. When S0 or S1 is derived during the second pass, scanning process is terminated.

## Implementation

### Software

This proposed technique is implemented into HM4.0 software.

# Experiments

## Test conditions

The common test conditions [3] are used.

Tests and testers are presented in Table 1.

Table 1 Tests and testers

| Test | Description | Related Doc. | Tester1 | Tester2 |
| --- | --- | --- | --- | --- |
| AMVP_SEL01 | one-pass scanning | JCTVC-F419 | JVC Kenwood | Sony |
| AMVP_SEL02 | two-pass scanning | JCTVC-F419 | JVC Kenwood | Sony |

## Experimental environment

The experimental environment used is presented in Table 2.

Table 2 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 3 and Table 4 present the simulation results of AMVP_SEL1 for random access and low delay B, respectively.

Table 5 and Table 6 present the simulation results of AMVP _SEL2 for random access and low delay B, respectively.

Table 3 Simulation results of AMVP_SEL01 for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 4 Simulation results of AMVP_SEL01 for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.0% | 0.3% | 0.0% | 0.1% | -0.2% |
| Class E | 0.1% | -0.1% | 0.6% | -0.1% | 0.4% | -0.4% |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | -0.1% |
|  | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 5 Simulation results of AMVP_SEL02 for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6 Simulation results of AMVP_SEL02 for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.4% | -0.1% |
| Class E | 0.0% | 0.1% | 0.4% | -0.1% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The simulation results report that AMVP_SEL01 and AMVP_SEL02 provide no coding loss for random access settings and low delay B settings. (See the attached excel sheet)

# Conclusions

This contribution reports that AMVP_SEL01 and AMVP_SEL02 achieve simplification without coding loss.

We recommend that AMVP_SEL01 or AMVP_SEL02 be adopted to the next WD and HM.

# References

[1] “Unification of derivation process for merge mode and MVP”,

JCTVC-F419, Torino, July 2011

[2] “Core Experiment 9: MV Coding and Skip/Merge Operations”,

JCTVC-F909, Torino, July 2011

[3] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[4] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD Changes

##### 8.4.2.1.7	Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause  is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, BS0, S1 as the output.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor candidate list, mvpListLX, is constructed as follows.

- mvLXAmvLXS0, if availableFlagLXAavailableFlagLXS0 is equal to 1

- mvLXBmvLXS1, if availableFlagLXBavailableFlagLXS1 is equal to 1

- mvLXCol, if availableFlagLXCol is equal to 1

- When motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- When mvpListLX is empty, a zero motion vector is added as follows.

mvpListLX[ 0 ][0] = 0	(8-1)

mvpListLX[ 0 ][1] = 0	(8-2)

- The variable numMVPCandLX is set to the number of elements within the mvpListLX and maxNumMVPCand is set to 2.

- The motion vector predictor list is modifed to contain exactly maxNumMVPCand motion vector predictor candidates as follows.

    - If numMVPCandLX is less than maxNumMVPCand, the derivation process for zero motion vector predictor candidates specified in subclause Error: Reference source not found is invoked with mvpListLX and numMVPCandLX given as input and the output is assigned to mvpListLX and numMVPCandLX.

    - Otherwise (numMVPCandLX is equal to or greater than maxNumMVPCand), all motion vector predictor candidates mvpListLX[ idx ] with idx greater than maxNumMVPCand − 1 are removed from the list.

- The motion vector of mvpListLX[ mvp_idx_lX[ xP, yP ] ] is assigned to mvpLX.

##### 8.4.2.1.8	Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or BS0 or S1)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-3 – Spatial motion vector neighbours

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

The variable isScaledFlagLXisScaledFlagLXSM with X being 0 or 1 and M being 0 or 1 is set equal to 0.

The motion vector mvLXAmvLXSM and the availability flag availableFlagLXAavailableFlagLXSM are derived in the following ordered steps:

- Let a set of five sample locations be (xAk, yAk), with k = 0, 1, 2, 3, 4 specifies sample locations with xA1,2or4 = xP – 1, xA0 = xP + nPSW, xBA3 = xA0 –MinPuSize, yA1 = yP + nPSH, yA2 = yA1 - MinPuSize and yA0,3or4 = yP – 1. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXS0 and availableFlagLXS1 be initially set equal to 0, the both components of mvLXS0 and mvLXS1 are set equal to 0, isScaledFlagLXS0 and isScaledFlagLXS1 be initially set equal to 0, and the variable M is set equal to 0.

- When availableFlagLXS0 or availableFlagLXS1 is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA4, yA4 ), the following applies repeatedly until M is equal to 2:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXSM is set equal to 1 and the motion vector mvLXSM is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxSM is set equal to refIdxLX[ xAk ][ yAk ], ListSM is set equal to LX and M is added by 1.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXSM is set equal to 1, the motion vector mvLXSM is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxSM is set equal to refIdxLY[ xAk ][ yAk ] ,  ListSM is set equal to LY and M is added by 1.

- Ohterwise if isScaledFlagLXS0 is equal to 0, the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXSM is set equal to 1, the motion vector mvLXSM is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxSM is set equal to refIdxLX[ xAk ][ yAk ], ListSM is set equal to LX, isScaledFlagLXSM is set equal to 1 and M is added by 1.

- Otherwise if isScaledFlagLXS0 is equal to 0, the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXSM is set equal to 1, the motion vector mvLXSM is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxSM is set equal to refIdxLY[ xAk ][ yAk ], ListSM is set equal to LY, isScaledFlagLXSM is set equal to 1 and M is added by 1.

- When availableFlagLXSM is equal to 1, isScaledFlagLXSM is set equal to 1 and mvLXSM is derived as specified below.

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, isScaledFlagLX is set equal to 1 and mvLXA is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-3)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-4)

mvLXAmvLXSM = ClipMv( Sign( DistScaleFactor * mvLXAmvLXSM ) * 			
			( (Abs( DistScaleFactor * mvLXAmvLXSM ) + 127 ) >> 8 ) )	(8-5)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxArefIdxSM, ListAListSM ) )	(8-6)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-7)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ),  availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When availableFlagLXA or availableFlagLXB is equal to 0 and isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- When availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-8)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-9)

mvLXB = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
			( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) ) 	(8-10)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-11)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-12)