| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G054 |
| --- | --- |

| Title: | CE4 Subtest 1.2.a: Study and cross-verification of Mitsubishi's signaling of Max and Min QP in slice (JCTVC-G462) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com hao@bu.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents the study and cross-verification results of Mitsubishi's signaling of Max and Min QP in slice (JCTVC-G462. The cross-verification task was successfully completed and it is confirmed that proponent's results are accurate. Additional simulation results (which set the maximum and minimum QP values are set to 0 and 51, respectively) show that the JCTVC-F174 decreases the dQP coding efficiency by 1.3%, 1.6%, and 1.8% for All intra HE, Random access HE, and Low delay HE, respectively, and that it improves the dQP coding efficiency by 0.9% for All intra LC, Random access LC, and Low delay LC. Another additional simulation results show that the performance decreases in dQP coding efficiency in HE settings can be avoided by using a modified binarization on top of HM4.0.

If the JCTVC-G462 is considered for HM5.0 adoption, the following are recommended:

- Discuss the dQP range issues including the necessity of cyclic QP reconstruction;

- Discuss its potential harmonization with other schemes such as the current HM, JCTVC-F495, etc.

- Software analysis

The following implementations of JCTVC-F174(JCTVC-G462) are confirmed through the software analysis:

- Signal maximum and minimum QP values in each of slice headers;

  - In each slice, the maximum and minimum QP values (QPSliceMax and QPSliceMin) of its CUs are derived prior to the entropy coding.

- Given a set of QPY,PREV,, QPSliceMax and QPSliceMin, determine a mapping between possible cu_qp_delta values and indices in a manner that assigns smaller index to smaller absolute cu_qp_delta value;

- Encode indices by using new entropy coding methods.

  - For CABAC case, the binary representation of an index is derived by using the truncated unary binarization with a maximum value QPSliceMax - QPSliceMin: each bin of the binary representation is encoded by BAC.

  - For CAVLC case, an index is encoded by using concatenated truncated Golomb code.

- Remarks

Regarding the QPSliceMax and QPSliceMin derivation, encoders based on one-pass encoding would not be able to determine those values by analyzing QP values of entire slice data prior to its entropy coding. Therefore it is expected that such encoders set QPSliceMax and QPSliceMin to 0 and 51, respectively. In such a case (that the span of QP values of a slice is large), the proposal can degrade coding efficiency of cu_qp_delta since, for example, its cu_qp_delta binarization is unconscious with the sign of cu_qp_delta value. Specifically, as reported in JCTVC-F046, sign-unconscious binary representations of cu_qp_delta double the average and maximum number of bins compared to those of the current HM4.0. We study the side-effects of the proposal’s binarization by conducting additional test as described in the next section.

Regarding the derivation of possible cu_qp_delta values from a set of QPY,PREV,, QPSliceMax and QPSliceMin, this proposes to disable cyclic QP reconstruction specified as QPY = ( ( ( QY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY. Note that the current WD draft text specifies the decoded value of cu_qp_delta is in the range of –( 26 + QpBdOffsetY / 2 ) to +( 25 + QpBdOffsetY / 2 ) in order to enable the cyclic QP reconstruction. Since the cyclic QP reconstruction is inherited from the H.264/AVC standard, the necessity of the cyclic QP reconstruction should be carefully discussed, as suggested in JCTVC-F663.

- Simulation results

Software provided by the proponent was successfully compiled by Microsoft Visual C++ 2008. We created its encoder-constraint version so as to force QPSliceMax and QPSliceMin to 0 and 51, respectively. Furthermore, we created its modified version that uses a binary representation described in JCTVC-F663 which is a combination of the JCTVC-F174 binarization and the current HM binarization (JCTVC-F745). (See its detail in Appendix.) The executables of the original version, encoder-constraint version, and modified version were tested on Windows-based computing platform listed below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Xeon 3.57GHz and Memory 32GB | Cross-verification test of original version.. Additional test of modified version (Note that it also forces QPSliceMax and QPSliceMin to 0 and 51, respectively) |
| Windows 7 64-bit Professional Core i7 2.80GHz and Memory 12GB | Additional test of encoder-constraint version |

Table 1 shows the simulation results of the original Mitsubishi’s software. The results match with proponent’s. Table 2 and Table 3 show the simulation results of the encoder-constraint version and modified version, respectively. (Detailed results are found in attached Excel sheets, 20110923-NEC-CE4-Anchor-vs-MitsubishiF174.xls, 20111002-NEC-CE4-Anchor-vs-MitsubishiF174-constraint-min0-max51.xls, and 20111024-NEC-CE4-Anchor-vs-MitsubishiF174-constraint-min0-max51-f745harmonization.xls.)

Table 1: Simulation results of JCTVC-F174.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | -0.1% | 0.0% | -0.9% | -0.2% | -0.2% | -0.2% | -5.8% |
| Class B | 0.0% | -0.1% | 0.0% | -0.5% | -0.2% | -0.2% | -0.2% | -4.4% |
| Class C | 0.0% | -0.1% | -0.1% | -1.0% | -0.2% | -0.2% | -0.2% | -5.4% |
| Class D | -0.1% | 0.0% | 0.0% | -1.9% | -0.3% | -0.2% | -0.2% | -6.9% |
| Class E | -0.1% | -0.1% | 0.0% | -1.4% | -0.3% | -0.3% | -0.3% | -5.5% |
| Overall | -0.1% | -0.1% | 0.0% | -1.1% | -0.2% | -0.2% | -0.2% | -5.5% |
|  | -0.1% | -0.1% | 0.0% |  | -0.2% | -0.2% | -0.2% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | 0.1% | -0.3% | -1.1% | -0.2% | -0.2% | -0.2% | -6.9% |
| Class B | 0.0% | 0.0% | 0.0% | -0.6% | -0.2% | -0.2% | -0.2% | -5.6% |
| Class C | 0.0% | 0.0% | -0.1% | -1.3% | -0.3% | -0.2% | -0.2% | -6.8% |
| Class D | 0.2% | 0.3% | 0.1% | -2.1% | -0.1% | -0.1% | -0.1% | -7.7% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | -1.3% | -0.2% | -0.2% | -0.2% | -6.7% |
|  | 0.0% | 0.1% | -0.1% |  | -0.2% | -0.2% | -0.2% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.3% | -0.8% | -0.3% | -0.3% | -0.3% | -6.1% |
| Class C | 0.0% | -0.2% | 0.2% | -1.5% | -0.3% | -0.3% | -0.3% | -7.2% |
| Class D | 0.1% | -0.1% | 0.4% | -2.1% | -0.2% | -0.2% | -0.2% | -7.8% |
| Class E | 0.1% | 1.2% | -0.4% | -3.1% | -0.3% | -0.3% | -0.3% | -8.8% |
| Overall | 0.0% | 0.2% | 0.0% | -1.7% | -0.3% | -0.3% | -0.3% | -7.3% |
|  | 0.0% | 0.2% | 0.1% |  | -0.3% | -0.3% | -0.3% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Table 2: Simulation results of the encoder-constrained version of JCTVC-F174.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | 0.0% | 0.0% | 1.4% | 0.0% | 0.0% | 0.0% | -1.1% |
| Class B | 0.0% | 0.0% | 0.0% | 1.3% | 0.0% | 0.0% | 0.0% | -0.7% |
| Class C | 0.0% | 0.0% | 0.0% | 1.2% | 0.0% | 0.0% | 0.0% | -0.7% |
| Class D | 0.1% | 0.1% | 0.1% | 1.3% | 0.0% | 0.0% | 0.0% | -0.9% |
| Class E | 0.0% | 0.0% | 0.1% | 1.1% | -0.1% | -0.1% | -0.1% | -1.1% |
| Overall | 0.0% | 0.0% | 0.1% | 1.3% | 0.0% | 0.0% | 0.0% | -0.9% |
|  | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 102% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.1% | 0.2% | -0.2% | 1.6% | 0.0% | 0.0% | 0.0% | -1.2% |
| Class B | 0.1% | 0.0% | 0.1% | 1.5% | 0.0% | 0.0% | 0.0% | -0.7% |
| Class C | 0.1% | 0.1% | 0.0% | 1.6% | 0.0% | 0.0% | 0.0% | -0.9% |
| Class D | 0.2% | 0.4% | 0.2% | 1.9% | 0.1% | 0.1% | 0.1% | -0.9% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.0% | 1.6% | 0.0% | 0.0% | 0.0% | -0.9% |
|  | 0.1% | 0.2% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 102% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | -0.2% | 1.5% | 0.0% | 0.0% | 0.0% | -0.8% |
| Class C | 0.1% | -0.1% | 0.3% | 1.7% | 0.0% | 0.0% | 0.0% | -0.9% |
| Class D | 0.2% | 0.0% | 0.5% | 2.2% | 0.1% | 0.1% | 0.1% | -0.8% |
| Class E | 0.3% | 1.4% | -0.2% | 1.8% | 0.1% | 0.1% | 0.1% | -1.3% |
| Overall | 0.1% | 0.3% | 0.1% | 1.8% | 0.0% | 0.0% | 0.0% | -0.9% |
|  | 0.1% | 0.3% | 0.2% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 102% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 102% |  |  |  |

Table 3: Simulation results of the modified version of JCTVC-F174. (Note that encoder forces QPSliceMax and QPSliceMin to 0 and 51, respectively.)

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access HE |  |  |  |
|  | Y | U | V | dQP incr. |
| Class A | 0.0% | 0.1% | -0.2% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.1% | 0.1% | 0.0% | 0.1% |
| Class D | 0.2% | 0.4% | 0.1% | -0.1% |
| Class E |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.1% | 0.0% |  |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low delay B HE |  |  |  |
|  | Y | U | V | dQP incr. |
| Class A |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.3% | -0.1% |
| Class C | 0.0% | -0.2% | 0.2% | 0.1% |
| Class D | 0.1% | -0.1% | 0.4% | 0.0% |
| Class E | 0.2% | 1.3% | -0.3% | 0.0% |
| Overall | 0.1% | 0.3% | 0.0% | 0.0% |
|  | 0.1% | 0.3% | 0.1% |  |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 98% |  |  |  |

The results show that:

- JCTVC-F174 attains consistent coding gains in both HE and LC cases when the maximum and minimum QP values are derived from slice data prior to its entropy coding;

- JCTVC-F174 drops the cu_qp_delta coding efficiency in HE settings when the maximum and minimum QP values are set to 0 and 51, respectively, that is, it drops the coding efficiency of the current HM to the level of previous one;

- JCTVC-F174 improves the cu_qp_delta coding efficiency in LC settings even when the maximum and minimum QP values are set to 0 and 51, respectively, that is, its concatenated truncated Golomb code works for different QP values.

- The drops in HE settings can be avoided by using the modified binarization on top of the current HM.

# Conclusions

The cross-verification task has been successfully completed and it is confirmed that proponent’s results are accurate. Additional simulation results (which set the maximum and minimum QP values are set to 0 and 51, respectively) have shown that the JCTVC-G462 decreases the cu_qp_delta coding efficiency by 1.3%, 1.6%, and 1.8% for All intra HE, Random access HE, and Low delay HE, respectively, and that it improves the cu_qp_delta coding efficiency by 0.9% for All intra LC, Random access LC, and Low delay LC. Another additional simulation results have demonstrated the coding efficiency decreases in HE settings can be avoided by using a modified binarization on top of HM4.0.

If the JCTVC-G462(or JCTVC-F174) is considered for HM5.0 adoption, the following actions are recommended:

- Discuss the cu_qp_delta range issue, as reported in JCTVC-F663;

  - Do we need cyclic QP reconstruction or not?

  - Do we allow encoder to switch between cyclic QP reconstruction on and off? (For example, we can specify it as follows: if both max and min QPs present in the slice header, the cyclic QP reconstruction is disabled: otherwise, the cyclic QP reconstruction is enabled.)

- Discuss harmonization of JCTVC-G462 and other schemes (such as the current HM, JCTVC-F495, etc).

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- K. Sugimoto and S. Sekiguchi, "Signaling of Max and Min QP in slice," JCTVC-F174, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- K. Sugimoto, A. Minezawa, and S.Sekiguchi, “CE4 subset 1.2.a: results of Max/Min QP signaling” JCTVC-G462, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

- K. Chono, H. Aoki, and Y. Senda, "Efficient binary representation of cu_qp_delta syntax for CABAC," JCTVC-F046, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.

- K. Chono and H. Aoki, “On cu_qp_delta range constraint,” JCTVC-F663, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.

- K. Chono, H. Aoki, Y. Senda, K. Kondo, K.Sato, and J. Xu, "BoG report on efficient binary representation of cu_qp_delta syntax for CABAC," JCTVC-F745, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.

# Appendix: Potential modifications of HEVC Working Draft 4.

(The changes in 9.3.2.7 Binarization process for cu_qp_delta are highlighted with yellow)

9.3.2.7 Binarization process for cu_qp_delta

Input to this process is a request for a binarization for the syntax element cu_qp_delta and a qp prediction value QPY,PREV..

Output of this process is the binarization of the syntax element.

The bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code.

absV = Abs( synVal )

if( absV == 0){

put( 0 )

} else {

put( 1 )

signV = (synVal > 0 )? 0 : 1

put (signV )

cMax = 24 + (QpBdOffsetY / 2) + signV

cNum = absV - 1

absVGreaterThan1Flag = ( absV == 1 )? 0 : 1

put (absVGreaterThan1Flag )

if( absVGreaterThan1Flag ){

while(--cNum)

{

put( 1 )

}

if(cMax > absV - 1 ) {

put( 0 )

}

}

if(signV){

cMax = QPY,PREV - MinQP - 1

}

else {

cMax = MaxQP - QPY,PREV - 1

}

cNum = absV – 1

if(cMax > 0){

absVGreaterThan1Flag = ( absV == 1 )? 0 : 1

put (absVGreaterThan1Flag )

if( absVGreaterThan1Flag ){

while(--cNum)

{

put( 1 )

}

if(cMax > absV - 1 ) {

put( 0 )

}

}

}

}