| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, November, 2011 | Document: JCTVC-G056 |
| --- | --- |

| Title: | CE8 Subtest G: Adaptive Loop Filtering of Chrominance Samples Using Luma Map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Hsan Guermazi Nader Mahdi Michael Horowitz | Email: | faouzi@ebriskvideo.com hsan@ebriskvideo.com nader@ebriskvideo.com michael@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This contribution presents a technique for the Adaptive Loop Filtering (ALF) of chrominance samples that employs the map used in ALF of the luminance samples. The proposed technique reduces the average decoding time (~1%) while also achieving better U/V BD-Rate (0.21%-0.19%) performance than those of HM4.0 [1].

# Proposed ALF Technique

In HM4.0, (1) ALF of the chrominance samples is applied only when ALF filtering is applied to the luminance samples and (2) CU-adaptive filtering is not applied for chrominance samples (i.e. ALF is applied to a whole frame of the chrominance samples, or it is not applied at all).

In the proposed technique, we apply either frame-based or CU-based ALF for chrominance samples as follows (see Figure 1):

- If ALF is not applied to the luminance samples, (a) it is also not applied to the chrominance samples.

- If frame-based ALF is applied to the luminance samples, either (b) ALF is not applied at all, or (c) frame-based ALF is also applied to the chrominance samples.

- If CU-based ALF is applied to the luminance samples, either (d) ALF is not applied at all, or (e) frame-based ALF is applied, or (f) CU-based ALF is also applied to the chrominance samples. When CU-based ALF is applied to the chrominance samples, we apply the same CU filter map generated for the luminance samples.

Figure 1 - ALF of the Luminance and Chrominance Samples

# Simulation Results

The simulation experiments were conducted following the testing conditions in [2]. The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @ 2.50 GHz.

- RAM: 4 Go.

- OS:  Windows XP Professional.

The coding efficiency (BD-Rate) numbers for the proposed ALF Algorithm were obtained for the Low Delay P HE (Table 1), Low Delay HE (Table 2), Random Access HE (Table 3), and Intra Only HE (Table 4) profiles.

Table 1 – Simulation Results: Low Delay P HE

|  | Low delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% |
| Class C | 0.0% | -0.1% | -0.4% |
| Class D | 0.0% | -0.1% | -0.5% |
| Class E | 0.0% | -0.7% | -0.5% |
| Class F | -0.1% | -0.3% | -0.4% |
| Overall | 0.0% | -0.3% | -0.4% |
| Overall - F | 0.0% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Table 2 – Simulation Results: Low Delay HE

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.4% | -0.1% |
| Class C | 0.0% | -0.3% | -0.4% |
| Class D | 0.0% | 0.0% | -0.2% |
| Class E | -0.1% | -0.7% | 0.2% |
| Class F | -0.1% | -0.4% | -0.4% |
| Overall | 0.0% | -0.3% | -0.2% |
| Overall - F | 0.0% | -0.3% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 3 – Simulation Results: Random Access HE

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% |
| Class B | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.2% | -0.2% |
| Class E |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | -0.2% | -0.1% |
| Overall - F | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

Table 4 – Simulation Results: Intra Only HE

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% |
| Overall - F | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

# Syntax & Semantics

#### Adaptive loop filter parameter syntax

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_region_adaptation_flag | 2 | u(1) |
| alf _shape_luma_flag | 2 | u(1) |
| alf_no_filters_minus1 | 2 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |  |
| alf_start_second_filter | 2 | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |  |
| for (i=1; i< 16; i++) |  |  |
| alf_filter_pattern[i] | 2 | u(1) |
| } |  |  |
| if (AlfNumFilters > 1) |  |  |
| alf_pred_method | 2 | u(1) |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |  |
| alf_coeff_luma[i][j] |  | ge(v) |
| alf_chroma_idc | 2 | ue(v) |
| if ( alf_chroma_idc ) { |  |  |
| alf _shape_chroma_flag | 2 | u(1) |
| if (alf_cu_control _flag) { |  |  |
| alf_cu_control_chroma_flag | 2 | u(1) |
| } |  |  |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |  |
| alf_coeff_chroma[i] |  | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |

alf_cu_control_chroma_flag specifies whether the adaptive loop filter process for chroma component shall be applied adaptively according to the coding unit. If alf_cu_control_chroma_flag is equal to 1, the filtering process shall be applied only when alf_cu_flag is equal to 1, otherwise, the filtering process shall be applied to all chroma samples in the current slice.

# Draft Text

### Adaptive loop filter process

An adaptive loop filtering process shall be conditionally performed on a treeblock basis after the completion of the sample adaptive offset process for the entire decoded picture, with all treeblocks in a picture processed in order of increasing treeblock addresses.

Each treeblock is processed on a coding unit basis with the same order as decoding process.

This process is invoked when both adaptive_loop_filter_enabled_flag and adaptive_loop_filter_flag are equal to 1.

This process is performed on a coding unit basis after the completion of the slice construction process prior to adaptive loop filter process for the entire decoded slice, with all coding units in a slice processed in order of coding unit scan order.

Filter coefficients cL for luma samples and cC for chroma samples are derived by invoking the process specified in subclause Error: Reference source not found.Filter index array fIdx for luma samples are derived by invoking the process specified in subclause Error: Reference source not found with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture and the coding unit size log2CUSize as inputs.

When interpreting the luma samples of the coding unit as to be filtered, depending on pcm_flag, pcm_loop_filter_disable_flag and alf_cu_control_flag, the following applies.

- If pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, the luma samples are not filtered.

- Otherwise, if alf_cu_control_flag is equal to 0, the luma samples are filtered,

- Otherwise (alf_cu_control_flag is equal to 1), if AlfCuFlag[ xC ][ yC ] is equal to 1 where xC and yC are the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the luma samples are filtered.

- Otherwise, the luma samples are not filtered.

When interpreting the chroma samples of the coding unit as to be filtered, depending on pcm_flag, pcm_loop_filter_disable_flag and alf_chroma_idc, the following applies.

- If pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, both chroma samples are not filtered.

- Otherwise, if alf_chroma_idc is equal to 1 and alf_cu_control_chroma_flag is equal to 0, the Cr samples are filtered.

- Otherwise, if alf_chroma_idc is equal to 1 and alf_cu_control_chroma_flag is equal to 1, if AlfCuFlag[ xC][ yC ] is equal to 1 where xC/2 and yC/2 are the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the Cr samples are filtered.

- Otherwise, if alf_chroma_idc is equal to 2 and alf_cu_control_chroma_flag is equal to 0, the Cb samples are filtered.

- Otherwise, if alf_chroma_idc is equal to 2 and alf_cu_control_chroma_flag is equal to 1, if AlfCuFlag[ xC ][ yC] is equal to 1 where xC/2 and yC/2 are the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the Cb samples are filtered.

- Otherwise, if alf_chroma_idc is equal to 3 and alf_cu_control_chroma_flag is equal to 0, both chroma samples are filtered.

- Otherwise, if alf_chroma_idc is equal to 3 and alf_cu_control_chroma_flag is equal to 1, if AlfCuFlag[ xC ][ yC] is equal to 1 where xC/2 and yC/2 are the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, both chroma samples are filtered.

- Otherwise (alf_chroma_idc is equal to 0), both chroma samples are not filtered.

For the luma samples of the coding unit interpreted as to be filtered, the filtering process for luma samples specified in subclause 4 is invoked with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the coding unit size log2CUSize, and the filter index array fIdx as inputs and the output is the modified filtered picture, recFiltPictureL.

For the chroma samples of the coding unit interpreted as to be filtered, the filtering process for chroma samples specified in subclause Error: Reference source not found is invoked with the chroma location ( xC/2, yC/2 ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the coding unit size log2CUSize-1 and the chroma component index cIdx equal to 1 as inputs and the output is the modified filtered picture, recFiltPictureCb.

For the chroma samples of the coding unit interpreted as to be filtered, the filtering process for chroma samples specified in subclause Error: Reference source not found is invoked with the chroma location ( xC/2, yC/2 ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the coding unit size log2CUSize-1 and the chroma component index cIdx equal to 2 as inputs and the output is the modified filtered picture, recFiltPictureCr.

[Ed.: (WJ) recPicture: deblocked/output picture and recFiltPicture: ALFed picture]

[Ed.: (WJ) depending adaptive_loop_filter_flag, recFiltPicture should be copied to recPicture in subclause Error: Reference source not found]

# Conclusions

Applying CU-based adaptation for the Adaptive Loop Filtering (ALF) of chrominance samples by employing the map used for ALF of the luminance samples is presented in this contribution. Comparison of the results of the proposed technique against those of HM4.0 shows a reduction in the average decoding time (~1%) and a better U/V BD-Rate (0.2%-0.2%) performance.

# Reference

[1] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

[2] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900.

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).