| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G058 |
| --- | --- |

| Title: | CE3: Interpolation using different-length horizontal and vertical filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Nader Mahdi Hsan Guermazi Michael Horowitz | Tel: Email: | faouzi@ebriskvideo.com nader@ebriskvideo.com hsan@ebriskvideo.com michael@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

In this contribution, an interpolation filtering technique is proposed for the motion compensation interpolation filtering of the luminance and chrominance samples of video sequences. This proposal consists of using one set of fixed vertical filters (V_F) for the vertical stage of filtering and a second set of fixed horizontal filters (H_F) for horizontal stage of filtering. Compared to HM4.0, the proposed technique yields average BD-rate reductions of -0.7% for LDP/LC, -0.2% for LDP/HE, -0.1% for LD/LC, 0.0% for LD/HE, 0.2% for RA/LC and 0.0% for RA/HE, while decreasing the decoding complexity. In fact, this technique reduces the required number of multiplications and additions by 5% and 6%, respectively.

# Introduction

This contribution describes an interpolation filtering technique for interpolation filtering during the motion compensation part of video encoding/decoding. The proposed technique is applied to the interpolation of the luminance and chrominance samples of video sequences, as described in the following section. The syntax changes in the WD text are described in Section 3. The simulation results are presented in Section 4. Section 5 presents some conclusions.

# Proposed Interpolation Technique with fixed filters:

The proposed interpolation technique, although based mainly on the HM4.0 technique in [1], is quite different. Instead of using the HM 4.0 filters, in this proposal, we employ the new filters shown in Table 1 and Table 2 for the interpolation of luminance and chrominance samples, respectively. More specifically, for the interpolation of the luminance samples, we employ (1) Samsung’s 6-tap ½-pel filter and 7-tap ¼-pel filter [5] during the vertical stage of luma filtering and (2) Motorola’s 8-tap ½-pel filter and 7-tap ¼-pel filter [6] during the horizontal stage of luma filtering. Similarly, we employ (1) Samsung’s 4-tap filters [8] during the vertical stage of chroma filtering and (2) Motorola’s 4-tap filters [7] during the horizontal stage of chroma filtering.

Table 1: Fixed Luma filter coefficients

| Filtering stage | Filter | Tap length | Filter coefficients |
| --- | --- | --- | --- |
| Horizontal | H_F1/4 | 7 | {-1, 3, -8, 60, 13, -4, 1, 0} |
|  | H_F1/2 | 8 | {-1, 4, -11, 40, 40, -11, 4, -1} |
| Vertical | V_F1/4 | 7 | {-2, 5, -11, 58, 18, -6, 2, 0} |
|  | V_F1/2 | 6 | { 0, 2, -9, 39, 39, -9, 2, 0} |

Table 2: Fixed Chroma filter coefficients

| Filtering stage | Filter | Tap length | Filter coefficients |
| --- | --- | --- | --- |
| Horizontal | H_F1/8 | 4 | {-2, 61, 6, -1} |
|  | H_F1/4 | 4 | {-4, 54, 16, -2} |
|  | H_F3/8 | 4 | {-5, 44, 29, -4} |
|  | H_F1/2 | 4 | {-4, 36, 36, -4} |
| Vertical | V_F1/8 | 4 | { -2, 58, 10, -2 } |
|  | V_F1/4 | 4 | { -4, 54, 16, -2 } |
|  | V_F3/8 | 4 | { -6, 46, 28, -4 } |
|  | V_F1/2 | 4 | { -4, 36, 36, -4 } |

# Proposed WD Text Changes (highlighted in Blue)

### 4.1.1. Luma sample interpolation process

…

…

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the corresponding filter to the nearest integer position samples:

a0,0 = ( −A−3,0 + 3*A−2,0 − 8*A−1,0 + 60*A0,0 +	 13*A1,0 − 4*A2,0 + 1*A3,0 ) >> shift1	(8-1)

b0,0 = ( −A−3,0 + 4*A−2,0 − 11*A−1,0 + 40*A0,0 +	 40*A1,0 − 11*A2,0 + 4*A3,0 − A4,0 ) >> shift1	(8-2)

c0,0 = (  1*A−2,0 − 4*A−1,0 + 13*A0,0 +	 60*A1,0 − 8*A2,0 + 3*A3,0 − A4,0 ) >> shift1	(8-3)

d0,0 = ( −2*A0,−3 + 5*A0,−2 − 11*A0,−1 + 58*A0,0 +	 18*A0,1 − 6*A0,2 + 2*A0,3 ) >> shift1	(8-4)

h0,0 = (  2*A0,−2 − 9*A0,−1 + 39*A0,0 +	 39*A0,1 − 9*A0,2 + 2*A0,3  ) >> shift1	(8-5)

n0,0 = (  2*A0,−2 − 6*A0,−1 + 18*A0,0 +	 58*A0,1 − 11*A0,2 + 5*A0,3 − 2*A0,4 ) >> shift1	(8-6)

–	The samples labelled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by applying the corresponding filter to the samples a0,i, b0,i and c0,i where i = −3..4 in vertical direction:

e0,0 = ( −2*a0,−3 + 5*a0,−2 − 11*a0,−1 + 58*a0,0 +	 18*a0,1 − 6*a0,2 + 2*a0,3 ) >> shift2	(8-7)

f0,0 = ( −2*b0,−3 + 5*b0,−2 − 11*b0,−1 + 58*b0,0 +	 18*b0,1 − 6*b0,2 + 2*b0,3 )  >> shift2	(8-8)

g0,0 = ( −2*c0,−3 + 5*c0,−2 − 11*c0,−1 + 58*c0,0 +	 18*c0,1 − 6*c0,2 + 2*c0,3 )  >> shift2	(8-9)

i0,0 = (  2*a0,−2 − 9*a0,−1 + 39*a0,0 +	 39*a0,1 − 9*a0,2 + 2*a0,3  ) >> shift2	(8-10)

j0,0 = (  2*b0,−2 − 9*b0,−1 + 39*b0,0 +	 39*b0,1 − 9*b0,2 + 2*b0,3  )  >> shift2	(8-11)

k0,0 = (  2*c0,−2 − 9*c0,−1 + 39*c0,0 +	 39*c0,1 − 9*c0,2 + 2*c0,3  )  >> shift2	(8-12)

p0,0 = (  2*a0,−2 − 6*a0,−1 + 18*a0,0 +	 58*a0,1 − 11*a0,2 + 5*a0,3 − 2*a0,4 ) >> shift2	(8-13)

q0,0 = (  2*b0,−2 − 6*b0,−1 + 18*b0,0 +	 58*b0,1 − 11*b0,2 + 5*b0,3 − 2*b0,4 ) >> shift2	(8-14)

r0,0 = (  2*c0,−2 − 6*c0,−1 + 18*c0,0 +	 58*c0,1 − 11*c0,2 + 5*c0,3 − 2*c0,4 ) >> shift2	(8-15)

…

...

...

### 4.1.1. Chroma sample interpolation process

…

…

…

The samples labelled ab0,0, ac0,0, ad0,0, ae0,0, af0,0, ag0,0, and ah0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ab0,0 = ( −2*B−1,0 + 61*B0,0 + 6*B1,0 – B2,0 ) >> shift1		(8-16)

ac0,0 = ( −4*B−1,0 + 54*B0,0 + 16*B1,0 – 2*B2,0 ) >> shift1	(8-17)

ad0,0 = ( −5*B−1,0 + 44*B0,0 + 29*B1,0 – 4*B2,0 ) >> shift1	(8-18)

ae0,0 = ( −4*B−1,0 + 36*B0,0 + 36*B1,0 – 4*B2,0 ) >> shift1	(8-19)

af0,0 = ( −4*B−1,0 + 29*B0,0 + 44*B1,0 – 5*B2,0 ) >> shift1	(8-20)

ag0,0 = ( −2*B−1,0 + 16*B0,0 + 54*B1,0 – 4*B2,0 ) >> shift1	(8-21)

ah0,0 = ( −B−1,0 + 6*B0,0 + 61*B1,0 – 2*B2,0 ) >> shift1		(8-22)

–	The samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ba0,0 = ( −2*B0,−1 + 58*B0,0 + 10*B0,1 – 2*B0,2 ) >> shift1	(8-23)

ca0,0 = ( −4*B0,−1 + 54*B0,0 + 16*B0,1 – 2*B0,2 ) >> shift1	(8-24)

da0,0 = ( −6*B0,−1 + 46*B0,0 + 28*B0,1 – 4*B0,2 ) >> shift1	(8-25)

ea0,0 = ( −4*B0,−1 + 36*B0,0 + 36*B0,1 – 4*B0,2 ) >> shift1	(8-26)

fa0,0 = ( −4*B0,−1 + 28*B0,0 + 46*B0,1 – 6*B0,2 ) >> shift1	(8-27)

ga0,0 = ( −2*B0,−1 + 16*B0,0 + 54*B0,1 – 4*B0,2 ) >> shift1	(8-28)

ha0,0 = ( −2*B0,−1 + 10*B0,0 + 58*B0,1 – 2*B0,2 ) >> shift1	(8-29)

–	The samples labelled bX0,0, cX0,0, dX0,0, eX0,0, fX0,0, gX0,0 and hX0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by applying the 4-tap filter to the intermediate values aX0,i where i = −1..2 in vertical direction:

bX0,0 = ( −2*aX0,−1 + 58*aX0,0 + 10*aX0,1 – 2*aX0,2 ) >> shift2	(8-30)

cX0,0 = ( −4*aX0,−1 + 54*aX0,0 + 16*aX0,1 – 2*aX0,2 ) >> shift2	(8-31)

dX0,0 = ( −6*aX0,−1 + 46*aX0,0 + 28*aX0,1 – 4*aX0,2 ) >> shift2	(8-32)

eX0,0 = ( −4*aX0,−1 + 36*aX0,0 + 36*aX0,1 – 4*aX0,2 ) >> shift2	(8-33)

fX0,0 = ( −4*aX0,−1 + 28*aX0,0 + 46*aX0,1 – 6*aX0,2 ) >> shift2	(8-34)

gX0,0 = ( −2*aX0,−1 + 16*aX0,0 + 54*aX0,1 – 4*aX0,2 ) >> shift2	(8-35)

hX0,0 = ( −2*aX0,−1 + 10*aX0,0 + 58*aX0,1 – 2*aX0,2 ) >> shift2	(8-36)

…

…

…

# Experimental Results

The simulation experiments were conducted following the testing conditions in JCTVC-F900 [3].

The proposed interpolation technique was compared to the HM4.0 technique. The coding efficiency (BD-Rate) numbers were obtained for the Low Delay P, Low Delay and Random Access configurations, and they are shown in Table 3. The summary of the complexity results (runtime, number of operations and memory bandwidth) are shown in Table 4.

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @2.50 GHz.

- RAM: 4 Go

- OS:  Windows XP Professional.

It is clear, from the tables, that the proposed interpolation method achieves coding efficiency gain over the HM4.0 fixed filtering technique, especially for 720p and higher resolution sequences. It is also clear that we achieve a significant reduction of the encoder/decoder complexity (-5% and -6% reduction in term of required multiplication and additions, respectively).

Table 3: Simulation Results (H/V Fixed filters)

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.1% | -0.8% | -0.2% | -0.5% |
| Class C | -0.2% | 0.1% | 0.3% | -0.3% | 0.2% | 0.2% |
| Class D | 0.2% | -0.1% | -0.2% | 0.1% | 0.2% | 0.1% |
| Class E | -0.9% | -0.3% | -0.2% | -2.1% | -1.4% | -0.7% |
| Overall | -0.2% | -0.1% | 0.0% | -0.7% | -0.2% | -0.2% |
|  | -0.2% | -0.1% | -0.1% | -0.7% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -7% |  |  | -7% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.4% | -0.2% | -0.4% | -0.4% | -0.4% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | 0.5% | 0.2% |
| Class D | 0.5% | 0.1% | 0.6% | 0.7% | 0.5% | 0.3% |
| Class E | -0.4% | 0.4% | 0.3% | -0.8% | -0.1% | -0.5% |
| Overall | 0.0% | 0.0% | 0.2% | -0.1% | 0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -7% |  |  | -7% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | -0.1% | 0.6% | 0.5% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class C | 0.1% | 0.1% | 0.2% | 0.3% | 0.2% | 0.3% |
| Class D | 0.1% | 0.3% | 0.2% | 0.7% | 0.2% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.3% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -7% |  |  | -7% |  |  |
| Access[%] | 1% |  |  | 0% |  |  |

Table 4: Summary of the Complexity Results (H/V Fixed filters)

| Gain Ave.[%] | -0.13% | 0.00% | 0.02% |
| --- | --- | --- | --- |
| Gain Min. [%] | -3.50% | -2.01% | -1.95% |
| Gain Max.[%] | 1.58% | 1.83% | 1.73% |
| Enc Time[%] | 100.21% |  |  |
| Dec Time[%] | 100.02% |  |  |
| Mult Ave.[%] | -5% |  |  |
| Mult Worst[%] | -5% |  |  |
| Adds Ave.[%] | -7% |  |  |
| Adds Worst[%] | -6% |  |  |
| Access[%] | 0% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |

# 7. Conclusion and recommendation

The proposed technique provides an average of 0.13% in Y BD-Rate reduction relative to the HM 4.0 fixed filtering technique, while also requiring essentially less encoding/decoding complexity. Therefore, we suggest that the proposed technique be adopted to the next HM.

# 8. References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

- S. Wittmann, T. Wedi, “Separable Adaptive Interpolation Filter,” ITU-T SG16/Q.6 Doc. T05-SG16-C-0219, Geneva, Switzerland, June 2007.

- F. Bossen, “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

- F. Kossentini, N. Mahdi, H. Guermazi, M. Horowitz, “CE3: An Adaptive Interpolation Filtering Technique”, JCTVC-G057, November 2011

- E. Alshina, A.Alshin  “CE3: DCT derived interpolation filter test by Samsung.”, JCTVC-F247, July 2011[3]

- J. Lou, K. Minoo, D. Baylon, K. Panusopone, L. Wang, Ajay Luthra, “Fixed interpolation filter tests by Motorola Mobility,” JCTVC-F574, Turin, Italy, July, 2011

- Koohyar Minoo, Jian Lou, Ajay Luthra, “Chroma interpolation filters,” JCTVC-F599, Turin, Italy, July, 2011

- Alexander Alshin, Elena Alshina, JeongHoon Park, Jian Lou, Koohyar Minoo “CE3: 7 taps interpolation filters for quarter pel position MC from Samsung and Motorola Mobility”, JCTVC-G778, Geneva, Switzerland, November 2011

# 9. Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).