| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, November, 2011 | Document: JCTVC-G064 |
| --- | --- |

| Title: | Results of the PA-Coder |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbo Zhu | Email: | hurricanezhb@hotmail.com |
| Source: | None. |  |  |

_____________________________

# Abstract

The proposal reports the design, fast decoding algorithm, probability estimation and coding results of the arithmetic coder based on the probability aggregation.

# Design of the PA-Coder

The arithmetic coder based on based on the probability aggregation (PA-Coder) is introduced in [1] and improved in [2-5]. As shown in fig. 1, the encoder of the PA-Coder maintains two variables, logprob and low. The logprob denotes the interval length while the low denotes the low bound of the interval. Each probability state is associated with two variables, LPSlogprob and MPSlogprob. The logprob denotes the log of the corresponding probability of the current interval. The LPSlogprob and MPSlogprob denote the log of the probabilities of the MPS and the LPS of the probability state. So when encoding the MPS, update the logprob as logprob=logprob+MPSlogprob and maintain the low unchanged. When encoding the LPS, update the logprob as logprob=logprob+LPSlogprob and update the low as low=low+I(logprob+MPSlogprob), in which the I(x) is a mapping from the logprob to the interval length.

Probability estimation

As the MPS∈[0.5,1.0), so MPSlogprob∈1,128. In the PA-Coder, the MPSlogprob is always integer. So use the paidx=MPSlogprob-1 as the index of the probability state, and estimate the probability using the scaled probability estimation method:

pmpsnxt=w+1−w×pmpscurr when MPSpmpsnxt=1−w×pmpscurr        when LPS

The parameter w is in range from 0.04 to 0.06. As the MPSlogprob of the updated probability is not exactly equal to integer, it is round to the nearest integer.

# Simulation Results

The HM4.0 r1354 is used as the platform to test the PA-Coder. The simulation experiments were conducted following the testing conditions in [6]. The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: amd phenom (tm) II x6 1075T @3.0GHz (6 cores).

- RAM: 6G DDR3 1333MHz.

- BUS: 1333MHz

- HardDisk: WD 1TB

- OS:  Win7x64.

- EXE: x64 version (VC2008)

Results of w=0.055

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.3% | 0.2% | 0.4% |
| Class B | 0.2% | 0.3% | 0.3% |
| Class C | 0.2% | 0.2% | 0.2% |
| Class D | 0.2% | 0.2% | 0.1% |
| Class E | 0.3% | 0.4% | 0.4% |
| Overall | 0.2% | 0.3% | 0.3% |
|  | 0.2% | 0.3% | 0.3% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 90% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.3% | 0.4% | 0.1% |
| Class B | 0.2% | 0.2% | 0.1% |
| Class C | 0.1% | 0.1% | 0.3% |
| Class D | 0.2% | 0.1% | 0.0% |
| Class E |  |  |  |
| Overall | 0.2% | 0.2% | 0.1% |
|  | 0.2% | 0.2% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.2% | 0.1% | -0.1% |
| Class C | 0.2% | 0.1% | 0.0% |
| Class D | 0.2% | 0.1% | -0.1% |
| Class E | 0.2% | -0.1% | 0.8% |
| Overall | 0.2% | 0.1% | 0.1% |
|  | 0.2% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.2% | 0.2% | 0.3% |
| Class C | 0.2% | 0.3% | 0.3% |
| Class D | 0.2% | 0.1% | -0.2% |
| Class E | 0.1% | 0.4% | -0.6% |
| Overall | 0.2% | 0.2% | 0.0% |
|  | 0.2% | 0.3% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 101% |  |  |

Table 1: Rate-distortion and speed performance comparison of PA-Coder with w=0.055

Results of w=0.044

Doing. It is expected that the new parameter w=0.044 eliminates the loss of the PA-Coder with w=0.055.

Results of w=0.044 with increased probability estimation accuracy by 1 bit

To be done. It is expected that the new parameter estimation give gain over the anchor.

# Software encoding/decoding speed comparison

To be done.

# Syntax & Semantics

# Draft Text

# Conclusions

It is suggested to adopt the PA-Coder into the next version HM and the WD.

# Reference

- Hongbo Zhu, “Arithmetic coding based on probability-aggregation and delayed-subdivision”, JCTVC-A027.

- Hongbo Zhu, “Some improvement of the arithmetic coding based on probability aggregation”, JCTVC-B091.

- Hongbo Zhu, “Decoding Improvement on the PA-Coder”, JCTVC-C296.

- Hongbo Zhu, “More improvements and results of the arithmetic coding based on probability aggregation”, JCTVC-D342.

- Hongbo Zhu, “Improved arithmetic coding based on the probability aggregation-more results”, JCTVC-F544.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900.

# Patent rights declaration(s)

Hongbo Zhu may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).