| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G067 |
| --- | --- |

| Title: | CE4 Subtest 1: Spatial QP prediction based on intra prediction and prediction mode (test 1.3.c) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN Masaaki Kobayashi, Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: Tel: Email: Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com +81-3-3758-2111 kobayashi.masaaki517@canon.co.jp shima.masato@canon.co.jp +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | NEC Corporation, Canon Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of test 1.3.c of CE4 Subtest 1. It is a spatial QP prediction method based on intra prediction direction and prediction mode matching. For CUs coded with angular intra prediction, the method employs a neighbouring CU along intra prediction direction for QP prediction. For other CUs, the method predicts QP as the average of reference QPs selected from above and left CUs according to prediction mode matching. . The experimental results show that the proposed QP prediction achieves 0.5% BD-rate reduction for AI_HE and AI_LC configurations, 0.3% BD-rate reduction for RA_HE configuration, 0.4% BD-rate reduction for RA_LC, RA_LC and LB_HE configurations under the test condition defined in CE4 Subtest 1.

# Introduction

In the former CE4 Subtest 2, it was reported that the performance of the QP prediction based on intra prediction (2.3.g) [1] was the best for AI_HE and AI_LC configurations under the test condition for CE4 Subtest 2 [2]. However, there were several other tools performing better for RA and LB configurations because the QP prediction based on intra prediction improves only QP coding of intra-coded CUs. Accordingly, it was agreed within the former CE4 that some combination of QP prediction based on intra prediction and other QP prediction tools is recommended.

Among combinations of intra prediction based QP prediction and other methods listed in the former CE4 Subtest 2, the best performing combination was that of 2.3.g and 2.3.f [3], i.e. prediction mode dependent QP prediction. Since there is further room to be combined with other methods in the combination of 2.3.g and 2.3.f and the second-best performing combination was that of 2.3.g, 2.3.e.r1 [4] and 2.3.b [5], i.e. averaging of neighbouring QPs, it was finally agreed within the former CE4 that the combination of test 2.3.g, 2.3.f, 2.3.e1 and 2.3.b should be recommended in the context of the former CE4. The former CE4 Subtest 2 has been continued as CE4 Subtest 1 and the combination is listed in CE4 Subtest 1 as the test 1.3.c [6]. This contribution reports the experimental results of the method described as the test 1.3.c.

# Algorithm description

For the proposed QP prediction, the proposed method employs the top-left-most CU and the top-left-most PU of each set of CUs where QP is assigned, which is referred to as quantization group of coding units in the current WD [7] and hereafter denoted as QU in this contribution QU, as shown in Figure 1

Brief algorithm of the proposed method is described as follows.

- If the top-left-most CU is intra-coded, predicted QP is calculated with the 1.3.b method [1][8], i.e. QP prediction based on intra prediction direction as shown in Figure 2.

- Otherwise, if at least either of the above CU or the left CU have the same prediction mode as that of the current CU, predicted QP is calculated with the former 2.3.f method [3], i.e. prediction mode dependent QP prediction.

- Otherwise if at least either of the above CU or the left CU is available, predicted QP is calculated with a simplified version of the former 2.3.e.r1 method [4] so that the use of above-left CU is omitted.

Detailed textual algorithm description in the form of WD text is shown in Appendix A.

Figure 1: CU, PU, neighbouring CUs and neighbouring QUs employed for QP prediction.

Figure 2: QP prediction based on intra prediction.

# Experiment

## Setup

The proposed QP prediction algorithm has been implemented onto the base software for CE4 Subtest 1 [6]. Experiments to evaluate performance of the proposed method were conducted under the CE4 Subtest 1 test conditions [6], with all applicable MinCUDQPSize values, i.e. 8x8, 16x16, 32x32 and 64x64.

The computing platform used for the experiments is shown below.

- OS: Linux 64-bit

- CPU: Intel Xeon X5670 2.93GHz, 6 physical cores x 2 CPUs (12 cores in total)

- Memory: 48GiB

## Results

Tables 1-5 show the summary of experimental results. Tables 2, 3, 4 and 5 show the summary of experimental results with MinCUDQPSize = 8x8, 16x16, 32x32 and 64x64, respectively. The brief summary of them is shown in Table 1. More detailed results are shown in the attached spreadsheets.

Note: As mentioned in the CE4 summary report, a defect was found on the software implementation submitted to CE4 at first. All results have been updated with fixing the defect.

Table 1: Brief summary results of test 1.3.c against CE4 anchor.

| MinCUDQP | BD-rates |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI HE | AI LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 | -0.46% | -0.49% | -0.30% | -0.38% | -0.36% | -0.43% |
| 16x16 | -0.23% | -0.24% | -0.19% | -0.23% | -0.24% | -0.28% |
| 32x32 | -0.09% | -0.09% | -0.08% | -0.10% | -0.12% | -0.19% |
| 64x64 | -0.03% | -0.03% | -0.05% | -0.04% | -0.06% | -0.08% |
| MinCUDQP | dQP code amount ratio |  |  |  |  |  |
|  | AI HE | AI LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 | -10.37% | -11.02% | -7.01% | -7.64% | -6.96% | -7.46% |
| 16x16 | -10.06% | -10.59% | -6.55% | -6.92% | -6.84% | -7.15% |
| 32x32 | -9.03% | -9.42% | -5.99% | -6.06% | -6.82% | -6.90% |
| 64x64 | -7.55% | -7.79% | -5.33% | -5.19% | -6.33% | -6.38% |

Table 2: Summary results of test 1.3.c against CE4 anchor with MinCUDQPSize = 8x8.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.3% | -0.3% | -6.5% | -0.2% | -0.2% | -0.2% | -6.6% |
| Class B | -0.4% | -0.4% | -0.4% | -9.8% | -0.4% | -0.4% | -0.4% | -10.2% |
| Class C | -0.5% | -0.5% | -0.4% | -11.3% | -0.6% | -0.5% | -0.5% | -12.3% |
| Class D | -0.5% | -0.4% | -0.4% | -10.8% | -0.5% | -0.5% | -0.5% | -12.0% |
| Class E | -0.8% | -0.8% | -0.8% | -14.7% | -0.8% | -0.8% | -0.8% | -15.3% |
| Overall | -0.5% | -0.4% | -0.4% | -10.4% | -0.5% | -0.5% | -0.5% | -11.0% |
|  | -0.5% | -0.4% | -0.4% |  | -0.5% | -0.5% | -0.5% |  |
| Enc Time[%] | 101% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | 0.2% | -0.3% | -4.7% | -0.3% | -0.2% | -0.3% | -4.6% |
| Class B | -0.3% | -0.3% | -0.2% | -6.5% | -0.3% | -0.3% | -0.4% | -6.9% |
| Class C | -0.4% | -0.4% | -0.5% | -9.0% | -0.5% | -0.5% | -0.5% | -10.3% |
| Class D | -0.3% | -0.2% | -0.1% | -7.9% | -0.4% | -0.5% | -0.5% | -9.0% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.2% | -0.3% | -7.0% | -0.4% | -0.4% | -0.4% | -7.6% |
|  | -0.3% | -0.2% | -0.3% |  | -0.4% | -0.4% | -0.4% |  |
| Enc Time[%] | 101% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.3% | -5.4% | -0.3% | -0.4% | -0.4% | -5.2% |
| Class C | -0.4% | -0.5% | -0.3% | -7.8% | -0.6% | -0.6% | -0.4% | -9.0% |
| Class D | -0.4% | -0.3% | -0.3% | -7.0% | -0.4% | -0.3% | -0.7% | -8.0% |
| Class E | -0.4% | -1.0% | -0.6% | -8.4% | -0.5% | -1.1% | -0.7% | -8.4% |
| Overall | -0.4% | -0.4% | -0.3% | -7.0% | -0.4% | -0.5% | -0.5% | -7.5% |
|  | -0.4% | -0.5% | -0.3% |  | -0.4% | -0.5% | -0.6% |  |
| Enc Time[%] | 101% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |

Table 3: Summary results of test 1.3.c against CE4 anchor with MinCUDQPSize = 16x16.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.1% | -5.9% | -0.1% | -0.1% | -0.1% | -5.8% |
| Class B | -0.2% | -0.2% | -0.2% | -9.7% | -0.2% | -0.2% | -0.2% | -9.9% |
| Class C | -0.2% | -0.2% | -0.2% | -10.9% | -0.3% | -0.2% | -0.2% | -12.1% |
| Class D | -0.2% | -0.1% | -0.1% | -9.6% | -0.2% | -0.2% | -0.2% | -10.7% |
| Class E | -0.5% | -0.4% | -0.5% | -15.7% | -0.5% | -0.5% | -0.5% | -16.0% |
| Overall | -0.2% | -0.2% | -0.2% | -10.1% | -0.2% | -0.2% | -0.2% | -10.6% |
|  | -0.2% | -0.2% | -0.2% |  | -0.2% | -0.2% | -0.2% |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.2% | 0.0% | -4.2% | -0.2% | -0.2% | -0.1% | -3.6% |
| Class B | -0.2% | -0.2% | -0.1% | -6.0% | -0.2% | -0.2% | -0.2% | -6.1% |
| Class C | -0.2% | 0.0% | -0.2% | -8.9% | -0.3% | -0.3% | -0.3% | -10.1% |
| Class D | -0.2% | -0.2% | -0.2% | -7.2% | -0.2% | 0.0% | -0.2% | -8.1% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.1% | -6.5% | -0.2% | -0.2% | -0.2% | -6.9% |
|  | -0.2% | -0.2% | -0.1% |  | -0.2% | -0.2% | -0.2% |  |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.4% | -0.1% | -4.9% | -0.2% | -0.3% | 0.2% | -4.7% |
| Class C | -0.3% | -0.4% | -0.5% | -8.1% | -0.4% | -0.7% | -0.4% | -9.0% |
| Class D | -0.2% | -0.9% | -0.3% | -6.8% | -0.2% | -0.7% | 0.0% | -7.4% |
| Class E | -0.3% | -1.4% | -0.4% | -8.4% | -0.3% | -0.1% | 0.2% | -8.5% |
| Overall | -0.2% | -0.7% | -0.3% | -6.8% | -0.3% | -0.4% | 0.0% | -7.2% |
|  | -0.2% | -0.7% | -0.3% |  | -0.3% | -0.4% | 0.0% |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |

Table 4: Summary results of test 1.3.c against CE4 anchor with MinCUDQPSize = 32x32.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | -0.1% | -0.1% | -5.0% | 0.0% | 0.0% | 0.0% | -4.6% |
| Class B | -0.1% | -0.1% | 0.0% | -8.6% | -0.1% | -0.1% | -0.1% | -8.5% |
| Class C | -0.1% | -0.1% | -0.1% | -9.7% | -0.1% | -0.1% | -0.1% | -10.7% |
| Class D | 0.0% | 0.0% | 0.0% | -7.6% | -0.1% | -0.1% | -0.1% | -8.9% |
| Class E | -0.2% | -0.2% | -0.2% | -16.1% | -0.2% | -0.2% | -0.2% | -16.4% |
| Overall | -0.1% | -0.1% | -0.1% | -9.0% | -0.1% | -0.1% | -0.1% | -9.4% |
|  | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | 102% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 102% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.5% | -3.6% | -0.1% | -0.1% | -0.1% | -2.9% |
| Class B | -0.1% | 0.0% | 0.0% | -5.4% | -0.1% | -0.1% | -0.1% | -5.1% |
| Class C | -0.1% | -0.1% | -0.2% | -8.6% | -0.1% | -0.1% | -0.1% | -9.2% |
| Class D | 0.0% | -0.1% | 0.2% | -6.5% | -0.1% | -0.1% | 0.0% | -7.2% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -6.0% | -0.1% | -0.1% | -0.1% | -6.1% |
|  | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | 101% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | -0.1% | -4.8% | -0.1% | -0.2% | -0.3% | -4.2% |
| Class C | -0.1% | -0.3% | -0.5% | -8.2% | -0.2% | -0.2% | -0.2% | -8.9% |
| Class D | -0.1% | -0.6% | -0.5% | -6.8% | -0.2% | 0.2% | -0.5% | -7.2% |
| Class E | -0.2% | 0.5% | -0.2% | -8.4% | -0.3% | 0.2% | -0.7% | -8.4% |
| Overall | -0.1% | -0.1% | -0.3% | -6.8% | -0.2% | 0.0% | -0.4% | -6.9% |
|  | -0.1% | -0.2% | -0.3% |  | -0.2% | 0.0% | -0.4% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |

Table 5: Summary results of test 1.3.c against CE4 anchor with MinCUDQPSize = 64x64.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | 0.0% | 0.0% | -4.9% | 0.0% | 0.0% | 0.0% | -4.7% |
| Class B | 0.0% | 0.0% | 0.0% | -7.4% | 0.0% | 0.0% | 0.0% | -7.0% |
| Class C | 0.0% | 0.0% | 0.0% | -8.5% | 0.0% | 0.0% | 0.0% | -9.3% |
| Class D | 0.0% | 0.0% | 0.0% | -4.7% | 0.0% | 0.0% | 0.0% | -4.4% |
| Class E | -0.1% | 0.0% | -0.1% | -14.0% | -0.1% | -0.1% | -0.1% | -15.7% |
| Overall | 0.0% | 0.0% | 0.0% | -7.6% | 0.0% | 0.0% | 0.0% | -7.8% |
|  | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.2% | -0.1% | -3.7% | 0.0% | 0.0% | 0.0% | -3.3% |
| Class B | -0.1% | -0.2% | -0.1% | -5.2% | 0.0% | 0.0% | 0.0% | -4.8% |
| Class C | 0.0% | -0.2% | 0.1% | -8.3% | -0.1% | 0.0% | -0.2% | -9.2% |
| Class D | 0.0% | 0.1% | -0.2% | -4.2% | 0.0% | 0.0% | -0.1% | -3.5% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | -5.3% | 0.0% | 0.0% | -0.1% | -5.2% |
|  | 0.0% | -0.1% | -0.1% |  | 0.0% | 0.0% | -0.1% |  |
| Enc Time[%] | 99% |  |  |  | 98% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.3% | -5.0% | -0.1% | -0.3% | 0.0% | -4.6% |
| Class C | -0.1% | -0.1% | 0.1% | -8.3% | -0.1% | 0.2% | -0.1% | -9.2% |
| Class D | -0.1% | 0.2% | -0.4% | -4.7% | 0.0% | -0.1% | -0.4% | -4.6% |
| Class E | -0.1% | -0.2% | 0.2% | -8.1% | -0.2% | 0.3% | 0.5% | -8.0% |
| Overall | -0.1% | 0.1% | 0.1% | -6.3% | -0.1% | 0.0% | 0.0% | -6.4% |
|  | -0.1% | 0.0% | 0.0% |  | -0.1% | 0.0% | -0.1% |  |
| Enc Time[%] | 100% |  |  |  | 98% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 98% |  |  |  |

# Conclusion

This contribution has reported the experimental results of test 1.3.c of CE4 Subtest 1, which is a spatial QP prediction method composed of a combination of test 2.3.g, 2.3.f, 2.3.e.r1 and 2.3.b of the former CE4 Subtest 2. The experimental results show that the proposed QP prediction achieves 0.5% BD-rate reduction for AI_HE and AI_LC configurations, 0.3% BD-rate reduction for RA_HE configuration, 0.4% BD-rate reduction for RA_LC, RA_LC and LB_HE configurations under the test condition defined in CE4 Subtest 1. It is recommended that the presented QP prediction method be incorporated into the next version of HM.

# References

[1] H. Aoki, et al., “CE4 Subtest 2: QP prediction based on intra prediction (test 2.3.g),” JCT-VC document JCTVC-F159, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[2] K. Sato, “CE4: Summary report of Core Experiment on quantization,” JCT-VC document JCTVC-F024, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[3] M. Kobayashi and M. Shima, “CE4 Subtest 2: Delta QP prediction results of test 2.2.b and 2.3.f,” JCT-VC document JCTVC-F300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[4] K. Sato, “CE4: Result of 2.3.d and 2.3.e,” JCT-VC document JCTVC-F420, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[5] M. Coban and M. Karczewicz, “CE4 Subtest 2: QP prediction from spatially neighboring CUs (test 2.3.b, 2.3.c),” JCT-VC document JCTVC-F332, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[6] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-F904, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[7] B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC document JCTVC-F803, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[8] H. Aoki, et al., “CE4 Subtest 1: Spatial QP prediction based on intra prediction (test 1.3.b),” JCT-VC document JCTVC-G066, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.

[9] K. Sato, et al., “CE4: Quantization,” JCT-VC document JCTVC-G034, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, July 2011.

# Patent rights declaration(s)

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 4

The modified or added portions are highlighted with yellow colour.

### A.1. Change of subclause 7.4.9 “Transform coefficient semantics”

cu_qp_delta can change the value of QPY specifies the difference between a luma quantisation parameter to be used and its prediction, for a quantisation group of coding units where the quantisation group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantisation group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantisation group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantisation group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

### A.2. Insertion of new subclause into clause 8.5

### 8.5.x	Derivation process for luma quantisation parameters

Inputs to this process are

–	a variable QpBdOffsetY,

–	a syntax element cu_qp_delta specifying the delta luma quantisation parameter of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Outputs of this process are

–	luma quantisation parameters QPY and QP’Y.

Depending on PredMode, the following applies.

–	If PredMode is equal to MODE_INTRA, quantisation parameter prediction process as specified in subclause 8.5.x.2 is invoked with the output being the reference quantisation parameter QPY, PRED.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED. .

With QPY,PRED, the value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-3)

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-4)

### 8.5.x.1		Default quantisation parameter prediction process

Inputs to this process are

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If all of the following conditions are true, then QPY, PRED is derived as follows

- The above neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the bottom-left coding unit of the above neighbouring quantisation group of coding units is the same as PredMode.

- The left neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the top-right coding unit of the left neighbouring quantisation group of coding units is the same as PredMode.

QPY,PRED = ( ( ( QPY,ABOVE + QPY,LEFT + 1 ) / 2               	(8-53)

where QPY,ABOVE is luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice, and QPY,LEFT is luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the above neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the bottom-left coding unit of the above neighbouring quantisation group of coding units is the same as PredMode, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the left neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the top-right coding unit of the left neighbouring quantisation group of coding units is the same as PredMode, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if all of the following conditions are true, then QPY, PRED is derived as defined in the equation 8-xx3.

- The above neighbouring quantisation group of coding units in the current slice is available.

- The left neighbouring quantisation group of coding units in the current slice is available.

–	Otherwise, if the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, QPY, PRED is set equal to QPY, PREV, where QPY,PREV is the luma quantisation parameter of the previous quantisation group of coding units in decoding order in the current slice. For the first quantisation group of coding units in the slice, QPY,PREV is initially set equal to SliceQPY at the start of each slice.

### 8.5.x.2		Intra quantisation parameter prediction process

Inputs to this process are

–	a syntax element pcm_flag specifying if the top-left coding unit of the current quantisation group of coding units is coded in PCM mode

–	a variable intraPredMode specifying the intra prediction mode of the top-left prediction unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The rearranged intra prediction order intraPredOrder is derived as follows.

–	If one of the following conditions is true, intraPredOrder is set to 0.

- pcm_flag is equal to 1.

- intraPredMode is equal to 0 (Intra_Planar) or 3 (Intra_DC).

–	Otherwise, if intraPredMode is equal to 1 (Intra_Vertical), intraPredOrder is set to 9.

–	Otherwise, if intraPredMode is equal to 2 (Intra_Horizontal), intraPredOrder is set to 25.

–	Otherwise, intraPredOrder is derived as specified in Table 8-7.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If intraPredOrder is greater than 0 and less than 18, and the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if intraPredOrder is greater than or equal to 18, and the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED.