| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G068 |
| --- | --- |

| Title: | CE4 Subtest 1: QP prediction based on intra/inter prediction (test 1.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN Muhammed Coban Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com +1 858 658 3973 mcoban@qualcomm.com martak@qualcomm.com |
| Source: | NEC Corporation, Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents the detailed description and the results of a QP prediction method based on intra/inter prediction (test 1.4) for CE4 subtest 1. The method predicts QP of the current block from spatially/temporally neighbouring blocks directed by its intra/inter prediction. For temporal prediction, QP values are stored in units of 16x16 blocks whereas QPs can be signalled at 8x8-block level. It is reportedly shown that QP prediction based on intra/inter prediction provides BD-rate gains by 0.4%, 0.4%, 0.5%, 0.6%, 0.7% and 1.0% for AI-HE, AI-LC, RA-HE, RA-LC, LB-HE and LB-LC configurations, respectively. Gains relative to QP prediction only with intra prediction (1.3.b) are 0.3%, 0.4%, 0.7%, 0.9% for RA-HE, RA-LC, LB-HE and LB-LC configurations, respectively.

# Introduction

The delta QP signalling at sub-LCU level was adopted at the 5th JCT-VC meeting, while there were several contributions presented to introduce alternative QP coding methods. CE4 Subtest 2 was therefore established at the 6th JCT-VC meeting for evaluating the performance of various QP coding methods and it has been continued as CE4 Subtest 1 at this meeting. In this contribution, QP prediction based on intra/inter prediction, originally proposed in JCTVC-F103 [1] and extended with a QP buffer compression technique proposed in JCTVC-F499 [2], is tested.

# Algorithm description

The concept of the proposed QP prediction can be simply described as follows.

If a subimage PD is well-predictable from another subimage PS, the QP variation applied for the subimage PD should be the same as that applied for the subimage PS,

where QP variation means QP offset value derived by some perceptual QP adaptation algorithm like MPEG-2 Test Model 5 Step 3 or its variants [3, 4].

The proposed method leverages information both of intra prediction and inter prediction to derive PS. Prediction is performed on a PU basis and QP is assigned to a set of CUs, which is referred to as quantization group of coding units in the current WD [5] and hereafter denoted as QU in this contribution. Therefore, it should be defined which PU is employed for QP prediction. For simplification, the proposed method employs the top-left-most PU (TLPU) of each QU for QP prediction as shown in Figure 1.

Figure 1: CU, PU and neighbouring QUs employed for the proposed QP prediction.

The proposed QP prediction switches between QP prediction based on intra prediction (spatial QP prediction) and that based on inter prediction (temporal QP prediction) according to prediction mode of the TLPU. If a TLPU is intra-predicted, spatial QP prediction shown in Figure 2 is employed for the QU. Otherwise, temporal QP prediction shown in Figure 3 is employed. Detailed algorithm description of spatial QP prediction is described in JCTVC-G066 [6] and therefore omitted here. In this contribution, we focus on temporal QP prediction.

First let us suppose the unidirectional prediction in order to simplify the explanation. Let xTL be the luma location of the top-left luma sample of the current QU relative to the top-left luma sample of the current picture, WTL be the width of the current TLPU, HTL be the height of the current TLPU, vTL be the motion vector of the current TLPU, and FRef be the reference frame indicated by the reference index of the current TLPU. The reference luma location xRef on the frame FRef is derived as follows.

xRef = xTL + (WTL/2, HTL/2) + vTL

This luma location specifies the center of the reference block for motion compensation of the current TLPU. For the proposed inter QP prediction, the QU covering the luma location (xQRef, yQRef) is employed as the reference QU. Let Q(F, x) be the luma quantization parameter of the QU covering a luma location x of the frame F, QSlice(F, x) be the value of SliceQPY of the slice covering a luma location x of the frame F and FCurr be the current frame. The predicted QP value QPred of the current QU is derived as follows:

QPred = Qave(FRef, xRef) + QSlice(FCurr, xTL) – QSlice(FRef, xRef),

where Qave(FRef, xRef) represents average values of Q(FRef, x) for all x within the QP storage unit covering the luma location xRef.

The offset term by slice QPs accounts for the inter-frame QP adaptation for rate control, quality control, etc. The size of temporal QP storage units is represented by MinCUTQPSizeLX, which is equal to or larger than the size of QP signalling units represented by MinCUDQPSize. MinCUTQPSizeLX can be signalled in the PPS header or set to a prefixed value.

When the bidirectional prediction is used for the TLPU, the predicted QP value QPred of the current QU is achieved simply by averaging two predicted QP values as follows.

QPred = (QPredL0 + QPredL1 + 1) / 2

Modification of semantics for derivation of quantization parameter of the current WD is described in Appendix A.

Figure 2: QP prediction based on intra prediction.

Figure 3: QP prediction based on inter prediction.

# Experiments

## Setup

The proposed QP prediction algorithm has been implemented onto the base software for CE4 Subtest 1 [7]. Experiments to evaluate performance of the proposed method were conducted under the CE4 Subtest 1 test conditions [7], with all applicable MinCUDQPSize values, i.e. 8x8, 16x16, 32x32 and 64x64. MinCUTQPSizeLX is set to 16x16 in this contribution.

Our computing platform used for the experiments is shown below.

- OS: Windows 7 Professional 64-bit

- CPU: Intel Xeon X5680 3.46GHz, 6 physical cores x 2 CPUs (12 cores in total)

- Memory: 32GiB

It should be noted that individual decoding/encoding time can be significantly varied since the CPU have automated over-clocking functionality and I/O speed varies by 12 processes running in parallel.

## Results

BD-Rates and processing time ratios against CE4 anchor were computed to evaluate total gains by intra and inter QP prediction combined with QP buffer compression scheme. Tables 1, 3-6 show the summary of experimental results. Tables 3, 4, 5 and 6 show the summary of experimental results with MinCUDQPSize = 8x8, 16x16, 32x32 and 64x64, respectively. The brief summary of them is shown in Table 1. More detailed results are shown in the attached spreadsheets.

In addition, to evaluate gains achieved by introducing temporal QP prediction compared with the test 1.3.b employing spatial QP prediction based on intra QP prediction, BD-Rates and processing time ratios against test 1.3.b were computed. Tables 2, 7-10 show the summary of experimental results. Tables 7, 8, 9 and 10 show the summary of experimental results with MinCUDQPSize = 8x8, 16x16, 32x32 and 64x64, respectively. The brief summary of them is shown in Table 2. More detailed results are shown in the attached spreadsheets. Note that results for all intra configurations are omitted in Tables 6-10 since there is no difference between test 1.3.b and test 1.4 in those configurations.

These results are summarized as follows.

- QP prediction based on inter prediction definitely improves coding efficiency in all configurations using inter prediction. Additional complexity introduced by employing inter prediction is negligible.

- When QP is signalled at 8x8-CU level, its coding gain over spatial QP prediction based on intra prediction is from 0.33 % (for RA-HE) to 0.88 % (for LB-LC). Even when QP is signalled at 64x64-CU level, coding gain is observed from 0.16 % (for RA-HE) to 0.54 % (for LB-HE) and the code amount of delta QPs is significantly reduced by up to 53.84 %.

- Total coding gain of QP prediction based on intra and inter sample prediction over the WD4 anchor is from 0.38 % (for AI-LC) to 0.98 % (for LB-LC) when QP is signalled at 8x8-CU level. Gains can be achieved even when QP is signalled at 64x64-CU level.

Table 1: Brief summary results of test 1.4 against CE4 anchor.

| MinCUDQP | Y BD-rates |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI HE | AI LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 | -0.38% | -0.42% | -0.46% | -0.55% | -0.74% | -0.98% |
| 16x16 | -0.18% | -0.21% | -0.41% | -0.48% | -0.75% | -0.96% |
| 32x32 | -0.06% | -0.07% | -0.27% | -0.31% | -0.71% | -0.87% |
| 64x64 | -0.02% | -0.02% | -0.17% | -0.19% | -0.50% | -0.57% |
| MinCUDQP | dQP code amount |  |  |  |  |  |
|  | AI HE | AI LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 | -8.41% | -9.38% | -13.46% | -15.21% | -17.05% | -19.66% |
| 16x16 | -7.94% | -8.86% | -19.46% | -22.13% | -25.07% | -28.52% |
| 32x32 | -6.39% | -7.25% | -26.24% | -29.04% | -37.93% | -40.43% |
| 64x64 | -4.45% | -5.11% | -35.41% | -37.70% | -53.86% | -54.12% |

Table 2: Brief summary results of test 1.4 against 1.3.b.

| MinCUDQP | Y BD-rates |  |  |  | dQP code amount |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HE | RA LC | LB HE | LB LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 | -0.33% | -0.35% | -0.69% | -0.88% | -10.47% | -11.58% | -15.87% | -18.13% |
| 16x16 | -0.34% | -0.39% | -0.73% | -0.94% | -17.39% | -19.59% | -24.30% | -27.50% |
| 32x32 | -0.26% | -0.28% | -0.73% | -0.82% | -25.13% | -27.67% | -37.47% | -39.88% |
| 64x64 | -0.16% | -0.19% | -0.47% | -0.54% | -34.91% | -37.11% | -53.69% | -53.84% |

Table 3: Summary results of test 1.4 against CE4 anchor with MinCUDQPSize = 8x8.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.1% | -0.2% | -4.7% | -0.2% | -0.2% | -0.2% | -5.3% |
| Class B | -0.3% | -0.3% | -0.3% | -7.7% | -0.3% | -0.3% | -0.3% | -8.6% |
| Class C | -0.4% | -0.4% | -0.4% | -9.1% | -0.5% | -0.4% | -0.4% | -10.4% |
| Class D | -0.3% | -0.3% | -0.3% | -8.4% | -0.4% | -0.4% | -0.4% | -9.8% |
| Class E | -0.7% | -0.7% | -0.7% | -13.5% | -0.8% | -0.7% | -0.8% | -14.3% |
| Overall | -0.4% | -0.3% | -0.4% | -8.4% | -0.4% | -0.4% | -0.4% | -9.4% |
|  | -0.4% | -0.3% | -0.4% |  | -0.4% | -0.4% | -0.4% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.3% | -0.5% | -0.6% | -11.7% | -0.3% | -0.5% | -0.7% | -12.5% |
| Class B | -0.4% | -0.5% | -0.5% | -13.2% | -0.5% | -0.6% | -0.6% | -14.7% |
| Class C | -0.6% | -0.5% | -0.7% | -14.6% | -0.7% | -0.9% | -0.9% | -17.1% |
| Class D | -0.5% | -0.4% | -0.4% | -14.4% | -0.7% | -0.8% | -0.7% | -16.6% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.5% | -0.5% | -0.6% | -13.5% | -0.6% | -0.7% | -0.7% | -15.2% |
|  | -0.5% | -0.5% | -0.5% |  | -0.6% | -0.7% | -0.7% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.7% | -0.8% | -15.6% | -0.7% | -0.8% | -0.9% | -17.6% |
| Class C | -0.8% | -0.9% | -0.7% | -16.1% | -1.0% | -1.0% | -1.0% | -19.0% |
| Class D | -0.7% | -1.2% | -0.6% | -16.7% | -1.0% | -1.1% | -1.1% | -19.6% |
| Class E | -0.9% | -0.8% | -1.0% | -21.1% | -1.3% | -1.9% | -1.6% | -24.1% |
| Overall | -0.7% | -0.9% | -0.8% | -17.0% | -1.0% | -1.2% | -1.1% | -19.7% |
|  | -0.7% | -0.9% | -0.8% |  | -1.0% | -1.2% | -1.1% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |

Table 4: Summary results of test 1.4 against CE4 anchor with MinCUDQPSize = 16x16.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.1% | -4.0% | -0.1% | -0.1% | -0.1% | -4.5% |
| Class B | -0.2% | -0.1% | -0.1% | -7.5% | -0.2% | -0.2% | -0.2% | -8.3% |
| Class C | -0.2% | -0.2% | -0.1% | -8.4% | -0.2% | -0.2% | -0.2% | -9.8% |
| Class D | -0.1% | -0.1% | -0.1% | -6.9% | -0.2% | -0.1% | -0.1% | -8.3% |
| Class E | -0.5% | -0.4% | -0.5% | -14.6% | -0.5% | -0.5% | -0.4% | -15.1% |
| Overall | -0.2% | -0.2% | -0.2% | -7.9% | -0.2% | -0.2% | -0.2% | -8.9% |
|  | -0.2% | -0.2% | -0.2% |  | -0.2% | -0.2% | -0.2% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 97% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.4% | -0.3% | -0.3% | -15.1% | -0.3% | -0.4% | -0.4% | -16.7% |
| Class B | -0.4% | -0.4% | -0.4% | -18.2% | -0.4% | -0.5% | -0.5% | -20.0% |
| Class C | -0.4% | -0.4% | -0.4% | -20.9% | -0.6% | -0.6% | -0.6% | -24.5% |
| Class D | -0.4% | -0.5% | -0.5% | -24.0% | -0.6% | -0.7% | -0.7% | -27.9% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.4% | -0.4% | -0.4% | -19.5% | -0.5% | -0.5% | -0.6% | -22.1% |
|  | -0.4% | -0.4% | -0.4% |  | -0.5% | -0.5% | -0.6% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.7% | -0.8% | -0.5% | -21.7% | -0.7% | -0.9% | -0.6% | -24.0% |
| Class C | -0.7% | -0.9% | -0.9% | -23.5% | -0.9% | -1.1% | -1.1% | -27.3% |
| Class D | -0.7% | -0.9% | -1.0% | -28.9% | -1.0% | -1.2% | -1.0% | -33.3% |
| Class E | -1.0% | -2.4% | -1.1% | -27.7% | -1.3% | -1.9% | -1.1% | -31.4% |
| Overall | -0.7% | -1.2% | -0.8% | -25.1% | -1.0% | -1.2% | -1.0% | -28.5% |
|  | -0.7% | -1.2% | -0.9% |  | -1.0% | -1.2% | -0.9% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |

Table 5: Summary results of test 1.4 against CE4 anchor with MinCUDQPSize = 32x32.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | 0.0% | 0.0% | -2.5% | 0.0% | 0.0% | 0.0% | -2.9% |
| Class B | -0.1% | -0.1% | 0.0% | -5.9% | -0.1% | -0.1% | -0.1% | -6.5% |
| Class C | 0.0% | -0.1% | 0.0% | -6.6% | -0.1% | 0.0% | 0.0% | -7.8% |
| Class D | 0.0% | 0.0% | 0.0% | -4.6% | 0.0% | 0.0% | 0.0% | -6.0% |
| Class E | -0.2% | -0.1% | -0.2% | -14.5% | -0.2% | -0.2% | -0.2% | -15.3% |
| Overall | -0.1% | -0.1% | 0.0% | -6.4% | -0.1% | -0.1% | -0.1% | -7.2% |
|  | -0.1% | 0.0% | 0.0% |  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 94% |  |  |  | 102% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.3% | -0.1% | -0.2% | -20.3% | -0.3% | -0.4% | -0.3% | -22.6% |
| Class B | -0.3% | -0.3% | -0.3% | -24.5% | -0.3% | -0.3% | -0.4% | -26.1% |
| Class C | -0.3% | -0.3% | -0.4% | -27.0% | -0.3% | -0.4% | -0.4% | -30.6% |
| Class D | -0.3% | -0.3% | -0.1% | -33.5% | -0.4% | -0.4% | -0.2% | -37.5% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.2% | -0.3% | -26.2% | -0.3% | -0.4% | -0.3% | -29.0% |
|  | -0.3% | -0.2% | -0.3% |  | -0.3% | -0.4% | -0.3% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 98% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.6% | -0.6% | -32.6% | -0.7% | -0.8% | -0.8% | -34.0% |
| Class C | -0.6% | -0.7% | -0.6% | -33.7% | -0.7% | -0.8% | -0.7% | -37.1% |
| Class D | -0.6% | -0.5% | -1.3% | -43.4% | -0.8% | -0.6% | -0.9% | -46.7% |
| Class E | -1.2% | -1.2% | -1.0% | -45.1% | -1.5% | -1.4% | -2.0% | -47.4% |
| Overall | -0.7% | -0.7% | -0.8% | -37.9% | -0.9% | -0.9% | -1.0% | -40.4% |
|  | -0.7% | -0.8% | -0.8% |  | -0.9% | -0.9% | -1.0% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  |

Table 6: Summary results of test 1.4 against CE4 anchor with MinCUDQPSize = 64x64.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | 0.0% | 0.0% | -1.4% | 0.0% | 0.0% | 0.0% | -1.8% |
| Class B | 0.0% | 0.0% | 0.0% | -3.9% | 0.0% | 0.0% | 0.0% | -4.4% |
| Class C | 0.0% | 0.0% | 0.0% | -5.0% | 0.0% | 0.0% | 0.0% | -5.9% |
| Class D | 0.0% | 0.0% | 0.0% | -1.5% | 0.0% | 0.0% | 0.0% | -1.5% |
| Class E | -0.1% | 0.0% | -0.1% | -12.6% | -0.1% | -0.1% | -0.1% | -14.7% |
| Overall | 0.0% | 0.0% | 0.0% | -4.4% | 0.0% | 0.0% | 0.0% | -5.1% |
|  | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 102% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.4% | -0.4% | -30.2% | -0.2% | -0.2% | -0.3% | -32.1% |
| Class B | -0.2% | -0.2% | -0.2% | -34.2% | -0.2% | -0.2% | -0.2% | -35.6% |
| Class C | -0.1% | -0.3% | -0.1% | -34.6% | -0.2% | -0.2% | -0.2% | -37.9% |
| Class D | -0.2% | -0.1% | -0.3% | -43.0% | -0.2% | -0.1% | -0.2% | -45.6% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.3% | -35.4% | -0.2% | -0.2% | -0.2% | -37.7% |
|  | -0.2% | -0.3% | -0.2% |  | -0.2% | -0.2% | -0.2% |  |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 98% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4% | -0.3% | -0.2% | -49.5% | -0.5% | -0.5% | -0.5% | -48.8% |
| Class C | -0.3% | -0.4% | -0.2% | -45.1% | -0.4% | -0.3% | -0.5% | -47.4% |
| Class D | -0.3% | -0.7% | -0.3% | -55.2% | -0.4% | 0.0% | -0.8% | -57.1% |
| Class E | -1.1% | -0.8% | -2.0% | -71.0% | -1.2% | -1.1% | -0.5% | -67.9% |
| Overall | -0.5% | -0.5% | -0.6% | -53.9% | -0.6% | -0.4% | -0.6% | -54.1% |
|  | -0.5% | -0.5% | -0.6% |  | -0.6% | -0.5% | -0.6% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |

Table 7: Summary results of test 1.4 against 1.3.b with MinCUDQPSize = 8x8.

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.3% | -0.3% | -0.1% | -9.9% | -0.2% | -0.4% | -0.4% | -10.5% |
| Class B | -0.3% | -0.4% | -0.4% | -10.0% | -0.3% | -0.4% | -0.4% | -10.9% |
| Class C | -0.4% | -0.4% | -0.5% | -10.7% | -0.4% | -0.6% | -0.5% | -12.3% |
| Class D | -0.4% | -0.4% | -0.5% | -11.3% | -0.5% | -0.5% | -0.6% | -12.8% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.4% | -0.4% | -10.5% | -0.4% | -0.5% | -0.5% | -11.6% |
|  | -0.3% | -0.4% | -0.4% |  | -0.4% | -0.5% | -0.5% |  |
| Enc Time[%] | 101% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.7% | -0.5% | -14.5% | -0.6% | -0.8% | -0.6% | -16.0% |
| Class C | -0.7% | -0.6% | -0.6% | -14.5% | -0.8% | -0.9% | -0.9% | -16.9% |
| Class D | -0.7% | -0.5% | -0.6% | -15.5% | -0.9% | -1.0% | -0.6% | -18.0% |
| Class E | -1.0% | -1.1% | -1.0% | -20.4% | -1.3% | -1.2% | -1.1% | -23.5% |
| Overall | -0.7% | -0.7% | -0.7% | -15.9% | -0.9% | -0.9% | -0.8% | -18.1% |
|  | -0.7% | -0.7% | -0.6% |  | -0.9% | -1.0% | -0.8% |  |
| Enc Time[%] | 101% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  |

Table 8: Summary results of test 1.4 against 1.3.b with MinCUDQPSize = 16x16.

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.3% | -0.4% | -0.3% | -13.9% | -0.2% | -0.3% | -0.3% | -15.1% |
| Class B | -0.3% | -0.4% | -0.4% | -15.8% | -0.3% | -0.4% | -0.4% | -17.1% |
| Class C | -0.4% | -0.4% | -0.5% | -18.1% | -0.5% | -0.6% | -0.5% | -21.1% |
| Class D | -0.4% | -0.4% | -0.4% | -22.2% | -0.6% | -0.6% | -0.6% | -25.6% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.4% | -0.4% | -17.4% | -0.4% | -0.5% | -0.5% | -19.6% |
|  | -0.3% | -0.4% | -0.4% |  | -0.4% | -0.5% | -0.5% |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.7% | -0.8% | -20.9% | -0.7% | -0.8% | -0.8% | -22.9% |
| Class C | -0.6% | -0.9% | -0.5% | -22.4% | -0.9% | -0.9% | -1.2% | -25.8% |
| Class D | -0.7% | -0.8% | -0.8% | -28.3% | -1.0% | -1.0% | -1.1% | -32.4% |
| Class E | -1.1% | -0.7% | -0.2% | -27.2% | -1.4% | -1.6% | -1.1% | -30.9% |
| Overall | -0.7% | -0.8% | -0.6% | -24.3% | -0.9% | -1.0% | -1.0% | -27.5% |
|  | -0.7% | -0.8% | -0.6% |  | -0.9% | -1.0% | -1.0% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |

Table 9: Summary results of test 1.4 against 1.3.b with MinCUDQPSize = 32x32.

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.2% | -0.3% | -19.6% | -0.2% | -0.2% | -0.3% | -21.8% |
| Class B | -0.2% | -0.3% | -0.3% | -23.0% | -0.2% | -0.3% | -0.4% | -24.4% |
| Class C | -0.2% | -0.2% | -0.3% | -25.5% | -0.3% | -0.3% | -0.3% | -28.9% |
| Class D | -0.3% | -0.3% | -0.1% | -32.9% | -0.3% | -0.3% | -0.2% | -36.4% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.2% | -25.1% | -0.3% | -0.3% | -0.3% | -27.7% |
|  | -0.3% | -0.3% | -0.2% |  | -0.3% | -0.3% | -0.3% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.7% | -0.8% | -32.1% | -0.6% | -0.6% | -0.5% | -33.2% |
| Class C | -0.6% | -0.7% | -0.6% | -33.1% | -0.7% | -0.6% | -0.7% | -36.3% |
| Class D | -0.6% | -0.4% | -1.2% | -42.9% | -0.8% | -0.6% | -0.7% | -46.2% |
| Class E | -1.3% | -1.0% | -1.4% | -44.9% | -1.4% | -2.2% | -1.8% | -47.2% |
| Overall | -0.7% | -0.7% | -0.9% | -37.5% | -0.8% | -0.9% | -0.9% | -39.9% |
|  | -0.7% | -0.7% | -0.9% |  | -0.8% | -0.9% | -0.8% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |

Table 10: Summary results of test 1.4 against 1.3.b with MinCUDQPSize = 64x64.

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.2% | -0.3% | -29.8% | -0.2% | -0.1% | -0.3% | -31.6% |
| Class B | -0.2% | -0.1% | -0.3% | -33.5% | -0.2% | -0.2% | -0.2% | -34.9% |
| Class C | -0.1% | -0.2% | -0.2% | -34.0% | -0.2% | -0.2% | -0.2% | -37.1% |
| Class D | -0.2% | -0.1% | -0.2% | -42.8% | -0.2% | -0.1% | -0.2% | -45.4% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.3% | -34.9% | -0.2% | -0.2% | -0.2% | -37.1% |
|  | -0.2% | -0.2% | -0.3% |  | -0.2% | -0.2% | -0.2% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4% | -0.6% | -0.6% | -49.3% | -0.5% | -0.2% | -0.4% | -48.5% |
| Class C | -0.3% | -0.2% | -0.2% | -44.7% | -0.4% | -0.3% | -0.5% | -47.0% |
| Class D | -0.3% | -0.6% | -0.3% | -55.2% | -0.4% | -0.1% | -0.8% | -56.9% |
| Class E | -1.0% | -1.2% | -1.6% | -70.9% | -1.0% | -1.2% | -0.7% | -67.7% |
| Overall | -0.5% | -0.6% | -0.6% | -53.7% | -0.5% | -0.4% | -0.6% | -53.8% |
|  | -0.5% | -0.6% | -0.7% |  | -0.5% | -0.4% | -0.6% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |

## Memory requirement

Temporal QP prediction (TQPP) introduces additional buffer for storing QPs of reference frames for temporal prediction. Here we show memory requirement of TQPP. According to the current WD specification [5], six bits are needed for storing a QP since QP is defined as being in the range of 0 to 51, inclusive. The size of QP buffer BTQP represented in bits per a reference frame is therefore calculated as follows.

PicWidthInMinCUTQPs = (PicWidthInSamplesL + MinCUTQPSizeLX-1) / MinCUTQPSizeLX

PicHeightInMinCUTQPs = (PicHeightInSamplesL + MinCUTQPSizeLX-1) / MinCUTQPSizeLX

BTQP = PicWidthInMinCUTQPs * PicHeightInMinCUTQPs * 6

Considering QP buffer memory may be byte-aligned, another definition of QP buffer size BTQP, aligned is given as follows.

BTQP, aligned = PicWidthInMinCUTQPs * PicHeightInMinCUTQPs * 8

Tables 11 and 12 show typical values of BTQP and BTQP, aligned, respectively, for sequences used in common test conditions [8]. The column highlighted in boldface type shows the values under the test condition in this contribution.

Table 11: Typical size of unaligned QP buffer.

|  | MinCUTQPSizeLX |  |  |  |
| --- | --- | --- | --- | --- |
|  | 8x8 | 16x16 | 32x32 | 64x64 |
| Class A (2560x1600) | 384000 bits | 96000 bits | 24000 bits | 6000 bits |
| Class B (1920x1080) | 194400 bits | 48960 bits | 12240 bits | 3060 bits |
| Class C (832x480) | 37440 bits | 9360 bits | 2340 bits | 624 bits |
| Class D (416x240) | 9360 bits | 2340 bits | 624 bits | 168 bits |
| Class E (1280x720) | 86400 bits | 21600 bits | 5520 bits | 1440 bits |

Table 12: Typical size of byte-aligned QP buffer.

|  | MinCUTQPSizeLX |  |  |  |
| --- | --- | --- | --- | --- |
|  | 8x8 | 16x16 | 32x32 | 64x64 |
| Class A (2560x1600) | 512000 bits | 128000 bits | 32000 bits | 8000 bits |
| Class B (1920x1080) | 259200 bits | 65280 bits | 16320 bits | 4080 bits |
| Class C (832x480) | 49920 bits | 12480 bits | 3120 bits | 832 bits |
| Class D (416x240) | 12480 bits | 3120 bits | 832 bits | 224 bits |
| Class E (1280x720) | 115200 bits | 28800 bits | 7360 bits | 1920 bits |

# Remarks

While experimental results have shown remarkable performance of temporal QP prediction, there may be some doubts whether temporal QP prediction should be effective in any cases. In real applications, some rate control would be applied since generated bitstreams must conform to some constraint imposed on transmission channel. Although the proposed temporal QP prediction introduces offset by Slice QP and it provides some adaptability to rate control, there may be cases where more strict rate control are needed and QP for each CU significantly varies within a slice. For example, at early frames of the whole sequence to be encoded, rate control tends to be unstable and thereby CU QP can be unstable. Supposing such cases, switching mechanism for enabling/disabling temporal QP prediction is better to be provided.

We therefore suggest introducing a flag to control availability of temporal QP prediction, for example, into PPS. The proposed modification of the WD4 text described in Appendix A includes the flag, namely temporal_qp_prediction_enabled_flag. Since the current HM has no rate control mechanism and the QP assignment in the common test conditions is perfectly stable, no additional experimental results are given here. It should be noted that fixing temporal_qp_prediction_enabled_flag equal to 1 corresponds to the method described in this contribution (test 1.4) and fixing temporal_qp_prediction_enabled_flag equal to 0 conrresponds to the method described in JCTVC-G066 (test 1.3.b) [6].

# Conclusions

This contribution has presented the detailed description and the results of a QP prediction method based on intra/inter prediction (test 1.4) for CE4 subtest 1. The method predicts QP of the current block from spatially/temporally neighbouring blocks directed by its intra/inter prediction. For temporal prediction, QP values are stored in units of 16x16 blocks whereas QPs can be signalled at 8x8-block level. It is reportedly shown that QP prediction based on intra/inter prediction provides BD-rate gains by 0.4%, 0.4%, 0.5%, 0.6%, 0.7% and 1.0% for AI-HE, AI-LC, RA-HE, RA-LC, LB-HE and LB-LC configurations, respectively. Gains relative to QP prediction only with intra prediction (1.3.b) are 0.3%, 0.4%, 0.7% and 0.9% for RA-HE, RA-LC, LB-HE and LB-LC configurations, respectively. It is recommended to adopt the proposed QP prediction based on intra/inter prediction into HM.

# References

[1] H. Aoki, et al., “CE4 Subtest2: QP prediction based on intra/inter prediction (test 2.4.b),” JCT-VC document JCTVC-F103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[2] M. Coban, et al., “Temporal QP memory compression,” JCT-VC document JCTVC-F499, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[3] H. Aoki, et al., “Prediction-based QP derivation,” JCT-VC document JCTVC-E215, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

[4] K. Sato, et al., “On LBS and Quantization,” JCT-VC document JCTVC-D308, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 4th meeting, Daegu, January 2011.

[5] B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC document JCTVC-F803, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[6] H. Aoki, et al., “CE4 Subtest 1: Spatial QP prediction based on intra prediction (test 1.3.b),” JCT-VC document JCTVC-G066, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.

[7] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-F904, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[8] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 4

The modified or added portions are highlighted with yellow or green colour.

Note: This modification includes semantics modification by spatial QP prediction based on intra prediction, which is described in JCTVC-G066 Error: Reference source not found. The portions described in Error: Reference source not found are highlighted with yellow colour and the portions introduced in this contribution are highlighted with green colour.

### A.1. Change of subclause 7.3.2.2 “Picture parameter set RBSP syntax”

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| max_cu_qp_delta_depth | u(4) |
| temporal_qp_prediction_enabled_flag | u(1) |
| if( temporal_qp_prediction_enabled_flag) { |  |
| max_cu_temporal_qp_depth | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### A.2. Change of subclause 7.4.2.2 “Picture parameter set RBSP semantics”

temporal_qp_prediction_enabled_flag equal to 1 specifies that temporal QP prediction is enabled in derivation process for quantisation parameters. temporal_qp_prediction_enabled_flag equal to 0 specifies that temporal QP prediction is disabled in derivation process for quantisation parameters. When temporal_qp_prediction_enabled_flag is not present, temporal_qp_prediction_enabled_flag shall be inferred to be equal to 0.

max_cu_temporal_qp_depth specifies the maximum allowed depth that is used for specifying temporal QPY values for coding unit. The value of max_cu_temporal_qp_depth shall be in the range of 0 to max_cu_qp_delta_depth, inclusive.

The variable Log2MinCUTQPSize specifying the minimum coding unit size that specifies the temporal QPY values for coding unit as follows:

Log2MinCUTQPSize = Log2MaxCUSize – max_cu_temporal_qp_depth

The variable MinCUTQPSize is set equal to (1 << Log2MinCUTQPSize).

### A.3. Change of subclause 7.4.9 “Transform coefficient semantics”

cu_qp_delta can change the value of QPY specifies the difference between a luma quantisation parameter to be used and its prediction, for a quantisation group of coding units where the quantisation group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantisation group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantisation group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantisation group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

### A.4. Insertion of new subclause into clause 8.5

### 8.5.x	Derivation process for luma quantisation parameters

Inputs to this process are

–	a variable QpBdOffsetY,

–	a syntax element cu_qp_delta specifying the delta luma quantisation parameter of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Outputs of this process are

–	luma quantisation parameters QPY and QP’Y.

Depending on PredMode, the following applies.

–	If PredMode is equal to MODE_INTRA, spatial quantisation parameter prediction process as specified in subclause 8.5.x.2 is invoked with the output being the reference quantisation parameter QPY, PRED.

–	Otherwise, if temporal_qp_prediction_enabled_flag is equal to 1, temporal quantisation parameter prediction process as specified in subclause 8.5.x.3 is invoked with the output being the predicted quantisation parameter QPY, PRED.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the predicted quantisation parameter QPY, PRED.

With QPY,PRED, the value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-xx1)

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-xx2)

### 8.5.x.1		Default quantisation parameter prediction process

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, QPY, PRED is set equal to QPY, PREV, where QPY,PREV is the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice. For the first quantisation group of coding units in the slice, QPY,PREV is initially set equal to SliceQPY at the start of each slice.

### 8.5.x.2		Spatial quantisation parameter prediction process

Inputs to this process are

–	a syntax element pcm_flag specifying if top-left coding unit of the current quantisation group of coding units is coded in PCM mode.

–	a variable intraPredMode specifying the intra prediction mode of the top-left prediction unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The rearranged intra prediction order intraPredOrder is derived as follows.

–	If one of the following conditions is true, intraPredOrder is set to 0.

- pcm_flag is equal to 1.

- intraPredMode is equal to 0 (Intra_Planar) or 3 (Intra_DC).

–	Otherwise, if intraPredMode is equal to 1 (Intra_Vertical), intraPredOrder is set to 9.

–	Otherwise, if intraPredMode is equal to 2 (Intra_Horizontal), intraPredOrder is set to 25.

–	Otherwise, intraPredOrder is derived as specified in Table 8-7.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If intraPredOrder is greater than 0 and less than 18 and the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter QPY of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if intraPredOrder is greater than or equal to 18 and the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter QPY of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED.

### 8.5.x.3		Temporal quantisation parameter prediction process

Inputs to this process are

–	a luma location ( xQ, yQ ) specifying the top-left luma sample of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	variables specifying the width and the height of the top-left prediction unit of the current quantisation group of coding units, nPSW and nPSH,

–	luma motion vectors mvL0 and mvL1 of the top-left prediction unit of the current quantisation group of coding units,

–	reference indices refIdxL0 and refIdxL1 of the top-left prediction unit of the current quantisation group of coding units,

–	prediction list utilization flags predFlagL0 and predFlagL1 of the top-left prediction unit of the current quantisation group of coding units.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

For X being replaced by 0 or 1, when predFlagLX is equal to 1, the uni-predicted quantisation parameter PredQPLX is derived in the following ordered steps.

- The variables xQRefLX and yQRefLX specifying the center of the reference block used for motion compensation of the top-left prediction unit of the current quantisation group of coding units are derived as follows.

xQRefLX = xQ + nPSW/2 + ( mvLX[ 0 ] >> 2 )					(8-xx3)
yQRefLX = yQ + nPSH/2 + ( mvLX[ 1 ] >> 2 )						(8-xx4)

- Let variables RefQPLX and SliceQPLX be set as follows.

- RefQPLX is derived in the following ordered steps.

- The variable Log2MinCUTQPSizeLX is set equal to Log2MinCUTQPSize of the reference frame specified by refIdxLX, and the variable MinCUTQPSizeLX is set equal to MinCUTQPSize of the reference frame specified by refIdxLX.

- The variables xQRefTlLX and yQRefTlLX are derived as follows.

xQRefTlLX = (xQRefLX >> Log2MinCUTQPSizeLX ) << Log2MinCUTQPSizeLX	(8-xx5)
yQRefTlLX = (yQRefLX >> Log2MinCUTQPSizeLX ) << Log2MinCUTQPSizeLX	(8-xx6)

- The variable RefQPLX is derived as

RefQPLX=1MinCUTQPSizeLX2((j=0MinCUTQPSizeLX−1i=0MinCUTQPSizeLX-1QLX(xQRefTlLX+i,yQRefTlLX+j))+MinCUTQPSizeLX2/2)										(8-xx7)

where QLX(x,y) specifies the luma quantisation parameter of the quantisation group of coding units covering luma location (x, y) in the reference frame specified by refIdxLX.

- SliceQPLX is the initial value of quantisation parameter of the slice covering luma location  (xQRefLX, yQRefLX) in the reference frame specified by refIdxLX.

- The uni-predicted quantisation parameter PredQPLX is derived as follows.

PredQPLX = RefQPLX + SliceQPY – SliceQPLX,					(8-xx8)

where SliceQPY is the initial value of quantisation parameter of the current slice.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

QPY,PRED = PredQPL0									(8-xx9)

–	Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

QPY,PRED = PredQPL1									(8-xx10)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1),

QPY,PRED = ( PredQPL0 + PredQPL1 + 1 ) >> 1						(8-xx11)