| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G069 |
| --- | --- |

| Title: | Cross-verification of KDDI's proposal on intra coding (JCTVC-G346) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Shohei Matsuo Seishi Takamura Hirohisa Jozawa 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-3382 jozawa.hirohisa@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This contribution reports the test results for KDDI's proposal on intra coding [1].

# Introduction

The software version was HM4.0. The reference was generated by using the common test conditions [2]. The provided source code was compiled on 64-bit Windows 7 environment by using Microsoft Visual Studio 2010 Professional edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7, Intel Core i7 2600 (3.4GHz) and 16GB RAM. The obtained results are shown in Table 1 and Table 2.

Table 1: All Intra HE and LC

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -12.0% | -14.9% | -0.1% | -11.1% | -13.7% |
| Class B | 0.0% | -2.6% | -2.5% | -0.1% | -2.2% | -1.7% |
| Class C | -0.1% | -2.2% | -2.7% | -0.1% | -1.7% | -2.0% |
| Class D | -0.1% | -2.1% | -2.3% | -0.1% | -1.5% | -1.7% |
| Class E | -0.1% | -1.1% | -0.9% | 0.0% | -0.8% | -0.7% |
| Overall | -0.1% | -4.1% | -4.7% | -0.1% | -3.5% | -4.0% |
|  | -0.1% | -4.0% | -4.6% | -0.1% | -3.4% | -3.9% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 2: All Intra HE and LC with class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -12.0% | -14.9% | -0.1% | -11.1% | -13.7% |
| Class B | 0.0% | -2.6% | -2.5% | -0.1% | -2.2% | -1.7% |
| Class C | -0.1% | -2.2% | -2.7% | -0.1% | -1.7% | -2.0% |
| Class D | -0.1% | -2.1% | -2.3% | -0.1% | -1.5% | -1.7% |
| Class E | -0.1% | -1.1% | -0.9% | 0.0% | -0.8% | -0.7% |
| Class F | -0.3% | -2.8% | -3.4% | -0.3% | -2.3% | -2.6% |
| Overall | -0.1% | -3.9% | -4.5% | -0.1% | -3.3% | -3.8% |
|  | -0.1% | -3.8% | -4.4% | -0.1% | -3.3% | -3.7% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

# Conclusion

This contribution reports the test results for KDDI's proposal (JCTVC-G346). Coding efficiency results matched completely. No mismatch between encoder’s local decoded picture and decoder’s output is observed. Relative encoding and decoding time are also confirmed. Source code was reviewed.

# References

- K. Kawamura, T. Yoshino, H. Kato, S. Naito, “Chroma intra prediction based on residual luma samples (G346),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Frank Bossen, “Common test conditions and software reference configurations (F900),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).