| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G070 m21616 |
| --- | --- |

| Title: | CE4 Subtest 1.2.b: Ericsson’s table-based delta QP coding method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg, Jiong Sun, Per Wennersten, Andrey Norkin Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document presents the description and results of QP coding methods (test 1.2.b) for CE4 Subtest 1. The method used in test 1.2.b was originally presented in the contribution JCTVC-F492. The anchor is CE4 QP scaling method with sparse QP mode on/off. The test is the QP coding method described in JCTVC-F492 with sparse QP mode on/off. The experimental results show that the proposed method with sparse mode=2 results in a BD-rate of -1.1% for AI_HE, RA_HE, AI_LC and RA_LC, -1.4% for LB_HE and -1.6% for LB_LC. The proposed method with sparse mode=1 achieves a BD-rate of -0.5% for AI_HE and RA_HE, -0.7% for LB_HE, -1.2% for AI_LC, -1.3% for RA_LC and -1.7% for LB_LC. When sparse mode is turned off, the BD-rate is 0.0% for LB_LC, +0.1% for RA_HE, RA_LC and LB_HE and +0.2% for AI_HE and AI_LC.

# Introduction

In CE4 Subtest 1, the performance and complexity of QP coding related methods are evaluated. This contribution reports the experimental results of a table-based delta QP coding method with sparse QP distribution (subtest 1.2.b).

# Algorithm description of table-based delta QP coding method

## CE4 Subtest 1.2.b

In the proposed method, both the encoder and decoder keep a QP-table containing all possible QP values. Where previously a delta QP would have been encoded, an index to a QP-table entry is sent instead. The QP-tables are updated for each QP such that the tables are sorted on how recently used the QP values are. This enables more recently used QP values to be represented by fewer bits compared to QP values that were used a long time ago or not yet used at all. At the decoder side, the QP-table indices are decoded in order to get the QP values to use. The decoder QP table will after decoding of each QP be updated using the same method as for the encoder.

The details of the table-based delta QP coding method is described as follows:

(1) Initialization of the QP table

It is important that the QP tables in the encoder and decoder are synchronized at all times. This means that the initialization of the tables for each slice has to be identical. We propose that the SliceQP is used for initialization and Table 1 shows an example QP table after initialization given a SliceQP of 25.

| Index | QP |
| --- | --- |
| 0 | 25 |
| 1 | 26 |
| 2 | 24 |
| 3 | 27 |
| 4 | 23 |
| 5 | 28 |
| … | … |

Table 1 - QP-table example

The initialization starts by inserting the SliceQP (25 in our example) at the first position in the QP table. Then QP values are added to the table with the QP value closest to the SliceQP first, then the second closest QP and so on.

(2) Updating the QP table and generating output

The encoder then encodes the CUs one by one. For each block, the encoder decides what QP to use. Let us call this QP currentQP. After a QP value is sent the QP table is updated. The encoder first finds currentQP in the QP table. Then the corresponding table index is encoded and sent to the decoder. CurrentQP is then removed from the table and all QP values above are shifted down 1 position, leaving index 0 empty. Finally, currentQP is put back at index 0. Note that if currentQP is found at index 0 (equivalent to a dQP equal to 0), the table is kept as-is.

For example, assume that currentQP is 23 and that Table 1 shows the QP-table before update. QP 23 is found at index 4, which is the index to send to the decoder. Table 2 then shows the updated QP table.

| Index | QP |
| --- | --- |
| 0 | 23 |
| 1 | 25 |
| 2 | 26 |
| 3 | 24 |
| 4 | 27 |
| 5 | 28 |
| … | … |

Table 2 - Updated QP-table example

The syntax element for delta QP used in HEVC is cu_qp_delta, for which the decoded value is a signed integer. Instead of using a delta QP value, the proposed method uses a QP table index which is an unsigned integer. The proposed method uses the same binarization, contexts and coding as cu_qp_delta with the mapping shown in Table 3.

| dQP, current method | Index, proposed method |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| -1 | 2 |
| 2 | 3 |
| -2 | 4 |
| 3 | 5 |
| … | … |

Table 3 – dQP in current method and table index in proposed method

For CAVLC coding the index is sent as a ue(v) which also is identical to the current HEVC method except for the mapping shown in Table 3.

# Experiments

Both anchor and test in subtest 1.2.b use an option to switch on/off sparse QP mode, which is a category QP assignment modification that results in sparse QP distributions.

The anchor is CE4 QP scaling method with sparse QP mode on (sparse mode=1,2) or off (sparse mode=0). The test is the table-based delta QP coding method (JCTVC-F492) with sparse QP mode on (sparse mode=1,2) or off (sparse mode=0).

In order to show the QP distribution when sparse QP mode is on, the following two metrics are collected from the experiments:

- A = Average QP span per slice. The QP span of a slice is the value of the maximum QP minus the minimum QP used in that particular slice. The average QP span per slice is the average of all QP spans in the bitstream.

- B = Average number of different QP values used per slice. The number of different QP values for each slice is counted. Then the mean of these values is calculated for the bitstream.

The value of A/B indicates how sparse/condense the QPs are distributed and is also shown in the experimental results.

## Experimental results on test 1.2.b, Sparse mode=2

Table 4 shows the summary of experimental results with MaxCuDQPDepth=3, Sparse mode=2 is used for both Anchor and Test.

Table 4: test 1.2.b results with sparse mode=2

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.8% | -0.8% | -0.8% | -30.3% | -0.8% | -0.8% | -0.7% | -28.2% |
| Class B | -1.0% | -0.9% | -0.9% | -31.0% | -0.9% | -0.8% | -0.8% | -26.9% |
| Class C | -1.2% | -1.1% | -1.0% | -32.2% | -1.2% | -1.1% | -1.1% | -28.8% |
| Class D | -1.1% | -1.0% | -1.0% | -30.9% | -1.1% | -1.0% | -1.0% | -28.6% |
| Class E | -1.5% | -1.4% | -1.5% | -32.2% | -1.5% | -1.4% | -1.4% | -29.0% |
| Overall | -1.1% | -1.0% | -1.0% | -31.2% | -1.1% | -1.0% | -1.0% | -28.2% |
|  | -1.1% | -1.0% | -1.0% |  | -1.1% | -1.0% | -1.0% |  |
| Enc Time[%] | 155% |  |  |  | 122% |  |  |  |
| Dec Time[%] | 99.5% |  |  |  | 99.7% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -1.0% | -1.3% | -0.8% | -32.7% | -0.9% | -0.9% | -0.8% | -31.1% |
| Class B | -1.1% | -1.0% | -1.1% | -33.9% | -1.1% | -1.1% | -1.1% | -30.7% |
| Class C | -1.2% | -1.2% | -1.2% | -33.9% | -1.3% | -1.3% | -1.3% | -32.0% |
| Class D | -1.1% | -1.2% | -0.9% | -32.8% | -1.2% | -1.2% | -1.2% | -31.6% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.1% | -1.2% | -1.0% | -33.4% | -1.1% | -1.1% | -1.1% | -31.3% |
|  | -1.1% | -1.2% | -1.0% |  | -1.2% | -1.1% | -1.1% |  |
| Enc Time[%] | 116% |  |  |  | 116% |  |  |  |
| Dec Time[%] | 99.8% |  |  |  | 100.2% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.4% | -1.4% | -1.0% | -35.0% | -1.4% | -1.4% | -1.2% | -32.5% |
| Class C | -1.5% | -1.3% | -2.1% | -34.7% | -1.7% | -1.7% | -1.5% | -33.2% |
| Class D | -1.3% | -1.1% | -1.3% | -33.6% | -1.5% | -1.6% | -1.4% | -32.7% |
| Class E | -1.5% | 0.0% | -1.4% | -34.6% | -1.7% | -1.4% | -1.8% | -33.8% |
| Overall | -1.4% | -1.0% | -1.4% | -34.5% | -1.6% | -1.5% | -1.5% | -32.9% |
|  | -1.4% | -1.1% | -1.4% |  | -1.6% | -1.5% | -1.5% |  |
| Enc Time[%] | 141% |  |  |  | 115% |  |  |  |
| Dec Time[%] | 99.6% |  |  |  | 100.0% |  |  |  |

The encoding time shown above is collected from simulations on Linux clusters. Therefore they do not reflect the true encoding time. Table 5 shows the decoding time on a single machine.

Table 5 – decoding time on a single machine

|  | HE | LC |
| --- | --- | --- |
| AI | 100.0% | 100.4% |
| RA | 100.3% | 99.4% |
| LD | 99.9% | 100.1% |

Table 6 shows the values of A/B when sparse mode=2. These values are the same for both Anchor and Test.

Table 6 – A/B, sparse mode=2

|  | AI HE | AI LC |
| --- | --- | --- |
| class A | 2.57 | 2.57 |
| class B | 2.56 | 2.56 |
| class C | 2.53 | 2.53 |
| class D | 2.50 | 2.50 |
| class E | 2.50 | 2.50 |
| Avg | 2.53 | 2.53 |
|  | RA HE | RA LC |
| class A | 2.53 | 2.51 |
| class B | 2.50 | 2.48 |
| class C | 2.43 | 2.39 |
| class D | 2.43 | 2.38 |
| class E |  |  |
| Avg | 2.47 | 2.44 |
|  | LD HE | LD LC |
| class A |  |  |
| class B | 2.50 | 2.49 |
| class C | 2.46 | 2.46 |
| class D | 2.41 | 2.38 |
| class E | 2.43 | 2.36 |
| Avg | 2.45 | 2.42 |

## Experimental results on test 1.2.b, Sparse mode=1

Table 7 shows the summary of experimental results with MaxCuDQPDepth=3, Sparse mode=1 is used for both Anchor and Test.

Table 7 : test 1.2.b results with sparse mode=1

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.4% | -0.4% | -0.3% | -11.9% | -0.9% | -0.8% | -0.8% | -26.4% |
| Class B | -0.4% | -0.3% | -0.3% | -11.2% | -1.0% | -0.9% | -0.9% | -26.4% |
| Class C | -0.5% | -0.4% | -0.4% | -13.5% | -1.3% | -1.2% | -1.2% | -27.8% |
| Class D | -0.5% | -0.4% | -0.4% | -12.8% | -1.2% | -1.1% | -1.1% | -27.2% |
| Class E | -0.7% | -0.6% | -0.6% | -14.1% | -1.6% | -1.5% | -1.5% | -27.6% |
| Overall | -0.5% | -0.4% | -0.4% | -12.6% | -1.2% | -1.1% | -1.1% | -27.0% |
|  | -0.5% | -0.4% | -0.4% |  | -1.2% | -1.1% | -1.1% |  |
| Enc Time[%] | 139% |  |  |  | 140% |  |  |  |
| Dec Time[%] | 99.7% |  |  |  | 100.4% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.4% | -0.5% | -0.5% | -14.6% | -1.0% | -1.0% | -1.0% | -28.9% |
| Class B | -0.5% | -0.5% | -0.5% | -14.7% | -1.2% | -1.2% | -1.2% | -29.4% |
| Class C | -0.6% | -0.5% | -0.5% | -16.2% | -1.5% | -1.4% | -1.4% | -30.2% |
| Class D | -0.5% | -0.7% | -0.8% | -15.4% | -1.3% | -1.4% | -1.3% | -29.4% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.5% | -0.5% | -0.6% | -15.2% | -1.3% | -1.2% | -1.2% | -29.5% |
|  | -0.5% | -0.6% | -0.6% |  | -1.3% | -1.2% | -1.2% |  |
| Enc Time[%] | 121% |  |  |  | 122% |  |  |  |
| Dec Time[%] | 99.7% |  |  |  | 100.1% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.6% | -0.4% | -16.1% | -1.5% | -1.4% | -1.4% | -30.8% |
| Class C | -0.8% | -0.8% | -0.8% | -17.0% | -1.8% | -1.8% | -1.7% | -31.1% |
| Class D | -0.7% | -0.4% | -0.3% | -16.4% | -1.6% | -1.6% | -1.9% | -30.2% |
| Class E | -0.7% | -1.0% | -1.5% | -17.8% | -1.9% | -2.0% | -2.2% | -31.8% |
| Overall | -0.7% | -0.7% | -0.6% | -16.7% | -1.7% | -1.7% | -1.7% | -30.9% |
|  | -0.7% | -0.7% | -0.6% |  | -1.7% | -1.7% | -1.7% |  |
| Enc Time[%] | 123% |  |  |  | 116% |  |  |  |
| Dec Time[%] | 99.7% |  |  |  | 99.8% |  |  |  |

The encoding time shown above is collected from simulations on Linux clusters. Therefore they do not reflect the true encoding time. Table 8 shows the decoding time on a single machine.

Table 8 – decoding time on a single machine

|  | HE | LC |
| --- | --- | --- |
| AI | 100.5% | 100.5% |
| RA | 100.2% | 100.3% |
| LD | 100.1% | 100.2% |

Table 9 shows the values of A/B when sparse mode=1. These values are the same for both Anchor and Test.

Table 9: A/B, sparse mode=1

|  | AI HE | AI LC |
| --- | --- | --- |
| class A | 1.80 | 1.80 |
| class B | 1.80 | 1.79 |
| class C | 1.79 | 1.78 |
| class D | 1.77 | 1.77 |
| class E | 1.78 | 1.78 |
| Avg | 1.79 | 1.79 |
|  | RA HE | RA LC |
| class A | 1.81 | 1.81 |
| class B | 1.82 | 1.79 |
| class C | 1.79 | 1.75 |
| class D | 1.86 | 1.80 |
| class E |  |  |
| Avg | 1.82 | 1.79 |
|  | LD HE | LD LC |
| class A |  |  |
| class B | 1.78 | 1.77 |
| class C | 1.76 | 1.76 |
| class D | 1.79 | 1.76 |
| class E | 1.82 | 1.76 |
| Avg | 1.79 | 1.76 |

## Experimental results on test 1.2.b, Sparse mode=0

Table 10 shows the summary of experimental results with MaxCuDQPDepth=3, Sparse mode=0 is used for both Anchor and Test.

Table 10: test 1.2.b results with sparse mode=0

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.2% | 0.1% | 0.2% | 5.6% | 0.2% | 0.2% | 0.2% | 7.0% |
| Class B | 0.2% | 0.2% | 0.2% | 5.6% | 0.2% | 0.2% | 0.2% | 6.6% |
| Class C | 0.1% | 0.1% | 0.1% | 3.7% | 0.2% | 0.2% | 0.2% | 4.6% |
| Class D | 0.2% | 0.2% | 0.2% | 5.1% | 0.2% | 0.2% | 0.2% | 6.0% |
| Class E | 0.2% | 0.1% | 0.2% | 2.9% | 0.3% | 0.3% | 0.3% | 4.7% |
| Overall | 0.2% | 0.1% | 0.2% | 4.7% | 0.2% | 0.2% | 0.2% | 5.9% |
|  | 0.2% | 0.1% | 0.2% |  | 0.2% | 0.2% | 0.2% |  |
| Enc Time[%] | 97% |  |  |  | 97% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 98% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.2% | 0.3% | 0.2% | 4.3% | 0.1% | 0.1% | 0.0% | 4.2% |
| Class B | 0.1% | 0.1% | 0.1% | 3.6% | 0.1% | 0.1% | 0.1% | 3.5% |
| Class C | 0.1% | 0.1% | -0.2% | 2.3% | 0.1% | 0.1% | 0.1% | 1.9% |
| Class D | 0.1% | 0.0% | 0.1% | 3.0% | 0.1% | 0.1% | 0.1% | 2.2% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 3.3% | 0.1% | 0.1% | 0.1% | 3.0% |
|  | 0.1% | 0.1% | 0.0% |  | 0.1% | 0.1% | 0.1% |  |
| Enc Time[%] | 98% |  |  |  | 98% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 98% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.0% | 2.6% | 0.1% | 0.1% | 0.0% | 2.1% |
| Class C | 0.0% | 0.0% | 0.1% | 1.7% | 0.0% | 0.1% | 0.1% | 0.9% |
| Class D | 0.1% | 0.0% | 0.5% | 2.1% | 0.0% | 0.1% | 0.0% | 0.8% |
| Class E | 0.1% | 0.3% | 0.1% | 1.0% | -0.1% | -0.4% | -0.1% | -0.5% |
| Overall | 0.1% | 0.2% | 0.2% | 2.0% | 0.0% | 0.0% | 0.0% | 1.0% |
|  | 0.1% | 0.1% | 0.2% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 98% |  |  |  | 98% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 100% |  |  |  |

The encoding time shown above is collected from simulations on Linux clusters. Therefore they do not reflect the true encoding time. Table 11 shows the decoding time on a single machine.

Table 11 – decoding time on a single machine

|  | HE | LC |
| --- | --- | --- |
| AI | 100.3% | 100.7% |
| RA | 99.8% | 100.1% |
| LD | 100.3% | 100.0% |

Table 12 shows the values of A/B when sparse mode=0. These values are the same for both Anchor and Test.

Table 12: A/B, sparse mode=0

|  | AI HE | AI LC |
| --- | --- | --- |
| class A | 0.95 | 0.95 |
| class B | 0.95 | 0.95 |
| class C | 0.94 | 0.94 |
| class D | 0.94 | 0.94 |
| class E | 0.94 | 0.94 |
| Avg | 0.94 | 0.94 |
|  | RA HE | RA LC |
| class A | 1.05 | 1.04 |
| class B | 1.07 | 1.05 |
| class C | 1.08 | 1.03 |
| class D | 1.23 | 1.19 |
| class E |  |  |
| Avg | 1.11 | 1.08 |
|  | LD HE | LD LC |
| class A |  |  |
| class B | 0.97 | 0.96 |
| class C | 0.97 | 0.96 |
| class D | 1.09 | 1.05 |
| class E | 1.16 | 1.09 |
| Avg | 1.05 | 1.02 |

# Conclusions

The experimental results show that the proposed method with sparse mode=2 results in a BD-rate of -1.1% for AI_HE, RA_HE, AI_LC and RA_LC, -1.4% for LB_HE and -1.6% for LB_LC. The proposed method with sparse mode=1 achieves a BD-rate of -0.5% for AI_HE and RA_HE, -0.7% for LB_HE, -1.2% for AI_LC, -1.3% for RA_LC and -1.7% for LB_LC. When sparse mode is turned off, the BD-rate is 0.0% for LB_LC, +0.1% for RA_HE, RA_LC and LB_HE and +0.2% for AI_HE and AI_LC.

# Proposal

We propose that JCT-VC adopts the Table-based delta-QP coding method.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).