| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G073 WG11 number: m21620 |
| --- | --- |

| Title: | CE4: Cross-check of Subtest 1.3.e – Spatial QP prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution presents a summary of the cross-check performed by the Institute for Infocomm Research on Qualcomm’s proposed spatial QP prediction scheme. The cross-check performance results reportedly match those provided by Qualcomm. The implemented algorithm is also reportedly in line with Qualcomm’s proposal.

# Introduction

In JCTVC-F705, Qualcomm has proposed a combination of earlier proposed methods for predicting QP when coding delta-QP parameters [1]. In the proposed method, for Intra coded CUs, the intra prediction mode is used to determine in which direction to predict QP from; for VER-8 to VER+8 modes, the above neighboring QP is used as a predictor, and for HOR-7 to HOR+8 modes, the left neighboring QP is used as a predictor, if those neighboring CUs are available. For DC and Planar modes, if either the left or above CU is available (checked in that order), then their QP is used as the predictor. For Inter coded CUs, the table below summarizes which QP is used as the predictor.

| Left CU | NA | NA | NA | INTRA | INTRA | INTRA | INTER | INTER | INTER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Above CU | NA | INTRA | INTER | NA | INTRA | INTER | NA | INTRA | INTER |
| QP Predictor | Last CU | Last CU | Last CU | Left CU | Left CU | Above CU | Left CU | Left CU | Left CU |

# Simulation Results

We have verified the results provided by Qualcomm. The test conditions as specified in CE4 Subtest 1 [2] are followed; specifically, both the anchor and the tested proposal uses TM-5 Step 3 for QP adjustment, with MaxCUDepthDQP=3. Tests were conducted on Intra, Random Access and Low Delay configurations for both high efficiency and low-complexity settings [3].

The table below summarizes the coding results.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.1% | -0.2% | -4.9% | -0.2% | -0.2% | -0.2% | -5.4% |
| Class B | -0.3% | -0.3% | -0.3% | -7.9% | -0.3% | -0.3% | -0.3% | -8.7% |
| Class C | -0.4% | -0.4% | -0.4% | -9.3% | -0.5% | -0.5% | -0.5% | -10.5% |
| Class D | -0.4% | -0.3% | -0.3% | -8.6% | -0.4% | -0.4% | -0.4% | -10.0% |
| Class E | -0.7% | -0.8% | -0.7% | -13.5% | -0.8% | -0.7% | -0.8% | -14.3% |
| Overall | -0.4% | -0.4% | -0.4% | -8.6% | -0.4% | -0.4% | -0.4% | -9.5% |
|  | -0.4% | -0.4% | -0.4% |  | -0.4% | -0.4% | -0.4% |  |
| Enc Time[%] | 102% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 101% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.3% | -1.7% | -0.1% | -0.1% | -0.2% | -2.0% |
| Class B | -0.2% | -0.2% | -0.1% | -3.6% | -0.2% | -0.2% | -0.2% | -4.3% |
| Class C | -0.2% | -0.1% | -0.3% | -4.7% | -0.3% | -0.4% | -0.4% | -5.9% |
| Class D | -0.1% | 0.1% | -0.1% | -3.7% | -0.2% | -0.4% | -0.1% | -4.7% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.2% | -3.4% | -0.2% | -0.3% | -0.2% | -4.2% |
|  | -0.1% | 0.0% | -0.2% |  | -0.2% | -0.3% | -0.2% |  |
| Enc Time[%] | 100% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 105% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.0% | -1.5% | -0.1% | -0.1% | -0.2% | -1.9% |
| Class C | -0.1% | -0.1% | 0.2% | -2.2% | -0.2% | -0.1% | -0.2% | -3.1% |
| Class D | -0.1% | -0.5% | 0.1% | -1.5% | -0.1% | -0.5% | -0.6% | -2.4% |
| Class E | 0.0% | 0.2% | 0.1% | -0.5% | -0.1% | -0.9% | -0.5% | -0.8% |
| Overall | -0.1% | -0.1% | 0.1% | -1.5% | -0.1% | -0.4% | -0.4% | -2.1% |
|  | -0.1% | -0.1% | 0.1% |  | -0.1% | -0.3% | -0.4% |  |
| Enc Time[%] | 102% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 103% |  |  |  |

# Conclusions

We have cross-checked Qualcomm’s proposal JCTVC-F705 (Subtest 1.3.e), studied the algorithm, and verified their coding simulation results.

# References

[1] M. Coban and M. Karczewicz, “CE4 Subtest 2: Spatial QP prediction: combination of test 2.3.g with 2.3.b/2.3.c,” in JCTVC-F705, Torino, Italy, Jul. 2011.

[2] K. Sato et al., “Description of Core Experiment 4: Quantization,” JCTVC-F904, Torino, Italy, Jul. 2011.

[3] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, Italy, Jul. 2011.