| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G074 WG11 number: m21621 |
| --- | --- |

| Title: | CE4: Cross-check of Subtest 2 on de-quantization offset (2.1.a, 2.3.d) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution presents a summary of the cross-checks performed by the Institute for Infocomm Research on two proposals submitted by MediatTek and Qualcomm. The cross-check performance results reportedly match those provided by MediaTek and Qualcomm. The implemented algorithm is also reportedly in line with the proposals from MediaTek and Qualcomm.

# Introduction

In CE4 Subtest 2, three proposals on de-quantization offset are to be tested under various settings [1]. The three proposals are JCTVC-F119 from MediaTek, JCTVC-F276 from RIM, and JCTVC-F610 from Qualcomm [2,3,4]. In this cross-check, the following two test cases are checked.

| Subtest | Anchor | Tested |
| --- | --- | --- |
| 2.1.a | HM4 | F119, w/o Lambda refinement |
| 2.3.d | HM4, RDOQ off, adaptive rounding on | F610, RDOQ off |

In HM4, de-quantization is conceptually carried out as follows:

Xk=ik⋅Δ

where Xkis the kth de-quantized coefficient, ikis the kth quantized coefficient index, andΔis the quantization step size. In practice, this is implemented with a multiply followed by a bit-shift.

## JCTVC-F119 (MediaTek)

In F119, the de-quantization is conceptually carried as follows:

Xk=0,ik=0ik+o⋅Δ,ik>0ik−o⋅Δ,ik<0

where o is the de-quantization offset to be used. The offset to be used depends on the color plane (Luma, Chroma U or Chroma V) but not the CU type.

To take advantage of the adaptive de-quantization offset, the modifications are made to the encoder:

- RDOQ is modified to compute the error with respect to the offset

- Lambda refinement can be performed for Inter slices

- Additional tables are used to keep track of coefficient statistics in order to compute de-quantization offsets to be used for the next slice

- Clipping (dependent on temporal level) is done on the computed offset to keep it within a desired range

Compared to F119, the current implementation does not appear to use a Laplacian model [2] and its associated look-up table to compute the offset. Instead, it uses a straightforward average offset computation.

## JCTVC-F610 (Qualcomm)

In F610, the de-quantization is conceptually carried as follows:

Xk=ik1+α⋅Δ

where α is an adaptive scaling parameter that can more finely control QP granularity. The scaling to be used depends on whether the CU is Inter or Intra, and whether it is Luma or Chroma residual. Scaling is only applied for Inter slices.

To take advantage of the adaptive de-quantization scaling, the modifications are made to the encoder:

- RDOQ is modified to compute the error taking into account the adaptive scaling

- Additional tables are used to keep track of coefficient statistics in order to compute the adaptive scaling to be used for the next slice

## JCTVC-F276 (RIM)

In F276, the de-quantization is conceptually carried as follows:

Xk=0,ik=0ik(1+α)+o⋅Δ,ik>0ik(1+α)−o⋅Δ,ik<0

The scaling and offsets to be used depends on whether the CU is Inter or Intra, and whether it is Luma or Chroma residual. This is done only for Inter slices.

# Simulation Results

We have verified the results provided by MediaTek and Qualcomm. The test conditions as specified in CE4 Subtest 2 [1] are followed. Tests were conducted on Random Access and Low Delay configurations for both high efficiency and low-complexity settings [5].

The results are shown below.

## Subtest 2.1.a – F119 vs HM4

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.3% | -2.4% | -2.7% | -1.6% | -2.8% | -3.4% |
| Class B | -1.0% | -3.4% | -3.9% | -1.1% | -3.2% | -3.7% |
| Class C | -1.3% | -2.6% | -2.7% | -1.6% | -3.0% | -2.7% |
| Class D | -0.9% | -2.7% | -2.7% | -1.2% | -2.8% | -2.6% |
| Class E |  |  |  |  |  |  |
| Overall | -1.1% | -2.8% | -3.1% | -1.4% | -3.0% | -3.1% |
|  | -0.7% | -2.7% | -2.9% | -1.3% | -2.9% | -3.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.3% | -1.0% | -0.9% | -1.7% | -1.1% | -1.2% |
| Class C | -1.3% | -1.2% | -0.9% | -1.9% | -1.3% | -1.5% |
| Class D | -0.9% | -0.7% | -0.7% | -1.4% | -1.0% | -1.3% |
| Class E | -0.6% | 0.7% | 0.0% | -0.8% | -1.0% | -0.9% |
| Overall | -1.1% | -0.7% | -0.7% | -1.5% | -1.1% | -1.2% |
|  | -1.1% | -0.7% | -0.8% | -1.5% | -1.2% | -1.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

An additional test was carried out for the case of low QP values (2, 7, 12, 17). The results of that area shown below. Note that timing results are not available. It is observed that coding gains at low QP values are not as high as the range of QPs in common conditions.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -1.2% | -1.4% | -1.0% | -2.0% | -2.2% |
| Class B | -0.2% | -1.8% | -3.3% | -1.1% | -2.3% | -3.7% |
| Class C | -0.2% | -2.2% | -2.5% | -1.1% | -2.3% | -3.0% |
| Class D | -0.3% | -2.1% | -2.2% | -1.0% | -2.7% | -2.9% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -1.8% | -2.4% | -1.1% | -2.3% | -3.0% |
|  | -0.3% | -1.5% | -2.1% | -1.1% | -1.9% | -2.5% |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | -0.4% | -2.0% | -0.5% | -1.3% | -2.5% |
| Class C | 0.2% | -0.6% | -1.3% | -0.5% | -1.4% | -2.2% |
| Class D | 0.1% | -0.8% | -1.1% | -0.5% | -2.2% | -2.5% |
| Class E | -0.2% | -1.0% | -1.1% | -0.3% | -2.7% | -2.8% |
| Overall | 0.1% | -0.6% | -1.4% | -0.5% | -1.8% | -2.5% |
|  | 0.1% | -0.6% | -1.4% | -0.5% | -1.2% | -1.9% |

## Subtest 2.3.d – F610 (RDOQ off) vs HM4 (RDOQ off, adaptive rounding on)

The first table shows the results of F610 with RDOQ off and adaptive rounding on.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.4% | 4.6% | 2.2% | -0.6% | 6.7% | 3.8% |
| Class B | -0.9% | 0.8% | 0.5% | -0.9% | 0.6% | 0.2% |
| Class C | -1.0% | 1.8% | 1.9% | -1.1% | 1.4% | 1.7% |
| Class D | -0.7% | 1.8% | 1.7% | -0.8% | 1.4% | 1.4% |
| Class E |  |  |  |  |  |  |
| Overall | -1.0% | 2.2% | 1.5% | -0.9% | 2.4% | 1.7% |
|  | -0.7% | 2.2% | 1.6% | -0.6% | 2.5% | 1.7% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.4% | -1.5% | -2.2% | -1.6% | -1.6% | -2.4% |
| Class C | -1.6% | -0.7% | -0.2% | -1.8% | -0.6% | 0.1% |
| Class D | -1.1% | -0.2% | -0.7% | -1.2% | -1.0% | -1.1% |
| Class E | -0.6% | -0.6% | -0.7% | -0.8% | -1.8% | -1.3% |
| Overall | -1.2% | -0.8% | -1.0% | -1.4% | -1.2% | -1.3% |
|  | -1.3% | -0.8% | -1.0% | -1.4% | -1.2% | -1.2% |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 101% |  |  |

The second table shows the results of F610 with RDOQ off and adaptive rounding off.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.2% | 3.4% | 0.9% | -0.4% | 5.7% | 2.8% |
| Class B | -0.6% | -1.2% | -2.0% | -0.6% | -0.7% | -1.5% |
| Class C | -1.0% | 1.4% | 1.6% | -1.0% | 0.8% | 1.4% |
| Class D | -0.7% | 1.8% | 1.5% | -0.8% | 1.3% | 1.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.8% | 1.2% | 0.3% | -0.7% | 1.6% | 0.9% |
|  | -0.6% | 1.3% | 0.5% | -0.4% | 1.7% | 1.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.4% | -1.6% | -2.3% | -1.6% | -1.6% | -2.3% |
| Class C | -1.6% | -0.4% | -0.2% | -1.7% | -0.5% | 0.1% |
| Class D | -1.1% | -0.1% | -0.8% | -1.2% | -1.0% | -1.3% |
| Class E | -0.6% | -0.2% | -1.1% | -0.8% | -2.0% | -2.7% |
| Overall | -1.2% | -0.7% | -1.2% | -1.4% | -1.3% | -1.5% |
|  | -1.3% | -0.7% | -1.1% | -1.4% | -1.2% | -1.4% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

# Conclusions

We have cross-checked F119 and F610, studied the algorithms, and verified their coding simulation results.

# References

[1] K. Sato et al., “Description of Core Experiment 4: Quantization,” JCTVC-F904, Torino, Italy, Jul. 2011.

[2] X. Li et al., “CE4: Adaptive de-quantization offset,” JCTVC-F119, Torino, Italy, Jul. 2011.

[3] X. Yu et al., “CE4-subtest 3.3.2: Quantization with Adaptive Reconstruction Levels,” JCTVC-F276, Torino, Italy, Jul. 2011.

[4] X. Wang et al., “Fine granularity QP change at slice level,” in JCTVC-F610, Torino, Italy, Jul. 2011.

[5] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, Italy, Jul. 2011.