| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G075 WG11 number: m21622 |
| --- | --- |

| Title: | CE7: Cross-check of Unified Tools 1 & 2 – Mode-Dependent Secondary Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution presents a summary of the cross-checks performed by the Institute for Infocomm Research on Samsung and Panasonic’s proposed Unified Tools 1 & 2 (Mode-Dependent Secondary Transform) in JCTVC-G108. The cross-check performance results reportedly match those provided by Samsung and Panasonic. The implemented algorithm is also reportedly in line with Samsung and Panasonic’s proposals.

# Introduction

Samsung and Panasonic had proposed applying mode-dependent secondary transforms coding intra prediction residual of block sizes 8x8 and larger in JCTVC-F224 [1] and JCTVC-F554 [2]. This was to be further tested in CE7 [3].

In the unified tool presented in JCTVC-G108 [5], for intra prediction residuals of block sizes 8x8 and larger, a mode-dependent secondary transform may be applied. The current mode to transform mapping table in HM4 for the mode-dependent DCT/DST coding of 4x4 intra prediction residual is reused. When the table implies the use of the DST in a particular direction (horizontal or vertical), the DCT is first applied, before a secondary transform is applied. In the tool, only the N lowest frequency coefficients are processed by the secondary transform, where N can range from 2 to 8.

In the implementation provided by the proponents, the NxN transform is implemented using a matrix multiply with 7 bits of fractional accuracy. Clipping to 16 bits is also performed after the each secondary transform stage.

# Simulation Results

We have run tests on the Tool for secondary transform sizes of 2x2, 7x7 and 8x8 on HM4.0, and have verified the results provided by Samsung and Panasonic. The test conditions as specified in CE7 [3] are followed. Tests were conducted on Intra and Random Access configurations for both high-efficiency and low-complexity settings [4].

The results for the 3 tested transform sizes are summarized in the tables below.

| Size | AI-HE |  |  |  |  | RA-HE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | U (%) | V (%) | Enc (%) | Dec (%) | Y (%) | U (%) | V (%) | Enc (%) | Dec (%) |
| 2x2 | -0.1 | 0.0 | 0.0 | 100 | 100 | -0.1 | 0.1 | -0.1 | 100 | 100 |
| 7x7 | -0.7 | -0.6 | -0.6 | 102 | 103 | -0.3 | -0.1 | -0.4 | 100 | 101 |
| 8x8 | -0.7 | -0.6 | -0.6 | 102 | 103 | -0.3 | -0.3 | -0.5 | 100 | 100 |

| Size | AI-LC |  |  |  |  | RA-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | U (%) | V (%) | Enc (%) | Dec (%) | Y (%) | U (%) | V (%) | Enc (%) | Dec (%) |
| 2x2 | -0.1 | -0.1 | -0.1 | 99 | 100 | -0.1 | 0.0 | 0.1 | 100 | 100 |
| 7x7 | -0.6 | -0.7 | -0.8 | 102 | 103 | -0.3 | -0.3 | -0.3 | 100 | 101 |
| 8x8 | -0.7 | -0.8 | -0.9 | 104 | 103 | -0.3 | -0.3 | -0.3 | 100 | 99 |

A more detailed breakdown is provided in the sub-sections below.

## 2x2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.1% | 0.1% | -0.1% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class D | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.3% | 0.0% | 0.0% | -0.3% | -0.3% | -0.3% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.1% | 0.0% | -0.1% | 0.3% | 0.3% |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class D | 0.0% | 0.2% | -0.2% | -0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.1% | -0.1% | -0.1% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## 7x7

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -0.5% | -0.6% | -0.8% | -0.7% | -1.0% |
| Class B | -0.8% | -0.7% | -0.6% | -0.6% | -0.9% | -1.0% |
| Class C | -0.5% | -0.5% | -0.5% | -0.6% | -0.7% | -0.6% |
| Class D | -0.3% | -0.4% | -0.4% | -0.4% | -0.3% | -0.4% |
| Class E | -1.1% | -0.7% | -0.8% | -0.9% | -1.0% | -1.1% |
| Overall | -0.7% | -0.6% | -0.6% | -0.6% | -0.7% | -0.8% |
|  | -0.7% | -0.6% | -0.6% | -0.7% | -0.7% | -0.8% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | 0.3% | -0.2% | -0.4% | -0.1% | -0.3% |
| Class B | -0.4% | -0.3% | -0.4% | -0.4% | -0.5% | -0.5% |
| Class C | -0.3% | -0.4% | -0.3% | -0.3% | -0.4% | -0.3% |
| Class D | -0.1% | 0.0% | -0.7% | -0.2% | -0.2% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.1% | -0.4% | -0.3% | -0.3% | -0.3% |
|  | -0.3% | -0.1% | -0.4% | -0.3% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

## 8x8

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -0.6% | -0.7% | -0.8% | -0.7% | -1.1% |
| Class B | -0.8% | -0.8% | -0.7% | -0.6% | -1.0% | -1.0% |
| Class C | -0.5% | -0.5% | -0.5% | -0.6% | -0.7% | -0.7% |
| Class D | -0.3% | -0.4% | -0.4% | -0.4% | -0.4% | -0.4% |
| Class E | -1.1% | -0.8% | -0.9% | -0.9% | -1.1% | -1.2% |
| Overall | -0.7% | -0.6% | -0.6% | -0.7% | -0.8% | -0.9% |
|  | -0.7% | -0.6% | -0.7% | -0.7% | -0.8% | -0.9% |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.2% | -0.9% | -0.4% | -0.1% | -0.3% |
| Class B | -0.4% | -0.3% | -0.3% | -0.4% | -0.5% | -0.5% |
| Class C | -0.3% | -0.6% | -0.2% | -0.3% | -0.4% | -0.2% |
| Class D | -0.2% | 0.0% | -0.6% | -0.2% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.5% | -0.3% | -0.3% | -0.3% |
|  | -0.3% | -0.3% | -0.5% | -0.3% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusions

We have cross-checked Samsung and Panasonic’s unified Tools 1 & 2 in CE7, studied the algorithm and verified their coding simulation results.

# References

[1] T. Shibahara and T. Nishi, “CE7: Mode Dependent 2-step Transform for Intra Coding,” in JCTVC-F224, Torino, Italy, Jul. 2011.

[2] A. Saxena and F. Fernandes, “On secondary transforms for intra prediction residual,” in JCTVC-F554, Torino, Italy, Jul. 2011.

[3] R. Cohen et al., “Description of Core Experiment 7: Additional Transforms,” JCTVC-F907, Torino, Italy, Jul. 2011.

[4] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, Italy, Jul. 2011.

[5] A. Saxena et al., “CE 7 : On secondary transforms for intra prediction residual,” JCTVC-G108, Geneva, Switzerland, Nov. 2011.