| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G076_r1 WG11 number: m21623 |
| --- | --- |

| Title: | CE7: Cross-check of Tool 4 – DST for Intra Chroma Residual Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution presents a summary of the cross-checks performed by the Institute for Infocomm Research on Samsung’s proposed Tool 4 (DST for  Intra Chroma Residual Coding). The cross-check performance results reportedly match those provided by Samsung. The implemented algorithm is also reportedly in line with Samsung’s proposal.

# Introduction

Samsung had proposed applying mode-dependent DCT/DST for coding chroma intra prediction residual in JCTVC-F553 [1]. This was to be further tested in CE7 [2].

In Tool 4, the basic concept is to apply a mode-dependent DCT/DST for transform coding of chroma intra prediction residual, depending on prediction mode used. In HM4, 6 chroma prediction modes can be used; they are: derived mode (DM), chroma-from-luma mode (LM), planar, vertical, horizontal and DC. The DM mode uses the mode signaled for luma intra prediction, and if it is one of planar, vertical, horizontal or DC, then the VER+8 mode would be used to replace the repeated mode. Note that if the planar, vertical, horizontal, DC or VER+8 mode is used, it can either be signaled using the DM, or explicitly signaled, i.e., not as a derived mode.

Due to the signaling used for chroma intra prediction mode, different variations of Tool 4 were tested. Table 1 below summarizes the different variations in terms of when the mode-dependent DCT/DST is applied. A ‘E’ signifies that DCT/DST is applied when that mode is explicitly signaled, and a ‘D’ signifies that DCT/DST is applied when that mode is signaled as a DM.

| Test | Planar | HOR | VER | DC | VER+8 | Others |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | - | E | E | - | - | - |
| Test 2 | E | E | E | - | - | - |
| Test 3 | E,D | E,D | E,D | - | - | - |
| Test 4 | E,D | E,D | E,D | - | E,D | D |
| Test 5 | E,D | - | - | - | - | - |
| Test 6 | - | E,D | E,D | - | - | - |
| Test 7 | E,D | E | E | - | E | - |
| Test 8 | E | E | E | - | E | - |
| Test 9 | E | - | - | - | - | - |
| Test 10 | E,D | E,D | E,D | - | E,D | - |

# Simulation Results

We have independently implemented the test cases 1, 3 and 4 on HM4.0, and have verified the results provided by Samsung. The test conditions as specified in CE7 [2] are followed. Tests were conducted on Intra and Random Access configurations for both high-efficiency and low-complexity settings [3].

## Test 1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.4% |
| Class B | 0.0% | -0.3% | -0.3% | 0.0% | -0.5% | -0.5% |
| Class C | 0.0% | -0.4% | -0.6% | 0.0% | -0.8% | -0.9% |
| Class D | 0.0% | -0.7% | -0.8% | 0.0% | -0.9% | -1.1% |
| Class E | 0.0% | -0.1% | -0.2% | 0.0% | -0.3% | -0.5% |
| Overall | 0.0% | -0.4% | -0.4% | 0.0% | -0.6% | -0.7% |
|  | 0.0% | -0.4% | -0.4% | 0.0% | -0.5% | -0.7% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.3% | 0.0% | -0.5% | -0.4% |
| Class B | 0.0% | -0.4% | -0.7% | 0.0% | -0.6% | -0.5% |
| Class C | 0.0% | -0.7% | -0.6% | 0.0% | -0.8% | -0.7% |
| Class D | 0.0% | -1.1% | -1.2% | 0.0% | -0.9% | -0.6% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.5% | -0.7% | 0.0% | -0.7% | -0.5% |
|  | 0.0% | -0.6% | -0.7% | 0.0% | -0.7% | -0.5% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

## Test 3

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% | 0.0% | -0.5% | -0.5% |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | -0.7% | -0.6% |
| Class C | -0.1% | -0.6% | -0.8% | -0.1% | -1.2% | -1.3% |
| Class D | -0.1% | -1.0% | -1.1% | -0.1% | -1.4% | -1.7% |
| Class E | 0.1% | 0.3% | 0.0% | 0.0% | -0.2% | -0.7% |
| Overall | 0.0% | -0.4% | -0.5% | 0.0% | -0.8% | -0.9% |
|  | 0.0% | -0.4% | -0.5% | 0.0% | -0.8% | -0.9% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.4% | -0.4% | 0.0% | -0.6% | -0.7% |
| Class B | 0.0% | -0.8% | -1.0% | 0.0% | -0.9% | -0.7% |
| Class C | 0.0% | -0.9% | -0.9% | 0.0% | -0.9% | -0.9% |
| Class D | 0.0% | -1.2% | -1.3% | 0.0% | -1.2% | -1.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.8% | -0.9% | 0.0% | -0.9% | -0.8% |
|  | 0.0% | -0.8% | -0.9% | 0.0% | -0.8% | -0.8% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |

## Test 4

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.4% | 0.0% | -0.2% | -0.1% |
| Class B | 0.0% | 0.5% | 0.6% | 0.0% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | -0.2% | -0.2% | -0.8% | -0.9% |
| Class D | -0.2% | -0.3% | -0.5% | -0.2% | -1.0% | -1.2% |
| Class E | 0.1% | 1.6% | 1.0% | 0.0% | 0.5% | -0.2% |
| Overall | -0.1% | 0.3% | 0.3% | -0.1% | -0.3% | -0.5% |
|  | -0.1% | 0.3% | 0.2% | -0.1% | -0.3% | -0.4% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | -0.4% | 0.0% | -0.2% | -0.1% |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.5% | -0.4% |
| Class C | 0.0% | -0.8% | -0.6% | -0.1% | -0.8% | -0.6% |
| Class D | 0.0% | -0.8% | -1.1% | 0.0% | -0.8% | -0.6% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.4% | -0.6% | 0.0% | -0.5% | -0.4% |
|  | 0.0% | -0.5% | -0.6% | 0.0% | -0.5% | -0.4% |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

# Operations count analysis

In addition, we have carried out an operations count analysis of the various tests. The analysis is carried out for 4x4 operations for chroma residual transform coding.

An equi-probable assumption is used in order to compute the average number of operations. In other words, the Luma mode (35 of them) and the Chroma mode (6 of them) are independent and equi-probable. Then each of the 35x6 combination can occur with a probability of 1/(6*35). The table below shows the resulting chroma intra prediction type used given all possible combinations of signaled Luma and Chroma intra modes.

| Luma Chroma | Planar | Ver | Hor | Ver+8 | DC | Others (X30) |
| --- | --- | --- | --- | --- | --- | --- |
| DM | Planar | Ver | Hor | Ver+8 | DC | Others |
| LM | LM | LM | LM | LM | LM | LM |
| E1 (Planar) | Ver+8 | Planar | Planar | Planar | Planar | Planar |
| E2 (Ver) | Ver | Ver+8 | Ver | Ver | Ver | Ver |
| E3 (Hor) | Hor | Hor | Ver+8 | Hor | Hor | Hor |
| E4 (DC) | DC | DC | DC | DC | Ver+8 | DC |

Each of the above cell occur with a probability of 1/(6*35), and the probability of each chroma intra prediction type being signaled explicitly or as a derived mode can be computed.

The table below summarizes the average operations count computed using this assumption. The accompanying spreadsheet captures the computations used to obtain this result.

| Case | HM4 | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 | Test 6 | Test 7 | Test 8 | Test 9 | Test 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mults | 48 | 50.59 | 53.18 | 53.33 | 55.24 | 50.67 | 50.67 | 54.41 | 53.33 | 50.59 | 53.52 |
| Adds | 96 | 99.89 | 103.77 | 104.00 | 106.86 | 100.00 | 100.00 | 104.11 | 104.00 | 99.89 | 104.29 |
| Shifts | 32 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 |

# Conclusions

We have cross-checked Samsung’s Tool 4 in CE7, studied the algorithm and verified their coding simulation results using an independent implementation.

# References

[1] A. Saxena, et al., “Mode-Dependent DCT/DST for 4x4 Chroma Blocks,” in JCTVC-F553, Torino, Italy, Jul. 2011.

[2] R. Cohen et al., “Description of Core Experiment 7: Additional Transforms,” JCTVC-F907, Torino, Italy, Jul. 2011.

[3] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, Italy, Jul. 2011.