| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G082 M21630 |
| --- | --- |

| Title: | Non-CE9: Modified H positions for memory bandwidth reduction in TMVP derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution advocates modification of the H TMVP positions to avoid co-located motion data required for TMVP derivation of the current LCU going beyond the current LCU row. In the proposed design, the H TVMP positions around the bottom boundary of the current LCU are replaced with closest ones inside the current LCU row (configuration 1), or with C3 TMVP positions (configuration 2). The proposed algorithm enables LCU-aligned motion data compression, storage and fetch, and can reduce the memory bandwidth for the TMVP derivation by half. The measured BD-rate penalty is 0.1% in RA-HE and RA-LC, 0.2% in LB-HE and LB-LC for configuration 1, and 0.2% in RA-HE and RA-LC, 0.3% in LB-HE and LB-LC for configuration 2.

# Introduction

This contribution modifies the H (bottom-right) TMVP (temporal MVP) positions in the merge/skip and AMVP list derivation process, to reduce memory bandwidth required by the TMVP derivation process.

# Problem statement

In the current HM4.0 design, both centre (CR) and bottom-right (H) TMVP positions are used in the derivation process of merge/skip and AMVP MVP lists. Figure 1 illustrates spatial and temporal MVP positions used in the HM4.0 merge/skip list derivation process.  For the merge/skip mode of the current PU, a total of five spatial neighboring PUs (i.e. left (1), upper (2), upper-right (3), bottom-left (4)  and corner (5) neighboring PUs) are checked. For temporal MVP, the H TMVP position is first checked, and if MVP at H position is unviable, the CR TMVP position is further checked.

Current PU

4

2

5

1

Co-located PU

CR 6

H 6

3

Figure 1. Illustration of the merge/skip MVP list construction of HM4.0

The major drawback of the current HM4.0 for the TMVP derivation is that the co-located motion data fetch area is not aligned with the current LCU. Because of H TMVP position, the co-located motion data fetch area can go beyond LCU boundaries. As shown in Figure 2, with the current HM4.0 design an 80x80 co-located motion data area (red box in Figure 2) needs to be pre-fetched for a LCU of 64x64 size. The 80x80 co-located area touches motion data of four LCUs in the co-located frame.

LCUs

HM4.0

Co-located motion data fetch area

Figure 2, Illustration of spatial relationship between LCU and co-located motion data fetch area in HM4.0

HM4.0 uses motion data compression to lower the memory bandwidth. It is highly desired that motion  data can be compressed, stored and fetched in 64x64 LCU aligned tile format to ensure DDR burst alignments and avoid page crossing, and thus to minimize the memory bandwidth requirement As shown in Table 1, after compression motion data size for each 16x16 block area is about 74 bits, roughly 10 bytes,. And for a 64x64 LCU, the total motion data after compression is about 148 bytes, which is roughly 19, 10, and 5 bursts for DDR burst  size of 8, 16, 32 bytes, respectively.

| Motion data | Size of data after compression for 16x16 block (bits) |
| --- | --- |
| Motion vectors | 64 (2 x 2 x 16) |
| Intra/inter flag + Prediction direction | 2 |
| Ref_idx | 8 ( 2 x 4) |
| Total for 16x16 (bits) | 74 (bits) = ~10 (bytes) |
| Total for 64x64 LCU (bytes) | 148 (bytes) |

Table 1.  Motion data size after compression for 16x16 and 64x64 block size

If the motion data is compressed and stored in 64x64 LCU aligned tile format, it would require to load motion data from four LCUs in the co-located frame for the TMVP derivation of the current LCU. This has high memory bandwidth implication. One solution to lower the memory bandwidth is to add a line buffer to maintain the co-located motion data for the previous LCU row, so that the motion data can be re-used by the TMVP derivation process of the current LCU row. But line buffer is costly and not preferred.

# Algorithm Description

We propose to modify the H TMVP positions to resolve the misalignment issue discussed above. Figure 3 depicts the TMVP positions for PUs of a LCU. In the proposed algorithm, if H TMVP positions are located in the next LCU row, they are replaced by closest ones of the current LCU row.

PU0

Central TMVP positions

(Inside LCU&PU)

Modified H TMVP

positions

PU1

PU2

PU6

PU3

PU7

PU4

PU8

CU0

CU1

CU2

CU3

CU/PU partitioning of the current LCU

Bottom neighboring LCU

Bottom-right neighboring LCU

Right neighboring LCU

H TMVP positions

PU5

Figure 3.  Modified H TMVP positions for PUs around the bottom LCU boundary (config. 1)

In the example shown in Figure 3, the H TMVP position for PU4, PU7 and PU8 are replaced with blue ones. In this way, the motion data required for  TMVP derivation of the current LCU won’t go beyond the current LCU row.

PU0

C3 TMVP positions

(Inside LCU&PU)

PU1

PU2

PU6

PU3

PU7

PU4

PU8

CU0

CU1

CU2

CU3

CU/PU partitioning of the current LCU

Bottom neighboring LCU

Bottom-right neighboring LCU

Right neighboring LCU

H TMVP positions

PU5

Figure 4.  Modified H TMVP positions for PUs around the bottom LCU boundary (config 2)

An alternative configuration is shown in Figure 4, in which center TMVP positions and H positions located in the next LCU row are all replaced with C3 TMVP positions.

LCUs

HM4.0

Co-located motion data fetch area

Figure 5, Illustration of spatial relationship between LCU and co-located motion data fetch area in proposed algorithm

As shown in Figure 5, with the proposed modification (both configuration 1 and 2) the motion data fetch area for a 64x64 LCU can be reduced from 80x80 to 80 x 64, motion data can be comfortably compressed, stored and fetched in 64x64 LCU aligned format. For the TMVP derivation of the current LCU, only the motion data of co-located LCU needs to be fetched, provided that the motion data of the left-neighboring LCU is buffered for re-use by the current LCU. That is, a sliding window of size of two LCU can be used in the proposed motion for motion data buffering to avoid fetching the same motion data multiple times. Compared to the current HM4.0 design, this effectively cuts the memory bandwidth for the TMVP derivation process by half.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

Table 2 provides the simulation results. Compared to HM4.0, there is an average loss of 0.1% in RA-HE and RA-LC, and 0.2% in LB-HE and LB-LC in configuration 1 (see Table 2), and average loss of 0.2% in RA-HE and RA-LC, and 0.3% in LB-HE and LB-LC in configuration 2 (see Table 3). The direct BD-rate comparison between configuration 1 and 2 in Table 4 reveals hardly BD-rate difference between two configurations.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.0% | 0.1% | 0.2% | 0.1% | 0.3% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class C | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.2% | -0.1% | 0.3% | 0.2% | 0.4% |
| Class C | 0.2% | 0.1% | 0.3% | 0.2% | 0.3% | 0.1% |
| Class D | 0.1% | 0.2% | -0.3% | 0.1% | 0.4% | 0.0% |
| Class E | 0.4% | 0.8% | 0.9% | 0.3% | 0.5% | 0.3% |
| Class F |  |  |  |  |  |  |
| Overall | 0.2% | 0.3% | 0.2% | 0.2% | 0.3% | 0.2% |
|  | 0.2% | 0.3% | 0.1% | 0.2% | 0.3% | 0.2% |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 93% |  |  | 94% |  |  |

Table 2.  BD-rate increase relative to HM4.0 anchor for configuration 1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% | 0.2% |
| Class B | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% |
| Class C | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.2% | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 110% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.2% | 0.2% | 0.3% | 0.3% | 0.1% |
| Class C | 0.3% | 0.2% | 0.1% | 0.3% | 0.3% | 0.0% |
| Class D | 0.2% | 0.3% | 0.3% | 0.2% | 0.2% | -0.2% |
| Class E | 0.4% | 0.7% | 0.6% | 0.3% | 0.5% | -0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.0% |
|  | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |

Table 3.  BD-rate increase relative to HM4.0 anchor for configuration 2

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 110% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.3% | 0.1% | 0.1% | -0.3% |
| Class C | 0.1% | 0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.6% | 0.1% | -0.2% | -0.1% |
| Class E | 0.0% | -0.1% | -0.3% | 0.0% | 0.0% | -0.6% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
|  | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | -0.3% |
| Enc Time[%] | 103% |  |  | 105% |  |  |
| Dec Time[%] | 109% |  |  | 114% |  |  |

Table 4.  BD-rate comparison (anchor configuration 1, tested configuration 2)

# Conclusions and recommendation

The proposed modifications of H TMVP positions can reduce the memory bandwidth for TMVP derivation by half. It is recommended to adopt one of the modifications into the test model to simplify the design.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Zhou, “CE1:  Evaluation results  on A.09, A.13-16 and an alternative solution”. JCT-VC Document, : JCTVC-F081, Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text changes

##### For configuration 1 which replaces the H positions outside the current LCU row with the closest ones inside the current LCU row

##### Replace 8.4.2.1.9

##### Derivation process for temporal luma motion vector prediction

With (changes are marked in yellow)

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-1)

if  (yP >> log2_lcu_size) is equal to (yPRb >>log2_lcu_size)

yPRb = yP + nPSH	(8-2)

Otherwise

yPRb =  yP + nPSH - 4	(8-3)

where log2_lcu_size =  log2_min_coding_block_size_minus3 +              log2_diff_max_min_coding_block_size + 3;

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-4)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-5)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

      - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-6)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-7)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-8)

mvLXCol = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
				( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-9)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-10)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-11)

##### For configuration 2 which replaces the H positions outside the current LCU row with appropriate C3 ones inside the current LCU row

##### Replace 8.4.2.1.9

##### Derivation process for temporal luma motion vector prediction

With (changes are marked in yellow)

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- The variable colPu is derived as follows

yPRb = yP + nPSH	(8-12)

  - If ( yP >> Log2MaxCuSize ) is equal to ( yPRb >> Log2MaxCuSize ), the horizontal component of the right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-13)

and the variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

  - Otherwise ( ( yP >> Log2MaxCuSize ) is not equal to ( yPRb >> Log2MaxCuSize ) ), colPu is marked as unavailable.

-

-

-

-

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic. Otherwise, colPu is marked as unavailable.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) 	(8-18)

yPCtr = ( yP + ( nPSH >> 1 ) 	(8-19)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

      - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-20)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-21)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-22)

mvLXCol = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
				( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-23)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-24)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-25)