| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G083 M21631 |
| --- | --- |

| Title: | CE4: Test results on compact representation of quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1-214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This document reports CE4 test results on compact representation of quantization matrices. Due to large block size transform and quantization used in the HEVC, carrying the quantization matrices in bitstream could lead to a significant overhead. In this document, a quantization matrix compression algorithm is proposed to achieve compact representation of quantization matrices for HEVC. Two tests are carried out, test1 evaluates the compression efficiency of the algorithm itself, test2 quantifies the BD-rate saving of the proposed algorithm against the AVC method when quantization matrices are changing from frame to frame. Test results showed that the algorithm provided up to 12x more compression (test1) and up to 56.8% BD-rate saving when compared to the AVC quantization matrix compression method if all the compression tools are enabled It is recommended to adopt the AVC quantization method into the test model, and conduct further CE on this subject to specify the compact quantization matrix representation format for efficient carriage of quantization matrices in HEVC.

# Introduction

In the current HEVC design the transform and quantization block sizes can go up to 32x32. The large block transforms can provide improved coding efficiency, but also lead to larger overhead for carrying the perceptual quantization matrices in the picture parameter sets. In HEVC there can be a total of 24 matrices (separate matrix for 4x4, 8x8, 16x16 and 32x32 four block sizes, intra/inter and Y, U, V components). It was reported in [3] that such an overhead can be roughly 10x of that of AVC if the AVC quantization matrix compression method is used. Therefore, it is desirable to design a compact representation format for encoding the quantization matrices of large block sizes.

In this contribution the quantization matrix compression algorithm proposed in [3] was further evaluated in CE4. The algorithm was integrated into the HM4.0 based CE4 software, and tested by using the test vectors agreed within the CE4 group.

# Algorithm Description

The diagram of proposed algorithm is depicted in Figure 1. On the top of AVC method, the following additional tools are added to enhance the capability of quantization matrix compression:

- quantization matrix down-sampling (down-sample)

- 135 degree symmetry processing (sym135)

- 45 degree symmetry processing with offset (sym45)

- unsigned exp-golomb coding (UEK)

- quantization matrix prediction of same size

The proposed algorithm is backward compatible to the AVC method, i.e. the additional compression tools can be enabled and disabled by high level flags in arbitrary manner, all the combinations of above tools are allowed. The detailed description of the algorithm can be found in [3].

Figure1.  Diagram of proposed quantization matrix compression algorithm

# Test Settings and Conditions

The quantization matrices used for testing are both asymmetry and symmetry ones provided to CE4 by SONY. Attached text files qmatrix_asymmetry1.txt and qmatrix_symmetry1.txt contain quantization matrices used in testing.

The simulation is intended to demonstrate the compression ratio and associated quantization matrix compression errors of the proposed algorithm relative to the AVC method. The effect of quantization matrix compression errors on video quality is not evaluated in the tests.

Two tests are defined. The first test (test1) is to evaluate the compression efficiency of the proposed algorithm  itself  for symmetric and asymmetric matrices; the second test (test2) is to quantify the BD-rate saving against the  AVC method when the quantization matrices change from frame to frame.

# Experimental results

For test 1 a summary of quantization matrix compression results is provided in Table 1. In this case the input matrices are modified by adding small offsets to the matrices (matrix dithering. In the table the number of bits is total of bits consumed by encoding a total of 24 quantization matrices (intra/inter, Y, U, V for 4x4, 8x8, 16x16 and 32x32 block sizes), the compression ratio is computed against lossless AVC method, the average quantization matrix compression error is calculated by averaging compression errors over all the 24 matrices. As shown in Table 1, the proposed algorithm supports both lossless mode (i.e. AVC method when all the four compression tools are turned off) and lossy mode of 15 different compression levels.

The major observations are:

- using AVC method as is for encoding quantization matrices of HEVC can lead to significant overhead (21 – 29K bits observed in this particular case).

- for symmetry matrices a factor  of 1.8 lossless compression can be achieved.

- matrix down-sampling  appears an efficient compression tool that offers 2.4x compression without introducing significant  compression error

- up to 12x more compression can be achieved relative to AVC method if all the four tools are turned on in the proposed algorithm.

| Qmatx_compression_ID |  |  |  | Symmetry matrices |  |  | Asymmetry matrices |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Down sample | UEK | sym45 | sym135 | Number of bits | Comp. ratio | Avg. error | Number of bits | Comp. ratio | Avg. error |
| 0 | 0 | 0 | 0 | 27226 | 1.00 | 0.00 | 29464 | 1.00 | 0.00 |
| 0 | 0 | 0 | 1 | 14803 | 1.84 | 0.00 | 14932 | 1.97 | 3.53 |
| 0 | 0 | 1 | 0 | 15418 | 1.77 | 5.06 | 13639 | 2.16 | 3.14 |
| 0 | 0 | 1 | 1 | 8585 | 3.17 | 5.06 | 8005 | 3.68 | 5.25 |
| 0 | 1 | 0 | 0 | 11168 | 2.44 | 3.69 | 10404 | 2.83 | 6.66 |
| 0 | 1 | 0 | 1 | 6749 | 4.03 | 3.47 | 6836 | 4.31 | 6.76 |
| 0 | 1 | 1 | 0 | 6554 | 4.15 | 7.83 | 5941 | 4.96 | 6.88 |
| 0 | 1 | 1 | 1 | 4223 | 6.45 | 7.47 | 4067 | 7.24 | 6.92 |
| 1 | 0 | 0 | 0 | 11551 | 2.36 | 0.43 | 12126 | 2.43 | 0.32 |
| 1 | 0 | 0 | 1 | 6685 | 4.07 | 0.43 | 6655 | 4.43 | 3.76 |
| 1 | 0 | 1 | 0 | 6996 | 3.89 | 4.73 | 5829 | 5.05 | 3.54 |
| 1 | 0 | 1 | 1 | 4330 | 6.29 | 4.73 | 3699 | 7.97 | 5.52 |
| 1 | 1 | 0 | 0 | 5733 | 4.75 | 3.52 | 4652 | 6.33 | 6.21 |
| 1 | 1 | 0 | 1 | 3683 | 7.39 | 3.16 | 3731 | 7.90 | 6.30 |
| 1 | 1 | 1 | 0 | 3764 | 7.23 | 6.92 | 2819 | 10.45 | 6.76 |
| 1 | 1 | 1 | 1 | 2566 | 10.61 | 6.49 | 2389 | 12.33 | 6.91 |

Table 1.  Summary of quantization matrix compression results (CE4 subset 3.2 test1, w/ input matrix “dithering”)

Additional tests are conducted. In this case the input matrices are not modified and matrix prediction of same size is used so that only unique matrices are coded. Because input matrices are identical for Y, U, V of a same size, the number of matrix bits in table 2 is roughly one third of that in table 1.

| Qmatx_compression_ID |  |  |  | Symmetry matrices |  |  | Asymmetry matrices |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Down sample | UEK | sym45 | sym135 | Number of bits | Comp. ratio | Avg. error | Number of bits | Comp. ratio | Avg. error |
| 0 | 0 | 0 | 0 | 8856 | 1.00 | 0.00 | 9512 | 1.00 | 0.00 |
| 0 | 0 | 0 | 1 | 4864 | 1.82 | 0.00 | 4894 | 1.94 | 3.53 |
| 0 | 0 | 1 | 0 | 5052 | 1.75 | 5.06 | 4558 | 2.09 | 3.14 |
| 0 | 0 | 1 | 1 | 2852 | 3.11 | 5.06 | 2626 | 3.62 | 5.25 |
| 0 | 1 | 0 | 0 | 3390 | 2.61 | 3.78 | 3442 | 2.76 | 6.68 |
| 0 | 1 | 0 | 1 | 2102 | 4.21 | 3.53 | 2126 | 4.47 | 6.83 |
| 0 | 1 | 1 | 0 | 2012 | 4.40 | 7.91 | 1968 | 4.83 | 6.90 |
| 0 | 1 | 1 | 1 | 1352 | 6.55 | 7.52 | 1286 | 7.40 | 6.95 |
| 1 | 0 | 0 | 0 | 3631 | 2.44 | 0.43 | 3784 | 2.51 | 0.32 |
| 1 | 0 | 0 | 1 | 2137 | 4.14 | 0.43 | 2094 | 4.54 | 3.76 |
| 1 | 0 | 1 | 0 | 2263 | 3.91 | 4.73 | 1923 | 4.95 | 3.54 |
| 1 | 0 | 1 | 1 | 1399 | 6.33 | 4.73 | 1249 | 7.62 | 5.52 |
| 1 | 1 | 0 | 0 | 1391 | 6.37 | 3.95 | 1380 | 6.89 | 6.35 |
| 1 | 1 | 0 | 1 | 1001 | 8.85 | 3.47 | 998 | 9.53 | 6.65 |
| 1 | 1 | 1 | 0 | 933 | 9.49 | 7.28 | 905 | 10.51 | 6.81 |
| 1 | 1 | 1 | 1 | 774 | 11.44 | 6.69 | 693 | 13.73 | 7.07 |

Table 2.  Summary of quantization matrix compression results (CE4 subset 3.2 test1, w/o input matrix “dithering”)

For test2 results for asymmetric and symmetric matrices are summarized in Table 3 and Table 4, respectively. For the consistency of CE4 results, here AVC anchor of SONY mode is used.  Actually Qmatx_compression_ID = 0 here is equlivent  to AVC mode, with the exception that the high-level compression mode signaling might be slightly different. As shown in Table 3 and Table 4, the proposed algorithm provides significant bit-rate saving when compared to the AVC method, up to 56.8% saving was observed when all the compression tools are turned on.

| Qmatx_compression_ID |  |  |  | BD-rate (Y) saving (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Down sample | UEK | sym45 | sym135 | AI-HE | AI-LC | RA-HE | RA-LC | LB-HE | LB-LC |
| 0 | 0 | 0 | 0 | 0.4 | 0.3 | 1.2 | 1.1 | 1.5 | 1.5 |
| 0 | 0 | 0 | 1 | -4.4 | -4.2 | -20.7 | -20.1 | -27.4 | -26.9 |
| 1 | 0 | 0 | 0 | -9.4 | -9.4 | -28.2 | -27.4 | -36.3 | -35.4 |
| 1 | 1 | 1 | 1 | -10.4 | -10.9 | -42.7 | -41.7 | -56.4 | -55.5 |

Table 3.  Summary of  BD-rate saving relative to AVC anchor for asymmetric matrices (CE4 subset 3.2 test2)

| Qmatx_compression_ID |  |  |  | BD-rate (Y) saving (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Down sample | UEK | sym45 | sym135 | AI-HE | AI-LC | RA-HE | RA-LC | LB-HE | LB-LC |
| 0 | 0 | 0 | 0 | 0.4 | 0.4 | 1.2 | 1.2 | 1.5 | 1.5 |
| 0 | 0 | 0 | 1 | -6.8 | -6.3 | -21.0 | -20.4 | -27.0 | -26.4 |
| 1 | 0 | 0 | 0 | -8.3 | -7.9 | -27.8 | -27.0 | -36.0 | -35.2 |
| 1 | 1 | 1 | 1 | -10.4 | -8.7 | -43.0 | -41.4 | -56.8 | -55.5 |

Table 4.  Summary of  BD-rate saving relative to AVC anchor for symmetric matrices (CE4 subset 3.2 test2)

For detailed results please refer to attached spreadsheets. Table 4 provides mapping between the spreadsheets and test settings.

| Tested Qmatx_compression_ID |  |  |  | Quantization Matrix | Results |
| --- | --- | --- | --- | --- | --- |
| Down sample | UEK | sym45 | sym135 |  |  |
| 0 | 0 | 0 | 0 | asymmetry | CE4_Test2_asymmetry_avc_vs_cmp00.xls |
| 0 | 0 | 0 | 1 | asymmetry | CE4_Test2_asymmetry_avc_vs_cmp01.xls |
| 1 | 0 | 0 | 0 | asymmetry | CE4_Test2_asymmetry_avc_vs_cmp08.xls |
| 1 | 1 | 1 | 1 | asymmetry | CE4_Test2_asymmetry_avc_vs_cmp15.xls |
| 0 | 0 | 0 | 0 | symmetry | CE4_Test2_symmetry_avc_vs_cmp00.xls |
| 0 | 0 | 0 | 1 | symmetry | CE4_Test2_symmetry_avc_vs_cmp01.xls |
| 1 | 0 | 0 | 0 | symmetry | CE4_Test2_symmetry_avc_vs_cmp08.xls |
| 1 | 1 | 1 | 1 | symmetry | CE4_Test2_symmetry_avc_vs_cmp15.xls |

Table 5.  Mapping of spreadsheets and test settings

# Conclusions and recommendation

Due to large block size transform and quantization used in the HEVC, carrying the quantization matrices can lead to significant overhead. The proposed quantization matrix compression algorithm provides significant BD-rate saving when compared to the AVC method. It is recommended to adopt the AVC quantization matrix compression algorithm into the HEVC test model, and conduct further CE on this subject to evaluate proposals in this category for specification of compact quantization matrix representation format for HEVC.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Zhou, V. Sze, “Compact representation of quantization matrices for HEVC”. JCT-VC Document, : JCTVC-D024, Daegu, Korea, January, 2011

[4] M. Zhou, V. Sze, “Further study on compact representation of quantization matrices for HEVC”. JCT-VC Document, : JCTVC-F085, Torino, IT, 14-22 July, 2011

[5] K.Sato, M. Budagavi, M. Coban, H.Aoki,  X.Li,  “Description of Core Experiment 4: Quantization”, JCT-VC Document,  JCTVC-F904, Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).