| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G084 M21632 |
| --- | --- |

| Title: | CE9: Test results on SP01, SP02, SP3 and SP04 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou | Tel: Email: | +1 214.480.3816 zhou@ti.com |
| Source: | Texas Instruments Inc |  |  |

_____________________________

# Abstract

This document reports CE9 test results on SP01, SP02, SP03 and SP04 which are related to temporal MVP. Test results reveal that disabling the centre TMVP position from both the merge/skip and AMVP MVP list derivation process leads to a loss of 0.1% in all configurations(SP01); removing TMVP from the AMVP list derivation process causes an average loss of 0.3% in RA-HE, 0.4% (0.3%) in RA-LC, 0.6% in LB-HE and 0.7% in LB-LC (SP03, SP04); and disabling TMVP from both the merge/skip and AMVP MVP list derivation process leads to an average loss of 2.4% in RA-HE, 2.6% in RA-LC, 3.4% in LB-HE and 4.0% in LB-LC (SP04).

# Introduction

# This document reports CE9 test results on the following simplifications:

- SP01: remove center TMVP position from both the merge/skip and AVP list derivation process

- SP02: disable TMVP in the AMVP list derivation process but still used in motion estimation as one of initial motion vectors

- SP03: disable TMVP in both the AMVP list derivation process and in motion estimation

- SP04: disable TMVP in both the merge/skip and AMVP list derivation process

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The test results are summarized in Table 1. From HM3.0 to HM4.0 the TMVP has become more important in the merge/skip and AMVP list derivation process. As shown in Table 1, disabling the centre TMVP position leads to a loss of 0.1% (SP01); removing TMVP from AMVP causes an average loss of 0.3% in RA-HE, 0.4% in RA-LC, 0.6% in LB-HE and 0.7% in LB-LC (SP03); and the gain of using TMVP in both merge/skip and AMVP is 2.4% in RA-HE, 2.6% in RA-LC, 3.4% in LB-HE and 4.0% in LB-LC (SP04);

| Tests | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| SP01 | 0.1 | 0.1 | 0.1 | 0.1 |
| SP02 | 0.3 | 0.3 | 0.6 | 0.7 |
| SP03 | 0.3 | 0.4 | 0.6 | 0.7 |
| SP04 | 2.4 | 2.6 | 3.4 | 4.0 |

Table 1, Summary of testing results

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% | 0.2% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.2% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 102% |  |  | 108% |  |  |
| Dec Time[%] | 105% |  |  | 120% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | 0.0% | 0.2% | 0.2% | 0.1% | -0.1% |
| Class D | 0.2% | 0.1% | 0.2% | 0.2% | 0.4% | 0.0% |
| Class E | 0.0% | 0.2% | 0.8% | 0.0% | 0.0% | 0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.3% | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 104% |  |  | 114% |  |  |

Table 2, BD-rate increase relative to HM4.0 anchor when the center TMVP position is removed (SP01)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.2% | 0.3% | 0.3% | 0.3% | 0.4% |
| Class B | 0.2% | 0.2% | 0.1% | 0.3% | 0.1% | 0.2% |
| Class C | 0.3% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% |
| Class D | 0.4% | 0.2% | 0.1% | 0.4% | 0.2% | 0.4% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.3% | 0.2% | 0.2% | 0.3% | 0.2% | 0.3% |
|  | 0.3% | 0.2% | 0.2% | 0.3% | 0.2% | 0.3% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5% | 0.5% | 0.4% | 0.6% | 0.9% | 1.0% |
| Class C | 0.6% | 0.5% | 0.5% | 0.7% | 0.9% | 0.6% |
| Class D | 0.8% | 1.2% | 0.8% | 0.9% | 1.2% | 1.2% |
| Class E | 0.3% | 0.1% | 0.4% | 0.3% | 0.4% | 0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.6% | 0.6% | 0.5% | 0.7% | 0.9% | 0.8% |
|  | 0.6% | 0.5% | 0.5% | 0.7% | 0.8% | 0.8% |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 108% |  |  | 101% |  |  |

Table 3, BD-rate increase relative to HM4.0 anchor when TMVP is disabled in AMVP but still used in ME (SP02)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | 0.3% | 0.3% | 0.4% | 0.5% | 0.6% |
| Class B | 0.2% | 0.2% | 0.2% | 0.3% | 0.2% | 0.2% |
| Class C | 0.4% | 0.3% | 0.4% | 0.4% | 0.3% | 0.4% |
| Class D | 0.4% | 0.4% | 0.2% | 0.4% | 0.2% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.3% | 0.3% | 0.3% | 0.4% | 0.3% | 0.4% |
|  | 0.3% | 0.3% | 0.3% | 0.4% | 0.3% | 0.4% |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.6% | 0.5% | 0.5% | 0.6% | 0.8% | 0.8% |
| Class C | 0.7% | 0.7% | 0.7% | 0.8% | 0.9% | 0.5% |
| Class D | 0.8% | 0.6% | 0.9% | 0.9% | 1.3% | 0.9% |
| Class E | 0.3% | -0.1% | 0.4% | 0.3% | 0.4% | 0.6% |
| Class F |  |  |  |  |  |  |
| Overall | 0.6% | 0.5% | 0.6% | 0.7% | 0.9% | 0.7% |
|  | 0.6% | 0.4% | 0.6% | 0.7% | 0.9% | 0.7% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

Table 4, BD-rate increase relative to HM4.0 anchor when TMVP is disabled in both AMVP and in ME (SP03)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 2.5% | 2.0% | 2.0% | 2.7% | 2.4% | 2.5% |
| Class B | 2.2% | 1.5% | 1.5% | 2.3% | 1.4% | 1.5% |
| Class C | 2.3% | 2.0% | 2.1% | 2.4% | 1.9% | 2.0% |
| Class D | 2.7% | 2.0% | 2.0% | 2.9% | 1.9% | 2.2% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 2.4% | 1.8% | 1.9% | 2.6% | 1.9% | 2.0% |
|  | 2.4% | 1.9% | 1.9% | 2.6% | 1.9% | 2.0% |
| Enc Time[%] | 95% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 3.3% | 2.8% | 2.4% | 4.3% | 2.8% | 3.2% |
| Class C | 3.6% | 3.4% | 3.5% | 4.0% | 3.4% | 3.6% |
| Class D | 3.2% | 2.8% | 2.5% | 3.6% | 3.0% | 2.9% |
| Class E | 3.5% | 3.2% | 3.8% | 4.2% | 2.8% | 3.1% |
| Class F |  |  |  |  |  |  |
| Overall | 3.4% | 3.0% | 2.9% | 4.0% | 3.0% | 3.2% |
|  | 3.4% | 3.0% | 2.9% | 4.0% | 3.0% | 3.2% |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 95% |  |  | 94% |  |  |

Table 4, BD-rate increase relative to HM4.0 anchor when TMVP is disabled in both AMVP and  in merge/skip (SP04)

# Conclusion and recommendation

Based on the test results, we recommend retain the center TMVP position and retain TMVP in the AMVP list derivation process, and re-test them in a later stage when the HEVC design is fully stabilized.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Zhou, “CE1: Evaluation results on A.09, A.13-16 and an alternative solution”. JCT-VC Document, JCTVC-F081, Torino, IT, July 2011.

[4] M. Zhou, “CE9: Evaluation results on disabling TMVP from AMVP list construction (ROB05)”. JCT-VC Document, JCTVC-F083, Torino, IT, July 2011.

[5] B. Bross, W. Chien, I. Kim, M. Zhou, “Core Experiment 9: MV Coding and Skip/Merge Operations”, JCT-VC Document, JCTVC-F909, Torino, IT, July 2011.

# Patent rights declaration(s)

Texas Instruments, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.