| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G085 M21633 |
| --- | --- |

| Title: | CE9: Test results on parallelized merge/skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou | Tel: Email: | +1 214.480.3816 zhou@ti.com |
| Source: | Texas Instruments Inc |  |  |

_____________________________

# Abstract

# This document reports CE9 test results on parallel merge/skip mode. The current HEVC merge/skip mode design is highly sequential and introduces dependency among neighboring PUs, which can lead to significant quality loss if motion estimation (ME) is performed in parallel for throughput or implementation cost reasons. For typical parallel ME level of 32x32, the measured average loss is 5.0% in RA-HE, 5.3% in RA-LC, 6.7% in LB-HE and 7.8% in LB-LC. The loss is caused by fact that the merge/skip mode cannot be tested for those PUs inside the 32x32 block whose neighboring motion data are still unavailable during the parallel processing process. It is proposed to add a high-level syntax element to signal the parallel level of merge/skip mode, divide a LCU into parallel motion estimation regions (MERs) and allow only those neighboring PUs which belong to different MERs from the current PU to be included in the merge/skip MVP list construction process.  Simulation results reveal that an average gain of 3.4% in RA-HE, 3.5% in RA-LC, 4.4% in LB-HE and 5.0% in LB-LC can be achieved for 32x32 block level parallel ME when compared to the current HM4.0 design, and for parallel level 16x16 that is used today, the average gain is 1.9% in RA-HE, 1.8% in RA-LC, 2.6% in LB-HE and 2.5% in LB-LC. The proposed design is backward compatible to the current design but offers flexibility for high throughput and high quality encoder designs.

# Introduction

# The current HEVC merge/skip mode design [2] is highly sequential and introduces dependency among neighboring PUs, which creates difficulty for the motion estimation on the encoder side for the pipelined architecture, in which motion vectors might be estimated in parallel for multiple PUs. The parallelized merge/skip mode [3] is further investigated in CE9. This document reports CE9 results on parallel merge/skip mode.

# Problem statement

Current PU

4

2

5

1

Co-located PU

CR 6

H 6

3

Figure 1. Illustration of the merge/skip MVP list construction of HM4.0

Figure 1 illustrates the MVP list construction of the merge/skip mode defined in HM4.0. For the merge/skip mode of the current PU, a total of five neighboring PUs (i.e. left (1), upper (2), upper-right (3), bottom-left (4)  and corner (5) neighboring PUs) and temporally co-located PUs (CR 6 and H6) are checked to form the initial merge/skip MVP list. A MVP candidate of a spatial/temporal neighboring PU of lower index in Figure 1 has higher priority in the MVP list.  The reference index for temporal TMVP derivation also depends on the availability and reference index of neighbors of the current PU.

2N x2N

N x 2N

2N xN

N xN

Figure 2. For a block of 2N x 2N, merge/skip search is only possible for the first PU of each partition if motion estimation is carried out in parallel for entire 2N x 2N region.

With the current HM4.0 design, if motion estimation is carried out in parallel for entire 2N x 2N block region, only the first PU of each partition (see green blocks in Figure 2) is able to have merge/skip mode searched in parallel to the regular motion estimation process (the regular motion estimation process estimates motion vectors for a PU independent of neighboring PUs), the rest of PUs will have to drop testing of the merge and skip mode because the neighboring motion data (motion vector, prediction direction, reference index, prediction size, intra/inter flag) are still unavailable when the parallel motion estimation is taking place. In general, for recursive motion estimation in which a 2N x 2N CU can be further partitioned into smaller CUs, only those PUs whose top-left corner sample is coincident with the top-left corner sample of the 2N x 2N block are able to test merge/skip mode in parallel motion estimation environment.   This can lead to significant quality loss because merge and skip mode have great impact on video quality. As shown in Table 1, significant quality loss is observed if the parallel motion estimation is carried out at 64x64 LCU level and 32x32 block level.

| Parallel ME level 2N x 2N (LCU = 64x64) | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| 64 x 64 | 6.8 | 7.7 | 8.1 | 10.9 |
| 32 x 32 | 5.0 | 5.3 | 6.7 | 7.8 |
| 16 x 16 | 2.4 | 2.3 | 3.4 | 3.3 |
| 8 x 8 | 0.2 | 0.2 | 0.4 | 0.3 |

Table 1, BD-rate increase relative to modified HM4.0 anchor when the parallel motion estimation is performed at 2N x 2N block level (N = 64, 32, 16 and 8, LCU size is fixed to 64x64)

Detailed results are reported in attached spreadsheets:

HM-4.0-pme_anchor4_vs_anchor8.xls

HM-4.0-pme_anchor4_vs_anchor16.xls

HM-4.0-pme_anchor4_vs_anchor32.xls

HM-4.0-pme_anchor4_vs_anchor64.xls

Parallel motion estimation is needed for practical video encoder implementation for meeting throughput requirements and lowering implementation cost of the motion estimation engine. For example, for the previous standards including AVC the typical parallel motion estimation level is 16x16, which is aligned with the macroblock size. For AVC, some encoders are doing 16 x 32 level parallel motion estimation for MBAFF support. For HEVC in which LCU size can go up to 64 x 64 and UHD (4K x 2K or 8K x 4K )  application is expected, performing parallel motion estimation at 32 x 32 can become a minimum requirement to meet real-time requirements  and cost constraints.

Please note that the anchor data used in this document has been modified. In HM4.0 regular motion estimation for AMVP is disabled under certain conditions for encoder speedup, but parallel merge/skip mode would disable merge estimation too in certain cases, this leads to encoder crash. In order to achieve fair comparison, the AMVP encoder speedup is disabled in the tests. The BD-rate difference between the modified HM4.0 anchor and HM4.0 anchor is listed in Table 2.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.7% | -0.8% | -0.2% | -0.3% | -0.3% |
| Class B | -0.3% | -0.4% | -0.3% | -0.2% | -0.3% | -0.1% |
| Class C | -0.2% | -0.3% | -0.2% | -0.2% | -0.1% | -0.2% |
| Class D | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -0.2% | -0.4% | -0.4% | -0.2% | -0.2% | -0.2% |
|  | -0.2% | -0.4% | -0.4% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 111% |  |  | 116% |  |  |
| Dec Time[%] | 99% |  |  | 109% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.4% | -0.6% | -0.3% | -0.3% | -0.4% |
| Class C | -0.3% | -0.2% | -0.5% | -0.2% | 0.0% | -0.3% |
| Class D | -0.3% | -0.3% | 0.2% | -0.2% | -0.4% | -0.1% |
| Class E | -0.6% | -0.4% | 0.2% | -0.4% | -0.5% | -0.7% |
| Class F |  |  |  |  |  |  |
| Overall | -0.4% | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% |
|  | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 119% |  |  | 112% |  |  |
| Dec Time[%] | 101% |  |  | 96% |  |  |

Table 2, BD-rate (Y) difference between the modified anchor and HM4.0 anchor (see HM-4.0-anchor_vs._pme_anchor4.xls).

# Algorithm description

To reduce the quality loss in the parallel motion estimation environment, we propose to modify the spatial MVP availability rules for merge/skip MVP list to enhance the parallelism of merge/skip mode, so that  merge/skip mode can be tested for all the PUs during the parallel motion estimation. The key parts of the algorithm are as follows:

- Define a high level syntax element (log2_parallel_merge_level_minus2) in picture parameter set to indicate parallel processing level of merge/skip mode.

- Based on the value of log2_parallel_merge_level_minus2, divide a LCU into a number of non-overlapped of motion estimation regions (MER). A MER has square shape.

- Modify the spatial MVP availability rules:  if a neighboring PU and the current PU belong to a same MER, this neighboring PU is treated as unavailable for spatial MVP derivation of the merge/skip MVP list construction process.

- Modify the temporal MVP reference index derivation rules: if a neighboring PU and the current PU belong to a same MER, this neighboring PU is treated as unavailable for the temporal MVP reference index derivation process.

## Definition of  log2_parallel_merge_level_minus2

A high level syntax element log2_parallel_merge_level_minus2 is added to slice header to signal the parallel merge/skip level. log2_parallel_merge_level_minus2 can have value of 0, 1, 2, 3 and 4, and 1 <<( log2_parallel_merge_level_minus2+2) shall not exceed the LCU size defined in the sequence parameter set.

The parallel merge/skip levels defined by log2_parallel_merge_level_minus2 are present in Table 3.

| log2_parallel_merge_level_minus2 | MER size | Remark |
| --- | --- | --- |
| 0 | 4x4 | Sequential merge/skip mode for all PUs in a LCU because minimum PU size allowed by HEVC is 4x4 |
| 1 | 8x8 | Parallel merge/skip mode search allowed for all PUs inside an 8x8 block |
| 2 | 16x16 | Parallel merge/skip mode search allowed for all PUs inside a 16x16 block |
| 3 | 32x32 | Parallel merge/skip mode search allowed for all PUs inside a 32x32 block |
| 4 | 64x64 | Parallel merge/skip mode search allowed for all PUs inside a 64x64 block |

Table 3, parallel merge/skip level defined by log2_parallel_merge_level_minus2

## Partitioning a LCU into motion estimation regions (MERs)

As shown in Figure 3, a LCU (e.g. 64x64 size) is divided into a number non-overlapped motion estimation regions of equal size. From MER to MER, the motion estimation is carried out sequentially, i.e. motion estimation is performed on the next MER after the motion estimation of the current MER is finished. Within a MER, the motion estimation is carried out in parallel for all the PUs residing in the MER. The MER size should be less or equal to LCU size. In the example shown in Figure 3, a LCU is divided into four MERs.

PU0

PU1

PU2

MER0

MER1

Not unavailable MVPs

(different MERs)

unavailable MVPs

(same MER)

unavailable MVPs

(neighboring PUs are not coded yet)

LCU

MER2

MER3

Figure 3. Illustration of motion estimation region (MER) partitioning, and availability rules of spatial MVPs for PUs at different locations

## Spatial MVP availability check for merge/skip MVP list construction

To enable testing merge/skip mode for every PU of a MER during the parallel motion estimation process, an additional check is introduced to define the MVP availability for spatial MVP derivation and for temporal MVP reference index derivation of merge/skip mode. Let (xP, yP) and (xN, yN) be the coordinates of the top-left corner pixel of the current PU and a neighboring PU (see Figure 4), respectively, whether the current PU and its neighboring PU belong to a same MER can be easily determined. This neighboring PU is treated unavailable if the following condition satisfies.

(xP >>( log2_parallel_merge_level_minus2+2))

== (xN’ >> ( log2_parallel_merge_level_minus2+2))

and

(yP >>( log2_parallel_merge_level_minus2+2))

== (yN’ >> ( log2_parallel_merge_level_minus2+2))

Current PU

N

(xN’, yN’)

(xP, yP)

Figure 4. Illustration of the current PU and its neighboring PU

| Spatial MVP index (see figure 1) | Position | (xN’ , yN’) |
| --- | --- | --- |
| 1 | Left-down | (xP -4 , yP + nPSH -4) |
| 2 | Above | (xP+nPSW-4, yP – 4) |
| 3 | Above Right | (xP+nPSW , yP – 4) |
| 4 | Left bottom | (xP – 4 , yP + nPSH) |
| 5 | Above left | (xP – 4 , yP – 4) |
| * nPSW x nPSH is the current PU size |  |  |

Table 3. (xN, yN) determination for merge/skip spatial MVP candidates

Otherwise, the spatial MVP availability rules defined in current HM4.0 [2] (see section 8.4.2) are used to determine the availability of the spatial MVP for merge/skip MVP list and for reference index determination of temporal MVP in merge/skip mode. The determination of (xN, yN) is provided in Table 3.

For example, in Figure 3 for PU0 all its spatial neighboring PUs are all located in different MERs, for PU1 all its spatial neighboring PUs are unavailable as they all belong to a same MER as the current PU, while for PU2 only part of its spatial neighboring PUs belong to different MERs.  In the case of all the spatial neighboring PUs belonging to a same MER, only temporal MVP is used for the merge/skip mode.

Note that log2_parallel_merge_level_minus2 equal to zero indicates 4x4 block level parallel motion estimation, which is essentially same as what being done in the HM reference software.  Therefore, the proposed algorithm is backward compatible to the existing HM4.0 design, but offers more flexibility on the encoder side to meet throughput/quality requirements and cost constraints.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software with the AMP encoder speedup disabled, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies, the common test conditions and reference configurations specified in [1] are followed,

# Experimental results

Following the common testing conditions specified in [1], simulation is carried out to verify the effectiveness of the proposed algorithm. As shown in Table 4, with the proposed change in the merge/skip list construction, two third of loss caused by the parallel motion estimation can be gained back.  Note that after employing the proposed parallelized merge/skip mode the coding loss becomes insignificant (less than 1%, see Table 4 16x16 row) for motion estimation parallel level 16x16 that is commonly used in video encoder implementations today.

| Parallel ME (merge/skip) level 2N x 2N (LCU = 64x64) | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| 64 x 64 | 2.5 | 2.9 | 3.5 | 4.3 |
| 32 x 32 | 1.5 | 1.6 | 2.0 | 2.3 |
| 16 x 16 | 0.5 | 0.5 | 0.6 | 0.7 |
| 8 x 8 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 4, BD-rate increase of the proposed algorithm relative to modified HM4.0 anchor when the parallel motion estimation is performed at 2N x 2N block level (N = 64, 32, 16 and 8, LCU size is fixed to 64x64)

Detailed results are reported in attached spreadsheets:

HM-4.0-pme_anchor4_vs_pme8.xls

HM-4.0-pme_anchor4_vs_pme16.xls

HM-4.0-pme_anchor4_vs_pme32.xls

HM-4.0-pme_anchor4_vs_pme64.xls

The relative bit-rate saving of the proposed algorithm compared to the current HM4.0 design in the parallel motion estimation environment is summarized in Table 5. For typical parallel motion estimation level of 32x32, an average gain of 3.4% in RA-HE, 3.5% in RA-LC, 4.4% in LB-HE and 5.0% in LB-LC are observed. For detailed results please refer to attached spreadsheets.  For parallel level 16x16 that is used today, the average gain is 1.9% in RA-HE, 1.8% in RA-LC, 2.6% in LB-HE and 2.5% in LB-LC.

| Parallel ME (merge/skip) level 2N x 2N (LCU = 64x64) | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| 64 x 64 | -4.0 | -4.5 | -4.3 | -5.9 |
| 32 x 32 | -3.4 | -3.5 | -4.4 | -5.0 |
| 16 x 16 | -1.9 | -1.8 | -2.6 | -2.5 |
| 8 x 8 | -0.2 | -0.2 | -0.3 | -0.3 |

Table 5, BD-rate saving of the proposed algorithm relative to the current HM4.0 design when the parallel motion estimation is performed at 2N x 2N block level (N = 64, 32, 16 and 8, LCU size is fixed to 64x64)

Detailed results are reported in attached spreadsheets:

HM-4.0-anchor8_vs_pme8.xls,

HM-4.0-anchor16_vs_pme16.xls

HM-4.0-anchor32_vs_pme32.xls

HM-4.0-anchor32_vs_pme32.xls

# Complexity analysis

# The proposed parallel merge/skip mode adds an additional check to determine whether the current PU and neighboring PU belong to a same MER during the spatial MVP derivation process, this is a negligible increase when compared to the amount of availability checks already needed for the HM4.0 spatial MVP derivation process. This is reflected in the attached WD text.

# In terms of bit-rate overhead, one-bit will be needed in slice header to signal encoder/decoder running in fully sequential merge/skip mode (i.e. the current HM common test conditions).

# In terms of runtime, our LSF normally cannot produce accurate runtime. This time, we have run anchor and proposed method for parallel block level of 16x16 and 32x32 side by side, just for example, to produce relatively accurate runtime comparison between anchor and proposed method and to demonstrate the proposed change won’t have impact on runtime. The results are summarized in Table 6.

| Parallel ME (merge/skip) level 2N x 2N (LCU = 64x64) | RA-HE (%) |  |  | RA-LC (%) |  |  | LB-HE (%) |  |  | LB-LC (%) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate | Enc Time | Dec Time | BD-rate | Enc Time | Dec Time | BD-rate | Enc Time | Dec Time | BD-rate | Enc Time | Dec Time |
| 32x32 | 1.5 | 99 | 99 | 1.6 | 100 | 99 | 2.0 | 100 | 99 | 2.3 | 101 | 98 |
| 16 x 16 | 0.5 | 99 | 101 | 0.5 | 99 | 97 | 0.6 | 99 | 99 | 0.7 | 100 | 97 |

Table 6, BD-rate and runtime comparison between proposed algorithm and HM4.0 anchor for parallel merge/skip level of 16x16 and 32x32

Detailed results are reported in attached spreadsheets: HM-4.0-pme_anchor4_vs_pme32_time.xls and HM-4.0-pme_anchor4_vs_pme16_time.xls.

# Conclusion and recommendations

The proposed algorithm achieves significant quality improvement in the parallel ME environment as compared to the current HM4.0 design. It improves parallelism of the merge/skip mode and offers flexibility for high quality and high throughput HEVC encoder designs. It is recommended to adopt this change into the HEVC test model.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Zhou, “Parallelized merge/skip mode for HEVC”. JCT-VC Document, JCTVC-F069, Torino, IT, July 2011.

[4] B. Bross, W. Chien, I. Kim, M. Zhou, “Core Experiment 9: MV Coding and Skip/Merge Operations”, JCT-VC Document, JCTVC-F909, Torino, IT, July 2011.

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD Text Changes (changes marked in yellow)

#### 7.3.3 Slice header syntax

#### 7.4.3 slice header semantics

##### 8.4.2.1.2 Derivation process for spatial merging candidates

##### 8.4.2.1.3 Derivation process of reference indices for temporal merging candidate

#### Replace 7.3.3 slice header syntax

#### with

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if (slice_type ==P ¦¦ slice_type == B) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| } |  |
| } |  |

#### in 7.4.3 slice header semantics

Add

log2_parallel_merge_level_minus2  specifies parallel processing level of merge/skip mode. The value of  log2_parallel_merge_level_minus2  should be in the range of [0: log2_min_coding_block_size_minus 3 + 1 + log2_diff_max_min_coding_block_size], inclusive.

Replace 8.4.2.1.2 Derivation process for spatial merging candidates

With

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N and N’ being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP – 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP – 1 ), ( xP + nPSW − 1,  yP – 1 ) or ( xP – 1,  yP – 1 ), and  (xN’, yN’) being ( xP – 4,  yP + nPSH ), ( xP − 4,  yP + nPSH − 4 ), ( xP + nPSW,  yP – 4 ), ( xP + nPSW − 4,  yP – 4 ) or ( xP – 4,  yP – 4 ), the following applies.

- If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - if (xP >> (log2_parallel_merge_level_minus2 +2)) is equal to (xN’ >> (log2_parallel_merge_level_minus2 +2)) and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to (yN’ >> (log2_parallel_merge_level_minus2 +2)).

  - N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

- PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

    - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

##### Replace   8.4.2.1.3 Derivation process of reference indices for temporal merging candidate

With

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

Outputs of this process are (with X being replaced by 0 or 1)

- the reference indices refIdxLX of the current prediction units.

For the derivation of refIdxLXA, the following applies.

- if (xP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((xP - 4) >> (log2_parallel_merge_level_minus2 +2)),  and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((yP + nPSH - 4) >> (log2_parallel_merge_level_minus2 +2)),  refIdxLXA is assigned to -1.

- Otherwise, if the prediction unit covering luma location ( xP-1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP + nPSH − 1 ].

- Otherwise, refIdxLXA is assigned to -1.

For the derivation of refIdxLXB, the following applies.

- if (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to ((xP + nPSW - 4 ) >> (log2_parallel_merge_level_minus2 +2)),  and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((yP - 4) >> (log2_parallel_merge_level_minus2 +2)),  refIdxLXB is assigned to -1.

- Otherwise, if the prediction unit covering luma location ( xP + nPSW − 1, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP + nPSW − 1, yP-1 ].

- Otherwise, refIdxLXB is assigned to -1.

For the derivation of refIdxLXC, the following applies.

- if (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to ((xP + nPSW) >> (log2_parallel_merge_level_minus2 +2)),  and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((yP - 4) >> (log2_parallel_merge_level_minus2 +2)),  refIdxLXC is assigned to -1.

- Otherwise if (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to ((xP -4) >> (log2_parallel_merge_level_minus2 +2)),  and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((yP + nPSH) >> (log2_parallel_merge_level_minus2 +2)),  refIdxLXC is assigned to -1.

- Otherwise if (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to ((xP -4) >> (log2_parallel_merge_level_minus2 +2)),  and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((yP -4) >> (log2_parallel_merge_level_minus2 +2)),  refIdxLXC is assigned to -1.

- Otherwise if the prediction unit covering luma location ( xP+nPSW, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[ xP+nPSW, yP-1 ].

- Otherwise if the prediction unit covering luma location ( xP-1, yP+nPSH ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[ xP-1, yP+nPSH ].

- Otherwise if the prediction unit covering luma location ( xP-1, yP-1 ) is available and PredMode is not MODE_INTRA refIdxLXC is assigned to refIdxLX[ xP-1, yP-1 ].

- Otherwise, refIdxLXC is assigned to -1.

The variable refIdxLX is derived as follows.

- If refIdxLXA is equal to refIdxLXB and refIdxLXB is equal to refIdxLXC, the following applies.

  - If  refIdxLXA is equal to -1 then refIdxLX = 0

  - Otherwise refIdxLX = refIdxLXA

- Otherwise if refIdxLXA is equal to refIdxLXB, the following applies.

  - If  refIdxLXA is equal to -1 then refIdxLX = refIdxLXC

  - Otherwise refIdxLX = refIdxLXA

- Otherwise if refIdxLXB is equal to refIdxLXC, the following applies.

  - If  refIdxLXB is equal to -1 then refIdxLX = refIdxLXA

  - Otherwise refIdxLX = refIdxLXB

- Otherwise if refIdxLXA is equal to refIdxLXC, the following applies.

  - If  refIdxLXA is equal to -1 then refIdxLX = refIdxLXB

  - Otherwise refIdxLX = refIdxLXA

- Otherwise if refIdxLXA is equal to -1, the following applies.

  - refIdxLX = min( refIdxLXB, refIdxLXC )

- Otherwise if refIdxLXB is equal to -1, the following applies.

  - refIdxLX = Min( refIdxLXA, refIdxLXC )

- Otherwise if refIdxLXC is equal to -1, the following applies.

  - refIdxLX = Min( refIdxLXA, refIdxLXB )

- Otherwise, the following applies.

  - refIdxLX = Min( Min( refIdxLXA, refIdxLXB ), refIdxLXC )