| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G086 M21634 |
| --- | --- |

| Title: | CE12 on subset 7.4: Test results decreasing worst case complexity of deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Osman Sezer Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1-214-479-0319 osman@ti.com +1-214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports test results on CE12 subset 7.4 “decreasing worst case complexity of deblocking filter”. By removing the motion vectors from the boundary strength (BS) calculation of the deblocking filter, the worst case number of operations, and memory access are reduced from (8, 10) to (1, 4), and from (39, 20) to (7, 4) for the BS calculation of a de-blocking edge in P-frame and B-frame, respectively. Subjective tests at TI observed visual quality improvement in BQMall + (random access-high efficiency) + QP 37 (ringing artifact around diagonal edge has been reduced), and no subjective difference in other CE12 selected sequences when compared to the HM4.0 anchor. The proposed simplification leads to an average BD-rate increase of 0.2% in RA-HE, RA-LC, LB-HE and LB-LC configuration.

.

# Introduction

This contribution reports CE12 subset on subset 7.4 “decreasing worst case complexity of deblocking filter”.

# Algorithm description

Figure 1 depicts the boundary strength (BS) calculation in the current HM4.0 de-blocking-filter design. The worst case is BS = 1 in which motion vectors are involved. Based on complexity study conducted in [3], in the worst case the total amount of (operations, memory access) amounts (8, 10) and (39, 20) for BS calculation of a deblockin edge in P-frame and B-frame, respectively.

Figure 1. Boundary calculation in HM4.0 deblocking filter

# The proposed algorithm which simplifies the worst case BS calculation is depicted in Figure 2. In the proposed algorithm, the motion vectors are removed from BS=1 calculation. This simplification has significantly reduced the complexity of the worst case BS calculation. Based on study of [3], the worst case number of operations and memory access are reduced from (8, 10) to (1, 4), and from (39, 20) to (7, 4) for the BS calculation of a de-blocking edge in P-frame and B-frame, respectively.

Figure 2. Boundary calculation in proposed simplified algorithm

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The experimental results are summarized in Table 1, which shows an average BD-rate loss of 0.2% in RA-HE, RA-LC, LB-HE and LB-LC configurations.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.1% | -0.1% | 0.2% | 0.1% | 0.0% |
| Class B | 0.2% | 0.1% | 0.0% | 0.3% | 0.0% | 0.0% |
| Class C | 0.2% | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% |
| Class D | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
|  | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | -0.1% | 0.0% | 0.3% | 0.0% | 0.0% |
| Class C | 0.2% | -0.1% | 0.0% | 0.2% | 0.1% | -0.2% |
| Class D | 0.1% | 0.3% | 0.1% | 0.1% | 0.3% | -0.4% |
| Class E | 0.3% | -0.2% | 1.0% | 0.2% | 0.2% | 0.4% |
| Class F |  |  |  |  |  |  |
| Overall | 0.2% | 0.0% | 0.2% | 0.2% | 0.1% | -0.1% |
|  | 0.2% | 0.0% | 0.1% | 0.2% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 1. Experimental results

Subjective viewing for CE12 sleeted sequences are conducted at TI, Subjective tests are carried out for the luma only change, visual quality improvement has been observed in BQMall + (random access-high efficiency) + QP 37 when compared to HM4.0 anchor, ringing artifact around diagonal edge has been reduced (see fig. 3),  no subjective difference has been observed in other sequences.

| BQMall sequence frame # 115, HM-4.0 Anchor |  |
| --- | --- |
| Detail, Proposal | Detail, Anchor |

Fig. 3 Subjective viewing results for BQMall sequence frame number 115 / 600.

# Conclusion and recommendation

The proposed algorithm can significantly lower the complexity of the worst case BS calculation and improve subjective quality in some sequences at a negligible BD-rate loss (0.2%). It is recommended to adopt this simplification in the HEVC test model to simplify the de-blocking filter design.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Sadafale,  “Improving worst case performance of Deblocking-filter”, JCTVC document JCTVC-F484, Torino, IT, 14-22 July, 2011

[4] A. Norkin, X. Guo, B. Jeon and M. Narroschke, “Description of Core Experiment 12: Deblocking Filtering “,JCTVC document JCTVC-F912, Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Required Changes in Working Draft

#### Replace 8.6.1.3 Derivation process of boundary filtering strength

With

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the block edge is also a coding unit edge and the following condition is true, the variable bS[ 0 ][ xEk ][ yEj ] is set equal to 4.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 3.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if the following condition istrue any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

- The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.