| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G088 M21636 |
| --- | --- |

| Title: | CE12 subset 2: Test results and architectural study on de-blocking filter without parallel on/off filter decision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Osman Sezer Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1-214-479-0319 osman@ti.com +1-214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports test results on CE12 subset 5.6 “removal of parallel on/off filter decision”. Architectural study shows that the parallel on/off decision actually restricts architecture choices, increases implementation costs in terms of memory reads and buffer size without intended throughput benefits. It is recommended to restore the AVC fashion of on/off filter decision, that is to use the un-filtered samples for the on/off filter decision of vertical edges, and the inter-mediate filtered samples (i.e. filtered samples after vertical edge filtering) for the on/off decision of horizontal edges. Test results reveal that this change leads to 0.0% BD-rate difference, and subjective viewing verifies that there is no visual difference for all the CE12 selected subjective testing sequences when compared to the HM4.0 anchor.

# Introduction

This contribution reports CE12 subset on subset 5.6 “removal of parallel on/off filter decision”.

# Algorithm description

The current HM4.0 has employed parallel on/off filter decision, which has reportedly [3] increased implementation cost without intended benefits. The major drawbacks of the current design are as follows:

- It restricts architecture choices. With the parallel filter decision the bottom three lines of the current LCU cannot perform vertical edge horizontal filtering before storing them in the line buffer. Otherwise, it will be three lines of line buffer size increase due to needs of buffering the un-filtered samples for the on/off filter decisions of upper LCU boundary of the next LCU row. In the AVC deblocking filter design, a common architecture is to filter as much as possible for the current macroblock and store the horizontally filtered intermediate samples into line buffer, which is now impossible without a significant area increase.

- It won’t improve throughput. Instead, for a same throughput six additional lines of un-filtered data will need to be loaded separately for the on/off filter decision of horizontal edges. This increases both memory reads and buffer size. Alternatively, for a same buffer size, memory reads and logic area, parallel decision actually slows down the vertical edge filtering because the vertical edge filtering cannot overwrite the data before the on/off decision on un-filtered samples for the horizontal edge is done.

This contribution advocates to restore the AVC fashion of the on/off filter decision, that is, the on/off decision for vertical edges use the un-filtered samples, while the on/off decision for horizontal edges use the inter-mediate filtered samples (i.e. filtered samples after vertical edge filtering).

# Architectural analysis

Figure 1 illustrates data storage format for a typical block-based de-blocking filter architecture. Each 8x8 block is TU block which needs to be filtered. The data is processed by 8x8 block size at a time, which is contained in the red square in Figure 1. Blue vertical and horizontal edges are to be filtered. Since the vertical edge has to be filtered before the horizontal edge, additional 4 columns need to be loaded for processing of the current filtering segment. Therefore, the actual data buffer size for an 8x8 filtering segment is 12 x 8, which is dashed red rectangular in Figure 1. The bottom six samples marked in green need to be loaded too for the vertical edge filter on/off decision. From one 8x8 filtering segment to the next, four columns that contain inter-mediate samples (after vertical edge filtering) are overlapped. In Figure 1, column 0 to 3 are inherited from the previous filtering segment to the left, while data of column 8 to 11 will be inherited by the next filtering segment after vertical edge filtering of the blue edge is done.

0

1

2

3

4

5

6

7

8

9

10

111

0  1   2  3  4  5  6  7   8  9 10 11

8x8 filtering segment

Data buffer

8x8 blocks

Figure 1. Block-based de-deblocking

With the following architecture assumptions

- Single-cycle 4x4 tile memory or 8 x2 line memory reads and writes

- Single-cycle filtering for a line or column

- Single-cycle filter on/off decision

- Single filter logic which is shared by horizontal and vertical edge filtering

And for a same buffer size, memory reads and logic area, the cycle count required for processing an 8x8 filtering segment is summarized in the table 1.

|  | 4x4 tile memory format | Line memory format |
| --- | --- | --- |
| w/o parallel decision | 22 | 21 |
| w/ parallel decision | 25 | 24 |

Table 1. Cycle count estimate for an 8x8 filtering segment

As shown in Table 1, the parallel decision actually slows down the de-blocking filtering process for a same mount of buffer size, memory reads and logic area, because vertical edge filtering cannot overwrite the un-filtered samples before the on/off filter decision for horizontal edge is done.

For achieving the same throughput, de-blocking filter with parallel on/off decision would need to buffer six un-filtered sample lines (size 8 x 6, line 1 – 6 in figure 1) for the horizontal edge on/off filter decision, which increases both memory reads and data buffer size (see Figure 2).

Loading unfiltered

samples (8x8) and vertical edge filtering

Horizontal

edge

filtering

Loading unfiltered

samples (8x6) and

on/off filter decision

Figure 2. Diagram of de-blocking  filter w/ parallel decision for a same throughput as that of w/o parallel decision

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The experimental results are summarized in Table 2. There is zero BD-rate difference is observed between the proposed method and the HM4.0 anchor.

Subjective tests were also carried out at TI, no visual difference was observed for all the CE12 selected subjective testing sequences.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | -0.2% |
| Class D | 0.0% | 0.4% | -0.3% | 0.0% | 0.2% | -0.2% |
| Class E | 0.1% | 0.0% | 0.4% | 0.0% | 0.1% | -0.3% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 2. Experimental results  (proposed vs. HM4.0 anchor)

# Conclusion and recommendation

The current HM4.0 de-blocking filter design with parallel on/off filter decision breaks commonly used de-blocking filter architecture without intended throughput benefits. Given the test results and architectural analysis, it is recommended to restore the AVC fashion of on/off filter decision, that is, the on/off decision for vertical edges uses the un-filtered samples, while the on/off decision for horizontal edges uses the inter-mediate filtered samples (i.e. filtered samples after vertical edge filtering).

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Sadafale,  “Improving Deblocking filter efficiency”, JCTVC document JCTVC-F256, Torino, IT, 14-22 July, 2011

[4] A. Norkin, X. Guo, B. Jeon and M. Narroschke, “Description of Core Experiment 12: Deblocking Filtering “,JCTVC document JCTVC-F912, Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

# Required Changes in Working Draft

#### Replace 8.6.1.4 Filtering process for coding unit (changes marked in yellow)

with

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- All elements of the three-dimensional array of size (2)x(nD)x(nD), dEdge are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(1<<log2CUSize), dSample are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(nD), bStrength ae initialized to zero.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7 	(8-1)

    - bStrength[1][k][m] is set equal to bSVer.

    - Decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs and the decision dEdge[1][k][m] and an array dS of size (8) as outputs.

    - dSample[1][k][(m<<3)+i] is set equal to dS[i] for i=0..7.

    - Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 	(8-2)

    - bStrength[0][k][m] is set equal to bSHor.

    - Decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m] and an array dS of size (8) as outputs.

    - dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7.

- For xDk set equal to xC+( k << 3 ), k=0..nD - 1, the following applies:

- For yDm set equal to yB+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - dS[i] is set equal to dSample[1][k][(m<<3)+i] for i=0..7.

    - Filtering process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, the boundary filtering strength bStrength[1][k][m], the decision dEdge[1][k][m], and the array of size (8), dS as inputs and the modified luma picture buffer as outputs.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

  - Boundary filtering strength bSHor is derived as follows:

  - bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 	(8-3)

  - bStrength[0][k][m] is set equal to bSHor.

  - Decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m] and an array dS of size (8) as outputs.

  - dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7.

- For yDm set equal to yC+( m << 3 ), m=0..nD - 1, the following applies:

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following ordered steps apply:

    - If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD – 1) << 3 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

    - dS[i] is set equal to dSample[0][m][ (k << 3) + i ] for i = 0..7.

    - Filtering process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bStrength[0][k][m], the decision dEdge[0][k][m], and the array of size (8), dS, xPOS, dSL[m][], dEL[m], bSL[m], and tCL[m], as inputs and the modified luma picture buffer as output.

- The elements of the two dimensional array of size (3)x(nD), dSL are set as follows. dSL[m][0], dSL[m][1], and dSL[m][2], are set equal to dS[5], dS[6] and dS[7].

- The elements of the array of size (nD), dEL are set as follows. dEL[m] is set equal to dEdge[0][k][m].

- The elements of the array of size (nD), bSL are set as follows. bSL[m] is set equal to bStrength[0][k][m].

- The elements of the array of size (nD), tC are set as follows. tCL[m] is set equal to tc.

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( Max( log2CUSize, 4 ) – 4 ).

- For xDk set equal to ( xC / 2 )+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to ( yC / 2)+( m << 2 ), m=0..nD*2 – 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = bS[ 0 ][ xDk*2 ][ yDm*2 ] 		(8-4)

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- For yDm set equal to ( yC / 2 )+( m << 3 ), m=0..nD – 1, the following applies:

- For xDk set equal to ( xC / 2 )+( k << 2 ), k=0..nD*2 – 1, the following ordered steps apply:

- If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD*2 – 1) << 2 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

- Boundary filtering strength bSHor is derived as follows:

bSHor = bS[ 1 ][ xDk*2 ][ yDm*2 ] 		(8-5)

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.