| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G089 M21637 |
| --- | --- |

| Title: | Non-CE12: Enable low-complexity and fully parallelized de-blocking filter with minor changes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Osman Sezer Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1-214-479-0319 osman@ti.com +1-214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution advocates the following two changes to the current HM4.0 de-blocking filter design: 1) compute the on/off decision on 4 lines/columns segment instead of 8 lines/columns segment, and 2) remove the parallel on/off filter decision to speed up vertical edge filtering. The proposed changes lead to a low-complexity, fully parallelized de-blocking filter design with enhanced architecture flexibility. Compared to the current HM4.0 design, the buffer size is reduced from 12x12 to 8x8, and number of memory access is reduced from 8x12 to 8x8, and from segment to segment the de-blocking filter is fully independent and parallelized. Experimental results revealed that the proposed algorithm led to 0.1% BD-rate reduction in LB-HE and LB-LC, and 0.0% BD-rate difference in other configurations when compared to HM4.0. Subjective tests showed that the proposed algorithm led to visible visual gain in BQMall (RA-HE, QP=37) sequence, and no visual difference in other CE12 selected subjective testing sequences.

# Introduction

De-blocking filter is not bottleneck for video codec and throughput has never been a concern for de-blocking filter for HD video applications.  This contribution proposes two minor changes to lower the de-blocking filter complexity, and as side benefit, to realize a fully parallelized de-blocking filter design with enhanced architecture flexibility.

# Algorithm description

In order to enable block-level parallelism, the deblocking filter on/off decision is modified.

Segment of 8 columns/lines

Figure 1 : Deblocking filter on/off decision in HM4.0

As shown in Figure 1, in the current HM4.0 design the deblocking filter on/off decision was carried out for a segment of 8 columns or lines, and based on the following equation

d2=|p22−2⋅p12+p02|+|q22−2⋅q12+q02|d5=|p25−2⋅p15+p05|+|q25−2⋅q15+q05|d2+d5<β                              (A.1)

Where ß is a threshold. The deblocking filter is tuned on if condition in eq. A.1 satisfies.

However, the fact that the on/off decision is made jointly for a segment of 8 columns or lines creates the filtering dependency between the top 4-line segment and bottom 4-line segment (or left 4-column segment and right 4-column segment if an edge is a horizontal edge), which prevents block-level parallelism at 8x8 grid described above.

In order to solve this problem, we propose to perform deblocking-filter on/off decision on a segment of 4 (instead of 8) columns or lines.

Top/left segment

Bottom/right segment

Segment of 8 columns/lines

Figure 2.  Proposed deblocking filter on/off decision

The proposed method is illustrated in Figure 2.  For a segment of 8 columns or lines, the on/off decision for top 4 lines segment (or left 4 columns segment if an edge is a horizontal edge) is done by

d1=|p21−2⋅p11+p01|+|q21−2⋅q11+q01|d2=|p22−2⋅p12+p02|+|q22−2⋅q12+q02|d1+d2<β                                 (A.2)

For bottom 4 lines segment (or right 4 columns segment if an edge is a horizontal edge) the on/off decision is done by:

d5=|p25−2⋅p15+p05|+|q25−2⋅q15+q05|d6=|p26−2⋅p16+p06|+|q26−2⋅q16+q06|d5+d6<β                                  (A.3)

In addition, it is proposed to use the un-filtered samples for the on/off filter decision of vertical edges, and the inter-mediate filtered samples (i.e. filtered samples after vertical edge filtering) for the on/off decision of horizontal edges.

# Architecture Analysis

Figure 1 illustrates data storage format for a block-based de-blocking filter architecture of the current HM4.0 design. Each 8x8 block is TU block which needs to be filtered. The data is processed by 8x8 block size at a time, which are contained in the red squares in Figure 3. Blue vertical and horizontal edges are to be filtered. Samples marked in green are the samples from the current 8x8 filtering segment involved in filter on/off decision, while samples marked in pink are samples from neighboring filtering segments that are also involved in the filter on/off decision of the current filtering segment.

0

1

2

3

4

5

6

7

8

9

10

11

0  1   2  3  4  5  6  7    8  9 10 11 12 1314 15

8x8 blocks

8x8 filtering segment

Figure 3. Block-based de-blocking filter of the current HM4.0 design

The fact that the filter on/off decision of the current 8x8 filtering segment requires samples from the neighboring filtering segments not only increases data buffer size (in this case, buffer size will be 12 x 12 for 4x4 tile format), but also creates filtering dependency among filtering segments because those pink samples cannot be overwritten before the on/off decision of the current filtering segment is done.

0

1

2

3

4

5

6

7

8

9

10

11

0  1   2  3  4   5   6  7 8  9 10 11 12 13 14 15

8x8 blocks

8x8 filtering segment

Figure 4. Block-based de-blocking filter of proposed simplified design

# With the proposed simplified design, all the samples that used for on/off filter decision are all located inside the current 8x8 filter segment (see figure 4, samples marked green and pink). Compared to the current design, this has the following advantages from the implementation point of view:

# reduced number of buffer size and memory access: the current HM4.0 design needs a buffer size of 12 x 12 samples and 8x12 memory reads for 4x4 tile memory format, while the proposed one reduces the buffer size to 8 x 8 and memory reads to 8x8 for each 8x8 filter segment.

# fully parallelized deblocking filter, there is no dependency among 8x8 filtering segments from segment to segment.

# enhanced architecture flexibility and simplicity: one can choose to replicate same filtering logic for high throughput if needed by processing multiple 8x8 filtering segments in parallel, but one can also choose not to do that by processing one 8x8 filtering segment at a time if throughput is not a concern.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The experimental results are summarized in Table 1. The proposed unification improves the coding efficiency in luma by 0.1% in LB-HE and LB-LC, leads to no BD-rate difference in other configurations.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class C | -0.2% | -0.1% | -0.2% | -0.2% | 0.2% | -0.1% |
| Class D | 0.0% | 0.5% | 0.1% | -0.1% | 0.1% | -0.1% |
| Class E | 0.0% | -0.1% | 0.7% | -0.1% | 0.0% | -0.1% |
| Class F |  |  |  |  |  |  |
| Overall | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.0% |
|  | -0.1% | 0.0% | 0.1% | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

Table 1. Experimental results (proposed method vs. HM4.0 anchor).

Subjective tests are carried at TI, visual improvement has been observed in BQMall + (random access-high efficiency) + QP 37 when compared to anchor, ringing artifact around diagonal edge has been reduced (see Fig. 5). No visual difference was observed for other CE12 selected visual testing sequences.

| BQMall sequence frame # 115, HM-4.0 Anchor |  |
| --- | --- |
| Detail, Proposal | Detail, Anchor |

Fig. 5 Subjective viewing results for BQMall sequence frame number 115 / 600.

# Conclusion and recommendation

The proposed change leads to a low-complexity and fully parallelized de-blocking filter design with enhanced architecture flexibility. Visual improvements have been observed in CE12 selected visual test sequences. It is therefore recommended to adopt this change into HEVC test model.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Sadafale,  “Improving Deblocking filter efficiency”, JCTVC document JCTVC-F256, Torino, IT, 14-22 July, 2011

[4] A. Norkin, X. Guo, B. Jeon and M. Narroschke, “Description of Core Experiment 12: Deblocking Filtering “,JCTVC document JCTVC-F912, Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Required Changes in Working Draft

related sections:

8.6.1.4 Filtering process for coding unit

8.6.1.4.1 Decision process for luma block edge

8.6.1.4.2 Filtering process for luma block edge

#### Replace 8.6.1.4 Filtering process for coding unit (changes marked in yellow)

with

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- All elements of the three-dimensional array of size (2)x(nD)x(nD), dEdge are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(1<<log2CUSize), dSample are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(nD), bStrength ae initialized to zero.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7 	(8-1)

    - bStrength[1][k][m] is set equal to bSVer.

    - Decision process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs and the decision dEdge[1][k][m] and an array dS of size (8) as outputs.

    - dSample[1][k][(m<<3)+i] is set equal to dS[i] for i=0..7.

    - Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 	(8-2)

    - bStrength[0][k][m] is set equal to bSHor.

    - Decision process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m] and an array dS of size (8) as outputs.

    - dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7.

- For xDk set equal to xC+( k << 3 ), k=0..nD - 1, the following applies:

- For yDm set equal to yB+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - dS[i] is set equal to dSample[1][k][(m<<3)+i] for i=0..7.

    - Filtering process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, the boundary filtering strength bStrength[1][k][m], the decision dEdge[1][k][m], and the array of size (8), dS as inputs and the modified luma picture buffer as outputs.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

  - Boundary filtering strength bSHor is derived as follows:

  - bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 	(8-3)

  - bStrength[0][k][m] is set equal to bSHor.

  - Decision process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m] and an array dS of size (8) as outputs.

  - dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7.

- For yDm set equal to yC+( m << 3 ), m=0..nD - 1, the following applies:

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following ordered steps apply:

    - If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD – 1) << 3 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

    - dS[i] is set equal to dSample[0][m][ (k << 3) + i ] for i = 0..7.

    - Filtering process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bStrength[0][k][m], the decision dEdge[0][k][m], and the array of size (8), dS, xPOS, dSL[m][], dEL[m], bSL[m], and tCL[m], as inputs and the modified luma picture buffer as output.

- The elements of the two dimensional array of size (3)x(nD), dSL are set as follows. dSL[m][0], dSL[m][1], and dSL[m][2], are set equal to dS[5], dS[6] and dS[7].

- The elements of the array of size (nD), dEL are set as follows. dEL[m] is set equal to dEdge[0][k][m].

- The elements of the array of size (nD), bSL are set as follows. bSL[m] is set equal to bStrength[0][k][m].

- The elements of the array of size (nD), tC are set as follows. tCL[m] is set equal to tc.

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( Max( log2CUSize, 4 ) – 4 ).

- For xDk set equal to ( xC / 2 )+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to ( yC / 2)+( m << 2 ), m=0..nD*2 – 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = bS[ 0 ][ xDk*2 ][ yDm*2 ] 		(8-4)

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- For yDm set equal to ( yC / 2 )+( m << 3 ), m=0..nD – 1, the following applies:

- For xDk set equal to ( xC / 2 )+( k << 2 ), k=0..nD*2 – 1, the following ordered steps apply:

- If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD*2 – 1) << 2 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

- Boundary filtering strength bSHor is derived as follows:

bSHor = bS[ 1 ][ xDk*2 ][ yDm*2 ] 		(8-5)

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.

Replace

##### Replace 8.6.1.4.1 Decision process for luma block edge

With

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp1[2] and dEq1[2] containing decisions,

–	one-dimensional array of size (8), dS containing decisions.

Let s’ represent the luma sample array recPictureL of the current picture.

A variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified specified as Error: Reference source not found with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5  1, 2, 5, 6 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-6)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-7)

- The variables dp, dq and d are derived as follows:

dp[0] = | p2,1 – 2*p1,1 + p0,1 | + | p2,2 – 2*p1,2 + p0,2 |		(8-8)

dq[0] = | q2,1 – 2*q1,1 + q0,1 | + | q2,2 – 2*q1,2 + q0,2 |	(8-9)

d[0] = dp[0] + dq[0]		(8-10)

dp[1] = | p2,5 – 2*p1,5 + p0,5 | + | p2,6 – 2*p1,6 + p0,6 |	(8-11)

dq[1] = | q2,5 – 2*q1,5 + q0,5 | + | q2,6 – 2*q1,6 + q0,6 |	(8-12)

d[1] = dp[1] + dq[1]		(8-13)

- The variables dE, dEp1[k] and dEq1[k] are set equal to 0, with k = 0, 1.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam

- The variable dE is set equal to 1.

- If dp[k] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp1[k] is set equal to 1, with k =0, 1

- If dq[k] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq1[k] is set equal to 1, with k = 0, 1

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5 1, 2, 5, 6 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-14)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-15)

- The variables dp, dq and d are derived as follows:

dp[0] = | p2,1 – 2*p1,1 + p0,1 | + | p2,2 – 2*p1,2 + p0,2 |		(8-16)

dq[0] = | q2,1 – 2*q1,1 + q0,1 | + | q2,2 – 2*q1,2 + q0,2 |		(8-17)

d[0] = dp[0] + dq[0]		(8-18)

dp[1] = | p2,5 – 2*p1,5 + p0,5 | + | p2,6 – 2*p1,6 + p0,6 |		(8-19)

dq[1] = | q2,6 – 2*q1,6 + q0,6 | + | q2,6 – 2*q1,6 + q0,6 |		(8-20)

d[1] = dp[1] + dq[1]		(8-21)

- The variables dE, dEp1[k] and dEq1[k] are set equal to 0, with k = 0, 1.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- For each sample location ( xC + xB + k, yC + yB ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam.

- The variable dE is set equal to 1.

- If dp[k] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp1[k] is set equal to 1, with k = 0, 1.

- If dq[k] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq1[k] is set equal to 1, with k = 0, 1.

##### Replace 8.6.1.4.2 Filtering process for luma block edge

With

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	variables dE, dEp1[2] and dEq1[2] containing decisions,

–	one-dimensional array of size (8), dS containing decisions,

–	a variable bSL,

–	a variable tCL,

Output of this process is:

–	modified reconstruction of the picture.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-22)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-23)

- If dE is not equal to 0, for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the decision dS[k], variables dEp1[k>>2] and dEq1[k>>2], the boundary filtering strength bS and the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary, and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +j, yC + yB + k ] = qj’		(8-24)

s’[ xC + xB – i – 1, yC + yB + k ] = pi’		(8-25)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- If xPOS is equal to 1, the parameters ks and ke are set to -3 and 4 respectively. If xD is equal to 2, the parameters ks and ke are set to 0 and 4 respectively. Otherwise ks and ke are set to 0 and 7 respectively.

- The sample values pi,k and qi,k with i = 0..3 and k = ks..ke are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-26)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-27)

- If xPOS is less than 0 and dEL is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = -3..-1, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision dSL[k+3], variables dEp1[k>>2] and dEq1[k>>2], the boundary filtering strength bSL and the variable tCL as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + j ] = qj’		(8-28)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-29)

- If dE is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = 0.. ke , the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision dS[k], dEp1[k>>2] and dEq1[k>>2], the boundary filtering strength bS and the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + j ] = qj’		(8-30)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-31)