| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G090 M21638 |
| --- | --- |

| Title: | Non-CE12: Testing results on using HM3.0 delta calculation for luma weak filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Osman Sezer Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1-214-479-0319 osman@ti.com +1-214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution proposes to restore the HM3.0 delta calculation for luma weak filter which has higher precision when compared to the HM4.0 one. Simulation results revealed that the proposed restoration improved the coding efficiency by 0.2% in AI-HE and AI-LC, and 0.1% in RA-HE, RA-LC, LB-HE and LB-LC. Subjective tests showed that the proposed change led to visible visual gain in BQMall (RA-HE, QP=37) sequence, and no visual difference in other CE12 selected subjective testing sequences. Based on test results, it is recommended to restore the HM3.0 delta calculation for luma weak filter for both visual quality and BD-rate improvements.

# Introduction

This contribution advocates restoring the HM3.0 delta calculation for luma weak filter for quality improvements. The precision of delta calculation for luma weak filter has been reduced from HM3.0 to HM4.0, which apparently led to quality degradation.

# Algorithm description

In HM4.0 the filter coefficient precision for delta calculation of luma weak filter has been decreased. In this contribution, we to restore HM3.0 delta calculation for luma weak filter. This will improve both subjective and objective video quality.

Luma Weak filter HM4.0:                     delta = (9*(m4-m3) -3*(m5-m2) + 8)>>4

Luma Weak filter proposed (HM3.0) : delta = (13*(m4-m3) - 4*(m5-m2) + 16) >> 5;

This change has no impact on software as total number of operations remains same.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

Table 1 summarizes the test results for using the HM3.0 delta calculation for luma weak filter, the chroma filters remains unchanged. As shown in Table 1, there is an average gain of 0.2% in AI-HE and AI-LC, and 0.1% in RA-HE, RA-LC, LB-HE and LB-LC. This indicates HM3.0 luma weak filter delta calculation is superior to the current HM4.0 design.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class B | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class E | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
|  | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.1% | -0.1% | 0.1% | 0.1% |
| Class B | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | -0.2% | -0.1% | -0.1% | 0.1% | -0.1% |
| Class D | -0.2% | 0.3% | -0.1% | -0.1% | 0.3% | -0.4% |
| Class E | 0.0% | 0.4% | 0.5% | -0.2% | 0.6% | 0.4% |
| Class F |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.2% | 0.0% |
|  | -0.1% | 0.0% | -0.1% | -0.1% | 0.2% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |

Table 1. Experimental results (using HM3.0 delta calculation for luma weak filter).

Subjective tests are carried out for the proposed restoration (i.e. restoring the HM3.0 delta calculation for luma weak filter), visual improvement has been observed in BQMall + (random access-high efficiency) + QP 37 when compared to anchor, ringing artifact around diagonal edge has been reduced (see Fig. 1). No visual difference was observed for other CE12 selected visual testing sequences.

| BQMall sequence frame # 115, HM-4.0 Anchor |  |
| --- | --- |
| Detail, Proposed | Detail, HM4.0 Anchor |

Fig. 1 Subjective viewing results for BQMall sequence frame number 115 / 600.

# Conclusion and recommendation

Based on the experimental results it is recommended to restore HM3.0 delta calculation for luma wek filter for subjective and objective quality improvement.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Sadafale,  “Improving Deblocking filter efficiency”, JCTVC document JCTVC-F256, Torino, IT, 14-22 July, 2011

[4] A. Norkin, X. Guo, B. Jeon and M. Narroschke, “Description of Core Experiment 12: Deblocking Filtering “,JCTVC document JCTVC-F912, Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

# Required Changes in Working Draft

In 8.6.1.4.5 Filtering process for a luma sample

Replace

 = ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4		(8-1)

With

 = ( 13 * ( q0 –  p0 ) – 4 * ( q1 – p1 ) + 16 ) >> 5		(8-2)