| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G091 M21639 |
| --- | --- |

| Title: | CE13: Test results on maxNumMergeCand signaling and simplification of merge MVP list pruning process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou | Tel: Email: | +1 214.480.3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

# This document reports CE13 test results on the following two tools:  signaling the maximum number of merge/skip MVP candidates (maxNumMergeCand) in slice header (signaling method), and simplification of the second pruning process of the merge/skip MVP list derivation process. (simplification method). The signaling method enables the truncated unary table of accurate length to be used for merge_idx coding, while the simplification method reduces the complexity of the second pruning process by 2x, thanks to the reduction of the total number of combined, non-scaled and zero MVP candidates from 8 to 5. Experimental results reveal that the proposed signaling method provides e.g. an average of gain of 0.1% in RA-HE, 0.3% in RA-LC, 0.2% in LB-HE and 0.7% in LB-LC when maxNumMergeCand equal to 3 when compared to the HM4.0 encoder only method, which uses a fixed truncated unary table of maximum length 5 for merge_idx coding regardless of number of merge candidates supported by the encoder. The gain is significantly larger in LC configurations. Results also showed that the proposed simplification method and adding it on the top of the signaling method does not cause coding loss.

# Introduction

This document reports CE13 [4] results on:

- subset 3.2.1: coding efficiency study on maximum truncated unary table sizes signaled in slice header for merge index coding.

- subset 3.2.2: simplification of the second pruning process of the merge MVP list construction by reducing the number of combined MVP candidates.

- And subset 3.3.3: combined study on subset 3.2.1 and subset 3.2.2

# Algorithm description

## Subset 3.2.1: coding efficiency study on maximum truncated unary table sizes signaled in slice header

On the encoder side, merge/skip mode search (i.e. merge motion estimation) is expensive in terms of both area and memory bandwidth. Motion search for each merge/skip MVP candidate can be seen as sub-pel motion vector refinement with zero refinement search range, and parallel logic needs to be created for merge/skip MVP candidates due to throughput constraints. In H.264/AVC sub-pel engine occupies a significant portion of entire motion estimation engine, the problem becomes much worse in HEVC because multiple sub-pel engines are required to support merge mode search up to five merge/skip MVP candidates. In addition, since merge/skip MVP candidates oftentimes go outside the major search window, they normally require separate loading of reference blocks. Supporting multiple merge/skip candidates again complicates the HEVC encoder design due to significant increase in memory bandwidth.

Therefore, for a practical HEVC encoder design, one may choose to cut the number of merge/skip MVP candidates during the motion estimation to lower the encoder cost and to meet cost budget. The current HM design already supports such an encoder design flexibility, as one can choose to do motion search only for high priority candidates and skip the rest of low priority candidates in the merge/skip MVP list.

The drawback of the current design is that the truncated unary table for merge/skip MVP index coding is designed for a maximum number of 5 candidates. If fewer number of merge/skip MVP candidates is supported on the encoder side, the coding efficiency is comprised since low-priority entries of the coding table remain unused.

To improve the coding efficiency, it is proposed to add a parameter in slice header to signal the maximum merge MVP list size (maxNumMergeCand), so that the truncated unary table for merge/skip MVP index coding can exactly match the maximum number of merge/skip candidates supported on the encoder side.

| Merge/skip MVP index | maxNumMergeCand = 4 |  | maxNumMergeCand = 3 |  | maxNumMergeCand = 2 |  | maxNumMergeCand = 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM4.0 | Proposed | HM4.0 | Proposed | HM4.0 | Proposed | HM4.0 | Proposed |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | N/A |
| 1 | 10 | 10 | 10 | 10 | 10 | 1 |  |  |
| 2 | 110 | 110 | 110 | 11 |  |  |  |  |
| 3 | 1110 | 111 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |

Table 1. Truncated unary table for merge_idx coding in HM4.0 and proposed method when encoder restricts number of merge/skip candidates in motion estimation

Table 1 highlights in difference of trunacted unary table for merge_idx coding under different maximum number of merge/skip MVP candidates supported by encoder. As shown in table 1, the proposed method cut the last entry of each table by 1 bit, which can lead coding efficiency improvements. Note that in the proposed method the merge_idx is not represent in bitstream if only one merge candidate is supported by the encoder.

## Subset 3.2.2: simplification of the second pruning process of the merge MVP list construction

Figure 1 depicts the diagram of the merge/skip MVP list derivation process in HM4.0.  There are two MVP pruning processes involved. The first pruning processing removes the redundant MVP candidates from a maximum of one temporal MVP and four spatial MVP incoming candidates.  After the first pruning process, the combined, non-scaled and zero MVP candidates can be appended to the list if the number of valid (native) MVPs is less than 5. This leads to the second MVP pruning process, in which a maximum of four native (i.e. resulting MVP candidates after the first pruning process), five combined, one non-scaled and two zero MVP candidates need to be pruned in the worst case. Compared to the first pruning process, the second one is quite complex because in the worst case there are a total of 12 input candidates. The number of comparison operations in the first pruning process is 10  (i.e. 5 input candidates, number of total compares N*(N-1)/2 for N = 5), while the number of comparison operations for the second pruning process amounts to 60 (i.e. 12 input candidates, first 4 are unique, number of total compares is (N-4)*(N+3)/2 for N = 12). There is 6x difference. Therefore, it is strongly desired to reduce the complexity of the second pruning process.

additional MVP candidates

Initial MVP list

2

3

6

4

5

Scale

2

3

6

4

5

2nd MVP pruning

0

1

3

1

2

1

3

2

1

1st MVP pruning

1

2

3

4

6

Non-Scale

11

MVP combinations

1

2

1

Legend:

MVP positions         
   MVP availability check              MVP pruning

MVP candidates                             virtual candidates               zero vector

0

Final MVP list

0

0

0

0

1

1

MUX

Figure 1. Diagram of merge/skip MVP list derivation process of HM4.0

In the proposed algorithm, the total number of combined, non-scaled and zero MVP candidates in the worst for the second pruning process is limited to 5, while the maximum number of non-scaled and zero MVP candidates remains unchanged (i.e. 1 for non-scaled, and 2 for zero MVP candidates). In this way, the worst case number of MVP candidates for the second pruning process can be reduced from 12 to 9, and number of comparison operations is decreased from 60 to 30, a factor of roughly 2X reduction.

## Subset 3.2.3: combined study on subset 3.2.1 and subset 3.2.2

In the combined design, the maximum merge MVP list size (maxNumMergeCand) is signaled in slice header, the proposed truncated unary tables in Table 1 are used for merge_idx coding based on the value of maxNumMergeCand, and the total number of combined, non-scaled and zero MVP candidates in the worst for the second pruning process is limited to 5 as specified in section 2.2.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

# Table 2 summarizes the coding loss when encoder chooses to support a smaller number of merge/skip MVP candidates during the motion estimation. The truncated unary table used for merge_idx coding here is the one for the maximum of 5 candidates as shown in HM4.0 columns of Table 1. As shown in Table 2, significant loss (i.e. 3.3% in RA-HE, 4.9% in RA-LC, 4.0% in LB-HE and 7.3% in LB-LC) has been observed when the maximum number of merge/skip candidates is set to 1 (i.e. AVC fashion), but relatively small loss (i.e. 0.4% in RA-HE, 0.7% in RA-LC, 0.6% in LB-HE and 1.5% in LB-LC) is seen e.g. if maximum number of merge candidates is set to 3.  The HM4.0 merge/skip design offers flexibility for encoder designs to choose proper cost and coding efficiency trade-offs to meet design constraints.

| CE13 Subset 3.2.1 Tests | maxNumMegeCand | RA-HE (%) | RA-LC (%) | LB-HE(%) | LB-LC(%) |
| --- | --- | --- | --- | --- | --- |
| Test 1b | 4 | 0.1 | 0.3 | 0.2 | 0.6 |
| Test 2b | 3 | 0.4 | 0.7 | 0.6 | 1.5 |
| Test 3b | 2 | 0.9 | 1.7 | 1.4 | 3.5 |
| Test 4b | 1 | 3.3 | 4.9 | 4.0 | 7.3 |

Table 2. Coding loss relative to HM4.0 anchor when encoder restricts maximum number of merge candidates to 4, 3, 2, 1 (HM4.0 encoder only method)

# Please refer to toe following spreadsheets for detailed results  of Table 2.

# HM-4.0-anchor_vs._anchorMRG1_CE13_subset3.2.1_test1b.xls

# HM-4.0-anchor_vs._anchorMRG1_CE13_subset3.2.1_test2b.xls

# HM-4.0-anchor_vs._anchorMRG1_CE13_subset3.2.1_test3b.xls

# HM-4.0-anchor_vs._anchorMRG1_CE13_subset3.2.1_test4b.xls

# Table 3 summarizes the coding loss when encoder chooses to support a smaller number of merge/skip MVP candidates during the motion estimation, and the maximum number of merge/skip MVP candidates is signaled in the slice header. Here the truncated unary table used for merge_idx coding depends on the actual value of the maximum number of merge/skip MVP candidates (see tables in the proposed columns of Table 1).  When compared to results in Table 2, the results in Table 3 revealed that the loss is reduced.

| CE13 Subset 3.2.1 Tests | maxNumMegeCand | RA-HE (%) | RA-LC (%) | LB-HE(%) | LB-LC(%) |
| --- | --- | --- | --- | --- | --- |
| Test 1a | 4 | 0.1 | 0.1 | 0.2 | 0.2 |
| Test 2a | 3 | 0.3 | 0.4 | 0.4 | 0.8 |
| Test 3a | 2 | 0.8 | 0.9 | 1.3 | 2.0 |
| Test 4a | 1 | 3.1 | 3.2 | 3.7 | 4.4 |

Table 3. Coding loss relative to HM4.0 anchor when maximum number of merge candidates (4, 3, 2, 1) is signaled in slice header (proposed signaling method)

# Please refer to the following spreadsheets for detailed results of Table 3.

- HM-4.0-anchor_vs._proposedMRG1_CE13_subset3.2.1_test1a.xls

- HM-4.0-anchor_vs._proposedMRG1_CE13_subset3.2.1_test2a.xls

- HM-4.0-anchor_vs._proposedMRG1_CE13_subset3.2.1_test3a.xls

- HM-4.0-anchor_vs._proposedMRG1_CE13_subset3.2.1_test4a.xls

The coding efficiency gain of the proposed signaling method over the HM4.0 encoder only method is summarized in Table 4. For the maximum number of merge/skip candidates equal to 3, the proposed method provides an average gain of 0.1% in RA-HE, 0.3% in RA-LC, 0.2% in LB-HE and 0.7% in LB-LC. The gain is largest when maximum number of merge/skip candidates is reduced to 1, in which an average gain of 0.2% in RA-HE, 1.6% in RA-LC, 0.3% in LB-HE and 2.6% in LB-LC is observed. The gain is also larger in low-complexity configurations in which CAVLC is used.

| CE13 Subset 3.2.1 Tests | maxNumMegeCand | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- | --- |
| Test 1b vs. 1a | 4 | 0.0 | -0.2 | -0.1 | -0.4 |
| Test 2b vs. 2a | 3 | -0.1 | -0.3 | -0.2 | -0.7 |
| Test 3b vs. 3a | 2 | -0.1 | -0.8 | -0.1 | -1.4 |
| Test 4b vs. 4a | 1 | -0.2 | -1.6 | -0.3 | -2.6 |

Table 4. Coding gain of proposed signaling method over HM4.0 encoder only method

# Please refer to the following spreadsheets for detailed results of Table 4.

- HM-4.0-CE13_subset3.2.1_test1b_vs_test1a.xls

- HM-4.0-CE13_subset3.2.1_test2b_vs_test2a.xls

- HM-4.0-CE13_subset3.2.1_test3b_vs_test3a.xls

- HM-4.0-CE13_subset3.2.1_test4b_vs_test4a.xls

Table 5 and Table 6 summarize the BD-rate difference between the proposed simplification method of the second pruning process and the HM4.0 encoder. As shown in Table 6, there is no coding loss when the total number of combined, non-scaled and zero MVP candidates is reduced from 8 to 5, while loss starts showing up (see table 5) when the total number of additional candidates is further reduced to 4. Therefore, setting the total number of combined, non-scaled and zero MVP candidates to 5 appears to be a good choice, which can reduce the complexity of the second pruning process by 2x without harming coding efficiency.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
|  | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | -0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.1% | 0.3% | 0.5% | 0.1% | 0.3% | 0.1% |
| Class E | 0.1% | 0.2% | 0.6% | 0.0% | 0.3% | -0.4% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

Table 5. BD-rate increase relative to HM4.0 anchor when the total number of combined, non-scaled and zero MVP candidates is limited to 4 (CE13 Subset 3.2.2 test1, see HM-4.0-anchor_vs._additionalMVP4_CE13_subset3.2.2_test1.xls)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.1% | 0.2% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.2% | 0.2% | 0.0% | 0.3% | -0.2% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.6% | 0.1% |
| Class E | 0.1% | 0.6% | 0.6% | 0.0% | 0.3% | 0.1% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.3% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.3% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 102% |  |  |

Table 6. BD-rate increase relative to HM4.0 anchor when the total number of combined, non-scaled and zero MVP candidates is limited to 5 (CE13 Subset 3.2.2 test2, see HM-4.0-anchor_vs._additionalMVP4_CE13_subset3.2.2_test1.xls)

Table 7 summarizes the combined study results on signaling the maximum number of merge/skip MVP candidates in slice header plus reducing number of MVP candidates from 12 to 9 for the second pruning process (combined method). The results in Table 7 are almost identical to Table 3,  indicating that adding the simplification of the second pruning process on the top of the signaling method won’t introduce any coding loss.

| CE13 Subset 3.2.3 Tests | maxNumMegeCand | RA-HE (%) | RA-LC (%) | LB-HE(%) | LB-LC(%) |
| --- | --- | --- | --- | --- | --- |
| Test 1b | 4 | 0.1 | 0.1 | 0.1 | 0.2 |
| Test 2b | 3 | 0.3 | 0.4 | 0.4 | 0.8 |
| Test 3b | 2 | 0.8 | 0.9 | 1.3 | 2.0 |
| Test 4b | 1 | 3.1 | 3.2 | 3.7 | 4.4 |

Table 7. Coding loss relative to HM4.0 anchor of combined method

# Please refer to the following spreadsheets for detailed results of Table 7.

- HM-4.0-anchor_vs_subset3.2.3_test1b.xls

- HM-4.0-anchor_vs_subset3.2.3_test2b.xls

- HM-4.0-anchor_vs_subset3.2.3_test3b.xls

- HM-4.0-anchor_vs_subset3.2.3_test4b.xls

The anchor method selected for Table 8 is the one which signals the maximum number of merge/skip MVP candidates in slice header (i.e. proposed signaling method). This direct comparison further verifies that adding the simplification of the second pruning process on the top of the signaling method won’t introduce any coding loss.

| CE13 Subset 3.2.1 Tests xa vs. CE13 subset 3.2.3 tests xb (x = 1,2 3,4) | maxNumMegeCand | RA-HE (%) | RA-LC (%) | LB-HE(%) | LB-LC(%) |
| --- | --- | --- | --- | --- | --- |
| Test 1a vs. 1b | 4 | 0.0 | 0.0 | 0.0 | 0.0 |
| Test 2a vs. 2b | 3 | 0.0 | 0.0 | 0.0 | 0.0 |
| Test 3a vs. 3b | 2 | 0.0 | 0.0 | 0.0 | 0.0 |
| Test 4a vs. 4b | 1 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 8. BD-rate difference between the combined method and proposed signaling method

# Please refer to the following spreadsheets for detailed results of Table 8.

- HM-4.0-CE13_subset3.2.1_test1a_vs_subset3.2.3_test1b.xls

- HM-4.0-CE13_subset3.2.1_test2a_vs_subset3.2.3_test2b.xls

- HM-4.0-CE13_subset3.2.1_test3a_vs_subset3.2.3_test3b.xls

- HM-4.0-CE13_subset3.2.1_test4a_vs_subset3.2.3_test4b.xls

# Conclusion and recommendation

The proposed signaling method and simplification method have been tested and verified in CE13. The proposed signaling method provides encoder design flexibility with maximum coding efficiency to meet design constraints. The proposed simplification method can reduce the implementation cost of the second pruning process of the merge/skip MVP derivation process by 2x without harming coding efficiency. The combined design of both methods has also been verified in CE13 and proved to be causing no coding loss. It is recommended to adopt both methods into HEVC test model 5.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Zhou, V. Sze, Y. Matsuba, “A study on HEVC parsing throughput issue”. JCT-VC Document, JCTVC-F068, Torino, IT, July 2011.

[4] J. Jung, B. Bross, J. Chen, P. Onno, M. Zhou, “Description of Core Experiment CE13: Motion data parsing robustness and throughput”, JCT-VC Document, JCTVC-F913, Torino, IT, July 2011.

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

WD texts

Related sections

### For maxNumMergeCand (changes are marked yellow)

### 7. 3. 3 Slice header syntax

### 7.4.3 Slice header semantics

### 7.3.7 Prediction unit syntax

##### Table 9-46

### For simplification of merge/skip second pruning process (changes are marked yellow)

- 8.4.2.1.5 Derivation process for combined bi-predictive merging candidates

### 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| If (slice_type == P ¦¦ slice_type == B) |  |
| 5minusmaxNumMergeCand | ue(v) |
| } |  |
| } |  |

### 7.4.3 Slice header semantics

Add :

5minusmaxNumMergeCand specifies vuale of 5 minus the maximum number of merging MVP candidates supported in the slice, 5minusmaxNumMergeCand is in the range of [0, 5], inclusive. The the maximum number of merging MVP candidates, maxNumMergeCand is equal to 5 - 5minusmaxNumMergeCand.

### 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if (maxNumMergeCand > 1) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if (maxNumMergeCand > 1) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

##### Changes in Table 9-46 (marked in yellow)

| Table 9-1 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| alf_cu_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| end_of_slice_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| split_coding_unit_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| prev_intra_luma_pred_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| … |  | … |  |  |  |
| rem_intra_luma_pred_mode | I | as specified in subclause Error: Reference source not found | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| intra_chroma_pred_mode ( IntraPredMode < 4 ) | I | TU, cMax = 3 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| intra_chroma_pred_mode ( 4 <= IntraPredMode < 34 ) | I | TU, cMax = 4 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| merge_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| merge_idx | P | TU, cMax = maxNumMergeCand | 3 | Error: Reference source not found | 0 |
|  | B |  | 3 | Error: Reference source not found | 4 |
| inter_pred_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |

##### Replace

# 8.4.2.1.5 Derivation process for combined bi-predictive merging candidates (changes marked in yellow)

With

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

- the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

The function RefPicOrderCnt( refidx, LX ) specifies the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of the current picture with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”. The variable PicOrderCnt specifies the PicOrderCnt of the current picture.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

When numOrigMergeCand is greater than 1 and less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables combIdx and combCnt are set to 0, the variable combStop is set to false and the following steps are repeated until combStop is equal to true.

- The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in             Table 8-2.

- The following assignments are made with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList ( l0Cand = mergeCandList[ l0CandIdx ] , l1Cand = mergeCandList[ l1CandIdx ] ).

- When all of the following conditions are true,

    - predFlagL0l0Cand = = 1

    - predFlagL1l1Cand = = 1

    - RefPicOrderCnt( refIdxL0l0Cand, L0 ) != RefPicOrderCnt( refIdxL1l1Cand, L1 ) | | mvL0l0Cand != mvL1l1Cand

the following applies.

    - When no candidate in mergeCandList has the same list0 motion vector and reference index as l0Cand and the same list1 motion vector and reference index as l1Cand, the candidate combCandk with k equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = combCandk ) and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are dervied as follows and numMergeCand is incremented by 1.

refIdxL0combCandk = refIdxL0l0Cand                                                                                (8-1)

refIdxL1combCandk = refIdxL1l1Cand                                                                                (8-2)

predFlagL0combCandk = 1                                                                                                    (8-3)

predFlagL1combCandk = 1                                                                                                    (8-4)

mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ]                                                                             (8-5)

mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ]                                                                              (8-6)

mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ]                                                                             (8-7)

mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ]                                                                              (8-8)

numMergeCand = numMergeCand + 1                                                                                 (8-9)

    - The variable combCnt is incremented by 1.

- The variable combIdx is incremented by 1.

- When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numMergeCand is equal to maxNumMergeCand or combCnt is equal to 5 – nscaleCnt – nzeroCnt , combStop is set 1. Where nscaleCnt and nzeroCnt are derived in subclause 8.4.2.1.5 and subclause 8.4.2.1.6, respectively.

Table 8-2 – Specification of l0CandIdx and l1CandIdx

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l0CandIdx | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 |
| l1CandIdx | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 |