| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G092 M21640 |
| --- | --- |

| Title: | AHG22: High-level signaling of lossless coding mode in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

Efficient lossless coding is required for real-word applications such as automotive vision, video conferencing and long distance education. This contribution proposes to signaling methods to enable lossless coding at picture,  region and LCU levels. Specifically, sps_lossless_coding_enabled_flag and pps_lossless_coding_enabled_flag are defined in SPS and PPS to signal whether a group of pictures are encoded losslessly; if pps_lossless_coding_enabled_flag is not set in PPS, aps_lossless_coding_enabled_flag is defined in APS to indicate whether there are regions in a picture are encoded losslessly, if yes, the lossless coding region information is encoded into in APS; if aps_lossless_coding_enabled_flag is not set in APS, slice_lossless_coding_enabled_flag is defined in slice_header() to signal whether 1-bit lossless coding flag is present at LCU level. This three-level signaling method provides a flexible way of signaling lossless coding for different use cases.

# Introduction

There are increasing needs of lossless video coding for real-world applications. For example, in the automotive vision application, video captured from cameras of a vehicle may need to be transmitted to the center processors losslessly for video analytics purpose. Video conferencing and long-distance education may need hybrid nature and syntactic video coding, in which part of video scene might contain syntactic contents such as presentation slides that need to be coded losslessly. One of problem facing the H.264/AVC high profile today is that it does not support efficient lossless coding mode. I_PCM mode is lossless but does not provide any compression.

Based on the requirements of different use cases, the HEVC lossless coding mode would need to support the following functionality

- encode entire sequence with lossless mode for use cases such as automotive vision.

- encode a sequence partly with lossless mode, and switch lossy and lossless coding as picture level for use cases such as video conferencing and remote education.

- encode a picture partly with lossless mode for use cases such as such as video conferencing and remote education. For example, part of picture may contain presentation slides, text that require lossless coding.

In this contribution, lossless mode signaling methods at picture, region and LCU-level are proposed to meet the requirements described above.

# Algorithm description

To meet requirements of lossless coding use cases scenarios, it is proposed to enable lossless coding at picture, region and LCU levels with the following signaling methods:

- Define a high-level flag in sequence parameter set (SPS), loss_coding_enabled_flag, to indicate whether lossless coding parameters are present in bitstream. lossless_coding_enabled_flag equal to 1 means that lossless coding parameters may present in SPS, PPS, APS and in slice_header().  Otherwise, lossless coding parameters are not present in bitstream.

In addition, a high-level flag in sequence parameter set (SPS),  sps_lossless_coding_enabled_flag), for use cases in which entire sequence is encoded with lossless modes (use case: video analytics for automotive vision). sps_lossless_coding_enabled_flag equal to 1 means that entire sequence is coded with lossless mode. If sps_lossless_coding_enabled_flag is not present in bitsteam, it is inferred 0.

| Seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … … |  |
| lossless_coding_enabled_flag | u(1) |
| If (lossless_coding_enabled_flag) |  |
| sps_lossless_coding_enabled_flag | u(1) |
| } |  |

- Define a high-level flag in picture parameter set (PPS) for use cases in which entire frame is encoded with lossless mode. (use case: video analytics for automotive vision). As shown in tables below, the lossless coding flag (pps_lossless_coding_enabled_flag ) is transmitted in PPS only if the lossless coding flag (sps_lossless_coding_enabled_flag) is not set in SPS.  sps_lossless_coding_enabled_flag equal to 1 means that entire frame referencing to the PPS is coded with lossless mode. If pps_lossless_coding_enabled_flag is not present in bitstream, it is inferred to the value of   sps_lossless_coding_enabled_flag .

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … … |  |
| If (lossless_coding_enabled_flag && !sps_loss_coding_enabled_flag) |  |
| pps_lossless_coding_enabled_flag | u(1) |
| } |  |

- Define a flag is adaptation parameter set (APS) to signal whether part of a frame is coded losslessly. If yes, define how many rectangular regions in the frame (in unit of LCUs) need to be encoded losslessly, followed by sending the parameters that define the losslessly coded rectangular regions (see table below). (use case: hybrid nature and syntactical video coding).  In HEVC a slice header references to both PPS and to APS, this mode is active only if lossless coding flag (i.e. pps_lossless_coding_enabled_flag) is not set in PPS that the slice header references to. aps_lossless_coding_enabled_flag equal to 1 means that regions of the frame are coded with lossless mode and region information is transmitted. If aps_lossless_coding_enabled_flag is not present in bitstream, it is inferred to 0.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … … |  |
| If (lossless_coding_enabled_flag) { |  |
| f (!pps_lossless_coding_enabled_flag) |  |
| aps_lossless_coding_enabled_flag | u(1) |
| if (aps_lossless_coding_enabled_flag) { |  |
| num_of_lossless_regions_minus1 | ue(v) |
| for (i = 0; i < num_of_lossless_regions_minus1 + 1; i++) { |  |
| region_upper_left_corner_x[i] | ue(v) |
| region_upper_left_corner_y[i] | ue(v) |
| region_width_minus1[i] | ue(v) |
| region_height_minus1[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Where

num_of_lossless_regions_minus1 + 1 specifies the number of losslessly coded regions in a frame.

region_upper_left_corner_x[i] specifies the  horizontal LCU address of the upper-left corner of the ith losslessly coded region in a frame

region_upper_left_corner_y[i] specifies the vertical  LCU address of the upper-left corner the ith losslessly coded region in a frame

region_width_minus1[i] + 1 specifies the horizontal size of the ith losslessly coded region in a frame (in unit of LCUs)

region_height_minus1[i] + 1 specifies the vertical size of the ith losslessly coded region in a frame (in unit of LCUs)

- Define a flag in slice header to signal whether the lossless coding is used for LCUs inside a frame. This flag allows switch between lossless and lossy coding from LCU to LCU, and provides the maximum flexibility. If slice_lossless_coding_enabled_flag is equal 1 in slice header, a 1-bit flag is transmitted for each LCU to signal lossless or lossy coding for the LCU. Note that this flag is only active if lossless coding flag in APS that the slice header refers to is not set.  If slice_lossless_coding_enabled_flag is not present in bitstream, it is inferred to 0.

| slice_header( ) { | Descriptor |
| --- | --- |
| If (lossless_coding_enabled_flag && !aps_lossless_coding_enabled_flag) |  |
| slice_lossless_coding_enabled_flag | u(1) |
| } |  |

Deblocking filter, SAO and ALF should be disabled for samples of LCUs that are coded losslessly. Along the LCU boundaries between the lossy and lossless coding areas in a frame, the de-blocking filter, SAO and ALF can be still enabled for samples of LCUs that use lossy coding. In particular, for the deblocking filter along boundary edges between lossy and lossless LCUs, samples on the lossy LCU side are filtered, while samples on the lossless LCU side are not filtered. The deblocking filter process (i.e. filter on/off decision and strong/weak filtering) is unchanged.

# Conclusion and recommendation

Efficient HEVC lossless coding is required for real-word applications such as automotive vision and video conferencing. It is recommended to conduct further study of lossless coding signaling methods in CE.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text changes

Related sections (changes are merged yellow in each section)

7.3.2. 1   Sequence parameter set RBSP syntax

7.4.2. 1   Sequence parameter set RBSP semantics

7.3.2. 2.  Picture parameter set RBSP syntax

7.4.2. 2.  Picture parameter set RBSP semantics

7.3.3       Slice header syntax

7.4.3       Slice header semantics

7.3.4       slice data RBSP syntax

7.4.4       slice data RBSP semantics

Adaptation parameter set RBSP syntax and semantics changes:

7.3.2. 1   Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| lossless_coding_enabled_flag | u(1) |
| if (lossless_coding_enabled_flag) |  |
| sps_lossless_coding_enbled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.4.2.1 Sequence parameter set RBSP semantics

Add

lossless_coding_enabled_flag equal to 1 specifies  that lossless coding parameters sps_lossless_coding_enbled_flag, pps_lossless_coding_enabled_flag, aps_lossless_coding_enabled_flag, slice_lossless_coding_flag and lossless coding region information may present in the bitstream.  Otherwise, lossless coding parameters are not present in bitstream.

sps_lossless_coding_enbled_flag equal to 1 specifies  that all the picture references to the active sequence parameter set are encoded with lossless mode. If sps_lossless_coding_enbled_flag is not present in the bitsteam, it is inferred to 0.

7.3.2. 2. Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |  |  |
| --- | --- | --- | --- |
| pic_parameter_set_id | ue(v) |  |  |
| seq_parameter_set_id | ue(v) |  |  |
| entropy_coding_mode_flag | u(1) |  |  |
| num_temporal_layer_switching_point_flags | ue(v) |  |  |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |  |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |  |  |
| num_ref_idx_l0_default_active_minus1 | ue(v) |  |  |
| num_ref_idx_l1_default_active_minus1 | ue(v) |  |  |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |  |  |
| constrained_intra_pred_flag | u(1) |  |  |
| slice_granularity | u(2) |  |  |
| shared_pps_info_enabled_flag | u(1) |  |  |
| if( shared_pps_info_enabled_flag ) |  |  |  |
| if( adaptive_loop_filter_enabled_flag ) |  |  |  |
| alf_param( ) |  |  |  |
| if( cu_qp_delta_enabled_flag ) |  |  |  |
| max_cu_qp_delta_depth | u(4) |  |  |
| if (lossless_coding_enabled_flag && !sps_loss_coding_enabled_flag) |  |  |  |
| pps_lossless_coding_enabled_flag |  | u(1) |  |
| rbsp_trailing_bits( ) |  |  |  |
| } |  |  |  |

7.4.2.2.  Picture parameter set RBSP semantics

Add

pps_lossless_coding_enabled_flag equal to 1 specifies  that a picture references to the active picture parameter set are encoded with lossless mode. If pps_lossless_coding_enabled_flag is not present in the bitstream, it is inferred to the value of sps_lossless_coding_enbled_flag.

7.3.3

Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if (lossless_coding_enabled_flag && !aps_lossless_coding_enabled_flag) |  |
| slice_lossless_coding_enabled_flag | u(1) |
| } |  |
| } |  |

7.4.3  Slice header semantics

slice_lossless_coding_enabled_flag equal to 1 specifies that lcu_lossless_coding_enabled_flag is present in slice data at beginning of each coding tree. If  slice_lossless_coding_enabled_flag is not present in the bitstream, it is inferred to 0.

7.3. 4 Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CurrTbAddr = LCUAddress |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = -1 |  |
| do { |  |
| xCU = HorLumaLocation( CurrTbAddr ) |  |
| yCU = VerLumaLocation( CurrTbAddr ) |  |
| If (moreDataFlag && lossless_coding_enabled_flag && slice_lossless_coding_enabled_flag) |  |
| lcu_lossless_coding_enabled_flag | u(1)/ue(1) |
| moreDataFlag = coding_tree( xCU, yCU, Log2TbSize, 0 ) |  |
| CurrTbAddr = NextTbAddress( CurrTbAddr ) |  |
| } while( moreDataFlag ) |  |
| } |  |

7.4.4    Slice data semantics

lcu_lossless_coding_enabled_flag equal to 1 specifies that the coding tree is coded with lossless mode.

If lcu_lossless_coding_enabled_flag is not present in the bitstream, it is inferred to 1 if any of the following conditions is true

- if sps_lossless_coding_enabled_flag is equal to 1

- if pps_lossless_coding_enabled_flag is equal to 1

- if aps_lossless_coding_enabled_flag is equal to 1 and the current coding tree belongs to a lossless coding region that is specified by region_upper_left_corner_x[i], region_upper_left_corner_y[i], region_width_minus1[i] and region_height_minus1[i] with i = 0, 1, .., num_of_lossless_regions_minus1

Otherwise, lcu_lossless_coding_enabled_flag is inferred to 0.

Adaptation parameter set RBSP syntax and semantics changes:

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … … |  |
| If (lossless_coding_enabled_flag) { |  |
| f (!pps_lossless_coding_enabled_flag) |  |
| aps_lossless_coding_enabled_flag | u(1) |
| if (aps_lossless_coding_enabled_flag) { |  |
| num_of_lossless_regions_minus1 | ue(v) |
| for (i = 0; i < num_of_lossless_regions_minus1 + 1; i++) { |  |
| region_upper_left_corner_x[i] | ue(v) |
| region_upper_left_corner_y[i] | ue(v) |
| region_width_minus1[i] | ue(v) |
| region_height_minus1[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics:

aps_lossless_coding_enabled_flag equal to 1 specifies that regions of a picture referencing to the active adaptation parameter set are encoded with lossless mode. If aps_lossless_coding_enabled_flag is not present in the bitstream, it is inferred to 0.

num_of_lossless_regions_minus1 + 1 specifies the number of losslessly coded regions in a frame.

region_upper_left_corner_x[i] specifies the  horizontal LCU address of the upper-left corner of the ith losslessly coded region in a frame

region_upper_left_corner_y[i] specifies the vertical  LCU address of the upper-left corner the ith losslessly coded region in a frame

region_width_minus1[i] + 1 specifies the horizontal size of the ith losslessly coded region in a frame (in unit of LCUs)

region_height_minus1[i] + 1 specifies the vertical size of the ith losslessly coded region in a frame (in unit of LCUs)