| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G094 M21642 |
| --- | --- |

| Title: | Non-CE04: Carriage of large block size quantization matrices with up-sampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1-214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution advocates use of up-sampled quantization matrices for 16x16 and 32x32 block sizes to reduce overhead spent on transmitting large block size quantization matrices. In the proposed algorithm both 16x16 and 32x32 quantization matrices are not present in the bitstream. Instead, they are derived from up-sampling 8x8 quantization matrices of same type on both the encoder and decoder sides.  For CE4 test2 in which quantization matrices change from frame to frame, the proposed algorithm provides an average bit-rate saving of 14.5% in AI-HE, 13.7% in AI-LC, 43.9% in RA-HE, 42.6% in RA-LC, 55.9% in LB-HE and 54.7% in LB-LC for symmetric matrices. The average bit-rate saving numbers for for asymmetric matrices are 13.6% in AI-HE, 12.6% in AI-LC, 43.1% in RA-HE, 41.4% in RA-LC, 55.3% in LB-HE and 53.7% in LB-LC.

# Introduction

In the current HEVC design the transform and quantization block sizes can go up to 32x32. The large block transforms can provide improved coding efficiency, but also lead to larger overhead for carrying the perceptual quantization matrices in the picture parameter sets. In HEVC there can be a total of 24 matrices (separate matrix for 4x4, 8x8, 16x16 and 32x32 four block sizes, intra/inter and Y, U, V components). It was reported in [3] that such an overhead can be roughly 10x of that of AVC if the AVC quantization matrix compression method is used. Therefore, it is desirable to design a compact representation format for encoding the quantization matrices of large block sizes.

This contribution advocates use of up-sampled quantization matrices for 16x16 and 32x32 block sizes to reduce overhead spent on transmitting large block size quantization matrices. The algorithm was integrated into the HM4.0 based CE4 software, and tested by using the test vectors agreed within the CE4 group.

# Algorithm Description

The decoder side diagram of proposed algorithm is depicted in Figure 1. In the proposed algorithm, 4x4 and 8x8 quantization matrices are decoded from bitstream, while 16x16 and 32x32 quantization matrices are directly up-sampled from reconstructed 8x8 ones of same type (Y, U, or V, inter/intra). Coding of 4x4 and 8x8 matrices can employ any method. In this contribution the AVC method is used.

bitstream

8x8 QMs

4x4 QMs

32x32 QMs

Factor 2 up-sampling

Decoded quantization matrices

decoding

Factor 2 up-sampling

16x16 QMs

Figure 1.  Diagram of proposed quantization matrix compression algorithm

# Test Settings and Conditions

The quantization matrices used for testing are both asymmetry and symmetry ones provided to CE4 by SONY. Attached text files qmatrix_asymmetry1.txt and qmatrix_symmetry1.txt contain quantization matrices used in testing.

CE4 test2 is carried out to quantify the BD-rate saving against the AVC method when the quantization matrices change from frame to frame.

# Experimental results

The experimental results are summarized in Table 1 and Table 2. For CE4 test2 in which quantization matrices change from frame to frame, the proposed algorithm provides an average bit-rate saving of 14.5% in AI-HE, 13.7% in AI-LC, 43.9% in RA-HE, 42.6% in RA-LC, 55.9% in LB-HE and 54.7% in LB-LC for symmetric matrices, and an average bit-rate saving of 13.6% in AI-HE, 12.6% in AI-LC, 43.1% in RA-HE, 41.4% in RA-LC, 55.3% in LB-HE and 53.7% in LB-LC for asymmetric matrices.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -5.1% | -0.5% | 0.6% | -5.2% | 2.5% | 3.4% |
| Class B | -6.4% | -5.6% | -5.6% | -6.0% | -4.1% | -4.2% |
| Class C | -14.7% | -13.7% | -13.7% | -13.6% | -12.5% | -12.6% |
| Class D | -31.9% | -29.9% | -30.2% | -30.4% | -28.6% | -28.8% |
| Class E | -17.4% | -15.9% | -15.5% | -15.8% | -13.4% | -13.4% |
| Class F |  |  |  |  |  |  |
| Overall | -14.5% | -12.6% | -12.4% | -13.7% | -10.7% | -10.6% |
|  | -14.5% | -12.7% | -12.4% | -13.7% | -10.8% | -10.7% |
| Enc Time[%] | 108% |  |  | 110% |  |  |
| Dec Time[%] | 104% |  |  | 106% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -20.6% | -15.5% | -13.2% | -19.3% | -13.7% | -10.7% |
| Class B | -34.3% | -31.2% | -30.1% | -32.4% | -29.4% | -28.2% |
| Class C | -51.5% | -49.5% | -49.5% | -50.3% | -48.3% | -48.3% |
| Class D | -71.5% | -70.0% | -70.2% | -70.9% | -69.6% | -69.7% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -43.9% | -40.9% | -40.1% | -42.6% | -39.6% | -38.6% |
|  | -43.9% | -41.1% | -40.3% | -42.6% | -39.7% | -38.7% |
| Enc Time[%] | 102% |  |  | 107% |  |  |
| Dec Time[%] | 106% |  |  | 112% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -35.9% | -31.4% | -30.6% | -34.3% | -30.3% | -29.0% |
| Class C | -51.6% | -49.0% | -48.8% | -50.5% | -48.0% | -48.1% |
| Class D | -71.1% | -69.4% | -69.5% | -70.6% | -68.8% | -69.2% |
| Class E | -74.4% | -72.2% | -72.4% | -73.1% | -70.9% | -71.5% |
| Class F |  |  |  |  |  |  |
| Overall | -55.9% | -52.9% | -52.7% | -54.7% | -52.0% | -51.8% |
|  | -55.9% | -53.0% | -52.8% | -54.7% | -52.0% | -51.9% |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 110% |  |  | 112% |  |  |

Table 1.  Bit-rate saving relative to CE4 AVC method for symmetry matrices

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -3.8% | -1.4% | -1.3% | -3.8% | -0.1% | -0.1% |
| Class B | -5.9% | -5.2% | -5.1% | -5.5% | -3.9% | -4.1% |
| Class C | -14.0% | -12.6% | -12.8% | -12.6% | -11.4% | -11.6% |
| Class D | -30.2% | -28.1% | -28.4% | -28.3% | -26.3% | -26.5% |
| Class E | -17.2% | -15.6% | -15.5% | -15.3% | -12.9% | -13.1% |
| Class F |  |  |  |  |  |  |
| Overall | -13.6% | -12.1% | -12.1% | -12.6% | -10.5% | -10.6% |
|  | -13.6% | -12.2% | -12.2% | -12.6% | -10.6% | -10.7% |
| Enc Time[%] | 89% |  |  | 102% |  |  |
| Dec Time[%] | 89% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -19.2% | -11.5% | -12.0% | -17.6% | -9.5% | -10.6% |
| Class B | -33.7% | -30.6% | -29.4% | -31.3% | -28.1% | -27.3% |
| Class C | -50.9% | -48.8% | -48.6% | -49.2% | -47.1% | -47.0% |
| Class D | -70.9% | -69.4% | -69.5% | -69.9% | -68.6% | -68.7% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -43.1% | -39.5% | -39.3% | -41.4% | -37.7% | -37.7% |
|  | -43.1% | -39.6% | -39.4% | -41.5% | -37.8% | -37.8% |
| Enc Time[%] | 97% |  |  | 105% |  |  |
| Dec Time[%] | 95% |  |  | 109% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -35.2% | -30.8% | -30.0% | -32.9% | -28.8% | -28.0% |
| Class C | -50.9% | -48.2% | -48.1% | -49.3% | -46.7% | -46.8% |
| Class D | -70.5% | -68.7% | -68.7% | -69.7% | -68.1% | -68.1% |
| Class E | -74.4% | -72.0% | -72.3% | -72.8% | -70.2% | -71.0% |
| Class F |  |  |  |  |  |  |
| Overall | -55.3% | -52.3% | -52.1% | -53.7% | -50.9% | -50.8% |
|  | -55.3% | -52.4% | -52.3% | -53.7% | -50.9% | -50.9% |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 108% |  |  |

Table 2.  Bit-rate saving relative to CE4 AVC method for asymmetry matrices

# Conclusion and recommendation

It has been demonstrated that simple matrix up-sampling for large block size quantization matrices can provide significant bit-rate saving when compared to the AVC method. It is recommended to include this method in the next round of CE for further study on the compact representation of quantization matrices for HEVC.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Zhou, V. Sze, “Compact representation of quantization matrices for HEVC”. JCT-VC Document, : JCTVC-D024, Daegu, Korea, January, 2011

[4] M. Zhou, V. Sze, “Further study on compact representation of quantization matrices for HEVC”. JCT-VC Document, : JCTVC-F085, Torino, IT, 14-22 July, 2011

[5] K.Sato, M. Budagavi, M. Coban, H.Aoki,  X.Li,  “Description of Core Experiment 4: Quantization”, JCT-VC Document,  JCTVC-F904, Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text

## Derivation process of 8x8 and 4x4 quantization matrices

Directly copy from relevant parts of the AVC spec

## Derivation process of 16x16 quantization matrices

Inputs to this process is 8x8 quantization matrix Qmatx8x8(c, x, y), with c = Y, U or V, and x, y = 0, 1, 2…7.

Output of this process is up-sampled quantization matrix Qmatx16x16(c, x, y), with c = Y, U or V, and x, y = 0, 1, 2…15

The quantization up-sampling process of subclause 8.4 is invoked with N equal to 8 and Qmatx8x8(c, x, y) as inputs to obtain up-sampled 16x16 matrix Qmatx16x16(c, x, y).

## Derivation process of 32x32 quantization matrices

Inputs to this process is 8x8 quantization matrix Qmatx8x8(c, x, y), with c = Y, U or V, and x, y = 0, 1, 2…7

Output of this process is up-sampled quantization matrix Qmatx32x32(c, x, y), with c = Y, U or V, and x, y = 0, 1, 2…32

A 32x32 is obtained by the following step:

- First, the quantization up-sampling process of subclause 8.4 is invoked with N equal to 8 and Qmatx8x8(c, x, y) as inputs to obtain up-sampled 16x16 matrix Qmatx16x16(c, x, y).

- Then, the quantization up-sampling process of subclause 8.4 is invoked with N equal to 16 and Qmatx16x16(c, x, y) as inputs to obtain up-sampled 32x32 matrix Qmatx32x32(c, x, y).

## Derivation process of quantization matrix up-sampling

Inputs to this process is NxN quantization matrix QmatxNxN(c, x, y), with c = Y, U or V, and x, y = 0, 1, 2…N-1.

Output of this process is up-sampled quantization matrix Qmatx2Nx2N(c, x, y), with c = Y, U or V, and x, y = 0, 1, 2…2*N-1.

The following ordered steps apply:

- Matrix copy

  - Qmatx2Nx2N(c, 0, 0)               = QmatxNxN(c, 0, 0)

  - Qmatx2Nx2N(c, 0, 2*j+1)        = QmatxNxN(c, 0, j) ; for j = 1, 2, .. N-1

  - Qmatx2Nx2N(c, 2*i+1, 0)        = QmatxNxN(c, i, 0); for i = 1, 2, .., N-1

  - Qmatx2Nx2N(c, 2*i+1, 2*j+1)  = QmatxNxN(c, i, j); for i,j = 1, 2, .., N-1

- Horizontal matrix up-sampling

  - Qmatx2Nx2N(c, 0, 1)               = (21*Qmatx2Nx2N(c, 0, 0) + 11* Qmatx2Nx2N(c, 0, 3) + 16)>>5

  - Qmatx2Nx2N(c, 0, 2)               = (11*Qmatx2Nx2N(c, 0, 0) + 21* Qmatx2Nx2N(c, 0, 3) + 16)>>5

  - Qmatx2Nx2N(c, 0, 2*j+2)        = (Qmatx2Nx2N(c, 0, 2*j+1) + Qmatx2Nx2N(c, 0, 2*j+3) + 1)>>1; for j = 1, 2, …, N-2

  - Qmatx2Nx2N(c, 2*i+1, 1)    = (21*Qmatx2Nx2N(c, 2*i+1, 0) + 11* Qmatx2Nx2N(c, 2*i+1, 3) + 16)>>5; for i = 1, 2, .., N-1

  - Qmatx2Nx2N(c, 2*i+1, 2)     = (11*Qmatx2Nx2N(c, 2*i+1, 0) + 21* Qmatx2Nx2N(c, 2*i+1, 3) + 16)>>5; for i = 1, 2, .., N-1

  - Qmatx2Nx2N(c, 2*i+1, 2*j+2) = (Qmatx2Nx2N(c, 2*i+1, 2*j+1) + Qmatx2Nx2N(c, 2*i+1, 2*j+3) + 1)>>1; for i = 1, 2, …, N-1, j = 1, 2, .., N-2

- Vertical matrix up-sampling

  - Qmatx2Nx2N(c, 1, j)               = (21*Qmatx2Nx2N(c, 0, j) + 11* Qmatx2Nx2N(c, 3, j) + 16)>>5; for j = 0,1, 2, …, 2*N-1

  - Qmatx2Nx2N(c, 2, j)               = (11*Qmatx2Nx2N(c, 0, j) + 21* Qmatx2Nx2N(c, 3, j) + 16)>>5; for j = 0,1, 2, …, 2*N-1

  - Qmatx2Nx2N(c, 2*i+2, j)       =  (Qmatx2Nx2N(c, 2*i+1, j) + 21* Qmatx2Nx2N(c, 2*i+3, j) + 1)>>1; for i = 1, 2, ..., N-2; j = 0, 1,2, .., 2*N-1