| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G095 M21643 |
| --- | --- |

| Title: | Virtual motion compensation memory bandwidth verifier (VMBV) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

# This contribution proposes an virtual motion compensation memory bandwidth verifier (VMBV) aimed to lower the worst case motion compensation memory bandwidth requirements without harming coding efficiency or imposing significant burden on the encoder side. The proposed VMBV advocates: 1) to define virtual maximum memory access bandwidth rate for profile & levels, say Rmax, measured in unit of bytes per second; 2) to specify a virtual memory access bandwidth measurement tool which quantifies the memory access bandwidth consumed by a decoded frame, say dn, measured in bytes; and 3) to impose the VMBV conformance on bitstreams: for any frame the consumed memory access bandwidth dn shall be less than or equal to R* Δtn, where R (R ≤ Rmax) is the virtual memory access bandwidth rate signaled in sequence parameter set,  and Δtn is time used for decoding the frame. It is recommended to conduct the further study on this topic to specify the proper VMBV memory bandwidth measurement tool, and to implement the VMBV constraint in the encoder motion estimation to investigate the practicality of the proposed VMBV model.

# Introduction

This contribution proposes a virtual motion compensation memory bandwidth verifier (VMBV) to lower the worst case motion compensation memory bandwidth requirements without harming coding efficiency or imposing significant burden on the encoder side.

# Problem statement

In the current HM4.0 design the 8-tap for luminance motion compensation is employed to improve the inter prediction accuracy. Compared to the AVC/H.264 in which 6-tap interpolation filter is use for luminance motion compensation, the HEVC needs higher memory access bandwidth on the decoder side.

Motion vector

Reference frame

Current frame

Reference data

Co-located PU

current PU

M

N

M+7

N+7

Figure 1. Illustration of reference block for the current PU in HEVC

As shown in Figure 1, for motion compensation of the current PU the reference block needs to be loaded from the off-chip memory, then 8-tap interpolation on the reference data is carried out to produce the motion compensated prediction block for the current PU.  Because of 8-tap filter used, a reference data block of (M+7) x (N+7) samples would needed to be loaded from the reference frame if the current PU has a size of M x N.

1

Reference frame

Current frame

current PUs

0

1

2

3

0

1

2

3

Reference blocks

Figure 2. Illustration of reference block overlapping in typical cases

In the typical cases, motion vectors from PU to PU are oftentimes similar. Therefore, there is usually lots of overlapping among reference blocks of neighboring PUs. As shown in Figure 2, the reference blocks for four neighboring PUs are largely overlapped. By leveraging the reference block overlapping, the reference blocks from neighboring PUs can be partially re-used for the motion compensation of the current PU if a proper reference block caching mechanism is implemented in place. This reduces the overall memory access bandwidth requirements for motion compensation because the decoder can avoid need of fully loading the reference block for the current PU.

However, in the worst cases (evil cases) an encoder can always generate a motion vector field in which the reference blocks for neighboring PUs are totally independently. This essentially breaks the assumption of the reference block caching model, and leads to much higher memory access bandwidth requirements. Figure 3 depicts such an example, in which the reference blocks for the four neighboring PUs totally separate from each other.   There is always a huge gap in terms of memory access bandwidth between the typical cases and worst cases, which leads to unnecessary high decoder cost without actual coding efficiency benefit, as the decoder designs have to cover the worst case. Early study [4] shows that such a gap can range form 2x to 3x.

Reference frame

Current frame

current PUs

0

1

2

3

0

1

2

3

Reference blocks

Figure 3. Illustration of reference block layout in the worst cases (evil cases).

Because of 8-tap filter used, the worst case memory access bandwidth requirements for HEVC motion compensation increases about 41% when compared to the H.264/AVC which uses 6-tap filter (Assuming 4x8 or 8x4 as minimum PU size).

Memory access bandwidth for motion compensation is always a bottleneck for decoder design. This bottleneck is becoming worse when the video resolutions are moving from HD to Ultra-HD. Therefore, it is highly desirable to develop a method which can impose motion compensation memory access bandwidth constraints on the HEVC bitstreams, to narrow the memory access bandwidth gap between the typical cases and worst access and to lower the decoder cost without harming coding efficiency.

# Algorithm description

To prevent encoder from generating evil bitstreams in which no reference block overlapping exists as shown in Figure 3, we propose a virtual memory access bandwidth verifier (VMBV) algorithm to impose the motion compensation memory access bandwidth constraints on the HEVC bitstreams.

Figure 4 depicts the diagram of the proposed virtual memory access bandwidth verifier algorithm which consists of the following parts:

- Define virtual maximum memory access bandwidth rate for profile & levels, say Rmax, measured in unit of bytes per second.

- Define a virtual memory access bandwidth measurement algorithm which quantifies the memory access bandwidth consumed by a decoded frame, say dn, measured in bytes.

- Impose the VMBV conformance on bitstreams: for any frame the consumed memory access bandwidth dn shall be less than or equal to R* Δtn, where R (R ≤ Rmax) is the virtual memory access bandwidth rate signaled in sequence parameter set,  and Δtn is time used for decoding the frame.

As shown in Figure 4, the motion vector field and DCT-coefficients for the current frame are decoded by the entropy decoding, then the reference blocks for LCUs are loaded and motion compensation is performed, the rest of decoding process is carried out to reconstruct the current frame. In the meantime, the motion compensation memory access bandwidth is measured by a virtual memory access bandwidth measurement algorithm LCU by LCU, and accumulated together for the current frame. After the current frame is decoded, the accumulated memory access bandwidth dn for the frame is compared with the memory access bandwidth budget R* Δtn to verify the bitstream conformance in terms of memory access bandwidth. A conformance bitstream has to satisfy   dn ≤ R* Δtn.

bitstreamm

Memory access bandwidth of a decoded frame

Decoded frame

+

+

Rest of decoding

process

Motion compensation

Entropy decoding

R*Δtn

Load reference blocks

Memory bandwidth measurement

dn

Conformance check

Memory access bandwidth budget

Figure 4. Diagram of decoder process with virtual memory access bandwidth verifier

The virtual motion compensation memory access bandwidth measurement algorithm needs to be standardized to quantify the amount of memory access bandwidth consumed by motion compensation of decoded frames during the decoder process. A good candidate is the one described in [3]. To simplify the encoder design, it is recommended to specify the maximum motion compensation memory access bandwidth rate Rmax that a compliant decoder needs to supply for just one typical reference block caching model. For example Rmax could be specified for the following typical reference block caching model for different profiles @ levels:

- Capacity (cache size): 128 KB

- Line size: 64 bytes

- Organization: 4-way set-associative

- Replacement strategy: LRU (Least Recently Used (LRU) strategy is industry standard)

- DDR alignment: 64 bits

- DDR burst size 128 bits

It is up to users to map the VMBV profile@level constraint Rmax defined in the standard to different reference block caching models, should users choose to use a caching model which is different from the one specified in the standard for the decoder design.

# Conclusion

# With the VMBV buffer model described about, the evil bitstreams which require huge amount of motion compensation memory access bandwidth can potentially be prevented, as long as the VMBV profile@level constraint Rmax is properly defined. The specification of Rmax shall consider minimizing the potential coding efficiency loss due to constraint that must be imposed on the encoder motion estimation for the VMBV compliance. It is recommended to conduct the further study in an AHG to specify the proper VMBV memory bandwidth measurement algorithm, and to implement the VMBV constraint in the encoder motion estimation to investigate the practicality of the proposed algorithm.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] K. Chono, T. Chujoh, C. Lim “JVT-VC AHG report: Memory compression”, JCTVC document JCTVC-C007, 3rd meeting, Guangzhou, CN, 7-15 October, 2010

[4] M. Zhou, M. Budagavi, “Testing results of TI reference frame compression algorithm using TMuC-0.9”, JCTVC document JCTVC-D023, 4th Meeting: Daegu, Korea, 20-28 January, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).