| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G096 M21644 |
| --- | --- |

| Title: | Items to be clarified in HEVC design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Request for clarifications |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Wade K. Wan Broadcom Corporation, USA T. Suzuki, A. Tabatabai Sony Corporation and Sony Electronics Inc | Tel: Email: Tel: Email: Tel: Email | +1-214-480-3816 zhou@ti.com +1-949-926-6387 wwan@broadcom.com +81-50-3750-2740 +1-408-352-4715 teruhikos@jp.sony.com ali.tabatabai@am.sony.com |
| Source: | Texas Instruments Inc; Broadcom Corporation; Sony Corporation and Sony Electronics Inc |  |  |

_____________________________

# Abstract

This contribution discusses several aspects of the HEVC codec that could benefit from explicitly specifying the operating range on their usage.  There may be different opinions about these items so it would be helpful to explicitly include language specifying their usage to initiate detailed discussion and/or confirm general agreement.  This would also benefit HEVC architectural studies to better analyze and identify implementation issues such as corner case scenarios.  The following aspects are discussed with recommended usage provided: minimum and maximum LCU size; minimum slice granularity; maximum number of reference frames; minimum inter-prediction block size, and minimum chroma de-blocking edge size.

# Introduction

Architectural and implementation studies to address complexity assessment often are dependent on correctly determining the operating range to identify issues such as corner case scenarios.  This contribution attempts to aid these studies by discussing several components of the current HEVC design that could benefit from explicitly clarifying their operating range.

# Items

## Maximum and minimum LCU size

The maximum and minimum LCU sizes have a significant impact on HEVC codec data flow and architecture. A maximum LCU size of 64 x 64 has proven to be a good trade-off between coding efficiency and complexity, and has been used in common test conditions since HM 1.0.  A minimum LCU size of 16x16 is what the current HM design can technically support due to the 8x8 block-based luma/chroma de-blocking filter. In the past, the finest pipeline granularity built for existing video standards is 16x16, which has proven to be a good trade-off between coding efficiency and latency. Despite the potential to reduce latency by 8 lines, supporting a minimum LCU size of 8x8 is undesirable due to disproportionally high control overhead for pipelining and poor coding efficiency.

It is recommended that a maximum LCU size of 64 x 64 and a minimum LCU size of 16 x 16 be specified in the HEVC standard as follows:

- Change/redefine the syntax element log2_min_coding_block_size_minus3 to log2_min_coding_block_size_minus4

- Constrain log2_min_coding_block_size_minus4 <= 2

- Constrain log2_diff_max_min_coding_block_size <=2 when log2_min_coding_block_size_minus4 = 0

- Constrain log2_diff_max_min_coding_block_size <=1 when log2_min_coding_block_size_minus4 = 1

- Constrain log2_diff_max_min_coding_block_size =0 when log2_min_coding_block_size_minus4 = 2

## Minimum slice granularity

Slice granularity provides a method to use different slice parameters within a CU and designed for providing granularity for large CU sizes, e.g. multiple slices within a 64 x 64 CU.  A minimum slice granularity of 16 x 16 appears to be a good trade-off between the benefits of slice granularity which can diminish for smaller slices and the processing needed to support it.  A minimum slice granularity of 16 x 16 would also correspond to what the current HM design can support due to the 8x8 block-based luma/chroma de-blocking filter.

It is recommended that a minimum slice granularity of 16 x 16 be specified in the HEVC standard as follows:

- The syntax element slice_granularity should have the additional requirement that (Log2MaxCUSize – slice_granularity) >= 4

## Maximum number of reference frames

In H.264/AVC, the maximum number of reference pictures is 16 and 32 for frame-picture and field-picture, respectively. AVC has demonstrated that up to 16 reference frames are sufficient for a wide range of use cases. Since HEVC supports progressive sequences only, it is logical to limit the maximum number of reference pictures to 16.  However, WD4.0 currently specifies that the maximum number of reference frames is allowed to be up to 32.

It is recommended that a maximum number of reference frames of 16 be specified in the HEVC standard as follows:

- num_ref_idx_l0_default_active_minus1 specifies how num_ref_idx_l0_active_minus1 is inferred for P and B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l0_default_active_minus1 shall be in the range of 0 to 15, inclusive.

- num_ref_idx_l1_default_active_minus1 specifies how num_ref_idx_l1_active_minus1 is inferred for B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l1_default_active_minus1 shall be in the range of 0 to 15, inclusive.

- num_ref_idx_l0_active_minus1 specifies the maximum reference index for reference picture list 0 that shall be used to decode the slice.

When the current slice is a P or B slice and num_ref_idx_l0_active_minus1 is not present, num_ref_idx_l0_active_minus1 shall be inferred to be equal to num_ref_idx_l0_default_active_minus1.

Otherwise, num_ref_idx_l0_active_minus1 shall be in the range of 0 to 15, inclusive.

- num_ref_idx_l1_active_minus1 specifies the maximum reference index for reference picture list 1 that shall be used to decode the slice.

When the current slice is a B slice and num_ref_idx_l1_active_minus1 is not present, num_ref_idx_l1_active_minus1 shall be inferred to be equal to num_ref_idx_l1_default_active_minus1.

The range of num_ref_idx_l1_active_minus1 is constrained as specified in the semantics for num_ref_idx_l0_active_minus1 with l0 and list 0 replaced by l1 and list 1, respectively.

## 4x4 inter-prediction

At the Torino meeting, a flag to enable/disable 4x4 inter-prediction was added to the HEVC test model and 4x4 inter prediction was disabled in the common testing conditions. A recent architectural study shows that removing 4x4 inter-prediction would lower the implementation cost for merge/skip and AMVP MVP list derivation. For a typical H/W implementation, the cycle budget for each 4x4 block is around 12 cycles. With this cycle budget, the MVPs of neighboring PUs are forced to be stored in registers to allow quick access of MVP data, because at least 1 cycle per MVP is needed for loading MVP data stored in SRAM. Buffering the motion vectors for an entire LCU in registers is very expensive, and could require 123 Kgates (assuming 64x64 LCU size, vector block size down to 4x4, bi-directional prediction, 12-bit motion vector precision, and 1-bit for 10 gates). With removal of 4x4 inter-prediction, the cycle budget would be doubled (minimum PU size becomes 4x8 or 8x4), which make it possible to buffer MVPs in SRAM instead of in registers and still meet throughput requirements.  Another reason to remove 4x4 inter-prediction is that this would reduce the worst case memory bandwidth usage for motion compensation.  More specifically, pixels around the prediction block to account for the interpolation filter as well as the prediction block must be fetched from memory for each block.  With the filter length being independent of the block size, the 4x4 case requires more memory bandwidth for the same pixel region than prediction using larger block sizes.

It is recommended to remove 4x4 inter-prediction from the HEVC standard as follows:

- Fix inter_4x4_enabled_flag to 0 for all profiles.

## Chroma deblocking block edge size

In current HM reference software, the size for chroma deblocking, similar to luma edge size, is set to 8x8. In JCTVC-F803_d5 WD (section 8.6.4.1.3), the filtering process for chroma block edge is defined for edge size of 4, as shown below:

If verticalEdgeFlag is equal to 1, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

qi = s’[ xC + xB +i, yC + yB + k ]                                                                                      (8-442)

pi = s’[ xC + xB – i – 1, yC + yB + k ]

It is recommended to harmonize the WD description with the HM SW, by changing the text of the WD for both horizontal and vertical edges as follows:

If verticalEdgeFlag is equal to 1, for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

qi = s’[ xC + xB +i, yC + yB + k ]                                                                                      (8-442)

pi = s’[ xC + xB – i – 1, yC + yB + k ]

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

# Patent rights declaration(s)

Texas Instruments, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

Broadcom Corporation does not have any current or pending patent rights relating to the technology described in this contribution.

Sony Corporation and Sony Electronics Inc do not have any current or pending patent rights relating to the technology described in this contribution.