| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G097 M21645 |
| --- | --- |

| Title: | Non-CE13: cross-verification of Broadcom’s proposal JCTVC-G288 complexity reduction of MVP list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for Broadcom’s proposal on simplification of the second pruning process of the merge/skip list derivation (CE13, subset 3.3). It is proposed to reduce number of combined MVPs to lower the complexity of the pruning process. Small losses are observed in the four settings tested, namely (1, 1), (1, 2), (2, 1) and (2, 2), where the first number is the number of combined MVPs and the second is number of no-scaled MVPs. The order of combined and non-scaled MVPs is also swapped in the tests.  The results match the ones reported by proponents. The source code is also checked to verify its consistency with the proposal description.

.

# Introduction

This document reports cross-check results for the following tests of CE13 subset 3.3 from Broadcom. The idea here is to find the best configuration of combined, no-scaled MVPs to simplify the second pruning process of the merge/skip list construction without harming quality.

|  | Maximum number of combined bi-predictive MVP candidates | Maximum number of non-scaled bi-predictive MVP candidates | Combined bi-predictive MVP candidates first | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- | --- |
| Test 1b | 1 | 1 | no | Broadcom | TI |
| Test 2b | 1 | 2 | no | Broadcom | TI |
| Test 3b | 2 | 1 | no | Broadcom | TI |
| Test 4b | 2 | 2 | no | Broadcom | TI |

Table 1. Test configurations.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The test results are summarized in table 2 to Table 4, small losses are observed. Those results match the ones reported by proponents. The runtime here may not be accurate.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -0.1% | 0.1% | 0.2% | 0.0% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.2% | 0.1% | 0.4% | 0.3% | 0.2% |
| Class C | 0.3% | 0.1% | 0.1% | 0.3% | 0.2% | 0.1% |
| Class D | 0.3% | 0.0% | 0.3% | 0.3% | 0.1% | 0.2% |
| Class E | 0.2% | 0.4% | 0.7% | 0.3% | 0.2% | -0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.2% | 0.2% | 0.3% | 0.3% | 0.2% | 0.1% |
|  | 0.2% | 0.2% | 0.2% | 0.3% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 2. Experimental results for test 1b

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -0.1% | 0.1% | 0.2% | 0.0% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.2% | 0.1% | 0.4% | 0.3% | 0.2% |
| Class C | 0.3% | 0.1% | 0.1% | 0.3% | 0.2% | 0.1% |
| Class D | 0.3% | 0.0% | 0.3% | 0.3% | 0.1% | 0.2% |
| Class E | 0.2% | 0.4% | 0.7% | 0.3% | 0.2% | -0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.2% | 0.2% | 0.3% | 0.3% | 0.2% | 0.1% |
|  | 0.2% | 0.2% | 0.2% | 0.3% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 3. Experimental results for test 2b

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.3% | 0.5% | 0.1% | 0.3% | 0.2% |
| Class E | 0.1% | 0.4% | 0.9% | 0.0% | 0.1% | -0.5% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 98% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 111% |  |  |

Table 4. Experimental results for test 3b

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 96% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.3% | 0.5% | 0.1% | 0.3% | 0.2% |
| Class E | 0.1% | 0.4% | 0.9% | 0.0% | 0.1% | -0.5% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 94% |  |  | 96% |  |  |

Table 5. Experimental results for test 4b

# Comments

# It appears to be that at least two combined MVPs are required for avoiding noticeable coding loss. It is recommended to consider it with other proposals in same category.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] J. Jung, B. Bross, J. Chen, P. Onno, M. Zhou, “Description of Core Experiment CE13: Motion data parsing robustness and throughput”, JCT-VC Document, JCTVC-F913, Torino, IT, July 2011.