| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G098 M21646 |
| --- | --- |

| Title: | Non-CE9: cross-verification of Canon’s proposal JCTVC-G241 on parallel derivation of the temporal predictor for Merge/Skip modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for Canon’s proposal JCTVC-G241 on parallel derivation of the temporal predictor for Merge/Skip modes. In the proposed method, the TMVP is removed from the first pruning process and the derivation process of combined, non-scaled MVPs, and is added only at the very end of the merge/skip list derivation process. Relative to HM4.0 anchor with Samsung encoder bug fix, the measured loss is 0.1% in RA-HE, 0.1% in RA-LC, 0.2% in LB-HE and 0.1% in LB-LC. The results match the ones reported by proponents. The source code is also checked to verify its consistency with the proposal description.

.

# Introduction

This document reports cross-check results for Canon’s proposal JCTVC-G241 on  parallel derivation of the temporal predictor for Merge/Skip modes. The proposal basically removes the TMVP from the first pruning process and the derivation process of combined, non-scaled MVPs, and adds it only at the very end of the merge/skip list derivation process. In this way, TMVP derivation process can run parallel to the rest of process.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The experimental results are summarized in Table 1. An average loss of 0.1% in RA-HE, 0.1% in RA-LC, 0.2% in LB-HE and 0.1% in LB-LC is observed.  The anchor used is here is the one which has Samsung encoder bug fix. Those results match what proponents have reported. Please be advised in Table 1 runtime may not be accurate.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class B | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | -0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.4% |
| Class D | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% | 0.2% |
| Class E | 0.2% | 0.1% | 0.1% | 0.0% | 0.3% | 0.1% |
| Class F |  |  |  |  |  |  |
| Overall | 0.2% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |
|  | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 95% |  |  | 97% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  |

Table 1. Experimental results

# Comments

The proposed parallel MVP scaling will improve the throughput of the merge/skip MVP list derivation process. The trade-off between coding efficiency and throughput improvement should be carefully considered here.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[4] B. Bross, W. Chien, I. Kim, M. Zhou, “Core Experiment 9: MV Coding and Skip/Merge Operations”, JCT-VC Document, JCTVC-F909, Torino, IT, July 2011.