| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G100 M21648 |
| --- | --- |

| Title: | Non-CE9: cross-verification of LGE’s proposal JCTVC-G164 on improvement on parallel merge/skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for LGE’s proposal JCTVC-G164 on improvement on parallel merge/skip mode. Instead of simply dropping unavailable MVP candidates from the merge/skip list derivation process of the current PU as proposed in JCTVC-G085, LGE advocates to use closest available MVPs to replace those unavailable ones for further quality loss reduction in the parallel motion estimation. It has verified that the results provided by proponents are accurate, and source code matches the proposal description.

.

# Introduction

This document reports cross-check results for LGE’s proposal JCTVC-G164 on improvement on parallel merge/skip mode. Instead of simply dropping unavailable MVP candidates from the merge/skip list derivation process of the current PU as proposed in JCTVC-G085[3], LGE advocates to use closest available MVPs to replace those unavailable ones to further cut the loss for the parallel motion estimation.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

Table 1 summarizes the relative gain of proposed method when compared [3]. For typical parallel level 32x32, average gain ranges from 0.6% to 0.9%.

| Parallel merge level | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| 8x8 | 0.0 | 0.0 | 0.0 | 0.0 |
| 16x16 | -0.2 | -0.3 | -0.3 | -0.3 |
| 32x32 | -0.6 | -0.7 | -0.7 | -0.9 |
| 64x64 | -0.8 | -1.0 | -0.9 | -1.3 |

Table 1, relative gain of LG PME method compared to [3]

The detailed results are listed in Table 2 to Table 5. The runtime here may not be accurate.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.1% | -0.1% | 0.2% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | -0.5% | 0.4% | 0.0% | 0.1% | -0.1% |
| Class E | 0.0% | -0.2% | 0.7% | 0.0% | -0.4% | 0.5% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.3% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.2% | 0.3% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 94% |  |  |

Table 2, BD-rate reduction relative to [3] for parallel merge level of 8x8

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.3% |
| Class B | -0.2% | 0.0% | 0.1% | -0.3% | 0.0% | -0.2% |
| Class C | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% | -0.3% |
| Class D | -0.2% | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.1% | -0.3% | -0.2% | -0.2% |
|  | -0.2% | -0.2% | -0.1% | -0.3% | -0.2% | -0.3% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.2% | 0.2% | -0.4% | -0.2% | -0.4% |
| Class C | -0.4% | -0.2% | -0.1% | -0.4% | -0.3% | -0.5% |
| Class D | -0.4% | -0.2% | -0.2% | -0.4% | -0.1% | -0.5% |
| Class E | -0.1% | 0.2% | -0.4% | -0.1% | -0.3% | -0.8% |
| Class F |  |  |  |  |  |  |
| Overall | -0.3% | 0.0% | -0.1% | -0.3% | -0.2% | -0.5% |
|  | -0.3% | 0.0% | -0.1% | -0.3% | -0.2% | -0.5% |
| Enc Time[%] | 104% |  |  | 97% |  |  |
| Dec Time[%] | 106% |  |  | 99% |  |  |

Table 3, BD-rate reduction relative to [3] for parallel merge level of 16x16

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.5% | -0.5% | -0.5% | -0.8% | -1.1% |
| Class B | -0.8% | -0.4% | -0.2% | -0.9% | -0.5% | -0.4% |
| Class C | -0.6% | -0.7% | -0.6% | -0.7% | -0.6% | -0.8% |
| Class D | -0.5% | -0.2% | -0.4% | -0.5% | -0.4% | -0.4% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -0.6% | -0.4% | -0.4% | -0.7% | -0.6% | -0.7% |
|  | -0.6% | -0.4% | -0.4% | -0.7% | -0.6% | -0.7% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.0% | -0.3% | -0.5% | -1.3% | -0.8% | -1.0% |
| Class C | -0.7% | -0.7% | -0.5% | -0.9% | -0.8% | -0.8% |
| Class D | -0.6% | -0.9% | -1.0% | -0.7% | -0.5% | -1.4% |
| Class E | -0.2% | -0.4% | -0.1% | -0.5% | -0.3% | -0.3% |
| Class F |  |  |  |  |  |  |
| Overall | -0.7% | -0.6% | -0.6% | -0.9% | -0.6% | -0.9% |
|  | -0.7% | -0.5% | -0.5% | -0.9% | -0.6% | -0.9% |
| Enc Time[%] | 98% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 4, BD-rate reduction relative to [3] for parallel merge level of  32x32

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.7% | -0.8% | -0.8% | -0.9% | -1.2% | -1.3% |
| Class B | -1.2% | -0.7% | -0.5% | -1.5% | -0.9% | -0.8% |
| Class C | -0.6% | -0.6% | -0.7% | -0.8% | -0.8% | -0.9% |
| Class D | -0.4% | -0.4% | -0.4% | -0.6% | -0.6% | -0.5% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -0.8% | -0.6% | -0.6% | -1.0% | -0.9% | -0.9% |
|  | -0.7% | -0.6% | -0.6% | -1.0% | -0.9% | -0.9% |
| Enc Time[%] | 111% |  |  | 99% |  |  |
| Dec Time[%] | 123% |  |  | 94% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.5% | -1.0% | -0.9% | -2.1% | -1.3% | -1.5% |
| Class C | -0.8% | -1.0% | -0.9% | -1.0% | -0.8% | -1.3% |
| Class D | -0.6% | -0.6% | -0.6% | -0.8% | -1.4% | -0.7% |
| Class E | -0.2% | -0.4% | -0.1% | -1.0% | -0.9% | -1.0% |
| Class F |  |  |  |  |  |  |
| Overall | -0.9% | -0.8% | -0.7% | -1.3% | -1.1% | -1.1% |
|  | -0.9% | -0.8% | -0.7% | -1.3% | -1.2% | -1.2% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 114% |  |  | 102% |  |  |

Table 5, BD-rate reduction relative to [3] for parallel merge level of  64x64

# Comments

The proposed tool can improve the coding efficiency in parallel motion estimation environment.

The proponents have recently revised the source code to improve the readability of the code. The new code was reviewed and verified to be producing identical results as the previous version. The new code reflects the intended behavior of the proposed tool in a much cleaner way and clears confusion on its implementation complexity.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3] M. Zhou, “CE9: Test results on parallelized merge/skip mode”, JCT-VC Document,  JCTVC-G085, 7th Meeting: Geneva, CH, 19-30 November, 2011

[4] B. Bross, W. Chien, I. Kim, M. Zhou, “Core Experiment 9: MV Coding and Skip/Merge Operations”, JCT-VC Document, JCTVC-F909, Torino, IT, July 2011.