| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1000 |
| --- | --- |

| Title: | Suggested revised common test conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen 3240 Hillview Ave Palo Alto CA 94304 USA | Tel: Email: | +1 650 496 4742 bossen@docomoinnovations.com |
| Source: |  |  |  |

_____________________________

# Abstract

Common test conditions used between the 6th and the 7th JCT-VC meeting (document JCTVC-F900) define two sets of common test conditions: “HE” and “LC”. These two sets are distinguised by three differences: 10-bit vs 8-bit encoding, CAVLC vs CABAC entropy coding, and ALF on vs off. Decisions at the 7th meeting have led to a single entropy coder and a focus on 8-bit encoding, thereby reducing the difference between the two sets to the setting of the ALF in-loop filter. It is thus desireable to revise common test conditions such that the difference between the two sets of configurations is more significant.

# Description

## Suggested configurations

| Tool | Configuration 1, “HE” | Configuration 2, “LC” | Configuration 1 with 10 bits, “HE-10” |
| --- | --- | --- | --- |
| Bit depth | InternalBitDepth:8 | InternalBitDepth:8 | InternalBitDepth:10 |
| Adaptive Loop Filter | ALF:1 | ALF:0 | ALF:1 |
| Sample-Adaptive Offset | SAO:1 | SAO:0 | SAO:1 |
| RDOQ | RDOQ:1 | RDOQ:0 | RDOQ:1 |
| Chroma from Luma intra mode | LMChroma:1 | LMChroma:0 | LMChroma:1 |
| NSQT | #define NSQT 1 | #define NSQT 0 | #define NSQT 1 |
| AMP | #define AMP 1 | #define AMP 0 | #define AMP 1 |

It should be noted that NSQT and AMP can currently only be switched at compilation time in the HM software. It may thus be desirable to add flags at the SPS level to allow for switching at run time.

## Suggested sequences

The table below defines the suggested “mandatory” combinations of configurations and sequences. Sequence classes are defined according to JCTVC-F900. AI, RA, LB, LP refer to the previously defined coding structures.

| Class | Configuration HE | Configuration LC | Configuration HE-10 |
| --- | --- | --- | --- |
| A (8 bit) | AI/RA | AI/RA | RA |
| A (10 bit) | - | - | RA |
| B | AI/RA/LB | AI/RA/LB | RA |
| C | AI/RA/LB | AI/RA/LB | - |
| D | AI/RA/LB | AI/RA/LB | - |
| E | AI/LB | AI/LB | - |
| F | - | - | - |