| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1001 |
| --- | --- |

| Title: | Cross-check of RIM's multi-level significant maps for large transform units (JCTVC-G644) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The contribution confirms the results presented in JCTVC-G644 on a multi-level significant maps for large transforms units using 4×4 sub-block based scanning process.

- Introduction

Software implementing JCTVC-G644 [1] on top of HM4.0 + JCTVC-G323 [2] has been provided by RIM. The code has been run under common test conditions and results match those provided by RIM.

JCTVC-G323 proposes a scanning process based on a diagonal scan within sub-blocks and a diagonal scan across sub-blocks, like figure 1 shows. This sub-block based scan replaces the large diagonal scans in blocks of sizes 16×16 and 32×32. JCTVC-G644 uses the case of a 4×4 sub-block size.

Figure 1. Diagonal sub-block scan (sub-block size 4×4)

JCTVC-G644 proposes a multi-level coding procedure for the significance map of large blocks (16×16 and 32×32). The coding process signals if a 4×4 sub-block has any significant coefficient and only codes the significance map within the sub-block if there is any significant coefficient in it. When the current scanning process for large blocks is replaced by the sub-block scan, then the multi-level coding process in JCTVC-G644 matches the scanning process. This simplifies the algorithm and code. Also, the efficiency performance is enhanced with respect to the large scan-based version of JCTVC-G644.

The number of coded bins has not been computed in this cross-check.

- Results

Coding results using common conditions (High-Efficiency settings) are summarized in the table below. HM4.0 is the anchor. Note that encoding/decoding times are not reliable since different types of machines were used.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.38% | -0.90% | -0.68% | -0.16% | -0.19% | 0.15% |  |  |  |
| Class B | -0.28% | -0.67% | -0.76% | -0.25% | -0.40% | -0.49% | -0.08% | -0.10% | -0.59% |
| Class C | -0.18% | -0.27% | -0.27% | -0.18% | -0.14% | -0.08% | -0.18% | -0.06% | -0.11% |
| Class D | -0.19% | -0.32% | -0.29% | -0.07% | 0.26% | -0.37% | -0.10% | 0.18% | 0.34% |
| Class E | -0.41% | -0.83% | -0.64% |  |  |  | -0.06% | -0.74% | 0.42% |
| Class F | -0.11% | -0.15% | -0.06% | -0.14% | -0.05% | -0.17% | -0.35% | -0.23% | -0.09% |
| Overall | -0.25% | -0.52% | -0.46% | -0.16% | -0.12% | -0.21% | -0.15% | -0.16% | -0.06% |
|  | -0.25% | -0.52% | -0.46% | -0.16% | -0.14% | -0.20% | -0.16% | -0.11% | -0.07% |
| Enc Time[%] | 104% |  |  | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 103% |  |  |
| Overall No F | -0.28% | -0.59% | -0.53% | -0.17% | -0.13% | -0.21% | -0.11% | -0.14% | -0.05% |

- References

- N. Nguyen, T. Ji, D. He, G. Martin-Cocher, L. Song, “Multi-level Significant Maps for Large Transform Units”, JCTVC-G644, 7th JCT-VC Meeting, Geneva, CH, Geneva 2011.

- J. Sole, R. Joshi, M. Karczewicz, “Non-CE11: Diagonal sub-block scan for residual coding”, JCTVC-G323, 7th JCT-VC Meeting, Geneva, CH, Geneva 2011.

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.