| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1002 |
| --- | --- |

| Title: | JCT-VC break-out report: Reference picture buffering and list construction (AHG21) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce Vidyo Rickard Sjöberg Ericsson Ye-Kui Wang Qualcomm |  | jill@vidyo.com rickard.sjoberg@ericsson.com yekuiw@qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

Break-out group meetings were held from 18:30 to 22:00 on November 21 2011 in Geneva, from 18:30 to 22:00 on November 22, from 16:15 to 18:15 on November 23, and from 18:10 to 19:45 on November 24.  Approximately 25 delegates were present.

The BoG met again on November 28 from 08:00 to 09:15.

# Mandates

The mandates of the BoG are:

- Briefly review and summarize the AHG21 related input documents

  - Presented by proponents if present

  - Review of input document otherwise

- Try to categorize proposal pieces

- Try to narrow down the list of proposal pieces to be further reviewed by JCT-VC

For the Nov 24 meeting specifically, the BoG mandate was to:

- To determine a long term reference picture solution to add to the AHG21 work draft text of JCTVC-G021

For the Nov 28 meeting specifically, BoG mandate was:

- To discuss test conditions for future reference picture buffering and list reordering experiments

# Recommendations

The BoG recommends adoption of the combination of the AHG21 working draft text modification, in JCTVC-G021, with the long term reference picture solution of JCTVC-G788 proposal #7a.  Draft specification text of this combination has been provided along with this BoG report in JCTVC-G1002.  While support of this long term reference picture solution was not unanimous in the BoG, it was the prevailing opinion of the BoG.    Details of this discussion are in section 5 below.

The BoG would like to meet again this meeting to define test conditions with various picture coding structure patterns for evaluation of long term reference picture contributions, and for bitrate savings based contribution, based on a draft test conditions document prepared by Miska Hannuksela, and circulated on reflector by Saturday.

The BoG also recommend to revisit contribution JCTVC-G198 with the full group.

Several contributions initially assigned to this BoG were considered to not be within its scope, so it is recommended that they be re-assigned: JCTVC-G717, JCTVC-G635, JCTVC-G157, and JCTVC-G549.  Additionally, we proposed postponing consideration of JCTVC-G715 until after the AHG18 discussion.

The BoG will prepare a document defining test conditions for reference picture buffering and list reordering experiments.  A draft has been uploaded as JCTVC-G1036, and a two-week editing period is requested, with Y.-K. Wang as editor.  Config files and anchor bitrate calculations corresponding to the use cases defined in the test case document will be gathered by T.K. Tan, for those use cases that are supported in the HM reference software.

The BoG recommends a normative restriction to forbid pictures following a CRA in output order to have a picture preceding the CRA in output order in its RPS.

# List of AHG related input documents

Monday meeting, chaired by Rickard Sjöberg.

### JCTVC-G0166 [Hendry, S. Park, B. Jeon (LG)] "AHG21: Explicit Reference Pictures Signaling with Output Latency Count Scheme"

The proposed scheme suggests signaling only reference pictures that are required by current pictures. To assist reference picture marking process, a new syntax element so called Output Latency Count (OLC) is introduced which can tell decoder when each reference picture should be marked as “unused for reference”. It is claimed that the proposed scheme can signal the list of reference pictures more efficiently. Furthermore, it is also claimed that the proposed scheme separates tools for reference picture marking process and error resilience feature so that encoder may have more flexibility to decide whether it wants to use them both.  It is reported that while proposed scheme can be use by its own, some part of its concept can be harmonized with signaling scheme developed in AHG21 to address the long-term reference picture issue.

Proposal: OLC part

Signal only reference pictures that are used by the current picture

A new syntax element so called Output Latency Count (OLC) is introduced which can tell decoder when each reference picture should be marked as “unused for reference”

OLC with frame number is one option and OLC with POC is another

olc_flag and erps_flag are introduced in the SPS

Comments

A shortcoming of the proposal is that the presence of the OLC keeps the reference picture for a fixed time which can not be altered on the fly.

Another comment was that the OLC should not contain "output" in its name.

Proposal: Long-term part

A long-term picture ID was introduced.

Comments

The error robustness property of the long-term picture ID was questioned when long-term picture IDs are re-used.

Another comment was that you must decide whether a picture should be a long-term picture at the time it is encoded.

### JCTVC-G0198 [T.K. Tan, C.S. Boon (NTT Docomo)] "AHG21: Inter reference picture set prediction syntax and semantics."

This contribution proposes to reduce the amount of bits necessary for signaling the reference picture set by predicting the ΔPOC values using the ΔPOC values from a reference picture set already present in the PPS. Based on the latest draft of the reference picture set syntax from the ad hoc group on Reference picture buffering and list construction (AHG21), the number of additional PPS signaling bits needed for the random access (RA) and low delay (LD) common conditions are 288 and 201 bits, respectively.  Using the proposed inter reference picture set prediction method, the numbers of bits needed are reduced to 144 and 106 bits, respectively.  This represents a reduction of 50% and 47%, respectively.

Full reference picture set

Predicting delta POC values using delta POC values from a reference picture set already present in the PPS.

A reference picture set is coded by: One codeword for each reference picture in the previous RPS indicating what to do with it. A codeword for indicating whether to include the reference picture or not. One delta POC for the current picture relative the previous picture.

Partial reference picture set

A partial reference picture set is coded by: One flag for each reference picture in the previous RPS indicating whether to keep or remove it. Introduce the syntax elements num_full_ref_pic_sets and num_partial_ref_pic_sets. A while loop to make partial reference picture sets from the full reference picture sets.

Comments

One comment was that there are other related proposals: JCTVC-G788, JCTVC-G637, JCTVC-G548

### JCTVC-G0314 [Y. -K. Wang, Y. Chen (Qualcomm)] "AHG21: On DPB management"

This document proposes to adopt the reference picture set based DPB management mechanism, including reference picture set signaling and derivation, reference picture list initialization and modification, as well as DPB operations, as specified in the AHG21 output text. Furthermore, a method for long-term reference picture support based on the reference picture set based DPB management mechanism is proposed.

Proposal

Adopt the AHG21 output text and in addition the following changes:

PPS: long-term pictures signaled with a presence flag and the number of bits to use for long-term POC delta values if flag is on

In the slice header if long-term pictures are present: The number of long-term pictures, their delta POC and used_by_curr_pic_lt_flag are signaled.

Two more RPS subsets for long-term picture support are added.

Comments

One comment was that the life-time of a long-term picture is limited by the number of bits to use for long-term in the PPS

### JCTVC-G0526 [Y. Park, I.-K. Kim, C. Kim (Samsung)] "AHG21: Combined signaling for reference picture set"

This contribution is proposed for AHG21 especially for supporting combined signaling. We suggest a method keeping long distance POC difference (theoretically infinite) between reference picture and current picture as well as providing the possibility to combine relatively short distance picture to reference picture set.

Proposal 1

Include combined coding by a codeword in the slice header indicating how many explicit pictures to add to the RPS from the PPS. The delta POCs are then encoded by se(v) codewords.

Proposal 2

Extension of the long-term delta POC to support virtually infinite delta POC values by introducing exception mechanisms for zero delta POC values. For instance, if the se(v) codeword mentioned above is 0, then the delta POC is extended to use more bits for its signaling, using u(v).

Comments

One comment on proposal 1 was that text for how to deal with the subsets are missing.

Monday session concluded.

Resume Tuesday, resume 22 Nov 6:30pm, chaired by Jill Boyce.

### JCTVC-G0548 [V. Wahadaniah, C. S. Lim, S. M. T. Naing (Panasonic)] "AHG21: Construction and modification of predefined reference picture sets and reference picture lists"

This contribution proposes a number of modifications on the design of reference picture set and reference picture list signalling in HEVC. The benefits of the proposed design reportedly include improved flexibility, reduced redundancy and higher coding efficiency of the reference picture set design. It is suggested that JCT-VC consider the aspects proposed in this contribution and further discusses these aspects in producing a final design of reference picture sets and reference picture lists.

Presentation not uploaded.

Proposal 1

Move RPS from PPS to SPS

Some support for proposal #1

Proposal 2

Long term ref pic using absolute POC is allowed in the PPS or the slice header

Question raised about error resiliency impact if in the PPS, encoder may be able to avoid the issue

msb is ue(v) which can be inefficient when msb value is big, and lsb is u(v)

Proposal 3

Hierarchical modification of predefined RPS

Question raised if the additional complexity of the design is justified.

Proposal 4

Ref pic modification syntax – can be in a PPS or a slice header.  Similar to MMCO.

5 pages of text, and a lot of decoding process. Question raised if the additional complexity of the design is justified.

Proposal 5

Predefined reference picture list, linked to same RPS index.

1 page of semantics text.  Question raised about gains in practical use cases.

Comments

Some similar aspects to G714 for proposal #2 and #4

Motivation is bitrate savings, but no data provided on bitrate savings.

No software implementation.  Text not fully mature.

### JCTVC-G0637 [R. Sjöberg, J. Samuelsson (Ericsson)] "AHG21: Long-term pictures and pruning of reference picture sets"

This document expresses Ericssons support of the AHG21 candidate Working Draft. Further, it is proposed to adopt the long-term syntax and semantics from the October 18 candidate Working Draft (ftp://ftp.hhi.de/ahg21/JCTVC-F803_d1_Buffer_Descriptions_display_process_suggestion_r1.doc) from AHG21. In addition, this document also proposes to include syntax for omitting reference pictures from the PPS reference picture set.

Presentation not uploaded.

Proposal 1

Include long term ref pic syntax from earlier draft of AFG21 WD

Signal presence of long term ref pics in PPS, and length of POC

In slice header send the long term ref pic delta pocs

Consensus that long term ref pic should be supported, but need to determine the correct solution

Proposal 2

Include syntax for omitting reference pictures from the RPS

Used for the first pictures following a random access point, either IDR or CRA or when encoder wants.

Add a flag in PPS, and in slice header if flag set, send a number

Simple decoding process change

Main motivation is bit savings.  No bitrate savings estimates provided.

Comment that this bitrate savings approach simpler than other proposals.

G714 has related proposal

### JCTVC-G0713 [K. Misra, S. Deshpande, A. Segall (Sharp)] "AHG21: Long term picture referencing using wrapped POC"

This document proposes a technique to signal long term reference pictures using “wrapped POC” scheme.  It is asserted that the method enables: (a) Each picture to be uniquely identifiable, (b) Reference picture signaling for a picture to be independent of other pictures, and (c) including the long term pictures in reference picture sets.  The method consists of using a POC cycle parameter when referencing long term pictures.

Proposal Comments

poc cycle is very similar to delta_msb of poc

entropy code both longterm_poc and longterm_poc_cycle, so many bits needed for a large value of poc_cycle

Proposal is based upon an earlier version of the AHG21 WD, would require some rewriting to accommodate the latest WD

### JCTVC-G0714 [K. Misra, S. Deshpande, A. Segall (Sharp)] "AHG21: On picture referencing"

This document proposes syntax to: signal long-term pictures using absolute POC; signal reference picture sets containing long term pictures in PPS; replace, delete or append entries to a reference picture set defined in the picture parameter set. It is asserted that this change enables: (a) Signaling of reference picture set containing long term pictures and (b) Efficient signaling of reference picture sets for the current picture if the desired reference picture set differs from that defined in the picture parameter set for only a small subset of entries.

Proposal Comments

ue(v) code absolute longterm_poc, so many bits needed for a large value

Motivation of replacement is to save bits, but no bitrate calculation provided.

9 new syntax elements, no text for decoding process provided. No software.

### JCTVC-G0788 [G. J. Sullivan, Y. Wu, J. Xu (Microsoft), B. Li (USTC)] "AHG21: Proposal on Decoded Picture Buffer Description Syntax Relating to JCTVC-F493"

This contribution supports the basic concepts previously proposed in JCTVC-F493 – namely, the elimination of the frame_num syntax element and the use of explicit buffer description syntax. However, number of specific refinements of the prior proposed scheme are suggested.

#1 Add mark_unused_after_decoding_flag .  Goal is to reduce memory picture buffer used

#2

#3

Several comments that #1 and #2 and #3 are not needed.

#4. temporal layers, not marking, some potential benefits to temporal scalability

Need more study to this further, text not ready

#5 Should go in APS, not PPS

Some earlier proposals would send in SPS, but with id

Some objections

Postpone until APS more resolved

#6 APS

Also postpone until APS more resolved

#7

#8

Some objections, violating spirit of new approach

#9

Is already in the AGH21 WD

#10

Some objections, need separate syntax to accomplish for probably small gain.

#11  poc_gaps_allowed_flag

Questions if necessary, now that we have better loss detection mechanisms

#12 Allow SPS properties to vary by temporal layer

OK in spirit, but no text provided.  Encourage text be provided.

#13. bitrate savings, no additional functionality

many objections

#14 te(v) coding

Asked to suggest which specific syntax elements might benefit

#15

OK in spirit, but not sure how to express in text.  No text for this appears to be in the contribution.

No software provided.  Text not mature for many of the proposals.

Conclude Tuesday session.

Meet again Wednesday 3:30, chaired by Jill Boyce.

### JCTVC-G0643 [M. M. Hannuksela, S. M. Gopalakrishna (Nokia)] "AHG21: On reference picture list construction and reference picture marking"

The contribution proposes reference picture list description of lists 0 and 1 as well as an idle reference picture list to be used instead of reference picture list initialization and modification and reference picture marking of HEVC WD4 or reference picture sets and reference picture list initialization and modification of JCTVC-F493 and its later revisions.

The contribution also proposes

- a two-part coding of POC differences as a “short-term” part and a ue(v)-coded “long-term” part;

- temporal_id-based POC granularity for POC difference coding; and

- use of the two-part coding of POC differences for determining the marking between “used for short-term reference” or “used for long-term reference”.

It is asserted that proposals 1 to 3 above could be used with JCTVC-F493 and its later revisions too.

Revision 1 of the proposal included the following additions:

- reference picture list subset description, which enables to specify a reference picture list by removing reference pictures from the tail of a certain earlier reference picture list description

- POC difference coding relative to a POC prediction value

It is asserted that the number of bits needed in the picture parameter set to describe the reference picture lists for the random access and low delay common conditions is 157 and 132 bits, respectively, according to the syntax in revision 1 of the contribution.

Text wasn’t mature, and wasn’t based on AHG21 text.  Software wasn’t available.

Group would like to see bitrate savings numbers for individual tools separately, especially proposal #5, for temporal_id based POC values.

### General discussion

AHG21 WD with following changes:

Philosophy: be cautious about adoptions at this meeting.  Proposals can be considered again at next meeting, and bitrate savings counts and use case definitions are encouraged.

Questions:

Where should RPS be signaled?

SPS, PPS, APS, slice-header?

Should there be hierarchy?

Decision: in PPS send multiple RPS with id, slice_header can refer to RPS id or send own RPS. This is what is in AHG21 WD.

Long term ref pic handling

ue(v) vs u(v)

msb and lsb?

implicit vs explicit long term vs short term?

Involvement of SPS, PPS, APS, slice_header?  At least a flag in SPS to indicate LT ref pic

Decision: Don’t adopt any special long term tool now – make use of existing short term ref pic syntax for long term refs (may be expensive).  Define use case for testing for proposal for next meeting.

Bitrate savings approaches

Delta delta POC, G198 Part 1 (TK)

applies only to short term ref pics

Revisit .

Temporal_id POC granularity, G643 (Miska)

RPS Modification

List modification, G526 (Samsung),

MMCO-like, G548 (Viktor)

9 new syntax elements, G714 (Kiran)

Reduce ref pics from RPS G637, (Ricard)

Reduce or add ref pics G643? (Miska)

Ref pic list initialization

G643 (Miska)

Ref pic list initialization – AHG21 method or AVC method?

Decision: Stick with AHG21 WD

CRA handling

Decision: postpone until after high level syntax discussion G319 about CRA conformance

Need to define some more test conditions – different prediction structures

Two classes of conditions to be defined during the meeting, as an output of BoG: Bit savings test conditions, long term reference pictures.

Recommend:

Continue AHG.  Adopt AHG21 WD.  Contributions to next meeting should be based on that text.

BoG meet again this meeting to define test conditions, based on draft prepared by Miska and circulated on reflector by Saturday.

Revisit G198 with full group.

Concluded 6:30 pm Wednesday.

# Outside the reference picture management scope

### JCTVC-G0717 [Y. Yu, K. Panusopone, X. Fang, L. Wang (Motorola Mobility)] "The Reference Picture Construction of Combined List for HEVC"

To be re-assiged.

### JCTVC-G0635 [M. Naccari (BBC), G. Van Wallendael (Ghent University), M. Mrak (BBC), D. Flynn (BBC)] "Coding with a unified reference picture list"

To be re-assiged.

### JCTVC-G0157 [Hendry, S. Park, B. Jeon (LG)] "Reference List Construction for Random Access Settings"

To be discussed in High-level syntax session

### JCTVC-G0549 [Y. Park, S. Jeong, C. Kim (Samsung)] "Syntax rearrangement for list combination"

To be discussed in High-level syntax session

### JCTVC-G0715 [K. Misra, S. Deshpande, L. Kerofsky, A. Segall (Sharp)] "AHG18/21: Absolute signaling for resolution switching"

AHG18

Postpone presentation until resolution switching discussion

--

# Long term reference picture discussion of Nov 24

Mandate of the Nov 24 meeting

- To determine a long term reference picture solution to add to the AHG21 work draft text of JCTVC-G021

Desirable properties in a long term ref pic solution:

- Separate signaling of long term and short term ref pics

- Encoder not required to decide when encoding current pic if it is a long term pic, i.e. can switch from a previously coded picture from short term to long term

- Bitrate efficiency

- Error resiliency

- Encoder should be able to discard long term ref pic at any time

- Clean, simple design

- Flexibility

- Temporal scalability

The table in attach Powerpoint document included with this BoG report document which summarized the various contributions long term reference picture solutions was considered.

BoG recommendation: Prevailing opinion supported recommending addition of the G788 (proposal #7a) long term ref pic handling to the AHG21 WD, and recommending the group adopt this.

Rickard and Jizheng to update a draft spec text and upload as part of the G1002 update doc.

# Nov 28 discussion on test conditions

Consider that some test conditions are more common than others, and perhaps should be given additional weight.

Ask for an editing period for this test conditions document.  2 weeks after end of meeting.

Error injection tool exists, but this group won’t necessarily use.

Anchor counts – of the current WD design, in a few different categories.  TK will provide.

Should the HM software have the capability to count the bits?   Perhaps as a #define option.

Theoretical analysis vs. actual experiments with software?  Suggestion that we should encourage actual implementation.  Some of the use cases may not have support in the current HM software.  Estimate is about half.

Can config files be provided for use cases?  Yekui to provide long term ref cases, after software is available, for cases that can be supported in the HM.

TK will gather together config files and anchor bit counts.  Wait to select schedule once HM 5 is available?  Or use the branch?  Better to have software version that includes new APS design.  Preference to use HM5, if it is feasible.  Initial work can begin on the branch.

Ericsson will volunteer to provide and integrate software for the RPS adoption, including both G021 and the G788 part.

CRA issue.  Can a picture following a CRA in output order have a picture preceding the CRA in output order in its RPS?  It already is forbidden from being used as a reference.  But possible to get pictures with different output orders following each other in coding order.  Alternative is an I frame and recovery point SEI message. Agreed. BoG recommends that restriction.  Suggested wording: “"There shall be no reference picture included in the reference picture set which precedes, in output order, any CRA picture that precedes the current picture both in decoding order and output order"

Add to the document: add slices – what size?  For high delay test cases, results for 1, 2, 4 slices.  For low delay slice size 1500 bytes.  What sequence at what QP?  Highest QP case.

Add to the document: Explicit adaptive reference picture selection in the slice header.  Decide in the AHG.  Possible using a random seed.

Ask for #define in the HM code to enable our modifications.

Yekui to be editor of test conditions output document, registered as JCTVC-G1036.