| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1003 |
| --- | --- |

| Title: | Integration of proposals on transform skipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Mrak, Andrea Gabriellini, David Flynn (BBC R&D) Glenn Van Wallendael (Ghent University) Rajan Joshi, Joel Sole, Xiangling Wang, Marta Karczewicz (Qualcomm) |  | marta.mrak@bbc.co.uk glenn.vanwallendael@ugent.be rajanj@qualcomm.com |
| Source: | British Broadcasting Corporation, Ghent University, Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents integration of 3 proposals from AHG19 on transform skipping. Transform skipping by omitting the transform on rows and/or columns is defined by four modes (including 2D transform). The proposals from AHG19 addressed entropy coding tools to support transform skipping, modified quantization to support different signal levels when the transform is skipped and proposed configurations for coding with transform skipping dependant on block parameters. The proposals have been integrated and the results for the unified solution are tested with transform skipping on 4x4 blocks.

# Transform skipping

Transform skipping and supporting tools are proposed for application on motion compensated blocks. Instead of performing transforms on rows and columns, in the proposed solution the transform of rows and/or transform of columns can be skipped. When the transform is skipped in only one direction, a transpose is applied, to keep the coefficients in the order required by other coding stages. Preserving the transforms on both columns and rows is still supported, and the choice of keeping the transform or skipping is defined by one of 4 Transform Skip Modes (TSM), [G575].

Following the transform stage, levels of signal that have the transform skipped in one or both directions are adjusted by changing default quantization factors. The entropy coding has been changed to support transform skip. When the transform is skipped on rows only or columns only, vertical or horizontal scans are used, respectively. The rest of the entropy coding description addresses CABAC only, as proposed in [G663]. The coding of last significant coefficient position is changed. When a transform is skipped in a particular direction, the last position in that direction is coded using fixed length code (dependent on the block size in that direction) in the bypass mode. The coding of last position in the direction in which transform is applied is unchanged from the current HEVC working draft. Modification for coding of significance flags are applied for coding of 4x4 and 8x8 blocks.

This integrated proposal introduces a mechanism for signaling the maximum block size up to which TSM is applied. It is proposed to include a parameter that defines maximum blocks size into the SPS - a TSM_maxblock_code parameter. For non-square blocks, both block dimensions have to be smaller or equal to the maximum block size defined by TSM.

Table 1 TSM block-size selection

| N (max block size) | - (TSM disabled) | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- | --- |
| TSM_maxblock_code | 0 | 1 | 2 | 3 | 4 |

The test is performed to evaluate the proposed solution. Maximal block size was set to 4x4, which was in the tested software driven by a macro setting, i.e. without implementing the TSM_maxblock in SPS

Table 2 Test results for integrate TSM proposal, compared to HM-4.0 under common conditions.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.2% | 0.1% | 0.2% | 0.2% |
| Class B | -0.1% | 0.2% | 0.1% | -0.1% | 0.4% | 0.6% |
| Class C | -0.3% | -0.2% | 0.1% | -0.3% | 0.2% | 0.2% |
| Class D | -0.5% | 0.2% | 0.1% | -0.5% | 0.5% | 0.4% |
| Class E |  |  |  |  |  |  |
| Class F | -1.9% | -1.1% | -1.0% | -1.3% | -0.5% | -0.5% |
| Overall | -0.5% | -0.2% | -0.2% | -0.4% | 0.2% | 0.2% |
|  | -0.5% | -0.2% | -0.2% | -0.4% | 0.1% | 0.2% |
| Enc Time[%] | 114% |  |  | 112% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.5% | 0.7% |
| Class C | -0.5% | 0.0% | 0.1% | -0.5% | 0.6% | 0.3% |
| Class D | -0.5% | 0.8% | 0.5% | -0.7% | 1.4% | 0.9% |
| Class E | 0.1% | 0.0% | 0.7% | -0.1% | -0.2% | 0.3% |
| Class F | -4.3% | -2.5% | -1.8% | -2.8% | -0.8% | -0.9% |
| Overall | -1.0% | -0.3% | -0.1% | -0.8% | 0.3% | 0.3% |
|  | -1.0% | -0.3% | -0.1% | -0.8% | 0.2% | 0.2% |
| Enc Time[%] | 111% |  |  | 109% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.5% | 0.3% |
| Class C | -0.6% | -0.1% | -0.1% | -0.7% | 0.3% | 0.4% |
| Class D | -0.8% | 0.5% | 0.4% | -1.0% | 0.9% | 0.8% |
| Class E | 0.0% | 0.3% | 0.3% | 0.0% | -0.2% | -0.1% |
| Class F | -4.6% | -2.6% | -2.5% | -3.2% | -1.0% | -0.6% |
| Overall | -1.2% | -0.4% | -0.4% | -1.0% | 0.1% | 0.2% |
|  | -1.2% | -0.5% | -0.5% | -1.0% | 0.1% | 0.1% |
| Enc Time[%] | 116% |  |  | 114% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Bit-stream modifications and semantics

This section describes the modification to the HEVC syntax that this proposal requires. The changes are described in reference to the latest version of The Working Draft [G803].

Modifications to Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| tsm_maxblock_code | u(3) |
| ... |  |

Addition to 7.4.9 Transform coefficient semantics

tsmIdx[ x0 ][ y0 ][ trafoDepth ] signals the choice of Transform Skip Mode (TSM) to use for the transform block for the luma and chroma components. The array indices x0 and y0 specify the location (x0, y0) of the top luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. Four possible values are allowed for tsmIdx[ x0 ][ y0 ][ trafoDepth ], as shown in the table below. The TSM index is only signalled when PredMode is different to MODE_INTRA and only for the luma component (cIdx = = 0).

When tsmIdx[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred to be equal to TS0.

log2TSMmax is derived from SPS parameter tsm_maxblock_code.

| tsmIdx | Transform on rows | Transform on columns | Scan Idx | Note |
| --- | --- | --- | --- | --- |
| 0 | + | + | 0 | 2D transform |
| 1 | + | - | 2 | 1D transform |
| 2 | - | + | 1 | 1D transform |
| 3 | - | - | 0 | no transform |

A transform skip mode set to 1 requires a scanning index scanIdx = 2, i.e. the vertical scanning. A transform skip mode set to 2 requires a scanning index scanIdx = 1, i.e. the horizontal scanning.

Notes related to 8.x sections related to transformation and quantisation

When Transform Skip Mode (TSM) is used on a block, the quantization (both forward and inverse) is performed using the following modification to the quantization factor:

QP' = (QP  bscale) >> bshift, 	(1)

where QP' = QP for full 2D transform, QP is a quantization factor for 2D transform (inverse or forward), and bscale and bshift are dependent on TSM modes and the current block dimensions. The block scale is defined as:

bscale=1,if TS0scaleheight,if TS1scalewidth,if TS2scaleheight⋅scalewidth,if TS3

where scale is defined in the following table:

Table 8-X Scaling factors for vectors of N elements.

| N | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- |
| scale | 128 | 181 | 256 | 362 |

The block shift bshift corresponds to the shifts of skipped transforms. Therefore, for the inverse quantization in HM-4.0, the blocks shift is defined as:

bshift=0,if TS07,if TS112 -trafoPrecisionExt,if TS219 - trafoPrecisionExt,if TS3

Notes related to 9.3 CABAC parsing process for slice data

Table 9-X Codeword / binarization for tsmIdx

| Value of tsmIdx | Codeword |
| --- | --- |
| 0 | 1 |
| 1 | 01 |
| 2 | 001 |
| 3 | 000 |

Inputs to this process are PredMode, cIdx and log2TrafoSize.

Table 9-Y contains the values of variables n and m used in the initialisation of context variable assigned to syntax element tsmIdx.

Table 9-Y Values of variable m and n for tsmIdx ctxIdx

| Initialisation variables | tsmIdx ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |

For tsmIdx , the ctxIdx is derived from log2TrafoSize and binIdx:

ctxIdx = (log2TrafoSize − 2) * 3 + binIdx

Modifications to 7.3.9 Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| … |  |
| } else { |  |
| … |  |
| } |  |
| transform_coeff( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) − ( cIdx > 0 ? 1 : 2 ) |  |
| if ( PredMode != MODE_INTRA && cIdx = = 0 && log2TrafoWidth <= log2TSMmax && log2TrafoHeight <= log2TSMmax ) |  |
| tsmIdx[ x0 ][ y0 ][ trafoDepth ] | ae(v) ¦ ce(v) |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredModeC ] |  |
| } else if ( tsmIdx[ x0 ][ y0 ][ trafoDepth ] = = TS1 ) { |  |
| scanIdx = 2 |  |
| } else if ( tsmIdx[ x0 ][ y0 ][ trafoDepth ] = = TS2 ) { |  |
| scanIdx = 1 |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| if ( scanIdx = = 0 ) |  |
| scanIdx = 3 |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, cIdx ) |  |
| } |  |
| } |  |
| } |  |

# References

[G575]	Marta Mrak, Andrea Gabriellini, David Flynn, "Transform skip mode," JCTVC-G575, November 2011

[G577]	Glenn Van Wallendael, Jan De Cock, Rik Van de Walle, Marta Mrak, "Transform skipping dependent on block parameters," JCTVC-G577, November 2011

[G663]	R. Joshi, J. Sole, X. Wang, M. Karczewicz, "AHG19: Modification to HE transform coefficient coding for transform skip mode," JCTVC-G663, November 2011

# Patent rights declarations

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ghent University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).