| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G1005_r1 WG11 Number: m22779 |
| --- | --- |

| Title: | BoG report on review of non-CE deblocking filter proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin | Email: | andrey.norkin@ericsson.com |
| Source: | BoG on review non-CE deblocking filter proposals |  |  |

_____________________________

This contribution summarizes activities of the break out group (BoG) on review of non-CE deblocking filter related proposals.

# Attendees

Geert Van Der Auwera (geertv@qualcomm.com)

Thomas Wedi (Thomas.Wedi@eu.panasonic.com)

Matthias Narroschke (Matthias.Narroschke@eu.panasonic.com)

Andrey Norkin (andrey.norkin@ericsson.com)

Byeungwoo Jeon  (bjeon@skku.edu)

Vivienne Sze (sze@ti.com)

Xun Guo (xun.guo@mediatek.com)

Seungwook Park (Seungwook.park@lge.com)

Anand Kotra (Anand.Kotra@eu.panasonic.com)

Minhua Zhou (zhou@ti.com)

Faouzi Kossentini (faouzi@ebriskvideo.com)

Kazuo Sugimoto (Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp)

Kenneth Andersson (kenneth.r.andersson@ericsson.com)

Hsan Guermazi (hsan@ebriskvideo.com)

and others.

# Review of proposals

The proposals are sorted into categories (although some proposals can belong to more than 1 category).

1. Parallel deblocking

2. Modifications to deblocking filter description

3. Modifications to Bs calculation process

4. Line buffer reduction

5. Deblocking filter simplifications

6. Signaling deblocking filter parameters in slice header

7. Varying QP deblocking and deblocking for IPCM blocks

## Parallel deblocking

JCTVC-G089 Non-CE12: Enable low-complexity and fully parallelized de-blocking filter with minor changes [M. Zhou, O. Sezer, V. Sze (TI)]

This contribution advocates the following two changes to the current HM4.0 de-blocking filter design: 1) compute the on/off decision on 4 lines/columns segment instead of 8 lines/columns segment, and 2) remove the parallel on/off filter decision to speed up vertical edge filtering. Experimental results revealed that the proposed algorithm led to 0.1% BD-rate reduction in LB-HE and LB-LC, and 0.0% BD-rate difference in other configurations when compared to HM4.0.

Results: 0.0/0.0, 0.0/0.0, -0.1/-0.1 for HE-AI/LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

Similar to G590 in spirit (however, does not change the strong filtering).

Recommendation: Depending on adoption of G590.

Subjective test: participate in the subjective test.

JCTVC-G171 Parallel deblocking filter [J. Yu, S. Yang, J. Byun, Y. Kim, J. Kim (Yonsei Univ.)]

The proposal was not presented since the presenters were absent.

Recommendation:  revisit in track A

Subjective test: revisit in track A

## Modifications to deblocking filter description

The proposals presented in this section do not modify HM4.0 behavior but modify the working draft text.

JCTVC-G175 BS decision tree simplification [S. Park, N. Park, B. Jeon (LGE), X. Guo, J. An, C. Hsu, Y. Huang, S. Lei (MediaTek)]

This contribution reports the simplification of BS (boundary strength) decision tree in the deblocking filter. The proposed simplification removes redundant BS values with related conditions. The number of BS values is changed from 5 to 3 and this modification provides the same BD rate and visual quality as HM4.0.

The check for the CU boundary (Bs = 4) is removed. The proposed BS values are 0, 1, 2.

Recommendation:  Makes sense, is related to contributions G620 part 1 and G638, parts 1 and 2.

Subjective test: not needed, identical to HM4.0

JCTVC-G620 Clean-up of deblocking filter description [A. Norkin (Ericsson)] [late]

Part 1: Removing clipping operations from the deblocking strong filter. Clipping is unnecessary since the output values cannot get outside the input range.

e.g. replace p1’ = Clip1Y( ( p2 + p1 + p0 + q0 + 2 ) >> 2 )  with  p1’ = ( p2 + p1 + p0 + q0 + 2 ) >> 2.

Recommendation: Check with software coordinators? Consider adopting if no objections from larger group and present deblocking strong filtering stays.

Part 2: Check for the CU boundary (Bs = 4) is removed. Decreasing the number of Bs values to 0, 1, 2. This part of the proposal is identical to G175.

Subjective test: Not needed, proposal is identical to HM4.0

JCTVC-G638 Deblocking boundary strength and filtering process simplifications [A. Kotra, M. Narroschke, T. Wedi (Panasonic)]

Part 1. Remove the Bs = 4 and the check for CU boundary. Same as in G620 and G175.

Part 2. Set Bs values to 0, 1 and 3. Remove the tc_offset from the WD text and replace tc_table [Clip3(0, 55, qPL + tc_offset] with  tc_table[Clip3(0, 55, qPL + bS-1)]. One participant supports this proposal, one is against.

Part 3. If Bs = 0, do not calculate Tc and Beta when Bs = 0. Suggests this change to the working draft text. Two participants support the proposal. Recommend adopting Part 3 to WD.

Subjective test: not needed, results are identical to HM4.0

Recommendations on Bs calcuation (G175, G620, G638):

Agreed on removing the check for the CU boundary (Bs = 4) and having 3 values for Bs.

Discuss further whether using:

1. Bs = 0, 1, 2 (G175, G620) or

2. Bs = 0, 1, 3 and removing tc_offset (G638).

## Modifications to Bs calculation process

JCTVC-G176 Simplified BS calculation process [S. Park, B. Jeon (LGE)]

Currently in HM4.0, Bs decision process for an 8-pel edge consists of finding BS decision for every 4-pel edge and then a maximum of two 4-pel Bs is found, because the minimum unit of BS and filtering is different.

The proposal suggests using the Bs of the first 4-pixel part as the Bs for the whole 8-pel edge to reduce Bs decision operation and remove BS merging process.

The proposal suggests using the Bs of the first 4-pixel part to determine Bs for the whole 8-pel edge. The average BD-rate change is 0.0% for all configurations.

Results: 0.0/0.0, 0.0/0.0, 0.0/0.0 for HE-AI/LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

One participant supports the proposal.

Recommendation: Test the proposal in the subjective test. Participate in CE12.

JCTVC-G616 On parallel deblocking [A. Norkin (Ericsson)]

The proposed solution is based on a parallel deblocking proposal G089 from TI with modifications to it. The proposal suggests using separate Bs for two 4-pixel parts of the 8-pixel block boundary in order to align Bs calculation with transform size. The other modification is to use lines 0 and 4 for deblocking filtering decision.

The proposal also addresses implementation of parallel deblocking in case of deblocking in CU order. The average gains BD-rate are gains are between 0.0%(AI) to 0.2%(LDB HE).

Results: 0.0/0.0, 0.0/0.0, -0.2/-0.1 for HE-AI / LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

Recommendation: participate in subjective test, discuss after.

## Line buffer reduction

JCTVC-G230 Non-CE12.3: Reducing pixel line buffers by modifying DF to R3W2 for horizontal LCU edges [C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek), M. Ikeda, T. Suzuki (Sony), S. Park, B. Jeon (LG)]

In this contribution, modified vertical filtering with reading 3 pixels and writing 2 pixles (R3W2) for deblocking filter (DF) is proposed for horizontal LCU boundaries, while DF of HM-4.0 is still used for the rest of edges. The proposed method reduces line buffer for DF compared with the current HM-4.0 by one line. Fewer pre-DF pixels above the horizontal LCU boundary are used for filtering decisions, filtering operations are modified.

Simulation results reportedly show 0% bit rate increase on average with unchanged run time.

Modification points: in strong filtering decision use p2 instead of p3. Strong filter is modified. Only the upper side of deblocking filter is modified, the bottom side is the same as in HM4.0.

One proponent suggested discussing trade-off between having one more loop filter and removing one line from line buffer after the subjective test.

Results: 0.0/0.0, 0.0/0.0, 0.0/0.0 for HE-AI / LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

Recommendations: Test the proposal and a combination with the non-CE8 line buffer reduction proposal in subjective test.

## Deblocking filter simplifications

JCTVC-G290 Deblocking Filter Simplifications [G. Van der Auwera, M. Karczewicz (Qualcomm)]

This contribution addresses the simplification of the deblocking filter.

Part 1. It is proposed to modify the weak filter delta. Modifying the weak filter delta results in BD-rate savings of -0.2% for AI, -0.1% for RA, and -0.2% for LD B. The number of strong/weak filter decisions for an edge segment is reduced from eight to two. The delta calculation for the first pixel from the block boundary is modified from (9*(q0-p0)-3*(q1-p1))/16) to (3*(q0-p0)-(q1-p1))/8 therefore reducing number of operation and resulting in weaker deblocking. E.g., in the absence of clipping, the blocking artifact 0 0 0 | 16 16 16 will be smoothed by the proposed deblocking to 0 2 4 | 12 14 16, whereas current HM4.0 deblocking results in 0 3 6 | 10 13 16.

Results: -0.2/-0.2, -0.1/-0.1, -0.2/-0.1 for HE-AI / LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

Part 2. It is proposed to modify strong/weak filter decision by performing the decision for the 4-pel edge based on one line.

It was claimed by one participant that complexity in hardware does not decrease comparing to testing two lines like in G590. The complexity in software probably decreases.

Results: 0.0/0.0, 0.1/0.0, 0.1/0.1 for HE-AI / LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

Recommendations: Test part 1 in subjective viewing. Do not test part 2.

JCTVC-G090 Non-CE12: Testing results on using HM3.0 delta calculation for luma weak filter [M. Zhou, O.Sezer, V. Sze (TI)]

This contribution proposes to modify delta calculation for luma weak filter which has higher precision when compared to HM4.0 one. Simulation results revealed that the proposed modification improved coding efficiency by 0.2% in AI-HE and AI-LC, and 0.1% in RA-HE, RA-LC, LB-HE and LB-LC. The proposed change reportedly led to visible visual gain in BQMall (RA-HE, QP=37) sequence and no visual difference in the other CE12 selected subjective testing sequences.

Results: -0.2/-0.2, -0.1/-0.1, -0.1/-0.1 for HE-AI / LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

The proponent does not think it is necessary to participate in subjective viewing (since the proposal is similar to G290).

JCTVC-G639 Deblocking simplification and rounding optimization [A. Kotra, M. Narroschke, T. Wedi (Panasonic)]

Part 1: Replace deblocking for the second pixel from the block boundary by smoothing operation that does not take into account pixels on the other side of the block boundary. The motivation is simplification of the delta calculation. The BDR results are: AI HE -0.3%, AI LC -0.1%, small BD-rate increase 0.1% on LDP configurations, other configurations are 0.0%.

Results: -0.3/-0.1, 0.0/0.0, 0.0/0.0 for HE-AI / LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

One participant liked the idea, one participant was concerned.

Part 2: Modification to deblocking rounding control. It is claimed that the current rounding in deblocking is not completely symmetric due to the shift operation of a signed delta-value introducing an offset in the deblocked signal and it is proposed to introduce another clipping function Clip4() with two  adjustment parameters. These adjustment parameters were determined based on statistics of the current test set. The objective gain is -0.1 on LC configurations and 0.0 on HE configurations.

Results: 0.0/-0.1, 0.0/-0.1, 0.0/-0.1 for HE-AI / LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

The proponent would like to have the proposal studied it in CE12 and participate in the subjective test with the combination of Part 1 and Part 2.

Recommendation: study in CE12.

## Signaling deblocking filter parameters in slice header

JCTVC-G291 Transform Dependent Deblocking Filter Parameter Adjustment in Slice Level [G. Van der Auwera, M. Karczewicz (Qualcomm)]

This contribution proposes to adjust the Tc and Beta parameters of the deblocking filter by enabling signaling of control data in the slice header. It is proposed to adjust Tc depending on the transform size and on intra or inter type of the blocks. The deblocking_filter_control_present_flag is proposed in the SPS to control the presence of the deblocking filter adjustment parameters in the slice header. There are BD-rate gains of -0.3% for all-intra HE and -0.1% for random access HE test conditions, other configurations similar to the anchor.  The proponent advocates importance of these parameters for subjective visual quality.

Different tc offsets are sent for blocks of different size. The control parameters are signaled in the slice header. Similar to proposal G174 but with sending additional parameters.

Results: -0.3/-0.1, -0.1/0.0, 0.1/0.1 for HE-AI / LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

Recommendation: participate in subjective viewing, revisit.

JCTVC-G619 Non-CE12: deblocking parameters signalling in slice header [A. Norkin, R. Sjöberg, K. Andersson, J. Enhorn (Ericsson)]

The document is a proposal for signaling deblocking filter parameters in the slice header. It is proposed to signal three deblocking parameters that control which pixels are being filtered and one clipping parameter that determines the largest possible modification of the pixels in the weak filtering mode.

The proposal sends a flag and four parameters in the slice header:

slice_tc_offset_div2

slice_beta_offset

slice_side_beta_offset_div4

slice_nat_edge_offset_div2

The encoder that makes a rough estimation of the parameters frame-wise on SSD has been implemented. BD-rate results are: -0.1% on AI, -0.4% on LDB HE and -0.3% on the rest of standard configurations.

The proposal can possibly participate in subjective viewing

Results: -0.1/-0.1, -0.3/-0.3, -0.4/-0.3 for HE-AI / LC-AI, HE-RA / LC-RA, HE-LD / LC-LD.

Recommendation: participate in CE12.

Recommendation for proposals on deblocking filtering parameters in CE12

Discuss what tests to perform and how to evaluate proposals. Consider possibility of harmonization?

## Varying QP deblocking and deblocking for IPCM blocks

JCTVC-G384 Varying QP Deblocking [G. Van der Auwera, M. Karczewicz (Qualcomm)]

This contribution proposes changes to the deblocking filter to support the varying QP capability enabled by delta-QP signaling. It is proposed to use the maximum of the QP values on both sides of the edge to lookup for Beta and tC threshold parameters. In case of weak filter decision for modifying one or two samples, it is proposed to use the QP value of the respective edge side to lookup a Beta parameter.

Related to proposal G640 part 1.

JCTVC-G640 Deblocking bug fix for CU-Varying QP’s and IPCM blocks [A. Kotra, M. Narroschke, T. Wedi (Panasonic)]

The contribution presents modifications to deblocking operations when CU-based multi-QP optimization is enabled (Part 1) and IPCM blocks (Part 2).

Part 1:

Five modifications are proposed related to varying QP deblocking.

Individual tc and ß thresholds are derived for the blocks corresponding to an edge on which weak filtering is performed.

Separate tc values are used in deriving the delta offsets for innermost samples in weak filtering

Separate ß values are used in decision for filtering of outermost samples in weak filtering.

It is proposed to use an average of the QP’s for deriving tc and ß values in filter decision process and strong filtering.

Experimental results provided for a QP adaptation control that randomly assigns QP to coding units and to CE4 testing conditions.

Part 2:

It is suggested to use a separate QP (QP_PCM) which is associated to IPCM blocks whenever deblocking filtering is desired to be performed over IPCM blocks. QP_PCM parameter is transmitted in the slice header.

JCTVC-G138 Deblocking of IPCM Blocks Containing Reconstructed Samples [G. Van der Auwera, M. Karczewicz (Qualcomm)]

This contribution proposes a modification to the HM4.0 deblocking loop filter in case of IPCM blocks containing reconstructed samples. HM4.0 deblocking filter always assigns quantization parameter value zero to the IPCM blocks, therefore deblocking filtering is disabled for the left and top edges of the IPCM blocks.

The proposal is to assign a quantization parameter value to the IPCM block, which is predicted from the neighboring quantization group.

Recommendation: discuss in track A.

Summary on JCTVC-G640, JCTVC-G384 and JCTVC-G138.

Varying QP deblocking modifications.

G384 uses maximum of the neighbouring blocks QPs, G640 part 1 uses average of QPs.

It is not possible currently to compare G383 and G640 subjectively or objectively since they are using different anchors (different varying QP settings).

There are certain similarities between two proposals (e.g. using separate QP values on the side of block boundaries). The proponents suggest adopting parts that are similar. This was supported by one non-proponent. Study the rest in CE12.

Otherwise, study everything in CE12.

IPCM deblocking

G138 proposes deriving QP for IPCM from neighbouring blocks, G640 proposes sending QP in the slice header for the all IPCM blocks in the slice.

# Cross-check documents

JCTVC-G675 [MediaTek] Non-CE12: Crosscheck for LGE's Simplified Bs Calculation in JCTVC-G176

JCTVC-G385 [Qualcomm] Cross-check of Panasonic's JCTVC-G638 and JCTVC-G639 on Deblocking

JCTVC-G973 [MediaTek] Non-CE12: Crosscheck for Ericsson's Parallel Deblocking in JCTVC-G616

JCTVC-G162 [LG] Non-CE12: cross-verification of JCTVC-G090 on testing results on using HM3.0 delta calculation for luma weak filter by TI

JCTVC-G793 [Panasonic] Non-CE12: Cross-check of Qualcomm’s deblocking of IPCM blocks containing reconstructed samples (JCTVC-G138)

JCTVC-G883 [NEC] Non-CE12: Cross-verification of Qualcomm’s Deblocking of IPCM Blocks Containing Reconstructed Samples (JCTVC-G138)

JCTVC-G982 [Toshiba] Non-CE12 Subtest 5: Cross-check of Qualcomm’s transform dependent deblocking filter parameter adjustment in slice level JCTVC-G291

JCTVC-G987 [Ericsson] Cross-check of Varying QP Deblocking (JCTVC-G384)

JCTVC-G929 [Panasonic] Cross check of Qualcomms deblocking filter simplifications (JCTVC-G290)

JCTVC-G485 [Mediatek] CE12.1.5: Crosscheck for Qualcomm’s deblocking of new non-square blocks: edge shift for AMP in JCTVC-G383

JCTVC-G1010 [Qualcomm] Cross-check of Ericsson's Proposal JCTVC-G619

JCTVC- G973 [MediaTek] Non-CE12: Crosscheck for Ericsson's Parallel Deblocking in JCTVC-G616

JCTVC-G844 [Samsung] Cross-verification of MediaTek and Sony's reducing pixel line buffers by modifying DF to R3W2 for horizontal LCU edges (JCTVC-G230)

JCTVC-G1013 [Qualcomm] Cross Check of Panasonic's Proposal JCTVC-G640 on Varying QP Deblocking