| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1006 |
| --- | --- |

| Title: | BoG Report on Non-CE MV Coding Proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | B. Bross J. Jung S. Oudin M. Zhou | Tel: Email: |  |
| Source: | BoG on Non-CE MV Coding Proposals |  |  |

_____________________________

# Abstract

# Reviewed proposals

JCTVC-G082 non-CE9: Modified H positions for memory bandwidth reduction in TMVP derivation [M. Zhou (TI)]

This contribution advocates modification of the H TMVP positions to avoid co-located motion data required for TMVP derivation of the current LCU going beyond the current LCU row. In the proposed design, the H TVMP positions around the bottom boundary of the current LCU are replaced with closest ones inside the current LCU row (configuration 1), or with C3 TMVP positions (configuration 2). The proposed algorithm enables LCU-aligned motion data compression, storage and fetch, and can reduce the memory bandwidth for the TMVP derivation by half. The measured BD-rate penalty is 0.1% in RA-HE and RA-LC, 0.2% in LB-HE and LB-LC for configuration 1, and 0.2% in RA-HE and RA-LC, 0.3% in LB-HE and LB-LC for configuration 2.

Slides not uploaded

It was requested to discuss the need/trade-off of memory bandwidth reduction that may vary depending on the implementation.

Simplification on the temporal candidate position

JCTVC-G274 Non-CE9: cross-check of contribution JCTVC-G082 (TI) on modified H TMVP position [J. Jung (Orange Labs)]

Confirmed.

This verification contribution confirms the results obtained by TI in JCTVC-G082. According to the results obtained, it is recommended to adopt configuration 2, which provides similar results as configuration 1, allowing however cleaner WD text and software.

JCTVC-G134 Motion Vector Predictor Candidate Clipping Removal [M. Coban, M. Karczewicz (Qualcomm)]

This contribution presents a modification to the motion vector predictor (MVP) candidate derivation process that removes the final normative motion vector clipping operation. In the current HM design, motion vector predictor values are clipped using the non-normative motion vector clipping operation in the motion compensation process. Removal of the motion vector clipping operation on the MVP candidates reduces MVP derivation complexity with negligible impact on coding efficiency. Simulation results show 0.0%, 0.0%, and 0.0% BD rate change for RA_HE, LB_HE and LP_HE cases, respectively.

One participant was asking whether clipping in motion compensation can be removed as well.

Recommendation: adopt

JCTVC-G344 Cross-verification report for motion vector predictor candidate clipping removal (JCTVC-G134) [H. Aoki, K. Chono (NEC)]

Nobody was there to report. Comments provided by email on the reflector:

Verification task was successfully completed; it was confirmed that the implementation conforms to the description in JCTVC-G134 and results exactly match those presented in JCTVC-G134.

The necessity of the clipping is also raised as one of open issues to be discussed in the WD4 document. It is recommended that the clipping issue is discussed and fixed at the 7th JCT-VC meeting.

JCTVC-G163 Non-CE9: simplification of merge/skip TMVP ref_idx derivation [Y. Jeon, S. Park, J. Park, B. Jeon (LG)]

This contribution reports the results when the derivation process of reference indices for temporal merging candidate is simplified. Five simplifications SP01 to SP06 are tested. Simulation results revealed that no coding loss is obtained in all cases except low delay configurations of SP01. Based on test results, it is recommended to adopt one of the five simplifications SP02, SP03, SP04, SP05 and SP06 because the refIdx derivation process can be simplified without any harm on coding efficiency. Among the four recommended simplifications, SP05 and SP06 are the most preferred since they are the simplest ones.

Related to the low delay coding option. What are the losses without low delay coding option?

Due to the lack of explicit reference list construction in current HM, the low delay coding option changes the decoder as well.

An outlier (BQSquare) with high loss was reporting when fixign the refidx to 0.

Simplification of temporal refidx derivation for merge

JCTVC-G099 Non-CE9: cross-verification of LGE’s proposal JCTVC-G163 on simplification of merge/skip ref_idx derivation [M. Zhou (TI)]

Confirmed.

JCTVC-G164 Non-CE9: improvement on parallelized merge/skip mode [Y. Jeon, S. Park, B. Jeon (LG)]

Parallelized merge/skip is introduced at the Torino meeting in JCTVC-F069 to compensate the significant quality loss that can be caused when the parallel motion estimation (PME) is performed with the current HM design for throughput or implementation cost reasons. This contribution advocates JCTVC-F069 method and additionally makes a change in the merge/skip list construction process of JCTVC-F069 to improve the coding efficiency of JCTVC-F069 further. The simulation results revealed that this change provides significant coding efficiency improvement relative to JCTVC-F069 method resulting in 1.0%, 0.7%, 0.3%, and 0.0% BD rate reductions on average of all configurations (RA-HE, RA-LC, LB-HE, and LB-LC) for PME level 64x64, 32x32, 16x16, and 8x8 respectively. Compared to the HM4.0 PME in which the merge and skip modes have to be disabled inside the motion estimation region called MER except the first partition of MER, 5.6%, 4.8%, 2.5%, and 0.3% BD rate reductions are achieved on average of all configurations for PME level 64x64, 32x32, 16x16, and 8x8 respectively.

General concerns on this parallel merge technique are to be discussed when presenting the CE9 proposals.

To be considered dependent on CE9 MRG_PAR (JCTVC-F069) discussion.

Improvement of CE9 test MRG_PAR JCTVC-F069.

JCTVC-G100 Non-CE9: cross-verification of LGE’s proposal JCTVC-G164 on improvement on parallel merge/skip mode [M. Zhou (TI)]

JCTVC-F069 proponent is cross-checker confirms that the code is clean is does what is proposed.

Cross-checker thinks that it is a good improvement but should use top candidate instead of top left or horizontal and vertical candidates rather than diagonal ones.

JCTVC-G922 Non-CE9: cross-check of LGE’s proposal JCTVC-G164 on improvement on parallel merge/skip mode [Y. Park, H. Yang (Samsung)] [late]

It is also recommended to consider this proposal dependent on JCTVC-F069.

JCTVC-G387 Non-CE9 Parallel Merge/skip Mode for HEVC [X. Wen, O. Au, W. Dai, C. Pang, J. Dai, F. Zou, X. Zhan (HKUST)]

The current HEVC merge/skip is copying motion parameters to the current PU from a candidate list which consists of spatial and temporal neighbouring PUs. However, it is hard for parallel encoding and decoding due to the data dependency. Furthermore, different shapes and position of PU would result in different definition of candidate lists, this would lead to potentially extra hardware cost and not easy to be efficiently implemented by the hardware. In this proposal, we propose a high lever syntax to signal parallel depth of merge/skip mode and divide a LCU into non-overlapped square merge regions (MRs). All the PUs located inside the same MR use same candidate list of PUs at both encode and decoder side. By doing this, all the PUs in the same MR can be checked by certain architecture in parallel. Simulation results reveal that an average loss of 1.5%, 1.5%, 2.4% and 2.5% in RA-HE, RA-LC, LB-HE and LB-LC for 32x32 block level parallel ME when compared to the current HM4.0 design.Provides different trade-off points between saving logic and coding efficiency.

Slides not uploaded

To be considered dependent on CE9 MRG_PAR (JCTVC-F069) discussion.

Improvement, it is basically the same as JCTVC-G164.

JCTVC-G407 Non-CE9: Cross-check of parallelized merge/skip mode for HEVC (JCTVC-G387) [B. Li (USTC), J. Xu (Microsoft)]

Confirmed.

JCTVC-G165 Non-CE9/Non-CE13: new MVP positions for merge/skip modes and its combination with replacing redundant MVPs [Y. Jeon, S. Park, B. Jeon (LG), J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

This contribution proposes new MVP positions for skip and merge mode. Two new spatial MVP candidate positions (A2 and B3) are introduced in addition to the spatial MVP positions (A0, A1, B0, B1, and B2) of the current HM. The proposed order for spatial MVP candidates is {A2, B3, A1, B1, B0, A0, A2} but there are some restrictions for adding the candidates to the MVP list in order to minimize the complexity which can be caused by introducing the two new candidates. Simulation results revealed that the proposed method achieves 0.1% gain for RA configurations and 0.2% gain for both LB and LP configurations without any increase in encoding and decoding time.

Improvement, adding two spatial merge candidates.

JCTVC-G860 Non-CE9/Non-CE13: Cross-check of JCTVC-G165 on new MVP positions for merge/skip modes [V. Seregin (Qualcomm)] [late]

Confirmed.

JCTVC-G104 Non-CE9: cross-verification of LGE and MediaTek’s proposal JCTVC-G165 on new MVP positions for merge/skip modes and its combination with replacing redundant MVPs [M. Zhou (TI)]

The gain here is associated with complexity increase of the first pruning process of the merge/skip list derivation, which should be studied.

Confirmed.

JCTVC-G181 Non-CE9: Merging candidate reordering [H. Takehara, S. Fukushima (JVC Kenwood)]

In the HM4.0, motion information of the merging candidate that is located inside of the CU is not to be used In this proposal, instead of avoiding such merging candidate, the merging candidate is moved to the last position in the merge list after the derivation process for merging candidates. The simulation results report that the proposed technique provides 0.1% BD-rate gain for random access and 0.1%/0.2% gain for low delay B settingMerge redundancy check removal

Same idea also proposed in JCTVC-G681, JCTVC-G396 and JCTVC-G593.

Simplification on merge partition redundancy removal

JCTVC-G468 Non-CE9: cross-verification of JVC KENWOOD's proposal on Merge candidate reordering (JCTVC-G181) [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

Confirmed.

JCTVC-G681 Non-CE9: Simplified Merge candidate derivation [Y. Zheng, X. Wang, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution proposes a change to the rules currently used in determining merge candidates for a PU under 2NxN, Nx2N, NxN, or AMP mode. In HM4.0, when a current PU under these partition modes is not the first PU in a CU, the motion information of each of its merge candidates is compared with that of a previous PU to avoid a situation that a number of PUs share the same motion information so that the current prediction information can be classified into a mode with less partitions. For example, every PU has the same motion information under a mode other than 2Nx2N. This contribution proposes to remove such comparison. The proposed changes reduce the operations and also enable parallel merge candidate generation.Merge redundancy check removal

Remove the possibly redundant candidate may improve parallelism because PU1 does not depend on PU0 anymore.

Simplification on merge partition redundancy removal

JCTVC-G830 Non-CE9: Crosscheck for Qualcomm's simplified merge candidate derivation in JCTVC-G681 [J.-L. Lin, Y.-W. Huang (MediaTek)] [late]

Confirmed.

JCTVC-G593 Non-CE13: Simplification of merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

This contribution comprises three parts aiming at reducing complexity of the merge process. In the first part, it is proposed to reduce number of motion comparisons during the first pruning process by comparing each spatial candidate with a limited number of spatial candidates and exempting the temporal motion vector prediction candidate from the pruning process. In the second part, it is proposed to derive redundancy information for the combined candidates during the first pruning process and exempting other additional candidates from pruning. In the third part, number of contexts used for the merge index is reduced to two and the contexts are assigned according to being a skip mode or inter-merge mode. Combination of all the three parts provides the merge mode having a total of one full motion comparison per PU where other motion comparisons are either removed or replaced by simpler operations. The reported compression efficiency impact in terms of BD rate is 0.1%, 0.1%, 0.1%, 0.2% for RA_HE, LB_HE, RA_LC and LB_LC, respectively.

Source code bundled with the proposal.

Merge redundancy check removal

Additional modification of the first and second pruning as well as context reduction for CABAC.

Simplification

JCTVC-G940 Cross-check for JCTVC-593: Non-CE13: Simplification of merge mode [Semih Esenlik (Panasonic)] [late]

Confirmed.

JCTVC-G396 Non-CE9: swapping of merge candidate [C. Kim, Y. Jeon, B. Jeon (LG)]

In this contribution, two methods for reordering the merge candidates are presented. The first method is applied only for 2Nx2N PUs of square shape. The first method (Proposed method 1) swaps A1 and B1 candidate order in the merge list if the propose condition is satisfied. It is reportedly shown in the experimental results that 0.1~0.2% BD rate reduction is achieved without increasing encoding and decoding time with the proposed method 1. The second method (Proposed method 2) is applied for the second PUs of 2NxN, 2NxnU, 2NxnD, Nx2N, nLx2N and nRx2N partitions of rectangular shape. The proposed method 2 uses the MVP candidate which belongs to the first PU of those rectangular partitions for creating the combined bi-pred. candidates even though this candidate shall not exist in the initial list due to the avoiding check operation. It is reportedly shown in the experimental results that 0.1~0.2% BD rate reduction is achieved without increasing encoding and decoding time with the proposed method 2. In addition, the proposed methods are tested with the new anchor in which the avoiding check operation is removed. It is reportedly shown from the simulation results that the proposed 2 achieves 0.0~0.1% BD rate saving without encoding/decoding time increase relative to the new anchor and the combination of the proposed method 1 and the proposed method 2 achieves 0.1~0.2% BD rate saving. The encoding and decoding time is almost same as the anchor.

Title was changed to “Non-CE9: reordering of merge candidate” after first registration.

It was mentioned that there is a dependency between first and second PU.

Improvement Method1

Simplification Method2

JCTVC-G484 Non-CE9: Crosscheck for LG's swapping of merge candidate in JCTVC-G396 [J.-L. Lin, Y.-W. Huang (MediaTek)] [late]

Nobody was there to report. Document confirms.

JCTVC-G927 Non-CE9: Cross-check of JCTVC-G396 on reordering of merge candidate [V. Seregin (Qualcomm)] [late]

Confirmed.

JCTVC-G944 Cross-check report on G396 [I.-K. Kim] [late]

Confirmed.

JCTVC-G542 Non-CE9/Non-CE13: Simplification on AMVP/Merge [T. Sugio, T. Nishi(Panasonic)]

In this contribution, three simplification methods on AMVP/Merge were proposed. 1) On avoiding merging candidate decision 2) On AMVP spatial scaling candidate 3) On zero motion vector merging candidate. Experimental results reportedly showed no loss on average relative to the HM4.0 with reducing complexity on AMVP/Merge.

Simplification 1 on merge partition redundancy removal

Simplification 2 on AMVP scaling for spatial candidates

Simplification 3 zero motion vector merge candidate

JCTVC-G891 Non-CE9: cross-verification of Panasonic's simplification on AMVP/Merge (JCTVC-G542) [M. Zhou (TI)] [late]

The proposed simplifications 2 and 3 of JCTVC-G542 are helpful for reducing the complexity of merge/skip and AMVP list derivation process. It is recommended to consider this proposal together with other contributions in this category.

Nobody was there to report. Document confirms.

JCTVC-G878 Non-CE9: Crosscheck for Panasonic's proposals (JCTVC-G542) by JVC-KENWOOD [Hiroya Nakamura, Shigeru Fukushima (JVC Kenwood)] [late]

Confirmed.

JCTVC-G182 Non-CE9: AMVP syntax for bi-prediction [H. Takehara, S. Fukushima (JVC Kenwood)]

The proposed simplifications 2 and 3 of JCTVC-G542 are helpful for reducing the complexity of merge/skip and AMVP list derivation process. It is recommended to consider this proposal together with other contributions in this category.

Changing the syntax order for ref_idx_l1. No impact CAVLC BD rate, small impact on CABAC due to context dependencies.

Simplification for parsing throughput.

JCTVC-G431 Non-CE9: Cross-verification of JVC Kenwood proposal on AMVP syntax for bi-prediction (JCTVC-G182) [T.Chujoh (Toshiba)]

Confirmed.

JCTVC-G195 Non-CE9/13: Averaged merge candidate [S. Shimada, K. Kazui, J. Koyama, A. Nakagawa (Fujitsu)]

This contribution proposes a new type of merge candidate, namely averaged merge candidate, in the derivation process for luma motion vectors for merge mode. An averaged merge candidate is derived by averaging a pair of two original merge candidates and added into the merge candidates list with higher priority than other candidates previously adopted in WD4.

The coding gains of the proposed scheme over HM4.0 are 0.2% (HE), 0.3% (LC) in RA configurations, 0.0% (HE), 0.0% (LC) in LDB configurations and 0.3% (HE), 0.6% (LC) in LDP configurations. When the proposed scheme works without non-scaled bi-predictive merge candidate in WD4, the additional coding gains are 0.2% (HE), 0.2% (LC) in RA configurations, 0.0% (HE), 0.0% (LC) in LDB configurations and 0.3% (HE), 0.6% (LC) in LDP configurations. Measured computational complexities for various configurations in encoding and decoding are 96% - 102% and 100% - 101%, respectively.

Test1: Adding averaged candidates

Test2: Replacing bi-predictive candidates by averaged ones.

It was remarked that additional shifts and adds are required for the averaging and it is checked whether the refidx of the two average candidates are equal.

It was assumed that removing the additional duplicate check may not have big impact in codign efficiency.

Removing all duplicate checks was proposed in JCTVC-G397.

Improvement

JCTVC-G467 Non-CE9/13: cross-verification of Fujitsu's proposal on Merge (JCTVC-G195) [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late] [miss]

Confirmed.

Reported runtimes are obtained on one type of computer and they are around 100%.

JCTVC-G217 Non-CE9: Derivation process of reference indices for temporal merge candidates [H. Nakamura, S. Fukushima (JVC Kenwood)]

In HM4.0, the reference indices of temporal merging candidates are derived by the majority decision of the reference indices of three neighboring blocks.

This contribution proposes that simplified derivation process of reference indices for temporal merging candidates.

The simulation results report that the proposed technique provides no coding loss for random access settings and low delay B settings.

Same as one simplification (test2) in JCTVC-G163.

JCTVC-G742 Cross-check report on derivation process of reference indices for temporal merge candidates (JCTVC-G217) [I.-K. Kim (Samsung)]

Confirmed

JCTVC-G219 Non-CE9: Construction of MVP list without using scaling [H. Nakamura, S. Fukushima (JVC Kenwood)]

In the HM4.0, when a MVP list is constructed, two-times scaling operations are performed at a maximum.

This contribution proposes a method which reduces the number of times of scaling operation.

The simulation results report that the proposed technique provides no gain and loss for random access and low delay B settings.

Simplification of AMVP list construction.

JCTVC-G105 Non-CE9: cross-verification of JVC’s proposal JCTVC-G219 on construction of MVP list without using scaling operation [M. Zhou (TI)]

Confirmed

On the encoder side, this technique won’t save the number of MVP scaling,  in the worst case still two scales are needed. But it will reduce the worst case scale from 2 to 1 on the decoder side. The penalties are increased latency because non-scaled and scaled MVP derivation processes now run sequentially (In the current design the non-scaled MVP derivation can be hidden by the scaled MVP derivation); motion vector reconstruction on decoder side needs two stages, i.e. first to use mvp_idx to compute the final MVP, and then add MVDs to obtain the motion vectors, and additional storage for buffering the temporal distances for all the MVPs.

JCTVC-G223 Non-CE9: Division-free MV scaling [T.-D. Chuang, Y.-W. Chen, J.-L. Lin, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

In HM-4.0 motion vector (MV) scaling for the derivation of spatial and temporal motion vector predictors (MVPs), a division operation is required to derive the scaling factor. In hardware and many DSP-based platforms, dividers are undesirable because of larger gate counts and more processing cycles. In this contribution, a division-free MV scaling is proposed to replace the general divider by a look-up table and simple arithmetic operations. Moreover, the effective scaling range is doubled to deal with reference pictures of longer temporal distances. Simulation results reportedly show that the proposed method has no bit rate increase in random access conditions and even 0.1% bit rate reduction in low delay conditions. The proposed design is also implemented in Verilog and synthesized with TSMC 40nm process. The synthesis results reportedly show that the gate count of the MV scaling module is reduced by 54-58%.

Proposal 1: Extending the clipping range in the scaling factor clipping

Proposal 2: Division free MV scaling by using a LUT

HEVC uses the same scaling as AVC. The implementation of this HM4.0 scaling the proposal is compared to may also be table-based. Additional slides were shown with comparing this with a tabled-based HM4.0 scheme.

JCTVC-F068 reports that scaling needs 3 cycles. That is not affected by the tested proposal.

JCTVC-G523 Non-CE9: Cross-check of division-free MV scaling [P. Onno (Canon)] [late]

Divsion-free scaling is confirmed.

Clipping range extension was not verified.

JCTVC-G225 Non-CE9: Simplified AMVP derivation for Inter mode [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

No presentation was requested.

JCTVC-G341 Crosscheck of MediaTek's AMVP simplification (JCTVC-G225) [J. Park, S. Park, B. Jeon (LG)]

JCTVC-G241 Non-CE9: On parallel derivation of the temporal predictor for Merge/Skip modes [G. Laroche, T. Poirier, P. Onno (Canon)]

This contribution presents a simplification of motion vector derivation process for the Merge/Skip modes. The aim of the proposed scheme is to reduce the number of cycles in hardware needed for this derivation process for the worst case. The proposed scheme consists in scaling the temporal candidates in parallel to the cascaded predictors and related pruning process. Based on the complexity analysis from Texas Instruments in JCTVC-F088 and JCTVC-F068, it is reported that 3 cycles are saved for the Merge/Skip mode derivation process for the worst case hardware implementation with less than 0.1% BDR loss.

Simplification of first pruning and last pruning process.

JCTVC-G098 Non-CE9: cross-verification of Canon’s proposal JCTVC-G241 on parallel derivation of the temporal predictor for Merge/Skip modes [M. Zhou (TI)]

The proposed parallel MVP scaling will improve the throughput of the merge/skip MVP list derivation process. The trade-off between coding efficiency and throughput improvement should be carefully considered here.

Cross-checker not present

JCTVC-G305 Non-CE9: Bi-prediction for low delay coding [Y. Suzuki, A. Fujibayashi (NTT DOCOMO)]

In this contribution the additional option of bi-prediction for low delay conditions and key pictures of random access conditions is proposed. In the proposed method, the motion vector difference for List 1 is not signaled and it sets to (0, 0) when POCs of all reference pictures are smaller than that of a target picture. The average Y BD-rate gains for low delay conditions and random access conditions are 0.9% and 0.2%, respectively.

Encoder only change (setting lis1 mvd to zero and signalling it) give 0.3% gain on average for LB.

There is no list 1 motion vector search for bi-prediction.

It was asked what would be the impact when the list1 motion vector search for bi-prediction is enabled at the encoder. The assumption is that if the list 1 estimation is better the loss of setting the list 1 mvd to zero is increasing.

Interesting candidate to be considered after CE9/13 conclusion is reached.

Improvement

JCTVC-G603 Non-CE9: Cross-check of Bi-prediction for low delay coding (JCTVC-G305) [P. Helle, B. Bross (Fraunhofer HHI)] [late]

Confirmed.

JCTVC-G343 Non-CE9: Improvement in temporal candidate of merge mode and AMVP [N. Zhang, X. Fan, S. Ma, D. Zhao (Harbin Inst. Tech.)]

This contribution proposes an improved scheme in temporal candidate of merge mode and AMVP. In order to utilize motion information from the below and right sides more effectively, the scheme considers three additional temporal candidate checking positions relative to HM4.0 in some cases. Simulation results show that the proposed scheme in average provides 0.1% bit rate saving for random access, and 0.2% bit rate saving for low delay with almost the same time cost. This proposal have been crosschecked by MSRA and the document number is JCTVC-G406.

Solution A conditional add

Solution B always add (prefered)

Improvement, add an additional temporal candidate for amvp and merge.

JCTVC-G406 Non-CE9: Improvement in temporal candidate of merge mode and AMVP (JCTVC-G343) [B. Li (USTC), J. Xu (Microsoft)]

Nobody was there to report. Document confirms.

JCTVC-G397 Non-CE9/Non-CE13: Simplification of adding new merge candidates [B. Li (USTC), J. Xu (Microsoft), H. Li (USTC)]

The parsing robustness issue was solved in JCTVC-F470 by a method similar to coding full index of merge candidate. For some other syntax elements, the parsing dependency of the surrounding information is also removed. One most important part of JCTVC-F470 is to compensate the loss introduced by coding full merge index. Some new merge candidates are also added to the set of merge candidates, which was adopted in the latest HM design. For each new added merge candidate, the duplication checking is also performed. This contribution evaluates the benefits of each individual step and proposes to remove some steps to simplify the design.

Simplification disable combined merge candidate and disable combined merge candidate duplicate checking

Very interesting data for the discussion on the combined merge candidates.

JCTVC-G426 Cross-check of Microsoft’s proposal on simplification of adding new merge candidates (JCTVC-G397) by Huawei [H. Yang, H. Yu (Huawei)]

Nobody was there to report. Document confirms.

JCTVC-G416 CU-based Merge Candidate List Construction [H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee, J. S. Choi (ETRI)]

This contribution proposes a CU-based approach for merge candidate list construction, where a CU can have at most one merge candidate list that can be constructed prior to encoding and decoding of the internal PUs. It is indicated that the proposed approach provides simpler design, reduced complexity, and improved parallelism compared to the PU-based one used in HM4. Also, it is reported that roughly 3~6 % encoding time reduction with the penalty of roughly 0.2~0.5% coding loss depending on the test configurations.

Simplification of the  merge candidate list construction

This is applied to all CU size whereas there are proposals restricting parallalism to certain sizes.

JCTVC-G899 Cross-check of CU-based Merge Candidate List Construction [S. Oudin, B. Bross(Fraunhofer HHI)] [late]

confirmed

JCTVC-G516 On Spatial MV Prediction [K. Sato (Sony)] [late]

Under current HM specification motion information of topright PU is one of the candidates for spatial motion vector prediction. When LCU-level parallel processing is applied for HEVC encoding/decoding, extraction of motion information of topright PU becomes bottleneck.  This contribution proposes not using motion information of topright PU at LCU boundaries for motion vector coding. Average losses with the proposed method is less than 0.2% for Y/U/V component with RA/RA_Loco/LD/LD_Loco conditions.  It is recommended that the proposed method be studied under the core experiment.

Simplification

JCTVC-G799 Non-CE9: Crosscheck for Sony's proposals (JCTVC-G516) by JVC KENWOOD [Hiroya Nakamura (JVC Kenwood)] [late]

Coding loss is considered acceptable.

Confirmed

JCTVC-G541 Non-CE9: Simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge [T. Sugio, T. Nishi(Panasonic)]

In this contribution, a simplified scaling calculation for AMVP/Merge was proposed. In the proposed method, the scaling factor is quantized to a power of 2 based on the distance among current picture, reference picture and co-located picture. Experimental results reportedly showed BD rate loss in the range of 0.0-0.1% with reducing the number of multiplier relative to the HM4.0 by applying the proposed method to temporal MVP in AMVP.

Test1:  simplified scaling for temporal amvp candidate

Test2: simplified scaling for temporal candidate

Test3: simplified scaling for spatial amvp candidates

Combined results are not reported.

Simplification of the scaling based on temporal distance used in both amvp and merge (temporal and combined candidates).

JCTVC-G102 Non-CE9: cross-verification of Panasonic’s proposal JCTVC-G541 simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge [M. Zhou (TI)]

The proposed scaling calculation will likely to reduce the cost in hardware implementation. Trade-off between coding efficiency and implementation cost reduction should be evaluated, its implication on the software implementations should be considered as well.

JCTVC-G551 Restriction on motion vector scaling for Merge and AMVP [I.-K. Kim, Y. Park, N. Shlyakhov, J. H. Park (Samsung)]

In this contribution, a restriction method on motion vector scaling is proposed. When POC difference between reference picture of current PU and reference picture of candidate PU (collocated PU or neighbor PU) are larger than pre-determined threshold, motion vector scaling is restricted. The motion vector predictor is marked as unavailable before the scaling process is performed. This restriction is applied to both spatial and temporal scaling. Average coding efficiency gain from this modification for each configuration are 0.0% (RAHE), 0.0% (RALC), 0.1% (LDHE), 0.0% (LDLC), 0.1% (LPHE), 0.1% (LPLC) with class F sequences. Encoding and decoding time are 100% and 101%, respectively. Coding efficiency gain for Class F only is 0.13%. It is recommended to adopt this restriction process in HEVC.

Should be considered in the AHG21 discussion.

There was support to have such a restriction.

JCTVC-G859 Crosscheck of JCTVC-G551 proposed by Samsung [X. Zheng] [late]

Nobody was there to report. Document confirms

JCTVC-G552 Simplification of temporal motion vector (TMVP) candidate derivation for Merge and AMVP [I.-K. Kim, N. Shlyakhov, J. H. Park(Samsung)]

In this contribution, two simplification methods are proposed to simplify TMVP derivation process of Merge and AMVP. The first item is utilization of upper-left position instead of center position. The second one is utilization of zero reference index instead of the value which has majority of occurrence among neighbours. By utilizing upper-left position, no additional calculation is required to get the specific position since it is easy to find upper-left position which is starting point of PU. In addition, utilizing zero reference index for Merge TMVP candidate saves the steps for checking and calculating the majority of occurrence. Checking which one is majority is complex process which involves several comparisons and conditional branches. When the both simplification is combined, the average loss is 0.0% with 100% and 99% encoding and decoding time, respectively, compared to HM-4.0. Based on these experimental results, it is recommended to adopt these two simplifications on Merge and AMVP.

Simplification1 of temporal motion vector position

Simplification2 of temporal motion vector index derivation

JCTVC-G227 Non-CE9: Cross-check for Samsung's proposals (JCTVC-G552) by JVC KENWOOD [H. Nakamura, S. Fukushima (JVC Kenwood)]

Confirmed

JCTVC-G515 Cross-check of Samsung's TMVP simplification by Sony [K. Sato (Sony)] [late]

Confirmed

JCTVC-G592 Non-CE9: Removal of reference index derivation for TMVP in merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

This was already discussed in several other documents JCTVC-G163/G217/G552/G690 (see 2.1.5)

JCTVC-G690 Non-CE9: Some possible motion vector coding related simplifications [Y. H. Tan, C. Yeo, Z. Li (I2R)]

This contribution presents several modifications that are aimed at simplifying motion vector coding. The first modification is related to the derivation of the L1 motion vector predictor (MVP) during bi-directional prediction. L0 and L1 MVP candidates are derived pair-wise such that both sets of candidates are derived from the same neighboring PU. This reportedly led to a coding loss of 0.0% and 0.1% BD-rate loss for RA and LB configurations respectively. The second modification restricts the reference index to zero during the derivation of the reference index of the temporal Merge candidate. This reportedly led to a coding loss of 0.0% and 0.1% BD-rate loss for RA and LB configurations respectively.  The third proposed modification is to remove the context for coding the MVP index when CABAC is the entropy coder, which reportedly resulted in an average of 0.0% BD-Rate loss. The last two modifications are related to the addition and pruning process of ‘combined’ Merge candidates. The modified process limits the number of combined candidates to five and uses a simplified pruning process. When applied together, these two modifications result in an average of 0.0% BD-Rate loss in all configurations.

Simplification 1 related to AMVP_SEL03, AMVP_SEL04 CE9 tests

A concern was raised on the dependency between list 0 and list 1 mvp derivation.

To be discussed when CE9 test are reviewed.

Simplification 2 also setting refidx to 0 for temporal merge candidate

Simplification 3 code mvp_idx in bypass mode

Simplification 4 reduce the possible combination of combined bi-predictive candidates to 5

Simplification 5 combined bi-predictive candidates duplicate check is replaced by checking the candidates before combination.

This is also addressed in JCTVC-G593 part 2

JCTVC-G970 Cross verification of Some possible motion vector coding related simplifications (JCTVC-G690) [M. Coban (Qualcomm)] [late]

Simplification 4 and 5 confirmed

JCTVC-G752 Non-CE9: Crosscheck of I2R's proposal on motion vector coding related simplifications (JCTVC-G690) [X. Zheng (HiSilicon)] [late]

Simplification 3, confirmed.

JCTVC-G733 cross check for I2R AMVP simplification [Yue Yu, Krit Panusopone, Limin Wang] [late] [miss]

Simplification 1 confirmed

JCTVC-G710 Non-CE9: The Parallel Friendly MVP Candidate Calculation for HEVC [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

This contribution proposes a parallel friendly MVP candidate calculation when a CU consists of two PUs. Simulation results show that there is a loss of 0.3% for random access condition and a loss of 0.1% for low delay condition compared to original AMVP while it is possible to parallel process two PUs.

Slides not uploaded

Results are reported for simplification combined with improvement.

Simplification of AMVP replace PU0 candidate for PU1 by one outside PU0

Improvement of AMVP by swapping indices, i.e. having different list orders for PU0 and PU1

JCTVC-G712 Non-CE9: The Simplification of MVP for HEVC [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

This contribution proposes a simplification of MVP design for HEVC. Simulation results show that a loss of 0.1% for LBHE and a loss of 0.3% for RAHE are observed compared to original AMVP while the complexity of the proposed method is reduced at least 50%, and up to 66.7% as compared to the MVP selection procedure of AMVP when a CU consist of two PUs.

Slides not uploaded

Simplification of AMVP where PU1 MVP is derived from PU0 MVP

Concern was raised on the additional scaling.

It cannot be combined with JCTVC-G710.

Losses of 1.1% for SteamLocomotive.

JCTVC-G740 Cross-check of Motorola's proposals on motion vector coding (G710, G712) [Y. H. Tan, C. Yeo (I2R)] [late]

Confirmed

JCTVC-G221 Non-CE13: The maximum number of merging candidates in P-slice [H. Nakamura, S. Fukushima, H. Takehara, M. Ueda (JVC Kenwood)]

In the HM 4.0, the maximum number of merging candidates is fixed to 5 as maxNumMergeCand in both P-slice and B-slice. This contribution reports the results of investigation of the variable maxNumMergeCand in low delay P settings. The simulation results report that 0.1% BD-rate gain is obtained under the setting of maxNumMergeCand equal to 3.

Slides not uploaded

Not cross-checked.

Proposal assumes a slice header flag signaling the maximum number of merge candidates as proposed in CE13 tests. These tests has to be considered first.

Simplification

JCTVC-G224 Non-CE13: Multiple-scaled merging candidates [H. Nakamura, S. Fukushima (JVC Kenwood)]

This proposal is for the improvement of the merge mode. In the HM4.0, for the merge mode, some spatial merging candidates and a temporal merging candidate are added into a merging candidate list. After that, if needed, combined bi-predictive merging candidates, a non-scaled bi-predictive merging candidate and zero motion vector merging candidates are added. The non-scaled bi-predictive merging candidate is added under a random access condition. Therefore, the multiple-scaled merging candidate by extending the non-scaled merging candidate for low delay conditions is proposed in this contribution. The simulation results report that the proposed techniques provide no gain for random access setting, 0.1-0.2% BD-rate gain for low delay B setting, and 0.1-0.2% BD-rate gain for low delay P setting.

Slides not uploaded

Improvement of non-scaled combined merge candidates by adding scaling.

JCTVC-G544 Non-CE13: Cross check report of JCTVC-G224 by panasonic [T. Sugio, T. Nishi(Panasonic)] [late]

Confirmed.

JCTVC-G288 Non-CE13: Complexity Reduction of MVP List Contruction [Y. Yu, T. Hellman (Broadcom)]

Proponent not there to present.

JCTVC-G097 Non-CE13: cross-verification of Broadcom’s proposal JCTVC-G288 complexity reduction of MVP list construction [M. Zhou (TI)]

JCTVC-G683 Non-CE13: Simplification and improvement of additional merge candidate [Y. Zheng, X. Wang, W.-J. Chien, M. Karczewicz (Qualcomm)]

This proposal presents several changes to simplify the additional merge candidates generating process. In the proposal, scaled bi-pred candidate is removed, the additional merge candidate pruning operation is limited to the combined bi-pred candidates only, and motion offset candidates are added. With fewer operations, the proposed changes provide 0.3% bitrate saving in average based on common test conditions.

Simplification: Disable remove duplicates for zero mv and remove the non-scaled bi-predictive merge candidates (also tested in JCTVC-G397)

Improvement: Add motion offset candidates

JCTVC-G872 Non-CE13: crosscheck of Qualcomm’s proposal G683 on simplification and improvement of additional merge candidate [Y. Jeon, B. Jeon (LG)] [late]

confirmed but reports slightly increased runtimes

JCTVC-G787 Non-CE13: Additional merge candidates with MV dependent offsets [T. Lee, J. Chen, J. H. Park (Samsung), G. Laroche, P. Onno (Canon), J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

This contribution presents additional Merge/AMVP candidates to compensate empty positions in the fixed length candidates list. The additional Merge/AMVP candidates are produced by adding offsets to the first existing candidate where the offset value is chosen according to the first candidate’s MV value. Relative to the HM4.0+MRG_ENC_FIX which is reported in JCTVC-G776, experimental results showed -0.3% BR savings in average of results in RA-HE, RA-LC, LB-HE, LB-LC, LP-HE, LP-LC.

Combination of CE13 tests. Should be revisited when a conclusion on CE9/13 tests is reached.

A concern was raised regarding losses in chroma and luma for class E sequences. This may be dues to removing zero motion vector merge candidates.

The fast encoding method can also be applied to other proposals.

Improvement on additional merge candidates

JCTVC-G926 Non-CE13: Cross-check of JCTVC-G787 on MV dependent offset [V. Seregin (Qualcomm)] [late]

It should be assured that the merge index encoder estimation bugfix is included in anchor and tested

Confirmed.

JCTVC-G839 Non-CE13: cross-verification of Samsung, Canon, MediaTek proposal JCTVC-G787 on additional merge candidates with MV dependent offsets [Y. Jeon, B. Jeon (LG)] [late]

Confirmed

JCTVC-G946 Non-CE13: Cross-check report of JCTVC-G787 (MvDepOffset+Fast) [S.-C. Lim, J. Lee, H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU)] [late]

# Classification of reviewed proposals

Figure 2-1 Different parts of the merge candidate list construction process based on which the reviewed proposals are classified.

## Simplifications

### Merge spatial candidate positions

JCTVC-G416 CU-based Merge Candidate List Construction [H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee, J. S. Choi (ETRI)]

JCTVC-G516 On Spatial MV Prediction [K. Sato (Sony)] [late]

### Merge partition redundancy removal

JCTVC-G181 Non-CE9: Merging candidate reordering [H. Takehara, S. Fukushima (JVC Kenwood)]

JCTVC-G681 Non-CE9: Simplified Merge candidate derivation [Y. Zheng, X. Wang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-G593 Non-CE13: Simplification of merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

JCTVC-G542 Non-CE9/Non-CE13: Simplification on AMVP/Merge [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G416 CU-based Merge Candidate List Construction [H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee, J. S. Choi (ETRI)]

### 1st pruning

JCTVC-G593 Non-CE13: Simplification of merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

JCTVC-G241 Non-CE9: On parallel derivation of the temporal predictor for Merge/Skip modes [G. Laroche, T. Poirier, P. Onno (Canon)]

JCTVC-G221 Non-CE13: The maximum number of merging candidates in P-slice [H. Nakamura, S. Fukushima, H. Takehara, M. Ueda (JVC Kenwood)]

### Merge additional candidates (combined, non-scaled, zero mv,...)

JCTVC-G593 Non-CE13: Simplification of merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

JCTVC-G396 Non-CE9: swapping of merge candidate [C. Kim, Y. Jeon, B. Jeon (LG)]

JCTVC-G542 Non-CE9/Non-CE13: Simplification on AMVP/Merge [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G397 Non-CE9/Non-CE13: Simplification of adding new merge candidates [B. Li (USTC), J. Xu (Microsoft), H. Li (USTC)]

JCTVC-G690 Non-CE9: Some possible motion vector coding related simplifications [Y. H. Tan, C. Yeo, Z. Li (I2R)]

JCTVC-G683 Non-CE13: Simplification and improvement of additional merge candidate [Y. Zheng, X. Wang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-G288 Non-CE13: Complexity Reduction of MVP List Contruction [Y. Yu, T. Hellman (Broadcom)]

### Merge temporal candidate refidx derivation

JCTVC-G163 Non-CE9: simplification of merge/skip TMVP ref_idx derivation [Y. Jeon, S. Park, J. Park, B. Jeon (LG)]

JCTVC-G217 Non-CE9: Derivation process of reference indices for temporal merge candidates [H. Nakamura, S. Fukushima (JVC Kenwood)]

JCTVC-G552 Simplification of temporal motion vector (TMVP) candidate derivation for Merge and AMVP [I.-K. Kim, N. Shlyakhov, J. H. Park(Samsung)]

JCTVC-G592 Non-CE9: Removal of reference index derivation for TMVP in merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

JCTVC-G690 Non-CE9: Some possible motion vector coding related simplifications [Y. H. Tan, C. Yeo, Z. Li (I2R)]

### Temporal candidate position

JCTVC-G082 non-CE9: Modified H positions for memory bandwidth reduction in TMVP derivation [M. Zhou (TI)]

JCTVC-G552 Simplification of temporal motion vector (TMVP) candidate derivation for Merge and AMVP [I.-K. Kim, N. Shlyakhov, J. H. Park(Samsung)]

### MV scaling

JCTVC-G223 Non-CE9: Division-free MV scaling [T.-D. Chuang, Y.-W. Chen, J.-L. Lin, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G541 Non-CE9: Simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G551 Restriction on motion vector scaling for Merge and AMVP [I.-K. Kim, Y. Park, N. Shlyakhov, J. H. Park (Samsung)]

### AMVP

JCTVC-G182 Non-CE9: AMVP syntax for bi-prediction [H. Takehara, S. Fukushima (JVC Kenwood)]

JCTVC-G516 On Spatial MV Prediction [K. Sato (Sony)] [late]

JCTVC-G710 Non-CE9: The Parallel Friendly MVP Candidate Calculation for HEVC [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-G712 Non-CE9: The Simplification of MVP for HEVC [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-G542 Non-CE9/Non-CE13: Simplification on AMVP/Merge [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G219 Non-CE9: Construction of MVP list without using scaling [H. Nakamura, S. Fukushima (JVC Kenwood)]

### Misc

JCTVC-G134 Motion Vector Predictor Candidate Clipping Removal [M. Coban, M. Karczewicz (Qualcomm)]

## Parallel merge

JCTVC-G164 Non-CE9: improvement on parallelized merge/skip mode [Y. Jeon, S. Park, B. Jeon (LG)]

JCTVC-G387 Non-CE9 Parallel Merge/skip Mode for HEVC [X. Wen, O. Au, W. Dai, C. Pang, J. Dai, F. Zou, X. Zhan (HKUST)]

JCTVC-G416 CU-based Merge Candidate List Construction [H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee, J. S. Choi (ETRI)]

## Coding efficiency improvements

JCTVC-G195 Non-CE9/13: Averaged merge candidate [S. Shimada, K. Kazui, J. Koyama, A. Nakagawa (Fujitsu)]

JCTVC-G305 Non-CE9: Bi-prediction for low delay coding [Y. Suzuki, A. Fujibayashi (NTT DOCOMO)]

JCTVC-G343 Non-CE9: Improvement in temporal candidate of merge mode and AMVP [N. Zhang, X. Fan, S. Ma, D. Zhao (Harbin Inst. Tech.)]

JCTVC-G224 Non-CE13: Multiple-scaled merging candidates [H. Nakamura, S. Fukushima (JVC Kenwood)]

JCTVC-G683 Non-CE13: Simplification and improvement of additional merge candidate [Y. Zheng, X. Wang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-G787 Non-CE13: Additional merge candidates with MV dependent offsets [T. Lee, J. Chen, J. H. Park (Samsung), G. Laroche, P. Onno (Canon), J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G165 Non-CE9/Non-CE13: new MVP positions for merge/skip modes and its combination with replacing redundant MVPs [Y. Jeon, S. Park, B. Jeon (LG), J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G223 Non-CE9: Division-free MV scaling [T.-D. Chuang, Y.-W. Chen, J.-L. Lin, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

(only the increased clipping range for the scale factor)

# Side activity report on simplification of the 2nd pruning process of merge/skip list derivation

## Software code base

HM4.0 +

- Samsung’s encoder bug fix for merge (JCTVC-Gxxx)

- Qualcomm/Panasonic’s simplification for merge partition redundancy removal (JCTVC-G681/JCTVC-G542 simplification 1)

- Panasonic’s Simplification 2 on AMVP scaling for spatial candidates (JCTVC-G542, simplification 2)

- Qualcomm’s removal of vector clipping in AMVP and merge/skip list derivation (JCTVC-G134)

- MediaTek’s extending the clipping range in the scaling factor clipping (JCTVC-G223 Proposal 1)

## Proposals to be tested

JCTVC-G593 Non-CE13: Simplification of merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

Section 3: Part 2: Simplification of the second pruning process.

Create the MVP combination map from the first pruning process for the combined MVPs, by sharing the duplication checks for uni-directional MVPs, and  adding more duplication checks for the bi-directional mode in the 1st pruning process.

The MVP combination map is used to avoid the second pruning process.

JCTVC-G690 Non-CE9: Some possible motion vector coding related simplifications [Y. H. Tan, C. Yeo, Z. Li (I2R)]

Simplification 5 combined bi-predictive candidates duplicate check is replaced by checking the candidates before combination. Plus removal of duplication check in no-scaled and zero MVP candidates.

Create combined MVP only if the list0 MVP and the list1 MVP have different X or Y component, remove the second pruning process

JCTVC-G397 Non-CE9/Non-CE13: Simplification of adding new merge candidates [B. Li (USTC), J. Xu (Microsoft), H. Li (USTC)]

No duplication checking for all

Note: number of combined, non-scaled and zero MVPs are kept same as HM4.0, i.e. 5, 1, 2 respectively.

## Test Plan

Panasonic creates the anchor software, proponents integrate tools into anchor software.

The simulation task is divided as follows:

| Proposal | Tester1 | Tester2 | Cross-check status |
| --- | --- | --- | --- |
| JCTVC-G593 | Nokia | I2R | confirmed |
| JCTVC-G690 | I2R | Nokia | Confirmed? |
| JCTVC-G397 | Microsoft | TI | confirmed |
| Anchor | Microsoft | TI | confirmed |

## Experimental results

Table 1. Summary of experimental results

| Proposal | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| JCTVC-G593 | 0.0 | 0.0 | 0.0 | 0.0 |
| JCTVC-G690 | 0.0 | 0.1 | 0.1 | 0.1 |
| JCTVC-G397 | 0.1 | 0.1 | 0.1 | 0.2 |
| JCTVC-G0397 plus removal of zero MVP cap (JCTVC-G542 simplification 3) | 0.0 | 0.1 | 0.0 | 0.0 |
| Anchor | 0.0 | 0.0 | -0.1 | -0.1 |

Table 2. Test results for JCTVC-G0593

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | -0.2% | 0.1% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 154% |  |  | 140% |  |  |
| Dec Time[%] | 87% |  |  | 80% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | 0.4% |
| Class E | 0.2% | 0.2% | 0.3% | 0.1% | 0.5% | -0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 147% |  |  | 145% |  |  |
| Dec Time[%] | 95% |  |  | 90% |  |  |

Table 3. Test results for JCTVC-G0690

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | -0.1% | 0.1% | -0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 199% |  |  | 183% |  |  |
| Dec Time[%] | 123% |  |  | 115% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.0% |
| Class E | 0.3% | -0.2% | 0.2% | 0.1% | 0.4% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 195% |  |  | 190% |  |  |
| Dec Time[%] | 120% |  |  | 119% |  |  |

Table 5. Test results for JCTVC-G0397

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | -0.1% | 0.1% | 0.1% | 0.0% |
| Class B | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 96% |  |  | 94% |  |  |
| Dec Time[%] | 87% |  |  | 90% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.2% | 0.2% | 0.0% | 0.0% |
| Class C | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% |
| Class D | 0.1% | 0.4% | 0.0% | 0.1% | 0.6% | 0.3% |
| Class E | 0.3% | 0.2% | 0.6% | 0.3% | 0.7% | -0.6% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.3% | 0.2% | 0.2% | 0.3% | 0.0% |
|  | 0.1% | 0.2% | 0.2% | 0.2% | 0.3% | 0.0% |
| Enc Time[%] | 97% |  |  | 95% |  |  |
| Dec Time[%] | 91% |  |  | 87% |  |  |

Table 6. New anchor related to HM4.0 anchor with JCTVC-Gxxx (encoder bug fix)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  | 109% |  |  |
| Dec Time[%] | 111% |  |  | 116% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | -0.4% | -0.2% | -0.3% | -0.4% |
| Class C | -0.1% | -0.3% | -0.2% | -0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.3% | 0.1% | -0.1% | -0.4% | 0.0% |
| Class E | 0.0% | 0.2% | -0.2% | -0.1% | -0.1% | 0.3% |
| Class F |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% | 0.0% |
|  | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% | -0.1% |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 106% |  |  | 111% |  |  |

Table 5. Test results for JCTVC-G0397 plus removal of zero MVP cap (JCTVC-542 simplification 3)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | -0.1% | 0.2% | 0.0% | 0.0% |
| Class B | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% |
| Class C | 0.1% | 0.2% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class D | 0.1% | 0.4% | 0.2% | 0.1% | 0.3% | 0.3% |
| Class E | 0.0% | -0.8% | -0.8% | 0.0% | -0.4% | -0.7% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

## WD text

Text for Joint of G397 and G542, i.e. disabling all 2nd pruning process and removing the limiter ZeroCnt.

8.4.2.1.4

When no candidate in mergeCandList has the same list0 motion vector and reference index as l0Cand and the same list1 motion vector and reference index as l1Cand, the candidate combCandk with k equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = combCandk ) and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are dervied as follows and numMergeCand is incremented by 1.

8.4.2.1.5

When no candidate in mergeCandList has a list0 motion vector equal to listX motion vector of origCand, a list1 motion vector equal to the listX motion vector of origCand multiplied by 1 and list0 and list1 reference indices equal to the listX reference index of origCand, the candidate nscaleCandl with l equal to ( numMergeCand − numInputMergeCand )is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = nscaleCandl ) and the reference indices, the prediction list utilization flags and the motion vectors of nscaleCandl are dervied as follows and numMergeCand is incremented by 1.

8.4.2.1.6

When numMergeCand is less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables zeroIdx and zeroCnt are is set to 0, the variable zeroStop is set to false and the following steps are repeated until zeroStop is equal to true.

no candidate in mergeCandList has a list0 motion vector equal to (0,0) and reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0),

no candidate in mergeCandList has a list0 motion vector equal to (0,0), a list0 reference index equal to zeroIdx, a list1 motion vector equal to (0,0) and a list1 reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0)

2.      The variables zeroCnt and zeroIdx are is modifed as follows.

If zeroIdx is equal to numRefIdx or zeroCnt is equal to 2, zeroIdx is set to 0.

Otherwise (zeroIdx is less than and zeroCnt is less than 2), zeroIdx and zeroCnt are is incremented by 1.

When numMergeCand is equal to maxNumMergeCand or zeroIdx is equal to or greater than numRefIdx, zeroStop is set to true.