| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1007 |
| --- | --- |

| Title: | Cross-check of JCTVC-G1003 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | T. Davies 14 Waterside Drive Langley, SL3 6EZ, UK | Email: | thdavies@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contribution is a cross-check of JCTVC-G1003 on integrated proposals for Transform Skip Mode.

# Introduction

Contribution JCTVC-G1003 [1] integrates tools from 3 proposals: JCTVC-G575 [2], JCTVC-G577 [3] and JCTVC-G663 [4]. The combined proposal: removes transform coefficient scaling in the case when a transform is skipped, using a QP offset instead; improves coefficient coding for the new block modes; and restricts TSM to 4x4 blocks.

# Simulation results

The summary simulation results are presented below and the spreadsheets are contained in this package. The simulations were performed on a homogenous Linux cluster.

The results matched those provided by the proponent. Simulation encoder times were slightly higher (around 5%) than reported by the proponent.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.2% | 0.1% | 0.2% | 0.2% |
| Class B | -0.1% | 0.2% | 0.1% | -0.1% | 0.4% | 0.6% |
| Class C | -0.3% | -0.2% | 0.1% | -0.3% | 0.2% | 0.2% |
| Class D | -0.5% | 0.2% | 0.1% | -0.5% | 0.5% | 0.4% |
| Class E |  |  |  |  |  |  |
| Class F | -1.9% | -1.1% | -1.0% | -1.3% | -0.5% | -0.5% |
| Overall | -0.5% | -0.2% | -0.2% | -0.4% | 0.2% | 0.2% |
|  | -0.5% | -0.2% | -0.2% | -0.4% | 0.1% | 0.2% |
| Enc Time[%] | 110% |  |  | 107% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.5% | 0.7% |
| Class C | -0.5% | 0.0% | 0.1% | -0.5% | 0.6% | 0.3% |
| Class D | -0.5% | 0.8% | 0.5% | -0.7% | 1.4% | 0.9% |
| Class E | 0.1% | 0.0% | 0.7% | -0.1% | -0.2% | 0.3% |
| Class F | -4.3% | -2.5% | -1.8% | -2.8% | -0.8% | -0.9% |
| Overall | -1.0% | -0.3% | -0.1% | -0.8% | 0.3% | 0.3% |
|  | -1.0% | -0.3% | -0.1% | -0.8% | 0.2% | 0.2% |
| Enc Time[%] | 108% |  |  | 119% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.5% | 0.3% |
| Class C | -0.6% | -0.1% | -0.1% | -0.7% | 0.3% | 0.4% |
| Class D | -0.8% | 0.5% | 0.4% | -1.0% | 0.9% | 0.8% |
| Class E | 0.0% | 0.3% | 0.3% | 0.0% | -0.2% | -0.1% |
| Class F | -4.6% | -2.6% | -2.5% | -3.2% | -1.0% | -0.6% |
| Overall | -1.2% | -0.4% | -0.4% | -1.0% | 0.1% | 0.2% |
|  | -1.2% | -0.5% | -0.5% | -1.0% | 0.1% | 0.1% |
| Enc Time[%] | 120% |  |  | 125% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |

# Review of software and Working Draft text

The software was briefly reviewed but not studied in depth. It appeared to be as described. The Working Draft text was also briefly reviewed and corresponds to the tool description.

# References

[1]	Marta Mrak, Andrea Gabriellini, David Flynn, “Integration of proposals on transform skipping”, JCTVC-G1003, November 2011

[2]	Marta Mrak, Andrea Gabriellini, David Flynn, "Transform skip mode," JCTVC-G575, November 2011

[3]	Glenn Van Wallendael, Jan De Cock, Rik Van de Walle, Marta Mrak, "Transform skipping dependent on block parameters," JCTVC-G577, November 2011

[4]	R. Joshi, J. Sole, X. Wang, M. Karczewicz, "AHG19: Modification to HE transform coefficient coding for transform skip mode," JCTVC-G663, November 2011