| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1008 |
| --- | --- |

| Title: | Cross-check of JCTVC-G569 Single entropy coder for HEVC with a high throughput binarization mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | T. Davies 14 Waterside Drive Langley, SL3 6EZ, UK | Email: | thdavies@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contribution is a cross-check of JCTVC-G569 on a single entropy coder for HEVC with a high throughput mode.

# Introduction

Contribution JCTVC-G569 [1] proposes to use the CABAC entropy coding engine with the CAVLC syntax for transform coefficient coding, interpreted as a binarization which is coded in CABAC bypass mode.

# Simulation results

The summary simulation results are presented below and the spreadsheets are contained in this package. The simulations were performed on a homogenous Linux cluster.

The results matched those provided by the proponent for classes A-E. Class F was not provided by the proponent.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 4.9% | 2.3% | 1.9% | -1.0% | 3.9% | 3.9% |
| Class B | 4.7% | 1.4% | 1.1% | -1.4% | 2.4% | 3.0% |
| Class C | 3.9% | 1.2% | 1.3% | -0.7% | 1.9% | 2.1% |
| Class D | 3.7% | 0.6% | 0.4% | -0.3% | 1.8% | 1.7% |
| Class E | 5.1% | 0.8% | 0.3% | -2.6% | 8.1% | 7.1% |
| Class F | 4.5% | 1.6% | 2.1% | -0.9% | 0.7% | 0.7% |
| Overall | 4.5% | 1.4% | 1.2% | -1.1% | 2.9% | 2.9% |
|  | 4.4% | 1.3% | 1.2% | -1.1% | 2.8% | 2.9% |
| Enc Time[%] | 119% |  |  | 118% |  |  |
| Dec Time[%] | 104% |  |  | 108% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 7.3% | -1.5% | -0.6% | -1.9% | 5.7% | 6.2% |
| Class B | 4.7% | 0.4% | 0.6% | -3.1% | 4.0% | 5.5% |
| Class C | 3.8% | 0.7% | 1.1% | -1.7% | 2.7% | 3.7% |
| Class D | 3.2% | -0.4% | -0.1% | -1.4% | 3.6% | 4.0% |
| Class E |  |  |  |  |  |  |
| Class F | 5.0% | 2.2% | 2.3% | -2.4% | -0.1% | 0.0% |
| Overall | 4.8% | 0.3% | 0.7% | -2.1% | 3.2% | 4.0% |
|  | 4.7% | 0.3% | 0.7% | -2.2% | 3.2% | 3.9% |
| Enc Time[%] | 111% |  |  | 102% |  |  |
| Dec Time[%] | 104% |  |  | 106% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 4.7% | 0.8% | 0.5% | -2.8% | 6.9% | 11.2% |
| Class C | 3.5% | 0.4% | 1.3% | -1.9% | 5.1% | 6.7% |
| Class D | 3.1% | 0.5% | 1.1% | -1.6% | 9.0% | 9.6% |
| Class E | 1.9% | 0.7% | 0.2% | -5.1% | 8.0% | 9.2% |
| Class F | 5.0% | 2.3% | 3.0% | -4.3% | -0.5% | -0.2% |
| Overall | 3.8% | 0.9% | 1.3% | -3.0% | 5.6% | 7.4% |
|  | 3.7% | 0.9% | 1.2% | -3.0% | 5.5% | 7.1% |
| Enc Time[%] | 111% |  |  | 105% |  |  |
| Dec Time[%] | 107% |  |  | 105% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 4.4% | 0.2% | 0.4% | -3.5% | 7.6% | 11.8% |
| Class C | 3.5% | 0.3% | 1.3% | -2.1% | 5.4% | 7.3% |
| Class D | 3.1% | -0.2% | 0.8% | -1.7% | 8.4% | 9.6% |
| Class E | 1.7% | 0.4% | -0.3% | -5.5% | 9.2% | 9.9% |
| Class F | 5.2% | 2.2% | 2.5% | -4.4% | -0.7% | 0.1% |
| Overall | 3.7% | 0.6% | 1.0% | -3.3% | 5.9% | 7.8% |
|  | 3.6% | 0.6% | 0.9% | -3.4% | 5.9% | 7.6% |
| Enc Time[%] | 113% |  |  | 109% |  |  |
| Dec Time[%] | 107% |  |  | 105% |  |  |

# Review of software

The software was briefly reviewed but not studied in depth, however it seems quite simple: the coefficient coding function from CAVLC is copied to TEncSbac, and the calls to binary coding functions use CABAC encodeBinEP instead. CAVLC RDOQ is used instead of CABAC RDOQ in this mode.

# References

[1] JCTVC-G569 “Single entropy coder for HEVC with a high throughput binarization mode”, J. Lainema, K. Ugur, and A. Hallapuro (Nokia), JCTVC 7th Meeting: Geneva, CH, 21-30 November, 2011