| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1009 WG11 Number: m22765 |
| --- | --- |

| Title: | A joint contribution on the coding tools of residual prediction for intra chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu, Yu Han, Lidong Xu, Wenhao Zhang, Hong Jiang Intel Corp. 2200 Mission College Blvd. Santa Clara CA USA 95054 Kei Kawamura, Tomonobu Yoshino, Haruhisa Kato, Sei Naito KDDI Corp. (KDDI R&D Laboratories) 2-1-15, Ohara, Fujimino-shi, Saitama, Japan | Tel: Email: Tel: Email: | +1-408-765-9494 yi-jen.chiu@intel.com +81-492787411 ki-kawamura@kddi.com |
| Source: | Intel Corp., KDDI Corp. (KDDI R&D Laboratories) |  |  |

_____________________________

# Abstract

This joint contribution presents the results of combining two coding tools on the techniques of residual prediction for intra chroma prediction. Namely the tools are “Cross-channel Intra chroma residual prediction” (JCTVC-G173, Intel), “Chroma intra prediction based on residual luma samples” (JCTVC-G346, KDDI). Compared to the individual tools, it is reported that, comparing to HM4.0, the combination of the tools is showing near-additive results, with BD rate equal to -0.4%, -4.8% and -6.9% for respectively Y, U and V components with average run time 101% for encoding and 101% for decoding for AIHE configuration, and -0.4%, -4.0% and -6.8% with average run time 100% for encoding and 102% for decoding for AILC configuration.

# Detailed results

## Results for combination results

The below tables report the summary data of the combined G173 & G346 which demonstrates an average BD bitrate reductions of -0.4% on Y, -4.8% on U, -6.9% on V for AI_HE case and an average -0.4% on Y, -4.0% on U, -6.8% on V for AI_LC case.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| JCTVC-G173 | -0.4% | -1.1% | -2.8% | -0.3% | -0.8% | -3.3% |
| JCTVC-G346 | -0.1% | -3.9% | -4.5% | -0.1% | -3.3% | -3.8% |
| combined | -0.4% | -4.8% | -6.9% | -0.4% | -4.0% | -6.8% |

Test results of combination of JCTVC-G173 and G346

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -12.2% | -17.3% | -0.3% | -11.1% | -16.4% |
| Class B | -0.1% | -2.9% | -5.3% | -0.1% | -1.8% | -4.9% |
| Class C | -0.6% | -3.8% | -5.0% | -0.5% | -3.0% | -5.5% |
| Class D | -0.6% | -3.7% | -4.7% | -0.5% | -2.8% | -5.1% |
| Class E | -0.3% | -2.1% | -3.7% | -0.2% | -1.7% | -3.9% |
| Class F | -0.8% | -4.1% | -5.0% | -0.7% | -3.5% | -4.6% |
| Overall A-F | -0.4% | -4.8% | -6.9% | -0.4% | -4.0% | -6.8% |
|  | -0.4% | -4.7% | -6.8% | -0.4% | -3.9% | -6.7% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

## Individual results

### Cross-channel intra chroma residual prediction (JCTVC-G173)

The performance of cross-channel intra chroma residual prediction on HM 4.0 reference software

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3% | -0.2% | -2.7% | -0.2% | 0.2% | -3.0% |
| Class B | -0.1% | -0.3% | -3.2% | -0.1% | 0.4% | -3.4% |
| Class C | -0.5% | -1.8% | -3.0% | -0.5% | -1.5% | -3.9% |
| Class D | -0.5% | -1.9% | -3.0% | -0.5% | -1.6% | -4.0% |
| Class E | -0.3% | -1.1% | -2.8% | -0.2% | -1.0% | -3.4% |
| Class F | -0.5% | -1.5% | -2.0% | -0.4% | -1.5% | -2.4% |
| Overall A-E | -0.3% | -1.0% | -3.0% | -0.3% | -0.6% | -3.5% |
| Overall A-F | -0.4% | -1.1% | -2.8% | -0.3% | -0.8% | -3.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

### Chroma intra prediction based on residual samplers (JCTVC-G346)

# Conclusion

This contribution reports the results of combining two coding tools on the techniques of residual prediction for intra chroma prediction. The data shows the near-additive results, with BD rate improvements of 0.4%/4.8%/6.9% (Y/U/V) for AIHE and 0.4%/4.0%/6.8% (Y/U/V) for AILC.

The proponents represented in this document recommend the techniques in this contribution to be studied in the Core Experiment to be further verified by broader community of experts.

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).