| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Switzerland, 21-30 November, 2011 | Document: JCTVC-G1012 WG11 Number: m22770 |
| --- | --- |

| Title: | Non-CE8: Supplementary results to JCTVC-G813 regarding ALF using one filter type (Luma only) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Hsan Guermazi | Email: | wlai@sta.samsung.com hsan@ebriskvideo.com |
| Source: | Samsung Electronics Co., Ltd. eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This contribution presents supplementary results to JCTVC-G813 regarding ALF using one filter type. During the presentation of JCTVC-G813, which does not have LDP results, it was recommended that the results of LDP to be recorded in this meeting. Note that the results provided in this meeting were obtained with one filter type (star-5x5 and cross-11x5) only for luma, chroma was not changed.

# Introduction

In the 6th JCT-VC meeting, ALF using two shapes, star(snowflake)-5x5 and cross-11x5 [1] as depicted in Figure 1, was adopted for both luma and chroma filtering.

Figure 1 Star-5×5 with 9 coefficients, and cross-11x5 with 8 coefficients [1]

In the 7th JCT-VC meeting, JCTVC-G813 [2] presented results by only using one of the filter types in Figure 1. It was noted that JCTVC-G813 [2] does not contain LDP results. And it was further recommended if LDP results are available among participants; it would be appreciated to be reported as formal input document to the JCT-VC meeting. Thus, this document present results for using one type of filters on the luma channel only for all four configurations: AI, RA, LD, and LDP.

# ALF using one filter type (Luma only)

To only use one filter type for luma, the following changes are made in the source code:

In "TEncAdaptiveLoopFilter.cpp", line 2465

2 shapes
for (Int filter_shape = 0; filter_shape < 2 ;filter_shape ++)

Star_Only
for (Int filter_shape = 0; filter_shape < 1 ;filter_shape ++)
Cross_Only
for (Int filter_shape = 1; filter_shape < 2 ;filter_shape ++)

Again please note we didn’t change the chroma part, thus only luma results are relevant here.

# Simulation Results

As compared to HM4.0 as anchor with 2 filter shapes, the performance is provided below.

Luma only, AI/RA/LD/LDP
Star_Only:  0.2/0.4/0.6/0.3
Cross_Only: 0.1/0.3/0.4/0.7

Details can be found in the Excel sheets.

# References

- “CE8 Subtest 4: ALF using vertical-size 5 filters with up to 9 coefficients”, JCTVC-F303, Torino, Italy, Jul. 2011

- “ALF with single filter type”, JCTVC-G813, Geneva, Switzerland, Nov. 2011

# Patent rights declaration(s)

Samsung and eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).