| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1015 |
| --- | --- |

| Title: | A combined proposal from JCTVC-G366, JCTVC-G657, and JCTVC-G768 on context reduction of significance map coding with CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Jun Xu 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 {cheung.auyeung, jun.xu}@am.sony.com |
|  | Gergely Korodi, Jinwen Zan, Da-ke He 295 Phillip Street Waterloo, Ontario, Canada N2L 3W8 | Tel: Email: | +1 519 888 7465 {gkorodi, jzan, dhe}@rim.com |
|  | Yinji Piao, Elena Alshina, Junghye Min, JeongHoon Park 416. Maetan 3-dong,Yeongtong-gu,Suwon-si,Gyeonggi-do Korea | Tel: Email: | +82-10-6296-4261 {yj1003.piao, elena_a.alshina , jh643.min, jeonghoon } @samsung.com |
| Source: | Sony Electronics Inc., Research In Motion Limited, Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution combines the contributions on significant_coeff_flag context reduction in JCTVC-G366 from Sony, JCTVC-G657 from RIM, and JCTVC-G768 from Samsung to provide 40 context reductions. The combined proposal resulted in an average BD-Rate of 0.0% for all test cases except I_HE at QP=12,17, 22, 27 with BD-Rate of 0.1% as specified in the testing conditions in CE11 for context modeling/selection for transform coefficient related syntax elements. This contribution also demonstrated that the proposed significance map context reduction can work with level context reduction such as the CE11 JCTVC-G121 with 0.0% BD-rate difference except 0.1% additional BD-Rate for I_HE at Low QP and RDOQ-off.

# Introduction

The JCTVC-G366[1], JCTVC-G657[2], and JCTVC-G768[3] proposed significant_coeff_flag context reduction with CABAC. There are many commonality in the approaches used in these three proposals. In light of the commonalities, the proponents worked together to identify the combination of the three proposals [1,2,3] with minimal impact on BDR that resulted in more total context reductions than that their individual proposal achieved.

# Joint proposal

In particular, this contribution proposes the following combination of context reduction for a total of 40 context reductions:

- 15 context reduction of 4x4 blocks from JCTC-G657 [2].

- 10 context reduction of 8x8 blocks from JCTC-G366 [1].

- 15 context reduction of 16x16/32x32 blocks from JCTVC-G768 [3].

# WD text

The corresponding technical and WD descriptions are in [2, 1, 3] respectively.

# Simulation results

The context reductions were integrated into HM4.0 with and without the level context reductions from CE11 JCTVC-G121/F132 [4, 5].

As shown in , the combined proposal resulted in an average BD-Rate of 0.0% for all test cases except the 0.1% BD-Rate for I_HE at QP = 12, 17, 22, 27.

As shown in , when the proposed 40 significance map context reductions were combined with the 36 level contexts reductions from CE11 JCTVC-G121/F132 [4,5] for a total of 76 context reductions, the BD-Rate for I_HE become 0.1%, 0.2%, 0.1% for normal QP, low QP and RDOQ-off respectively. The impact on BDR for other test cases was less. In particular, the BD-rate for RA_HE and LB_HE became 0.0%, 0.0%, 0.0% for normal QP, low QP and RDOQ-off respectively. The BD-rate for LP_HE became 0.1%, 0.0%, 0.0% for normal QP, low QP and RDOQ-off respectively.

TI kindly agreed to provide the crosscheck for this joint proposal.

|  | All Intra HE |  |  | All Intra (Low QP) |  |  | All Intra (RDOQ off) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.3% | 0.4% | 0.1% | 0.1% | 0.1% | 0.1% | 0.3% | 0.2% |
| Class B | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class E | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% | 0.4% | 0.3% |
| Class F |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access (Low QP) |  |  | Random Access(RDOQ off) |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.4% | 0.6% | 0.0% | 0.2% | 0.3% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.3% | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% | 0.0% | -0.3% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B (Low QP) |  |  | Low delay B (RDOQ off) |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | -0.3% | -0.3% |
| Class C | 0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.2% |
| Class D | 0.0% | 0.4% | -0.3% | 0.0% | -0.1% | -0.2% | 0.0% | -1.1% | -0.3% |
| Class E | 0.0% | -0.4% | 0.1% | 0.0% | -0.3% | -0.1% | 0.0% | -0.3% | 0.3% |
| Class F |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | -0.4% | -0.2% |
|  | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | -0.2% | 0.0% | -0.4% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P (Low QP) |  |  | Low delay P (RDOQ off) |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.3% | 0.0% | 0.0% | -0.1% | 0.0% | -0.3% | -0.4% |
| Class C | 0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.1% | -0.3% | -0.6% | 0.0% | -0.1% | -0.3% | 0.0% | -1.0% | -0.6% |
| Class E | 0.0% | -0.1% | -0.8% | 0.0% | -0.3% | -0.3% | 0.0% | -1.0% | -0.5% |
| Class F |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% | 0.0% | -0.6% | -0.4% |
|  | 0.0% | -0.1% | -0.4% | 0.0% | -0.1% | -0.2% | 0.0% | -0.5% | -0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |

Table 1: The 40 context reduction proposed in this contribution resulted in average BD-Rate of 0.0% for all test cases in the common test conditions.

|  | All Intra HE |  |  | All Intra (Low QP) |  |  | All Intra (RDOQ off) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.3% | 0.4% | 0.2% | 0.2% | 0.2% | 0.1% | 0.3% | 0.3% |
| Class B | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class E | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.3% | 0.1% | 0.4% | 0.3% |
| Class F |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |
|  | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access (Low QP) |  |  | Random Access(RDOQ off) |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.4% | 0.3% | 0.0% | 0.2% | 0.3% | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.1% | 0.0% | -0.2% | 0.0% | 0.0% | -0.3% | 0.1% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B (Low QP) |  |  | Low delay B (RDOQ off) |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.1% | 0.1% | -0.1% | 0.0% | -0.1% | -0.4% |
| Class C | 0.1% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | -0.2% |
| Class D | 0.0% | -0.1% | -0.5% | 0.0% | -0.1% | -0.1% | 0.0% | -0.7% | -0.7% |
| Class E | 0.1% | -0.5% | 0.1% | 0.0% | -0.5% | -0.1% | -0.1% | -1.0% | -0.1% |
| Class F |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.3% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | -0.4% | -0.4% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.1% | 0.0% | -0.4% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P (Low QP) |  |  | Low delay P (RDOQ off) |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.3% | 0.1% | 0.1% | -0.1% | 0.0% | -0.3% | -0.5% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.2% |
| Class D | 0.1% | -0.1% | -0.6% | 0.0% | -0.1% | -0.2% | 0.0% | -0.8% | -1.1% |
| Class E | 0.2% | -0.1% | -0.1% | 0.1% | 0.0% | 0.3% | 0.0% | -1.3% | 0.4% |
| Class F |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | -0.6% | -0.4% |
|  | 0.1% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |

Table 2: BD-Rate of 76 context reduction: the proposed 40 significance map context reductions combined with 36 level context reductions from JCTVC-G121.

# Summary

This contribution reduced 40 contexts for the coding of significance map with CABAC and resulted in the average luminance BD-Rate in Table 3:

|  | I_HE | RA_HE | LB_HE | LP_HE |
| --- | --- | --- | --- | --- |
| QP(22,27,32,37) | 0.0 | 0.0 | 0.0 | 0.0 |
| LQP(12,17,22,27) | 0.1 | 0.0 | 0.0 | 0.0 |
| RDOQ-OFF | 0.0 | 0.0 | 0.0 | 0.0 |

Table 3: Average BD-Rate of the proposed 40 significance map context reduction.

When this contribution is combined with the 36 level context reductions from CE11 [6] on JCTVC-G121/F132 [4, 5] for a total of 76 context reductions, the BD-Rate became the results in Table 4.

|  | I_HE | RA_HE | LB_HE | LP_HE |
| --- | --- | --- | --- | --- |
| QP(22,27,32,37) | 0.1 | 0.0 | 0.0 | 0.1 |
| LQP(12,17,22,27) | 0.2 | 0.0 | 0.0 | 0.0 |
| RDOQ-OFF | 0.1 | 0.0 | 0.0 | 0.0 |

Table 4: Average BD-Rate of proposed significance map context reduction combined with level context reduction from JCTVC-G121/F132 for a total of 76 context reductions.

As reference, the BD-Rate results from JCTVC-G121 are shown in Table 5.

|  | I_HE | RA_HE | LD_HE |
| --- | --- | --- | --- |
| QP(22,27,32,37) | 0.1 | 0.0 | 0.0 |
| LQP(12,17,22,27) | 0.1 | 0.0 | 0.0 |
| RDOQ-OFF | 0.0 | 0.0 | 0.0 |

Table 5: Average BD-Rate of JCTVC-G121 for 36 level context reduction as benchmark.

By comparing the results in Table 4 and Table 5, this contribution demonstrated that the proposed significance map context reduction can work with level context reduction in JCTVC-G121 with at the most 0.1% BDR in three out of 12 test cases.

# References

- Cheung Auyeung and Jun Xu, “Description Context reduction of significance map coding with CABAC”, JCTVC-G366, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Gergely Korodi, Jinwen Zan, and Da-ke He, “Encoding and decoding significant coefficient flags for small Transform Units using partition sets”, JCTVC-G657, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Yinji Piao, Junghye Min, and JeongHoon Park, “Reduced contexts for significance map coding of large transform in CABAC”, JCTVC-G768, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Vivienne Sze, “Reduction in contexts used for significant_coeff_flag and coefficient level”, JCTVC-F132, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Vivienne Sze, “CE11: Reduction in contexts used for coefficient level”, JCTVC-G121, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Vivienne Sze, Tung Nguyen, Jianle Chen, Joel Sole, Krit Panusopone, “Description of Core Experiment (CE11): Coefficient scanning and coding”, JCTVC-F911, 6th Meeting: Torino, IT, 14-22 July, 2011.

- B. Bross, W-J Han, J-R Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 6th Meeting, Torino, July 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Proposed Working Draft text

This section describes the change request of the current proposal with regard to the Working Draft 4 of HEVC [7], where the new changes are highlighted in yellow:.

Table 9-6 – Values of variable m and n for significant_coeff_flag ctxIdx (A)

| Initialisation variables | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| m | -11 | -10 | -11 | -11 | -9 | -14 | -13 | -19 | -10 | 0 | 0 | 0 | 0 | 0 | 0 | -8 |
| n | 103 | 94 | 93 | 92 | 87 | 89 | 91 | 94 | 89 | 64 | 64 | 64 | 64 | 64 | 64 | 90 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| m | -4 | 0 | -6 | -8 | -4 | 1 | -4 | -8 | -3 | 3 | -3 | -17 | -5 | -1 | -8 | -15 |
| n | 72 | 58 | 54 | 80 | 70 | 57 | 54 | 72 | 63 | 54 | 55 | 79 | 59 | 56 | 90 | 119 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| m | -4 | -2 | -15 | -4 | -4 | -10 | 0 | 0 | 0 | 0 | 0 | 0 | 34 | 8 | -11 | -14 |
| n | 49 | 72 | 112 | 28 | 72 | 96 | 64 | 64 | 64 | 64 | 64 | 64 | 22 | 52 | 83 | 83 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| m | -59 | 60 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 14 | 6 | -30 | -35 | -7 |
| n | 149 | -35 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 48 | 50 | 103 | 74 | 68 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| m | 14 | 8 | -25 | -37 | -21 | 9 | 7 | -37 | 2 | -36 | 14 | 15 | -9 | 7 | 13 | 0 |
| n | 36 | 43 | 63 | 104 | 83 | 43 | 22 | 80 | 24 | 103 | 48 | 59 | 38 | 49 | 47 | 64 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 |  |  |  |  |  |  |  |  |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |  |  |  |  |  |  |  |  |

Table 9-7 – Values of variable m and n for significant_coeff_flag ctxIdx (B)

| Initialisation variables | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| m | -2 | -5 | -8 | -8 | -6 | -19 | -10 | -20 | -4 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| n | 74 | 76 | 77 | 75 | 72 | 89 | 73 | 88 | 69 | 64 | 64 | 64 | 64 | 64 | 64 | 61 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| m | -1 | -2 | -15 | -9 | -5 | -1 | -8 | -16 | -10 | -1 | -4 | -24 | -20 | -10 | 2 | 0 |
| n | 56 | 51 | 67 | 66 | 58 | 49 | 55 | 72 | 62 | 49 | 52 | 88 | 84 | 70 | 61 | 78 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| m | 2 | 0 | -9 | -4 | 1 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 41 | 13 | 16 | -24 |
| n | 31 | 65 | 93 | 20 | 57 | 72 | 64 | 64 | 64 | 64 | 64 | 64 | 3 | 34 | 33 | 84 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| m | 4 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 18 | 22 | 13 | -32 | 3 |
| n | 43 | 52 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 31 | 11 | 10 | 58 | 36 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| m | 25 | 21 | 37 | -30 | 36 | 27 | 11 | -56 | 20 | 16 | 18 | 28 | 0 | 5 | 20 | 0 |
| n | -6 | -3 | -34 | 81 | -36 | -2 | 36 | 143 | 3 | 19 | 31 | 29 | 0 | 49 | 32 | 64 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 |  |  |  |  |  |  |  |  |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |  |  |  |  |  |  |  |  |

Table 9-8 – Values of variable m and n for significant_coeff_flag ctxIdx (C)

| Initialisation variables | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| m | -2 | -5 | -8 | -8 | -6 | -19 | -10 | -20 | -4 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| n | 74 | 76 | 77 | 75 | 72 | 89 | 73 | 88 | 69 | 64 | 64 | 64 | 64 | 64 | 64 | 61 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| m | -1 | -2 | -15 | -9 | -5 | -1 | -8 | -16 | -10 | -1 | -4 | -24 | -20 | -10 | 2 | 0 |
| n | 56 | 51 | 67 | 66 | 58 | 49 | 55 | 72 | 62 | 49 | 52 | 88 | 84 | 70 | 61 | 78 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| m | 2 | 0 | -9 | -4 | 1 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 41 | 13 | 16 | -24 |
| n | 31 | 65 | 93 | 20 | 57 | 72 | 64 | 64 | 64 | 64 | 64 | 64 | 3 | 34 | 33 | 84 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| m | 4 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 18 | 22 | 13 | -32 | 3 |
| n | 43 | 52 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 31 | 11 | 10 | 58 | 36 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| m | 25 | 21 | 37 | -30 | 36 | 27 | 11 | -56 | 20 | 16 | 18 | 28 | 0 | 5 | 20 | 0 |
| n | -6 | -3 | -34 | 81 | -36 | -2 | 36 | 143 | 3 | 19 | 31 | 29 | 0 | 49 | 32 | 64 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 |  |  |  |  |  |  |  |  |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |  |  |  |  |  |  |  |  |

## Proposed changes to “9.3.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag”

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size log2TrafoSize.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the color component index cIdx, the transform block size and previously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- -1

- If log2TrafoSize is less than or equal to 2, and cIdx is equal to 0, sigCtx is derived as follows.

sigCtx  =  CTX_IND_MAP_4x4[(yC << 2) + xC ] 					(9-2)

where CTX_IND_MAP_4x4 is defined in Table A-1.

- If log2TrafoSize is less than or equal to 2, and cIdx is equal to 1, sigCtx is derived as follows.

sigCtx  =  CTX_IND_MAP_4x4[15 + (yC << 2) + xC ] 				(9-3)

where CTX_IND_MAP_4x4 is defined in Table A-1.

- Otherwise if log2TrafoSize equals to 3, sigCtx is derived as follows.

sigCtxTemp  =  ( ( yC  >>  1 )  <<  2 )  +  ( xC  >>  1 )

sigCtx  =  ( (xC+yC) == 0 )  ?  30  :  15  +  CTX_IND_MAP_8x8[ sigCtxTemp ] 	(9-4)

where CTX_IND_MAP_8x8 is defined in Table A-2.

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  31

- Otherwise if xC + yC is less than 1 << (uiLog2BlkSize-2) and cIdx is equal to 0 , sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-4)
temp=(temp+1)>>1;

sigCtx  =  32  +  Min( 2, temp )

- Otherwise (xC  +  yC is greater than 1 << (uiLog2BlkSize-2)-1 or cIdx is greater than 0), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable ctxCnt is initialized as follows.

ctxCnt =  0		(9-5)

- When xC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

ctxCnt = ctxCnt +  significant_coeff_flag[ xC + 1 ][ yC ]			(9-5)

- When xC and yC are less than ( 1 << log2TrafoSize ) − 1, the following applies.

ctxCnt = ctxCnt +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]			(9-6)

- When xC is less than ( 1 << log2TrafoSize ) − 2, the following applies.

ctxCnt = ctxCnt +  significant_coeff_flag[ xC + 2 ][ yC ]			(9-7)

- When yC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

ctxCnt = ctxCnt +  significant_coeff_flag[ xC ][ yC + 1 ]			(9-8)

- When yC is less than ( 1 << log2TrafoSize ) − 2 and ctxCnt is less than 4, the following applies.

ctxCnt = ctxCnt +  significant_coeff_flag[ xC ][ yC + 2 ] 			(9-9)

- ctxCnt =(ctxCnt + 1) >> 1

- If cIdx is equal to 0

sigCtx = 35 + ctxCnt

- Otherwise (cIdx is greater than 0),

sigCtx = 32 + ctxCnt

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx								(9-10)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  44  +  sigCtx							(9-11)

Table A-1 - Context assignment for 4x4 significance maps

|  | Significant coefficient index |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| CTX_IND_MAP_4x4 | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| CTX_IND_MAP_4x4 | 0 | 1 | 2 | 4 | 1 | 1 | 2 | 4 | 3 | 3 | 5 | 5 | 4 | 4 | 5 |

Table A-2 - Context assignment for 8x8 significance maps

|  | Significant coefficient index |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| CTX_IND_MAP_8x8 | 0 | 1 | 2 | 3 | 4 | 5 | 10 | 3 | 8 | 10 | 10 | 11 | 12 | 12 | 11 | 11 |