| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1017 |
| --- | --- |

| Title: | BoG Report on Intra Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vivienne Sze (chair) Rik Allen | Email: | sze@ti.com |
| Source: | Break out Group |  |  |

_____________________________

The mandate of this break out group is to look into the details of G243, analyze “evaluate text, implementation complexity, destabilization, and amount of gain”.  Take note of any non-CE proposals which may be related to this contribution.  Only contributions relating exclusively to entropy coding of intra mode were discussed; contributions which affected pixel values (e.g. G358 and part of G119) were not discussed. Meeting was held at 5pm-7pm on November 25, 2011. Section 2 and 3 contain meeting notes.

# Summary

| Tools | Changes/Complexity | Coding Gains* | Alternative Tools |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | G153 -0.2Y | G359 -0.3Y | G423 0.2U/0.3V |
| +2 MPRM | -Compute four additional MPRM via LUT or operations & comparison for angle conversion -Additional 3(?) comparisons to select 2 MPRM -Additional bin (mprm_idx) | -0.3(Y) | X** | X*** |  |
| Remainder coding with mode ranking | -variable registers that need to be updated for rank -rank mapped to exp-golomb rather than FLC -bypass coding of mprm_idx and remainder | -0.2 (Y) | X | X |  |
| Chroma mode coding | - replace ver/hor/DC with luma+/-1 and perp (computed with LUT or operations & comparison for angle conversion) | -0.3 (U), -0.4 (V) |  |  | X |
| additional 19th mode Intra_4x4 | Additional mode | 0.0? | X | X |  |

*estimated from Appendix B

**treats first remaining mode as MPM; additional bin

*** no additional MPM, but requires angle conversion

X – denotes that proposal provides an alternative to tool in G243

# G243 – Complexity Assessment

- Addition of 2 MPRMs

  - Two additional LUT 10bitsx19, 12bitsx35 to get 2 MPM+/- 1

    - Select based on PU size; x2 for each original MPM

Table 1. Two closest adjacent modes of MPM for PU size equal to 4x4

| Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPM_Minus_1 | 1 | 12 | 17 | 1 | 15 | 11 | 13 | 14 | 16 | 18 | 7 | 4 | 5 | 1 | 6 | 8 | 2 | 9 | 10 |
| MPM_Plus_1 | 2 | 13 | 16 | 2 | 11 | 12 | 14 | 10 | 15 | 17 | 18 | 5 | 1 | 6 | 7 | 4 | 8 | 2 | 9 |

Table 2. Two closest adjacent modes of MPM for PU size equal to 8x8, 16x16, or 32x32

| Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPM_Minus_1 | 1 | 22 | 31 | 1 | 27 | 20 | 24 | 26 | 29 | 33 | 7 | 19 | 21 | 23 | 25 | 28 | 30 | 32 | 34 |
| MPM_Plus_1 | 2 | 23 | 30 | 2 | 19 | 21 | 25 | 10 | 28 | 32 | 34 | 20 | 22 | 24 | 26 | 27 | 29 | 31 | 33 |
| Mode | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |  |  |
| MPM_Minus_1 | 4 | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 15 | 8 | 16 | 2 | 17 | 9 | 18 | 10 |  |  |  |
| MPM_Plus_1 | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 7 | 4 | 15 | 8 | 16 | 2 | 17 | 9 | 18 |  |  |  |

Alternatively, can reuse the mode to angle LUT and then add or subtract value.  Need to check if resulting value exceeds range; alternatively extend table by 4 entries to cover edge cases. Only need to do one look up since mpm_idx will indicate which MPM to use. [Last statement incorrect. After BoG meeting, confirmed with proponent that two look ups need to be performed to generate all candidates.]

  - ## Comparisons after LUT (3 serial comparisons)

  - 4 additional contexts in code, but does not seem to be used (confirmed)

- Coding of remaining mode is based on mode ranking, applied to the complete set of modes.

  - Additional registers of 35x6bits & 19x5bits  that are updated for rank

  - Rank mapped to unified binarization/VLC tables rather than FLC -  Exp-Golomb Codes (Note: no need to unify with CAVLC)

  - CABAC coding is simplified by using bypass coding for MPRM index and remaining mode bins (save one context model compared to HM-4.0). MPM index is coded with context.

- Chroma coding

  - Replace vertical, horizontal, DC with luma mode+/-1 (reuse LUT or calculation from MPRM used in luma) and perpendicular luma (10bits x 19, 12bits x 35 LUT for perpendicular, or compute from angle, but check range)

- Restore (additional) mode for Intra_4x4 (originally removed due to FLC of remainder)

Proponents of G243 also tested with G280; gain 1%, 0.8%, 1%

# Non-CE6 with alternative solutions to G243 (discussed but not presented)

G153 – Alternative to remaining mode + additional MPM.  Gives -0.2% gain; comment from floor – additional bin represent planar/vertical/horizontal (first remaining mode) which can be viewed as another MPM

G359 – Uses exiting MPMs in HM-4.0; meaning, computation (get index of angle, averaging index, logic to check corner cases – e.g. MPMs are not angular, large difference between angles) and binarization of remaining mode different; Gives -0.3% gain.

G423 – Alternative to chroma coding aimed for simplification by removing modes – check flag in SPS to determine whether LM is enable; if enabled, only DM and LM used; if disabled, only use DM; reported loss 0.0%, 0.2%, 0.3% (LM enabled) account for chroma during luma decision in RDO.

G767, G707, G153 – Alternative coding of mpm_idx use bypass. 0.0%

Related Intra Mode Coding proposals

G109, G144, G418 is related to G119 (changing remapping table --- “converting” mode for different PU size)

# Appendix A (Summary of proposals and coding efficiencies)

| Doc# (CE in blue box) | Summary of Changes | HE-AI |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| G243 (MTK, Qualcomm, Canon, LG, Sony) | - 4 MPMs (2 MPMs from HM-4.0 + 2 MPRMs) - Mode ranking for coding remaining modes bins (swapping & VLC) - Bypass coding for the MPRMs and remaining modes bins. - Chroma coding – replace ver, hor, DC with luma pred modes - Additional mode in 4x4 PU(+1 mode) | -0.5 | -0.4 | -0.5 |
| G203 (MTK, Qualcomm) | - 5 MPMs (2 MPMs from HM-4.0 + 3 MPRMs) - Chroma coding – replace ver, hor, DC with luma pred modes - Additional mode in 4x4 PU(+1 mode) | -0.4 | -0.4 | -0.5 |
| G869 (Sony) | - Mode ranking for coding remaining modes bins - Additional mode in 4x4 PU(+1 mode) - 3 MPM (2 MPMs from HM-4.0 + 1 MPM) | -0.3, -0.4 | -0.1, -0.1 | -0.1, -0.1 |
| G242 (Canon) | - Mode ranking for coding remaining modes bins (swapping & VLC) - Additional mode in 4x4 PU(+1 mode) - 3 MPM (2 MPMs from HM-4.0 + 1 MPM) | -0.2, -0.3 | -0.1, -0.1 | -0.1, -0.1 |
| G192 (LG) | Mode ranking for coding remaining modes bins (swapping & VLC) | -0.2 (LC) | -0.1 (LC) | -0.1 (LC) |
| G153 (I2R) | - Remove ctx from mpm_idx (no coding loss) – also in combo with G243 - For rem_intra_luma_pred_mode, additional bin (ctx coded ) + FLC (bypass coded) - Total 2 contexts entire intra mode coding - Additional mode in 4x4 PU(+1 mode) | -0.2 |  |  |
| G145 (LG) | Line buffer compression 2:1 Remove line buffer (assume planar mode if LCU boundary) | 0.0, 0.1 |  |  |
| G184 (JVC) | Change MPM position to top right, bottom left (to match AMVP) | 0.0 |  |  |
| G359 (MERL) | - Signal difference between the current mode and an estimate = average of MPMs or adjacent mode (if only one MPM) - No ranking table; smaller differences are assigned smaller codewords fixed mapping (no syntax changes) | -0.3 | 0.0 | 0.0 |
| G423 (Huawei) | Chroma mode coding (remove redundancy between DM & four luma mode based on if LM is enabled SPS flag – remove ver/hor/DC/plan); account for chroma during luma mode decision in RDO | 0.0 | 0.2 | 0.3 |
| G707 (Sharp) | Bypass coding of both mpm_idx and rem_intra_luma_pred_mode | 0.0 | 0.0 | 0.0 |
| G767(Samsung) | Bypass coding of both mpm_idx and rem_intra_luma_pred_mode | 0.0 | 0.0 | 0.0 |
| G119 (Huawei, HiSilicon) | (2) Removal of the remapping tables used in MPM prediction (if can’t use neighbor due to range, not intra, unavailable, use Planar) | 0.0 | 0.1 | 0.0 |
| G109 (MTK)* | Avoid mapping if PU sizes different; if out of range, use planar | 0.0 | 0.0 | 0.1 |
| G144 (LG)* | (1) reduce mapping table size (2) avoid mapping if PU sizes different; if out of range, use planar | 0.0, 0.0 | 0.0, 0.0 | 0.0, 0.1 |
| G418 (ETRI)* | Avoid mapping if PU sizes different; if out of range, use planar | 0.04 | 0.05 | 0.07 |
| G185 (JVC)* | (1) change order of LM and DM in list based on partition size (2) replace ver/hor/DC with neighboring and orthogonal mode seems to be the same as G243 | 0.0, 0.0 | -0.1, -0.4 | -0.1, -0.5 |

*contributions added to list after BoG meeting

## Overlapping ideas in proposals

Avoid mapping if PU sizes different; if out of range, use planar (part (2) of G119, G109, G144, G418)

- Note: part (2) of G119 also uses planar for unavailable and non-intra

Bypass coding of mpm_idx (G153, G707, G767)

Bypass coding of rem_intra_luma_pred_mode (G707, G767)

## Contributions in 5.14.2 Intra Mode Coding of meeting notes which may belong to other sections

JCTVC-G358 New modes for chroma intra prediction [X. Zhang, O. C. Au, J. Dai, F. Zou, C. Pang, X. Wen (HKUST)]

| Chroma mode coding (4 modes DM, LM but includes two new modes LML, LMA) [not entropy cod.] | -0.1 | -3.3 | -3.4 |
| --- | --- | --- | --- |

JCTVC-G119 Modifications to Intra-frame coding [Y. Lin, H. Yang, L. Liu, J. Zheng, H. Yu (Huawei)]

Part (1) and (3)

| (3) Merge two modes VER+8 & HOR +8 with bi-prediction [not entropy cod.] | -0.1 | 0.0 | 0.0 |
| --- | --- | --- | --- |

JCTVC-G245 Non-CE6a: Use of chroma phase in LM mode [E. François, C. Gisquet, S. Pautet (Canon)]

JCTVC-G419 Inconsistency of intra LM mode between HM and WD [J. Lee, S.-C. Lim, H. Y. Kim, J. S. Choi (ETRI)]

JCTVC-G754 Non-CE6: Line buffer reduction for CABAC context of SDIP syntax [L. Guo, X. Wang, M. Karczewicz (Qualcomm)]

# Appendix B (Coding gain results provided for G243 by proponents)

1) G243 with only 2 MPMs (4 MPMs in total) + chroma improvements

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.3% | 0.0% | 0.1% |
| Class B | -0.3% | -0.4% | -0.4% |
| Class C | -0.3% | -0.6% | -0.7% |
| Class D | -0.3% | -0.5% | -0.6% |
| Class E | -0.5% | -0.2% | -0.4% |
| Overall | -0.3% | -0.3% | -0.4% |
|  | -0.3% | -0.4% | -0.4% |

2) remainder coding+bypass with 2 MPMs

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.3% | -0.1% | 0.0% |
| Class B | -0.3% | 0.0% | -0.1% |
| Class C | -0.3% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | -0.1% |
| Class E | -0.1% | 0.0% | 0.0% |
| Overall | -0.2% | 0.0% | -0.1% |
|  | -0.2% | 0.0% | -0.1% |

Note that G869 (Sony proposal with 2 MPMs+mode ranking), using improved binarization, has slightly better results:

| Overall | -0.3% | -0.1% | -0.1% |
| --- | --- | --- | --- |
|  | -0.3% | -0.1% | -0.1% |

3) G243 without chroma improvements

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.5% | 0.1% | 0.0% |
| Class B | -0.5% | 0.0% | 0.0% |
| Class C | -0.5% | -0.1% | -0.2% |
| Class D | -0.4% | -0.1% | -0.1% |
| Class E | -0.6% | 0.0% | 0.1% |
| Overall | -0.5% | 0.0% | 0.0% |
|  | -0.5% | 0.0% | 0.0% |