INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2011/m22768

November 2011, Geneva, Switzerland

| Source | Samsung Electronics Co., Ltd. |
| --- | --- |
| Status | Proposal |
| Title | Samsung’s comments on HEVC Extension |
| Author | Ken McCann, Chanyul Kim, Byeongdoo Choi and JeongHoon Park |

# Introduction

This document provides Samsung’s comments on the requirements for the proposed HEVC extension.

# Comments on HEVC Extension

## Coding standard scalability

The AVC and MPEG-2 video coding standards have been widely implemented in terrestrial, cable and satellite broadcasting systems throughout the world. Samsung therefore requests that a coding standard scalability mode should be carefully considered as a desirable requirement of an HEVC scalability extension, where the base layer is compliant with one of the existing MPEG video codecs.

The main purpose of this requirement would be to provide a flexible extension to the next generation multimedia services such as UHDTV or high quality 3D broadcast to new TV receivers, while keeping base layer compatibility with existing TV receivers. The coding standard scalability mechanism should be able to support both an MPEG-2 compliant base layer  and an AVC compliant base layer. To meet immediate industry needs, coding standard scalability modes should be added at the same time as the initial scalability extension.

## View scalability

There are two categories of approach to view scalability:

- 2 or more views are captured and all views are compressed by exploiting redundancy across the views (e.g. MVC extension of AVC),

- 2 or more views are captured, depth maps and other related information is extracted and this supplementary data is sent in addition to the base view. The supplementary data is used by the decoder to generate intermediate views.

Although interest in multiview 3D video is increasing, so far only stereoscopic 3D systems have been deployed in the market. The existing TV systems typically display left and right views by decoding half resolution stereoscopic video streams transmitted in a frame compatible mode.

The next generation broadcasting services can be expected to provide higher resolution (and higher quality) stereoscopic video services using HEVC-based compression systems and the first category of view scalability. The latter category of view scalability would also enable support of auto-stereoscopic N-view displays, but may not be well-suited to the real-time encoding of live content. In order to address industry needs, MVC-like functionality should be added in the same timescale as that for other initial scalability modes, while further investigation of depth map and other supplementary data based view scalability should be performed over a longer timeframe.

## Complexity

If scalable functionality causes a significant increase in computational or algorithmic complexity relative to the base layer, it would act as an obstacle to its adoption by the industry. HEVC is expected to have a very broad range of application from mobile devices to UHDTV systems and packaged media players such as home cinema devices. The complexity increase required to support the addition of the HEVC extension should be evaluated to ensure that the extension is readily implementable by the next generation multimedia devices.

# Conclusion

In this contribution, a few comments on the proposed HEVC extension have been made, together with suggestions to clarify the requirements by consideration of market demands.

To summarize, Samsung suggests the following three points to address industry needs:

- MPEG-2 as well as AVC base layer compliance should be taken into account when considering coding standard scalability modes.

- View scalability that exploits redundancy over multiple views (such as MVC for AVC) should be standardized for HEVC with the same timeline as the initial scalability extension, while 3D video coding using supplementary data (e.g. depth maps) should be considered over a longer timeframe.

- The complexity increase required to add scalability should be seriously investigated in order to confirm the suitability of the HEVC extension for the next generation of multimedia devices.