| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G102 M21650 |
| --- | --- |

| Title: | Non-CE9: cross-verification of Panasonic’s proposal JCTVC-G541 simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for Panasonic’s proposal JCTVC-G541 simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge. In Panasonic’s proposal the MVP scaling is realized by using comparators and shifts. The test results of the following three tests are verified: 1) apply the simplified MVP scale in the TMVP derivation of AMVP only; 2) apply the simplified MVP scale in the spatial MVP derivation of AMVP only and 3) apply the simplified MVP scale in the TMVP derivation of merge/skip only. Small quality loss (ranging from 0.1% to 0.2%) is observed in all three settings. The results match the ones reported by proponents. The source code is also checked to verify its consistency with the proposal description.

.

# Introduction

This document reports cross-check results for Panasonic’s proposal JCTVC-G541 simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge. In the current HM4.0 design, MVP scaling is used for derivation of temporal MVP in both merge/skip and AMVP list derivation and for derivation of spatial MVPs in AMVP from difference reference frames. The MVP scale is expensive because multiply are table look up are involved. Panasonic’s proposal simplifies the MVP scaling operation by using comparator and bit-shift only. Based on the temporal distances the amount of shift is determined, the final scaled MVP is obtained by left or right-shifting the MVP accordingly.

The following three tests are cross-checked:

- test a: apply the simplified MVP scale in the TMVP derivation of AMVP only

- test b: apply the simplified MVP scale in the spatial MVP derivation of AMVP only

- test c: apply the simplified MVP scale in the TMVP derivation of merge/skip only

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The experimental results are summarized in Table 1, Table 2 and Test 3.  Small quality loss (ranging from 0.1% to 0.2%) is observed in all three settings. The results match the ones reported by proponents. Please be advised  runtime here may not be accurate.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% | 0.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  | 98% |  |  |
| Dec Time[%] | 92% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.2% | -0.1% | 0.2% | 0.2% | 0.3% |
| Class C | 0.1% | -0.1% | 0.1% | 0.1% | 0.2% | 0.0% |
| Class D | 0.2% | 0.3% | 0.0% | 0.2% | 0.4% | 0.1% |
| Class E | -0.1% | 0.5% | 0.4% | 0.0% | 0.3% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.1% | 0.1% | 0.3% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.3% | 0.2% |
| Enc Time[%] | 102% |  |  | 96% |  |  |
| Dec Time[%] | 98% |  |  | 95% |  |  |

Table 1. Experimental results for test a (simplified MVP scaling in AMVP TMVP derivation only)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.1% | -0.1% | 0.2% | 0.3% | 0.5% |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class C | 0.2% | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% |
| Class D | 0.2% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% |
|  | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | -0.2% | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | -0.1% | -0.1% | 0.1% | 0.3% | 0.0% |
| Class D | 0.2% | 0.3% | 0.3% | 0.2% | 0.4% | 0.3% |
| Class E | 0.0% | 0.2% | 0.3% | -0.1% | 0.1% | -0.4% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.3% | 0.0% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 95% |  |  |

Table 2. Experimental results for test b (simplified MVP scaling in AMVP Spatial MVP derivation only)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.3% | 0.2% | 0.1% | 0.2% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 105% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | -0.1% | 0.1% | 0.0% | -0.1% |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.3% | 0.1% |
| Class D | 0.2% | 0.2% | 0.6% | 0.3% | 0.1% | 0.1% |
| Class E | 0.1% | -0.4% | 0.6% | 0.0% | 0.1% | 0.1% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | -0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
|  | 0.1% | -0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 102% |  |  |

Table 3. Experimental results for test c (simplified MVP scaling in Merge TMVP derivation only)

# Comments

# The proposed scaling calculation will likely to reduce the cost in hardware implementation. Trade-off between coding efficiency and implementation cost reduction should be evaluated, its implication on the software implementations should be considered as well.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[4] B. Bross, W. Chien, I. Kim, M. Zhou, “Core Experiment 9: MV Coding and Skip/Merge Operations”, JCT-VC Document, JCTVC-F909, Torino, IT, July 2011.