| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1021 |
| --- | --- |

| Title: | Non-CE11: Cross check result of JCVC-G323 proposed by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jungsun Kim Chulkeun Kim #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Email: | jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports the verification results of JCTVC-G323 [1], which is on scanning order of residual coefficient block, combined with the unification of scanning passes for residual coding proposed by Qualcomm [2]. The proponent provides their source codes and the experimental results. The crosschecker checked source codes and compared it to the proposals. It succeeded and produced as same result as the proponent provided. The spreadsheet contains the detailed data is accompanied.

# Methods Proposed

In the contribution of the proponent, they propose to:

- Unify scanning orders for significant coefficient coding and other coefficient coding

- Scan coefficients by subblocks which themselves are scanned diagonally.

Each of proposals is briefly described below. Final scanning order proposed is as shown in Fig. 1.

Fig. 1 The scanning order for coefficient coding in 16x16 block

## Scanning passes of Residual Data

In current HM4.0, the scan order for coding significance coefficient map is different from that for other coefficient coding (e.g. coefficient sign coding, coefficient level coding). The former is done by each TU and the latter done by a set of consecutive 16 coefficients in the order of predefined scanning order as shown in Fig. 1. To keep consistency, the proponent proposes to use same scanning orders for every coefficient coding.

Fig. 2 Scanning orders of coefficient coding in HM4.0

## Diagonal sub-block scan for HE residual coding

Basically this method proposes to scan residual coefficients by subblocks. In the proposed scanning order, subblocks are scanned by diagonal scanning order and then coefficients in a subblock are scanned by diagonal scanning order when a block is larger than 8x8 as shown in Fig. 3. Therefore final scanning order can be depicted as Fig. 1, for example.

Fig. 3 blue lines : The scanning order for subblocks when a block is larger than 8x8, red lines : The scanning order for coefficients in a subblock

It also includes relevant changes on significance map coding. The context indices of top-left corner and above bottom-right corner of each subblock are modified to considering existence of non-zero coefficient below one-row. In other word, (0,0) and (3,2) are the positions where context indexes would be changed if a subblock is 4x4.

# Code Review

The source code provided includes 2 macros which are for scanning pass unification and for subblock scanning respectively. In the source code, the size of subblock is defined as 4 and it could be easily changed to 8 and 16 without problem. Each macro changes one or two parts of current source code less than 30 lines.

# WD Review

The WD text provided is clearly described what proposes in the contribution.

# Experimental Results

## Simulation environment

The encoding environment is as following:

OS: Linux version 2.6.18-274.el5 (gcc version 4.1.2 20080704 (Red Hat 4.1.2-50))

CPU: Intel Xeon Quad Core E31230 (SANDY BRIDGE3.2GHz)

Memory : 12GB DDR3 UDIMM Memory

And decoding environment is as following:

OS: Win XP 64 bit

CPU: i7 875K 2.93Hz

Memory: DDR3 16G

## Simulation conditions and results

The simulation follows the common test conditions [3] : AIHE, RAHE, LBHE and LPHE. It produces as same BD rate in Table 1 and Table 2. Encoding and decoding time shows differences but the proponent informs that their encoding/decoding time could be incorrect.

Table 1 The results cross checked

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.23% | 0.02% | 0.13% | 0.19% | 0.64% |  |  |  |  |  |  |
| Class B | 0.08% | 0.09% | 0.11% | 0.08% | 0.13% | 0.06% | 0.11% | 0.04% | -0.13% | 0.07% | 0.19% | -0.31% |
| Class C | 0.01% | 0.01% | 0.02% | 0.02% | -0.13% | -0.10% | 0.06% | -0.10% | -0.09% | 0.06% | 0.12% | 0.09% |
| Class D | 0.01% | 0.02% | -0.03% | 0.07% | 0.16% | -0.13% | -0.02% | 0.58% | 0.50% | 0.04% | 0.02% | -0.33% |
| Class E | 0.00% | -0.08% | -0.05% |  |  |  | 0.17% | -1.32% | 0.51% | 0.17% | -1.42% | -0.16% |
| Class F | -0.01% | -0.05% | -0.06% | 0.02% | -0.17% | -0.16% | -0.19% | -0.27% | 0.01% | -0.03% | -0.04% | -0.31% |
| Overall | 0.02% | -0.03% | 0.01% | 0.06% | 0.04% | 0.06% | 0.02% | -0.15% | 0.13% | 0.06% | -0.15% | -0.21% |
|  | 0.02% | -0.04% | -0.02% | 0.06% | 0.01% | 0.05% | 0.02% | -0.14% | 0.11% | 0.06% | -0.09% | -0.23% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |

Table 2 The results provided by the proponent

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.23% | 0.02% | 0.13% | 0.19% | 0.64% |  |  |  |  |  |  |
| Class B | 0.08% | 0.09% | 0.11% | 0.08% | 0.13% | 0.06% | 0.11% | 0.04% | -0.13% | 0.07% | 0.19% | -0.31% |
| Class C | 0.01% | 0.01% | 0.02% | 0.02% | -0.13% | -0.10% | 0.06% | -0.10% | -0.09% | 0.06% | 0.12% | 0.09% |
| Class D | 0.01% | 0.02% | -0.03% | 0.07% | 0.16% | -0.13% | -0.02% | 0.58% | 0.50% | 0.04% | 0.02% | -0.33% |
| Class E | 0.00% | -0.08% | -0.05% |  |  |  | 0.17% | -1.32% | 0.51% | 0.17% | -1.42% | -0.16% |
| Class F | -0.01% | -0.05% | -0.06% | 0.02% | -0.17% | -0.16% | -0.19% | -0.27% | 0.01% | -0.03% | -0.04% | -0.31% |
| Overall | 0.02% | -0.03% | 0.01% | 0.06% | 0.04% | 0.06% | 0.02% | -0.15% | 0.13% | 0.06% | -0.15% | -0.21% |
|  | 0.02% | -0.04% | -0.02% | 0.06% | 0.01% | 0.05% | 0.02% | -0.14% | 0.11% | 0.06% | -0.09% | -0.23% |
| Enc Time[%] | 103% |  |  | 100% |  |  | 103% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 100% |  |  | 97% |  |  |

# References

- J. Sole, R. Joshi, M. Karczewicz , “Non-CE11: Diagonal sub-block scan for HE residual coding”, JCTVC-G323, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Nov. 2011 - J. Sole, R. Joshi, M. Karczewicz , “CE11: Scanning Passes of Residual Data in HE”, JCTVC-G320, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Nov. 2011 - F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, Italy, Aug 2011