| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1022 |
| --- | --- |

| Title: | On line buffer removal for CU split flag and skip flag context model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park Tzu-Der Chuang Ching-Yeh Chen Yu-Wen Huang Shawmin Lei Wei-Jung Chien Marta Karczewicz | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com peter.chuang@mediatek.com chingyeh.chen@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com wchien@qualcomm.com martak@qualcomm.com |
| Source: | Samsung Electronics Co., Ltd., MediaTek Inc., Qualcomm |  |  |

_____________________________

# Abstract

This document reports the results of line buffer removal in context models. By disabling dependency on upper blocks on split flag and skip flag, the line buffer is totally removed in the current HM model. In this contribution which is based on G769, G200 and G829, it is proposed to use the depth information of the current block and the left block for the context formation of split_coding_unit_flag and to use the data of the left block for the context formation of skip_flag when the upper block belongs to the upper LCU. It is reported that the proposed context modeling for split flag and split flag causes 0.01%, 0.05, 0.12% BD rate loss for, respectively, all intra, random access and low delay configuration.

# Introduction

In the 6th JCT-VC meeting in Torino, several proposals related to context reduction and context modeling simplification were adopted. Most line buffers for CABAC are removed. In HM-4.0 CABAC, only two syntax elements, split_coding_unit_flag and skip_flag, need information from upper blocks, which has to be stored in line buffers. In this contribution, context modeling modifications for split_coding_unit_flag and skip_flag are proposed to remove line buffers in HM-4.0 CABAC.

# Split flag

In current HEVC test model, context model index for split_coding_unit_flag is derived as following,

ctxIdx = isSplit(A) + isSplit(B)                           (1)

Whether the neighbouring unit is split or not can be derived by comparing the CU depth or CU size of neighbouring CU to current CU depth or size, so CU size or CU depth line buffer is needed to decode split_coding_unit_flag syntax.

In this contribution, context model index for split flag is determined as following, jointly by split information of left CU and the current CU size.

ctxIdx = isSplit(A) +(CU_Size>16)                         (2)

With this modification, the context model number is not changed with line buffer removal. From G769, initialization values for B slice type are changed because G769 used the HM4.0 values even though the new context model is proposed.

# Skip flag

In HM-4.0, the context model index for skip_flag is derived as:

ctxIdx = isSkip(A) + isSkip (L),

where the isSkip (A) means whether the upper PU is coded in Skip mode, and isSkip (L) means whether the left PU is coded in Skip mode. A skip_flag line buffer is required.

In this contribution, context model index for split skip_flag is modified as:

ctxIdx = isSkip(L) + ((is the upper PU in the upper LCU) ? isSkip(L) : isSkip (A))

In the proposed skip_flag coding, when the upper block is not in the current LCU and belongs to the upper LCU, the information of the left block is used to replace that of the upper block for context formation, so the line buffer can be removed.

# Experimental results

Experiments are conducted with the common condition. Table 1 shows that 0.01%, 0.04, 0.09% BD rate loss is observed for, respectively, all intra, random access and low delay configuration by removing the line buffer for split_coding_unit_flag.

Table 1. Results of modified ctxIdx selection for Split flag

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.06% | -0.04% | -0.01% | -0.3% | 0.0% |  |  |  |
| Class B | 0.01% | -0.02% | -0.04% | 0.02% | 0.0% | -0.1% | 0.03% | 0.1% | -0.1% |
| Class C | 0.00% | -0.04% | -0.09% | 0.08% | -0.1% | 0.1% | 0.05% | -0.2% | 0.0% |
| Class D | 0.02% | -0.04% | -0.04% | 0.06% | 0.0% | -0.1% | 0.03% | 0.5% | 0.1% |
| Class E | 0.05% | -0.03% | -0.04% |  |  |  | 0.29% | -0.3% | 0.8% |
| Overall | 0.01% | -0.04% | -0.05% | 0.04% | -0.1% | 0.0% | 0.09% | 0.0% | 0.1% |
|  | 0.01% | -0.03% | -0.05% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |

Table 2 shows that 0.04%, 0.04% BD rate loss is observed for, respectively, random access and low delay configuration by removing the line buffer for skip_flag.

Table 2. Results of modified ctxIdx selection for Skip flag

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.07% | -0.06% |  |  |  |
| Class B | 0.06% | 0.08% | 0.07% | 0.08% | -0.06% | 0.04% |
| Class C | 0.03% | -0.06% | 0.07% | 0.01% | -0.15% | -0.12% |
| Class D | 0.03% | -0.08% | -0.15% | 0.03% | 0.42% | 0.19% |
| Class E |  |  |  | 0.04% | 0.04% | 0.38% |
| Overall | 0.04% | -0.03% | -0.01% | 0.04% | 0.06% | 0.10% |
|  | 0.04% | -0.03% | 0.00% | 0.05% | 0.02% | 0.04% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 3 shows the results of the proposed split_coding_unit_flag and skip_flag coding. No bit rate increase is observed in HE-AI, and the bit rate increases in HE-RA and HE-LDB are 0.05% and 0.12%, respectively. Encoding time and decoding time are unchanged. Line buffers for split_coding_unit_flag and skip_flag coding can be removed.

Table 3. Combined results of modified ctxIdx selection for Split flag and Skip_flag

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.06% | -0.04% | -0.03% | -0.31% | -0.03% |  |  |  |
| Class B | 0.01% | -0.02% | -0.04% | 0.06% | 0.06% | 0.00% | 0.11% | 0.16% | -0.17% |
| Class C | 0.00% | -0.04% | -0.09% | 0.11% | 0.04% | 0.01% | 0.09% | 0.02% | -0.04% |
| Class D | 0.02% | -0.04% | -0.04% | 0.07% | 0.20% | 0.08% | 0.02% | 0.23% | -0.12% |
| Class E | 0.05% | -0.03% | -0.04% |  |  |  | 0.31% | -0.35% | 0.97% |
| Overall | 0.01% | -0.04% | -0.05% | 0.05% | 0.00% | 0.01% | 0.12% | 0.05% | 0.09% |
|  | 0.01% | -0.03% | -0.05% | 0.05% | -0.02% | 0.02% | 0.12% | 0.07% | 0.11% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |

# References

- T. Lee, J. Chen, J. H. Park, “CE1 subtest A: Line buffer removal for CU split flag context model”, JCTVC-G769, Nov., 2011.

- T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei , “CE1.A.3: Reducing line buffers for CABAC”, JCTVC-G200, Nov., 2011.

- W.-J. Chien, M. Karczewicz, “Context modeling of split flag for CABAC” , JCTVC-G829, Nov., 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

Proposed working draft text is included below using the following text conventions:

- Removed text from JCTVC-F803_d5 is shown with strikethrough.

- Newly added text is shown in red.

| Table 9-51 - Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| alf_cu_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| split_coding_unit_flag | Error: Reference source not found | 0 | 0,1,2 (subclause 89.3.3.1.1.7) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 89.3.3.1.1.7) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause 89.3.3.1.1.7) | na | na | na | na |

Table 9-52 – Specifcation of ctxIdxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| split_coding_unit_flag | cuDepth[ xL ][ yL ] > cuDepth[ xP ][ yP ] | cuDepth[ xA ][ yA ] > cuDepth[ xP ][ yP ] | ( condL && availableL ) + ( condA && availableA ) |
| skip_flag | skip_flag[ xL ][ yL ] | skip_flag[ xA ][ yA ] | ( condL && availableL ) + ((availableA _LCU)? ( condA && availableA ) : ( condL && availableL )) |

###### Derivation process of ctxIdxInc using left and above syntax elements

Input to this process is are the luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture and the luma location ( xLC, yLC ) specifying the top-left luma sample of the current largest coding unit relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit with xL = xP − MinPuSize and yL = yP, the variable availableL is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If the prediction unit covering location ( xL, yL ) is available, availableL is set equal to 1.

- Otherwise (the prediction unit covering location ( xL, yL ) is not available), availableL is set equal to 0.

Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit with xA = xP and yA = yP − MinPuSize, the variable availableA is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If the prediction unit covering location ( xA, yA ) is available, availableA is set equal to 1.

- Otherwise (the prediction unit covering location ( xA, yA ) is not available), availableA is set equal to 0

Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit with xA = xP and yA = yP − MinPuSize, the variable availableA_LCU is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If yA is smaller than yLC, the variable availableA_LCU is set equal to 1.

- Otherwise, the variable availableA _LCU is set equal to 0.

- 9.3.3.1.1.7 Derivation process of ctxIdxInc using left and above syntax elements

Input to this process is the luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit with xL = xP − MinPuSize and yL = yP, the variable condL is derived as follows. [Ed.: (BB) MinPuSize should be defined somewhere]

- If the prediction unit covering location ( xL, yL ) is available and cuDepth[ xL ][ yL ] > cuDepth[ xP ][ yP ], condL is set to equal to 1.

- Otherwise (the prediction unit covering location ( xL, yL ) is not available or cuDepth[ xL ][ yL ] <= cuDepth[ xP ][ yP ]), condL is set equal to 0

The variable ctxIdxInc is derived as follows.

- If condL is equal to 1 and log2CUSize > 4, ctxIdxInc is set to equal to 2.

- Otherwise if condL is equal to 1 or log2CUSize > 4, ctxIdxInc is set to equal to 1.

- Otherwise (condL is equal to 0 and log2CUSize <= 4), ctxIdxInc is set to equal to 0.

# Alternative solution

In this proposal, alternative solution for split_coding_unit_flag which was proposed in G200 and G829 is also considered. The alternative context formation method for split_coding_unit_flag is specified as:

ctxIdx = isSplit(L) + (min(2,current_depth)<<1)

Since this also utilize only the depth of the left CU and the depth of current CU, the line buffer can be removed as well. Three additional context models are used in this method.

Table 4 shows the result of split_coding_flag as an alternative context modeling for split_coding_unit_flag.

Table 4. Results of alternative context modeling for split_coding_unit_flag

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.03% | -0.10% | -0.08% | -0.07% | -0.18% | 0.09% |  |  |  |
| Class B | 0.00% | -0.09% | -0.11% | -0.01% | 0.03% | -0.21% | -0.01% | -0.12% | -0.30% |
| Class C | 0.00% | -0.05% | -0.07% | 0.05% | -0.14% | -0.03% | 0.07% | -0.11% | 0.02% |
| Class D | 0.02% | -0.02% | -0.05% | 0.05% | 0.08% | -0.02% | 0.07% | 0.08% | -0.47% |
| Class E | 0.04% | -0.10% | -0.07% |  |  |  | 0.12% | 0.45% | -0.56% |
| Overall | 0.00% | -0.07% | -0.08% | 0.00% | -0.05% | -0.05% | 0.06% | 0.04% | -0.31% |
|  | 0.00% | -0.07% | -0.09% | 0.01% | -0.06% | -0.04% | 0.06% | 0.00% | -0.32% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |

Table 5 shows the combined results of split_coding_unit_flag and skip_flag coding with the alternative context modeling for split_coding_unit_flag. No bit rate increase is observed in HE-AI, and the bit rate increases in HE-RA and HE-LDB are less than 0.07%. Encoding time and decoding time are unchanged. Line buffers for split_coding_unit_flag and skip_flag coding can be removed as well.

Table 5. Combined results of modified ctxIdx selection for Split flag and Skip_flag 
with alternative context modeling for split_coding_unit_flag

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.03% | -0.10% | -0.08% | 0.01% | -0.15% | 0.16% |  |  |  |
| Class B | 0.00% | -0.09% | -0.11% | 0.04% | 0.06% | -0.19% | 0.05% | 0.02% | -0.27% |
| Class C | 0.00% | -0.05% | -0.07% | 0.06% | 0.01% | 0.10% | 0.07% | 0.12% | 0.02% |
| Class D | 0.02% | -0.02% | -0.05% | 0.08% | 0.17% | 0.11% | 0.08% | 0.01% | -0.33% |
| Class E | 0.04% | -0.10% | -0.07% |  |  |  | 0.07% | -0.33% | -0.14% |
| Overall | 0.00% | -0.07% | -0.08% | 0.05% | 0.02% | 0.03% | 0.07% | -0.02% | -0.19% |
|  | 0.00% | -0.07% | -0.09% | 0.05% | 0.01% | 0.04% | 0.07% | -0.05% | -0.21% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |