| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1023 |
| --- | --- |

| Title: | CE8 subset 0: Improved ALF N pass encoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | In Suk Chong1 Marta Karczewicz1 Tomoo Yamakage2 Takashi Watanabe2 Takeshi Chujoh2 Ching-Yeh Chen3 Chih-Ming Fu3 Chia-Yang Tsai3 Yu-Wen Huang3 Shawmin Lei3 | Email: | ichong@qualcomm.com martak@qualcomm.com tomoo.yamakage@toshiba.co.jp takashi39.watanabe@toshiba.co.jp takeshi.chujoh@toshiba.co.jp chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | Qualcomm1, TOSHIBA Corporation2, and MediaTek Inc.3 |  |  |

_____________________________

# Abstract

This evaluates a modified ALF N pass encoding algorithm. This includes improvement and bugfix of ALF N pass encoding. Coding efficiency gain for luma is 0.0 %, 0.0 %, 0.1% and 0.2 % in HE-AI, RA, LB, and LP without encoding/decoding time increase on average.

# Introduction

In the current HM4.0 ALF N pass encoder design, new design of N pass desgin was introduced due to ALF filter shape changes adopted during the last Torino meeting. We found out some bugs and inefficiencies within the algorithm, and fixes are introduced for those parts.

# Simulation results

Table 1  Results of HM4.0 14-pass ALF vs modified 1-pass/2-pass

|  |  | 14-pass vs 1-pass |  |  |  | 14-pass vs 2-pass |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB | LDP | AI | RA | LDB | LDP |
| BD-rate | Class A | 0.0% | 0.2% |  |  | 0.0% | 0.1% |  |  |
|  | Class B | 0.0% | 0.2% | 0.2% | 0.4% | 0.0% | 0.1% | 0.1% | 0.2% |
|  | Class C | 0.0% | 0.2% | 0.2% | 0.3% | 0.0% | 0.1% | 0.0% | 0.1% |
|  | Class D | 0.0% | 0.2% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% | 0.2% |
|  | Class E | 0.0% |  | 0.2% | 0.3% | 0.0% |  | 0.0% | -0.1% |
|  | Overall | 0.0% | 0.2% | 0.2% | 0.3% | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc. time |  | 95% | 99% | 99% | 99% | 95% | 99% | 99% | 99% |
| Dec. time |  | 101% | 99% | 99% | 99% | 101% | 99% | 99% | 99% |

# Conclusions

In this proposal, the coding efficiency is improved by fixing some bugs in ALF N Pass encoding algorithm in HM4.0. Coding efficiency gain for luma is 0.0 %, 0.0 %, 0.1% and 0.2 % in HE-AI, RA, LB, and LP. There was no encoding/decoding time increase on average.

# Patent rights declaration(s)

Qualcomm, TOSHIBA Corporation, and MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).