| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1024 WG11 Number: m22799 |
| --- | --- |

| Title: | Report of combining two coding tools for Chroma intra prediction (G173 and G358) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xingyu Zhang, Oscar C. Au, Ketan Tang , Xing Wen, The Hong Kong University of Science and Technology Yi-Jen Chiu, Yu Han, Lidong Xu, Wenhao Zhang, Hong Jiang Intel Corp. 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: Tel: Email: | +852-23588529 eexyzhang@ust.hk eeau@ust.hk wxxab@ust.hk +1-408-765-9494 yi-jen.chiu@intel.com |
| Source: | HKUST, Intel Corp. |  |  |

_____________________________

# Abstract

This joint report presents the results of combining two chroma intra prediction tools. Namely the tools are “Cross-channel Intra chroma residual prediction” (JCTVC-G173, Intel), and “New modes for Chroma intra prediction” (JCTVC-G358, HKUST). Compared to the individual tools, the gain of the combination is basically additive. The simulation is based on HM4.0 bug-fixed version. It is reported that, the combination yields an average of -0.4%, -3.9%, -5.7% BD-rate reduction for Y, U and V respectively under AIHE configuration, and -0.3%, -4.1%, -6.3% BD-rate reduction for Y, U and V respectively under AILC configuration without class F.

# Detailed results

## Individual results of G358 and G173

### New modes for Chroma intra prediction (JCTVC-G358)

The performance of scheme 3 in G358 is based on bugfixed HM4.0.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2% | -9.0% | -9.7% | -0.2% | -10.9% | -11.0% |
| Class B | -0.1% | -2.4% | -1.9% | -0.1% | -2.9% | -2.1% |
| Class C | 0.0% | -1.9% | -2.3% | -0.1% | -1.8% | -2.2% |
| Class D | 0.0% | -1.5% | -1.6% | 0.1% | -1.5% | -1.4% |
| Class E | -0.1% | -1.5% | -1.5% | 0.0% | -2.3% | -2.0% |
| Class F | -0.2% | -2.4% | -2.8% | -0.2% | -2.1% | -2.6% |
| Overall A-E | -0.1% | -3.3% | -3.4% | -0.1% | -3.9% | -3.7% |
| Overall A-F | -0.1% | -3.1% | -3.3% | -0.1% | -3.6% | -3.5% |
| Enc Time[%] | 94% |  |  | 92% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

### Cross-channel intra chroma residual prediction (JCTVC-G173)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3% | 0.1% | -2.4% | -0.2% | 0.3% | -2.9% |
| Class B | -0.1% | -0.3% | -3.2% | -0.1% | 0.5% | -3.4% |
| Class C | -0.5% | -1.8% | -2.9% | -0.4% | -1.5% | -3.9% |
| Class D | -0.5% | -1.9% | -3.0% | -0.4% | -1.6% | -4.0% |
| Class E | -0.3% | -1.0% | -2.8% | -0.2% | -1.0% | -3.3% |
| Class F | -0.5% | -1.6% | -1.8% | -0.4% | -1.5% | -2.5% |
| Overall A-E | -0.3% | -1.0% | -2.9% | -0.3% | -0.6% | -3.5% |
| Overall A-F | -0.4% | -1.1% | -2.7% | -0.3% | -0.7% | -3.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Note:

Both of these two modes do not need to add new syntax to current HM4.0.

G173 uses the residue of U to prediction the residue of V in all modes excluding LM mode.

G358 Scheme 3 uses 4 modes only (DM, LM, LML and LMA).

## Results of combination of G173 and G358

The combination applies G173 to the DM mode in G358 Scheme 3 only.

The below tables report the summary data of the combined G173 & G358 Scheme3, which demonstrates an average BD bit rate reductions of -0.4% on Y, -3.9% on U, -5.7% on V for AI_HE case and an average -0.3% on Y, -4.1% on U, -6.3% on V for AI_LC case without class F. With class F, the BD-rate reduction achieves -0.5% on Y, -3.7% on U, -5.5% on V for AI_HE case and an average -0.4% on Y, -3.9% on U, -6.0% on V for AI_LC case.

| Class A-E | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | Y | U | V |
| G173 | -0.3% | -1.0% | -2.9% | -0.3% | -0.6% | -3.5% |
| G358_4modes | -0.1% | -3.3% | -3.4% | -0.1% | -3.9% | -3.7% |
| G173+G358_4modes | -0.4% | -3.9% | -5.7% | -0.3% | -4.1% | -6.3% |

| Class A-F | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | Y | U | V |
| G173 | -0.4% | -1.1% | -2.7% | -0.3% | -0.7% | -3.3% |
| G358_4modes | -0.1% | -3.1% | -3.3% | -0.1% | -3.6% | -3.5% |
| G173+G358_4modes | -0.5% | -3.7% | -5.5% | -0.4% | -3.9% | -6.0% |

Test results of combination of JCTVC-G173 and G358

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -8.8% | -11.8% | -0.4% | -10.6% | -13.0% |
| Class B | -0.2% | -2.8% | -4.7% | -0.1% | -2.6% | -4.7% |
| Class C | -0.6% | -2.9% | -4.4% | -0.5% | -2.4% | -5.0% |
| Class D | -0.5% | -2.4% | -3.7% | -0.4% | -2.1% | -4.2% |
| Class E | -0.3% | -2.2% | -3.9% | -0.2% | -2.5% | -4.6% |
| Class F | -0.7% | -3.1% | -4.1% | -0.6% | -2.8% | -4.3% |
| Overall A-E | -0.4% | -3.9% | -5.7% | -0.3% | -4.1% | -6.3% |
| Overall A-F | -0.5% | -3.7% | -5.5% | -0.4% | -3.9% | -6.0% |
| Enc Time[%] | 92% |  |  | 91% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  |

# Conclusion

This contribution provides the information of combining two coding tools (G173 & G358) on the techniques of residual prediction for intra chroma prediction. The data shows the near-additive results, with BD rate improvements of 0.4%/3.9%/5.7% (Y/U/V) for AIHE and 0.3%/4.1%/6.3% (Y/U/V) for AILC.

Both of these two new modes do not need to add new syntax to current HM4.0.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).